Preface

Document Overview

The CM680 series inverter is a low-voltage high-performance engineering inverter that supports the control of three-phase AC permanent magnet synchronous motors, asynchronous motors, and synchronous reluctance motors. The inverter employs advanced vector control technology, providing high torque output at low speeds, excellent dynamic characteristics, and strong overload capability. It features user-programmable functions, background software monitoring, communication bus functionality, and support for various encoder types, offering rich and powerful combined functions with stable performance.

This manual introduces detailed content on product functionality, communication, fault codes, function codes, and more.

More Information

| Document Name | Content Summary |
| --- | --- |
| CM680 Series General Purpose Inverters Quick Installation and Commissioning Manual | Introduces detailed content on product installation, wiring, commissioning, troubleshooting, function codes, fault codes, etc. |
| CM680 Series General Purpose Inverters Hardware Manual | Introduces the system composition, technical specifications, components, dimensions, optional accessories (installation accessories, cables, peripheral electrical components), expansion cards, as well as daily maintenance and care guidelines, compliance certifications, and standards related to the product. |
| CM680 Series General Purpose Inverters Installation Manual | Introduces the installation dimensions, space design, detailed installation steps, wiring requirements, cabling requirements, installation requirements for optional accessories, and common EMC problem resolution suggestions, among other detailed content. |
| CM680 Series General Purpose Inverters Communication Manual | This section provides a detailed introduction to the communication expansion card of the product, including its overview, composition, dimensions, installation, electrical connections, communication parameter configuration, fault codes, etc. |

Version Change Record

| Revision Date | Release Version | Change Content |
| --- | --- | --- |
| 2023–12 | Ver 1.0 | First release of the manual. |
| 2024-05 | Ver 1.1 | Addition and modification of some software function codes. |
| 2024-09 | Ver 1.2 | Addition and modification of some software function codes. |

About Obtaining the Manual

This manual is not shipped with the product. If you need to obtain the electronic PDF version, you can do so through the following methods:

Log in to the official website of Changsha Riye Electric (www.cssunye.com), go to the “Download Center,” search for keywords, and download.

Table of Contents

Preface1

1 Parameter Overview	6

2 Parameter Group Description41

2.1 F0 Group Basic Function Codes	41

2.2 F1 Group Start/Stop Control Parameters	46

2.3 F2 Group VF Control Parameters	53

2.4 F3 Group Vector Control Parameters54

2.5 F4 Group First Motor Parameters	57

2.6 F5 Group Input Terminals	61

2.7 F6 Group Output Terminals	76

2.8 F7 Group Auxiliary Functions and Keyboard Display	82

2.9 F8 Group Communication Parameters	86

2.10 F9 Group Fault and Protection Parameters	89

2.11 FA Group PID Function	100

2.12 FB Group Tension Control Parameters108

2.13 FC Group Position Control Parameters121

2.14 FD Group Multi-Speed and Simple PLC Function126

2.15 FE Group Torque Control Parameters127

2.16 U0 Group Fault Record Parameters130

2.17 U1 Group Status Monitoring Parameters132

2.18 H0 Group Other Motor Parameters133

2.19 H1 Group Other Motor VF Control134

2.20 H3 Group Other Motor Fault Parameters136

2.21 L0 Group System Control Parameters145

2.22 L1 Group User Function Code Customization145

2.23 L2 Group Optimized Control Parameters146

2.24 L4 Group Brake Function Parameters146

2.25 L5 Group Sleep Wake Function Parameters147

2.26 L6 Group Counting149

2.27 L7 Group AI Multi-Point Curve Setting150

2.28 L8 Group Application Macro Parameter 151

3 Basic Configuration and Function Application	158

3.1 Basic VFD Application	158

3.1.1 Operation Command Setting 158

3.1.2 Frequency Command Setting	161

3.1.3 Start/Stop Methods	162

3.2 MotorCommissioning Methods	167

3.2.1 Asynchronous Motor VF and VVC Control Commissioning Methods	167

3.2.2 Permanent Magnet Motor VVC Commissioning Method 169

3.2.3 Asynchronous Motor and Permanent Magnet Motor SVC Commissioning Methods 175

3.3 Input and Output Terminal Description 181

3.3.1 Digital Input Terminal Functions (DI) 181

3.3.2 Digital Output Terminal Functions (DO) 185

3.3.3 Analog Input Terminal Function (AI) 188

3.3.4 Analog Output Terminal Function (AO) 190

3.4 Control Performance	190

3.4.1 VF Curve Setting	191

3.4.2 Torque Compensation	192

3.4.3 Slip Compensation	192

3.4.4 Current Limiting	193

3.4.5 Parameter Management Setting	193

3.4.6 Command Source Setting (AUTO)	194

3.4.7 Stopping Method	195

3.4.8 Forward/Reverse Prohibition Selection	195

3.4.9 Auxiliary Frequency Function (AUTO)	197

3.4.10 Braking Energy Suppression	198

3.4.11 Zero Speed Operation Selection	198

3.4.12 Frequency Setting Processing	198

3.4.13 External Command Frequency Limit	199

3.4.14 Speed Curve	199

3.4.15 Skip Frequency Function	201

3.4.16 Automatic Acceleration and Deceleration	201

3.4.17 Overcurrent Stall	202

3.4.18 Overvoltage Stall	204

3.4.19 Incremental PG Card	206

3.4.20 PID Function	207

3.5 Application Control	212

3.5.1 Jog Operation	212

3.5.2 Fan Control	213

3.5.3 Load Torque Compensation	213

3.5.4 Base Lockout	213

3.5.5 Bus Voltage Compensation	213

3.5.6 Frequency Hold	214

3.5.7 Start/Stop DC Braking	214

3.5.8 Instantaneous Power Failure Restart	215

3.5.9 No Stop on Instantaneous Power Failure	216

3.5.10 Speed Tracking	216

3.5.11 Abnormal Restart	217

3.5.12 Vibration Suppression	217

3.5.13 Braking Unit Control	218

3.5.14 Excessive Slip	218

3.5.15 Automatic Energy Saving	218

3.6 Tension Control	219

3.7 Faults and Protection	227

3.7.1 IGBT Overtemperature Protection	227

3.7.2 Low Current Protection	227

3.7.3 Motor Overheat Protection	227

3.7.4 Motor Overload Protection	228

3.7.5 Fault Recording Function	230

3.7.6 Over-torque Detection	231

3.7.8 Ground Protection	233

3.7.9 Undervoltage Protection	234

3.7.10 Output Phase Loss	234

3.7.11 Input Phase Loss	238

3.8 Monitoring	238

3.9 User Settings	240

3.9.1 Application Macros	240

3.9.2 Sleep and Wake-up	241

4 Fault Diagnosis and Countermeasures	244

4.1 Fault Handling 244

4.2 Warning Handling 254

Appendix English Abbreviation Meanings	260

1 Parameter Overview

Function Code Symbol Explanation as Follows:

“☆”: Indicates that the inverter parameter can be modified during both stop and operation

“★”: Indicates that the inverter parameter cannot be modified while in operation

“○”: Indicates that the parameter is a manufacturer's parameter and cannot be changed by the user

“●”: Indicates the actual detection value of the inverter or the manufacturer's fixed value, which cannot be changed

The communication addresses in the function parameter table are written in hexadecimal.

| Function Code | Name | Content | Default Value | Change | Communication Address |
| --- | --- | --- | --- | --- | --- |
| F0 Group - Basic Function Group |  |  |  |  |  |
| F0-00 | Model Number | Model Number: Input Voltage, Power, Input Voltage Phases | Model Confirmation | ● | 0000 |
| F0-01 | Light/Heavy Load Selection | 1: Heavy Load | 1 | ★ | 0001 |
| F0-02 | Rated Current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | Model Confirmation | ● | 0002 |
| F0-03 | Control Mode | 0: Speed Mode 1: Point-to-Point Mode 2: Torque Mode 3: Homing Mode | 0 | ★ | 0003 |
| F0-04 | Speed Mode Selection | 0: V/F Control 2: VVC Voltage Vector Control 3: Induction Motor Closed-loop Control 4: Permanent Magnet Motor Closed-loop Control 6: SVC Open-loop Vector Control | 0 | ★ | 0004 |
| F0-05 | Run Command Source Selection | 0: Digital Operator 1: External Terminal Input 2: RS485 Communication Input 3: CANopen Input 4: Reserved 5: Communication Card Input | 0 | ★ | 0005 |
| F0-06 | Frequency Source Selection | 0: Digital Operator 1: RS485 Communication 2: Analog Input 3: External Up/Down Input 4: Pulse Input Without Direction 5: Pulse Input With Direction 6: CANopen Input 7: Reserved 8: Communication Card Input 9: PID 10: Digital Terminal Multi-Speed 11: Simple PLC | 0 | ★ | 0006 |
| F0-07 | Auxiliary Frequency Source | 0: Disabled 1: Digital Operator 2: RS485 Communication 3: Analog Input 4: External Up/Down Input 5: Pulse Input 6: CANopen Input 7: Reserved 8: Communication Card Input | 0 | ★ | 0007 |
| F0-08 | Frequency Source Superposition Selection | 0: Main Freq + Aux Freq 1: Main Freq - Aux Freq 2: Aux Freq - Main Freq 3: Auxiliary Frequency | 0 | ★ | 0008 |
| F0-09 | Forward/Reverse Prohibited Selection | 0: Forward/Reverse Enabled 1: Reverse Prohibited 2: Forward Prohibited | 0 | ★ | 0009 |
| F0-10 | Upper Limit Frequency | 0.00Hz～599.00Hz | 599.00Hz | ☆ | 000A |
| F0-11 | Lower Limit Frequency | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 000B |
| F0-12 | Speed curve time unit | 0: Acceleration/deceleration unit is 0.01s 1: Acceleration/deceleration unit is 0.1s | 0 | ★ | 000C |
| F0-13 | Acceleration time 1 | 0.00s～600.00s | Model Confirmation | ☆ | 000D |
| F0-14 | Deceleration time 1 | 0.00s～600.00s | Model Confirmation | ☆ | 000E |
| F0-15 | Carrier frequency | 2kHz～15kHz | 6kHz | ★ | 000F |
| F0-16 | Reduced Carrier Method | 0: Reduce carrier based on current temperature 1: Fixed carrier and limit current 2: Same as setting 0 | 0 | ☆ | 0010 |
| F0-17 | High-speed Machine Mode | 0～65535 | 0 | ★ | 0011 |
| F0-18 | Parameter Management Settings | 0: No Function 1: Parameter Write Protection 5: Reset kWH Display to Zero 7: Reset CAN Slave 9: Reset to 50Hz Factory Values | 0 | ★ | 0012 |
| F0-19 | Keyboard Set Frequency | 0.00～599.00 | 50.00Hz | ★ | 0013 |
| F1 Group - Start/Stop Control Parameters |  |  |  |  |  |
| F1-00 | Startup Speed Tracking | 0: No Action 1: Track from Maximum Frequency 2: Track Frequency at Startup 3: Track from Minimum Frequency | 0 | ☆ | 0100 |
| F1-01 | Initial Position Identification Method | 0: Do Not Perform Initial Angle Identification 1: Attraction Method 3: Pulse Injection Method 1 4: Pulse Injection Method 2 | 0 | ☆ | 0101 |
| F1-02 | Abnormal Restart Method | 0: Stop Operation 1: Current Speed Tracking 2: Minimum Frequency Tracking | 0 | ☆ | 0102 |
| F1-03 | Speed Tracking Maximum Current | 20%～200% | 100% | ☆ | 0103 |
| F1-04 | Voltage Increase Rate | 1%～200% | 100% | ☆ | 0104 |
| F1-05 | Start Frequency | 0.00Hz～599.00Hz | 0.50Hz | ★ | 0105 |
| F1-06 | Startup Hold Time | 0.00s～600.00s | 0.00s | ☆ | 0106 |
| F1-07 | Startup Hold Frequency | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 0107 |
| F1-08 | Braking Current Level | 0%～100% | 0% | ☆ | 0108 |
| F1-09 | Startup Braking Time | 0.0s～60.0s | 0.0s | ☆ | 0109 |
| F1-10 | DC Braking Ratio Coefficient | 0～65535 | 2000 | ☆ | 010A |
| F1-11 | DC Braking Integral Coefficient | 0～65535 | 100 | ☆ | 010B |
| F1-12 | Stopping Method | 0: Deceleration Stop 1: Free Stop | 0 | ☆ | 010C |
| F1-13 | Deceleration Method | 0: Deceleration Method 0 1: Deceleration Method 1 2: Deceleration Method 2 | 0 | ★ | 010D |
| F1-15 | 1-4 Segment Acceleration/Deceleration Switching Points | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 010F |
| F1-16 | S Acceleration Time 1 | 0.00s～25.00s | 0.20s | ☆ | 0110 |
| F1-17 | S Acceleration Time 2 | 0.00s～25.00s | 0.20s | ☆ | 0111 |
| F1-18 | S Deceleration Time 1 | 0.00s～25.00s | 0.20s | ☆ | 0112 |
| F1-19 | S Deceleration Time 2 | 0.00s～25.00s | 0.20s | ☆ | 0113 |
| F1-20 | Automatic Acceleration and Deceleration Selection | 0: Linear Acceleration and Deceleration 1: Automatic Acceleration and Linear Deceleration 2: Linear Acceleration and Automatic Deceleration 3: Automatic Acceleration and Deceleration 4: Automatic Acceleration and Deceleration Suppression | 0 | ☆ | 0114 |
| F1-21 | Automatic Acceleration and Deceleration Kp | 0～65535 | 20 | ☆ | 0115 |
| F1-22 | Automatic Acceleration and Deceleration Ki | 0.000～65.535 | 0.400 | ☆ | 0116 |
| F1-23 | Abnormal Shutdown Method | 0: Free Stop 1: According to First Deceleration Time 2: According to Second Deceleration Time 3: According to Third Deceleration Time 4: According to Fourth Deceleration Time 5: According to System Deceleration Time 6: Automatic Deceleration | 0 | ☆ | 0117 |
| F1-24 | Braking Time | 0.0s～60.0s | 0.0s | ☆ | 0118 |
| F1-25 | Braking Start Frequency | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 0119 |
| F1-26 | Stop Holding Time | 0.00s～600.00s | 0.00s | ☆ | 011A |
| F1-27 | Parking Hold Frequency | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 011B |
| F1-28 | Speed Tracking Demagnetization Time | 0～65535 | 50 | ☆ | 011C |
| F1-29 | Instant Power Failure Restart Method | 0: Stop Operation 1: Current Speed Tracking 2: Minimum Frequency Tracking | 0 | ☆ | 011D |
| F1-30 | Allowed Power Failure Time | 0.0s～20.0s | 2.0s | ☆ | 011E |
| F1-31 | Base Blockade Interruption Time | 0.0s～5.0s | 0.5s | ☆ | 011F |
| F1-32 | dEb recovery threshold | 0.0～200.0 | 40.0 | ☆ | 0120 |
| F1-33 | dEb action bias threshold | 0.0～200.0 | 40.0 | ☆ | 0121 |
| F1-34 | dEb deceleration selection | 0: No Action 1: Enable, no recovery 2: Enable, recovery | 0 | ☆ | 0122 |
| F1-35 | dEb recovery time | 0.0s～25.0s | 3.0s | ☆ | 0123 |
| F1-36 | PM Voltage Pulse Width | 0.0ms～3.0ms | 1.0ms | ☆ | 0124 |
| F1-37 | Reserved | 0～65535 | 0 | ☆ | 0125 |
| F1-38 | Reserved | 0～65535 | 0 | ☆ | 0126 |
| F1-39 | Reserved | 0～65535 | 0 | ☆ | 0127 |
| F1-40 | Reserved | 0～65535 | 0 | ☆ | 0128 |
| F1-41 | Flux Zero Position Alignment Time | 0.0～60.0s | 2.0 | ☆ | 0129 |
| F1-42 | Stop Frequency Mode | 0:Follow Current Frequency Command 1:Reset Frequency Command 2:Parameter F2-16 Setting Value | 0 | ☆ | 012A |
| F1-43 | Initial Stop Frequency | 0.00Hz～599.00Hz | 50.00Hz | ★ | 012B |
| F2 Group - VF Control Parameters |  |  |  |  |  |
| F2-00 | V/F Voltage Selection | 0: General V/F Curve 1: 1.5 Power V/F Curve 2: 2 Power V/F Curve | 0 | ★ | 0200 |
| F2-01 | Torque Compensation Gain | 0～10 | 1 | ☆ | 0201 |
| F2-02 | Torque Filter Time | 0.001s～10.000s | 0.500s | ☆ | 0202 |
| F2-03 | Reserved | 0～65535 | 0 | ● | 0203 |
| F2-04 | M1 Multi-point VF Frequency Point 1 | 0.00Hz～599.00Hz | 0.50Hz | ★ | 0204 |
| F2-05 | Multi-point VF Voltage Point 1 | 0.0V～480.0V | 2.0V | ★ | 0205 |
| F2-06 | Multi-point VF Frequency Point 2 | 0.00Hz～599.00Hz | 1.50Hz | ★ | 0206 |
| F2-07 | Multi-point VF Voltage Point 2 | 0.0V～480.0V | 10.0V | ★ | 0207 |
| F2-08 | Multi-point VF Frequency Point 3 | 0.00Hz～599.00Hz | 3.00Hz | ★ | 0208 |
| F2-09 | Multi-point VF Voltage Point 3 | 0.0V～480.0V | 22.0V | ★ | 0209 |
| F2-10 | Slip Filter Time | 0.001s～10.000s | 0.100s | ☆ | 020A |
| F2-11 | Slip Compensation Gain | 0.00～10.00 | 0.00 | ☆ | 020B |
| F2-12 | Power Generation Slip Compensation Gain | 0.00～1.00 | 1.00 | ☆ | 020C |
| F2-13 | Maximum Slip Limit Value | 0.00Hz～200.00Hz | 20.00Hz | ☆ | 020D |
| F2-14 | Oscillation Suppression Gain | 0～10000 | 1000 | ☆ | 020E |
| F2-15 | Overexcitation Braking Gain | 0～65535 | 30 | ☆ | 020F |
| F2-16 | Voltage Stability Gain | 0～200Hz | 10 | ☆ | 0210 |
| F2-17 | Reserved | 0～65535 | 0 | ☆ | 0211 |
| F2-18 | Reserved | 0～65535 | 0 | ☆ | 0212 |
| F2-19 | Reserved | 0～65535 | 0 | ☆ | 0213 |
| F2-20 | Reserved | 0～65535 | 0 | ☆ | 0214 |
| F3 Group - Vector Control Parameters |  |  |  |  |  |
| F3-00 | System Control | 0000H~FFFFH | 0 | ★ | 0300 |
| F3-01 | ASR1/2 Switching Frequency | 5.00Hz～599.00Hz | 7.00Hz | ☆ | 0301 |
| F3-02 | Zero Speed Bandwidth | 1Hz～40Hz | 10Hz | ☆ | 0302 |
| F3-03 | ASR1 Low-Speed Bandwidth | 1Hz～40Hz | 10Hz | ☆ | 0303 |
| F3-04 | ASR2 High-Speed Bandwidth | 1Hz～40Hz | 10Hz | ☆ | 0304 |
| F3-05 | ASR Zero Speed Gain | 0Hz～40Hz | 20Hz | ☆ | 0305 |
| F3-06 | ASR Zero Speed Integral | 0.000s～10.000s | 0.100s | ☆ | 0306 |
| F3-07 | ASR1 Low-Speed Gain | 0Hz～40Hz | 10Hz | ☆ | 0307 |
| F3-08 | ASR1 Low-Speed Integral Time | 0.000s～10.000s | 0.100s | ☆ | 0308 |
| F3-09 | ASR2 High-Speed Gain | 0Hz～40Hz | 10Hz | ☆ | 0309 |
| F3-10 | ASR2 High-Speed Integral Time | 0.000s～10.000s | 0.100s | ☆ | 030A |
| F3-11 | ASR Speed Feedforward | 0%～200% | 0% | ☆ | 030B |
| F3-12 | PDF Coefficient | 0～200 | 30 | ☆ | 030C |
| F3-13 | DOB Compensation Gain | 0Hz～5000Hz | 0Hz | ☆ | 030D |
| F3-14 | ASR Output Filter Time | 0.000s～0.350s | 0.008s | ☆ | 030E |
| F3-15 | Notch Depth | 0dB～20dB | 0dB | ☆ | 030F |
| F3-16 | Notch Filter Frequency | 0.00Hz～200.00Hz | 0.00Hz | ☆ | 0310 |
| F3-17 | Speed Limit Selection | 0: Parameters FE-03&FE-04 1: Frequency Source 2: Absolute Value of Frequency Source 3: Tension Control Line Speed | 0 | ★ | 0311 |
| F3-18 | Current Loop Bandwidth | 0Hz～65535Hz | 290Hz | ☆ | 0312 |
| F3-19 | Iq Current Loop Proportional Gain | 0%～65535% | 100% | ☆ | 0313 |
| F3-20 | Iq Current Loop Integral Time | 0s～65535s | 100s | ☆ | 0314 |
| F3-21 | Id Current Loop Proportional Gain | 0%～65535% | 100s | ☆ | 0315 |
| F3-22 | Id Current Loop Integral Time | 0s～65535s | 100s | ☆ | 0316 |
| F3-23 | I/F Mode Current Command | 0%～150% | 40% | ☆ | 0317 |
| F3-24 | IF Switching Frequency 1 | 0.00Hz～599.00Hz | 20.00Hz | ☆ | 0318 |
| F3-25 | IF Switching Frequency 2 | 0.00Hz～599.00Hz | 20.00Hz | ☆ | 0319 |
| F3-26 | Weak Magnetism Proportion Coefficient | 0.00～655.35 | 1.00 | ☆ | 031A |
| F3-27 | Weak Magnetism Integral Coefficient | 0.00～655.35 | 0.20 | ☆ | 031B |
| F3-28 | Output Voltage Limit Value | 0～115 | 100 | ☆ | 031C |
| F3-29 | Torque Filter Time | 0.001～10.000 | 0.500 | ☆ | 031D |
| F3-30 | VVC Voltage Feedforward Gain | 0.00～2.00 | 1.00 | ☆ | 031E |
| F3-31 | VVC Frequency Compensation Low-pass | 0.00s～600.00s | 0.05s | ☆ | 031F |
| F3-32 | Forward Motor Torque Upper Limit | 0%～500% | 500% | ☆ | 0320 |
| F3-33 | Forward Braking Torque Upper Limit | 0%～500% | 500% | ☆ | 0321 |
| F3-34 | Reverse Electric Torque Upper Limit | 0%～500% | 500% | ☆ | 0322 |
| F3-35 | Reverse Braking Torque Upper Limit | 0%～500% | 500% | ☆ | 0323 |
| F3-36 | FLUXBEMF Bandwidth | 20.0Hz～100.0Hz | 40.0Hz | ☆ | 0324 |
| F3-37 | PM Flux Weakening Ki | 0～65535 | 32 | ☆ | 0325 |
| F3-39 | Flux Observer Filter | 1ms～1000ms | 50ms | ☆ | 0327 |
| F3-40 | Flux Rise Time | 33%～100% | 100% | ☆ | 0328 |
| F3-41 | VVC CompensationHigh Pass | 0.00s～600.00s | 5.00s | ☆ | 0329 |
| F3-42 | VVC Compensation Gain | 0.00～655.35 | 1.00 | ☆ | 032A |
| F3-43 | PM Sensorless Control Bit | 0000H~FFFFH | 0 | ☆ | 032B |
| F3-44 | Id filtering time | 0.0s～6.0s | 0.2s | ☆ | 032C |
| F3-45 | MTPA low-passfilter coefficient | 0～1000 | 10 | ☆ | 032D |
| F3-46 | M1 flux weakening curve | 0～200 | 90 | ☆ | 032E |
| F3-47 | M2 flux weakening curve | 0～200 | 90 | ☆ | 032F |
| F3-48 | Weak Magnetization Speed Response | 0～150 | 65 | ☆ | 0330 |
| F3-49 | Weak Magnetization ASR Proportional Coefficient | 0%～65535% | 7618% | ☆ | 0331 |
| F3-50 | Weak Magnetization ASR Integral Coefficient | 0s～65535s | 30s | ☆ | 0332 |
| F3-51 | I Maximum Value | 0～250 | 150 | ★ | 0333 |
| F3-52 | Torque Limit Value | 0～500 | 200 | ☆ | 0334 |
| F3-53 | Demagnetizing Current | 0～65535 | 100 | ☆ | 0335 |
| F3-54 | Frequency Filter Coefficient | 0～1000 | 10 | ★ | 0336 |
| F3-55 | Us Maximum Value | 0～100 | 100 | ★ | 0337 |
| F3-56 | Id Demagnetizing Bandwidth | 0～546 | 273 | ☆ | 0338 |
| F3-57 | MTPA Flux Weakening Voltage Limit | 0.0%～12.0% | 10.5% | ☆ | 0339 |
| F4 Group - First Motor Parameters |  |  |  |  |  |
| F4-00 | Motor Type Selection | 0: Induction Motor 1: Surface-Mounted Permanent Magnet Synchronous Motor 2: Interior Permanent Magnet Synchronous Motor 3: Synchronous Reluctance Motor | 0 | ★ | 0400 |
| F4-01 | Motor Parameter Identification | 0: No Function 1: IM Advanced Rotational Identification 2: IM Static Identification 4: PM Motor Magnetic Pole Recognition 5: PM Rotation Recognition 6: IM Rotation Recognition 12: Inertia Recognition 13: PM Static Recognition | 0 | ★ | 0401 |
| F4-02 | M1 Maximum Frequency | 0.00Hz～599.00Hz | 50.00Hz | ★ | 0402 |
| F4-03 | M1 Rated Frequency | 0.00Hz～599.00Hz | 50.00Hz | ★ | 0403 |
| F4-04 | M1 Rated Voltage | 0.0V～510.0V | 380.0V | ★ | 0404 |
| F4-05 | IM1 Rated Power | 0.00kW～655.35kW | Model Confirmation | ★ | 0405 |
| F4-06 | IM1 Number of Poles | 2～20 | Model Confirmation | ★ | 0406 |
| F4-07 | IM1 Rated Current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | F4-05 Confirm | ★ | 0407 |
| F4-08 | IM1 Rated Speed | 0rpm～65535rpm | F4-05 Confirm | ★ | 0408 |
| F4-09 | Inertia Per Unit Value | 1pu～65535pu | Model Confirmation | ★ | 0409 |
| F4-10 | IM1 No-load Current | 0.00A～F4-07 | Model Confirmation | ★ | 040A |
| F4-11 | IM1 Stator Resistance | 0.000Ω～65.535Ω | Model Confirmation | ★ | 040B |
| F4-12 | IM1 Rotor Resistance | 0.000Ω～65.535Ω | Model Confirmation | ★ | 040C |
| F4-13 | IM1 Mutual Inductance | 0.0mH～6553.5mH | Model Confirmation | ★ | 040D |
| F4-14 | IM1 Leakage Inductance | 0.00mH～655.35mH | Model Confirmation | ★ | 040E |
| F4-15 | PM rated power | 0.00kW～655.35kW | Model Confirmation | ★ | 040F |
| F4-16 | PM pole number | 0～65535 | Model Confirmation | ★ | 0410 |
| F4-17 | PM rated current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | F4-15 confirmation | ★ | 0411 |
| F4-18 | PM rated speed | 0rpm～65535rpm | F4-15 confirmation | ★ | 0412 |
| F4-19 | PM motor inertia | 0.0kgm2～6553.5kgm2 | Model Confirmation | ★ | 0413 |
| F4-20 | PM Stator Resistance | 0.000Ω～65.535Ω | Model Confirmation | ★ | 0414 |
| F4-21 | PM D-axis Inductance | 0.00mH～655.35mH | Model Confirmation | ★ | 0415 |
| F4-22 | PM Q-axis Inductance | 0.00mH～655.35mH | Model Confirmation | ★ | 0416 |
| F4-23 | PM Ke Parameter | 0V～65535V | Model Confirmation | ★ | 0417 |
| F4-24 | PM Back EMF Coefficient | 0.0～6553.5 | Model Confirmation | ★ | 0418 |
| F4-25 | PM Current Recognition Threshold | 0.00%～100.00% | Model Confirmation | ☆ | 0419 |
| F4-26 | PM Magnetic Pole Angle | 0.0DEG～360.0DEG | Model Confirmation | ★ | 041A |
| F4-27 | Encoder Type Selection | 0: No PG Card 1: ABZ Encoder 2: Reserved 3: Resolver | 0 | ★ | 041B |
| F4-28 | Encoder Pulse Count | 6ppr～20000ppr | 1024ppr | ★ | 041C |
| F4-29 | Encoder Pulse Type | 0: No Function 1: A phase leads B phase for forward rotation 2: B phase leads A phase for forward rotation 3: A pulse B direction HForwardLReverse 4: A pulse B direction LForwardHReverse 5: Single-phase Input | 1 | ★ | 041D |
| F4-30 | Reference Pulse Input Type | 0: No Function 1: A phase leads B phase for forward rotation 2: B phase leads A phase for forward rotation 3: A pulse B direction HForwardLReverse 4: A pulse B direction LForwardHReverse 5: Single-phase Input | 0 | ★ | 041E |
| F4-31 | Frequency Division Output Setting | 1～255 | 1 | ☆ | 041F |
| F4-32 | Rotary Encoder Pole Pairs | 1～50 | 1 | ★ | 0420 |
| F4-33 | PG Digital Filter Value | 0CNT～65535CNT | 100CNT | ★ | 0421 |
| F4-34 | PG Feedback High-Speed Filter | 0Hz～2000Hz | 100Hz | ☆ | 0422 |
| F4-35 | PG Feedback Low-Speed Filter | 0Hz～2000Hz | 50Hz | ☆ | 0423 |
| F4-36 | Load-side Gear A1 | 1～65535 | 100 | ☆ | 0424 |
| F4-37 | Load-side Gear B1 | 1～65535 | 100 | ☆ | 0425 |
| F4-38 | Load-side Gear A2 | 1～65535 | 100 | ☆ | 0426 |
| F4-39 | Load-side Gear B2 | 1～65535 | 100 | ☆ | 0427 |
| F4-40 | Electronic Gear A | 1～65535 | 100 | ☆ | 0428 |
| F4-41 | Electronic Gear B | 1～65535 | 100 | ☆ | 0429 |
| F5 Group - Input Terminals |  |  |  |  |  |
| F5-00 | DI1 Terminal Function Selection | 0: No Function 1: Multi-speed/Multi-point Position 1 2: Multi-speed/Multi-point Position 2 3: Multi-speed/Multi-point Position 3 4: Multi-speed/Multi-point Position 4 5: Fault Reset 6: Jog 7: Speed Pause 8: 1-2 Segment Acceleration/Deceleration Switching 9: 3-4 Segment Acceleration/Deceleration Switching 10: External Fault (F1-20) 11: Base Block 12: Stop Output 13: Automatic Acceleration/Deceleration Prohibited 15: AI1 Input Frequency Command 16: AI2 Input Frequency Command 17: AI3 Input Frequency Command 18: Deceleration Stop (F1-20) 19: External Command for Frequency Increase 20: External Command for Frequency Decrease 21: PID Function Prohibited 22: Clear Counter 23: Input Count 24: External Forward Jog 25: External Reverse Jog 26: Torque/Speed Mode Switch 27: Speed Loop 1/2 Switch 28: External Fault Free Stop 31: High Torque Compensation 32: Intermediate Torque Compensation 33: Low Torque Compensation 34: Multi-Speed/Position Switching 35: Internal Positioning Enable 36: Multi-Point Position Input 37: Pulse Input Position Control 39: Torque Command Direction Switching 40: Motor Free Stop 41: Manual Mode Enable 42: Automatic Mode Enable 44: NL Reverse Limit 45: PL Forward Limit 46: ORG Return to Origin 47: Return to Origin Action Enable 48: Mechanical Gear Ratio Switching 49: Inverter Enable 50: Slave dEb Execution 53: CANopen Quick Stop 56: LOC/REM Switching 70: Auxiliary Frequency Prohibition 71: PID Prohibition, Zero Output 72: PID Prohibition, Maintain Output 73: PID I Gain is 0 74: PID Feedback Reversal 83: Motor Switching Bit0 84: Motor Switching Bit1 | 0 | ★ | 0500 |
| F5-01 | DI2 Terminal Function Selection |  | 0 | ★ | 0501 |
| F5-02 | DI3 Terminal Function Selection |  | 1 | ★ | 0502 |
| F5-03 | DI4 Terminal Function Selection |  | 2 | ★ | 0503 |
| F5-04 | DI5 Terminal Function Selection |  | 3 | ★ | 0504 |
| F5-05 | DI6 Terminal Function Selection |  | 4 | ★ | 0505 |
| F5-06 | DI7 Terminal Function Selection |  | 0 | ★ | 0506 |
| F5-07 | HDI8 Terminal Function Selection |  | 0 | ★ | 0507 |
| F5-08 | Terminal Command Mode | 0: No Function 1: 2-Wire Mode 1 2: 2-wire Mode 2 3: 3-wire 4: 2-wire Mode 1/Fast Start 5: 2-wire Mode 2/Fast Start 6: 3-wire Fast Start | 1 | ★ | 0508 |
| F5-09 | UP/DOWN Mode Selection | 0: System Acceleration/Deceleration Time 1: F5-10 Fixed Acceleration/Deceleration 2: F5-10 Pulse Signal 3: External Up/Down Input 4: External Up/Down Input 1 | 0 | ☆ | 0509 |
| F5-10 | UP/DOWN Rate of Change | 0.001Hzms～1.000Hzms | 0.001Hzms | ☆ | 050A |
| F5-15 | DI Terminal Valid Logic | 0000H~FFFFH | 0 | ☆ | 050F |
| F5-16 | DI Terminal Response Time | 0.000s～30.000s | 0.005s | ☆ | 0510 |
| F5-17 | Virtual/Real DI Terminal Selection | 0000H~FFFFH | 0 | ☆ | 0511 |
| F5-18 | Virtual Terminal Status Setting | 0～65535 | 0 | ☆ | 0512 |
| F5-19 | External Operation Selection | 0: Invalid 1: Run if there is an operation command | 0 | ☆ | 0513 |
| F5-20 | AI1 signal type selection | 0: 0-10V input selection 1: 0-20mA input selection 2: 4-20mA input selection | 0 | ☆ | 0514 |
| F5-21 | AI1 function selection | 0: No Function 1: Frequency setting 2: Torque setting 3: Torque compensation setting 4: PID target value 5: PID feedback value 6: Thermistor PTC Input 7: Forward Torque Limit 8: Reverse Torque Limit 9: Regenerative Torque Limit 10: Forward/Reverse Torque Limit 11: Thermistor PT100 Value 12: Auxiliary Frequency Setting 13: PID Offset 14: Tension PID Feedback Value 15: Line Speed Feedback 16: Roll Diameter Feedback 17: Tension PID Setpoint 18: Tension Setpoint 19: Zero-speed Tension Setting 20: Tension Taper Setting | 1 | ☆ | 0515 |
| F5-22 | AI1 Input Bias | -100.0%～100.0% | 0.0% | ☆ | 0516 |
| F5-23 | AI1 Bias Mode Selection | 0: No Bias 1: Below Bias = Bias 2: Above Bias = Bias 3: Absolute Value Bias as Center 4: Bias as Center | 0 | ☆ | 0517 |
| F5-24 | AI1 Gain | -500.0%～500.0% | 100.0% | ☆ | 0518 |
| F5-25 | AI1 Filter Time | 0.00s～20.00s | 0.01s | ☆ | 0519 |
| F5-26 | AI2 Signal Type Selection | 0: 0-10V input selection 1: 0-20mA input selection 2: 4-20mA input selection | 0 | ☆ | 051A |
| F5-27 | AI2 Function Selection | 0: No Function 1: Frequency setting 2: Torque setting 3: Torque compensation setting 4: PID target value 5: PID feedback value 6: Thermistor PTC Input 7: Forward Torque Limit 8: Reverse Torque Limit 9: Regenerative Torque Limit 10: Forward/Reverse Torque Limit 11: Thermistor PT100 Value 12: Auxiliary Frequency Setting 13: PID Offset 14: Tension PID Feedback Value 15: Line Speed Feedback 16: Roll Diameter Feedback 17: Tension PID Setpoint 18: Tension Setpoint 19: Zero-speed Tension Setting 20: Tension Taper Setting | 0 | ☆ | 051B |
| F5-28 | AI2 Input Bias | -100.0%～100.0% | 0.0% | ☆ | 051C |
| F5-29 | AI2 Bias Mode Selection | 0: No Bias 1: Below Bias = Bias 2: Above Bias = Bias 3: Absolute Value Bias as Center 4: Bias as Center | 0 | ☆ | 051D |
| F5-30 | AI2 Gain | -500.0%～500.0% | 100.0% | ☆ | 051E |
| F5-31 | AI2 Filter Time | 0.00s～20.00s | 0.01s | ☆ | 051F |
| F5-32 | AI3 Signal Type Selection | 0: 0-10V input selection 1: 0-20mA input selection 2: 4-20mA input selection | 0 | ☆ | 0520 |
| F5-33 | AI3 Function Selection | 0: No Function 1: Frequency setting 2: Torque setting 3: Torque compensation setting 4: PID target value 5: PID feedback value 6: Thermistor PTC Input 7: Forward Torque Limit 8: Reverse Torque Limit 9: Regenerative Torque Limit 10: Forward/Reverse Torque Limit 11: Thermistor PT100 Value 12: Auxiliary Frequency Setting 13: PID Offset 14: Tension PID Feedback Value 15: Line Speed Feedback 16: Roll Diameter Feedback 17: Tension PID Setpoint 18: Tension Setpoint 19: Zero-speed Tension Setting 20: Tension Taper Setting | 0 | ☆ | 0521 |
| F5-34 | AI3 Input Bias | -100.0%～100.0% | 0.0% | ☆ | 0522 |
| F5-35 | AI3 Bias Mode Selection | 0: No Bias 1: Below Bias = Bias 2: Above Bias = Bias 3: Absolute Value Bias as Center 4: Bias as Center | 0 | ☆ | 0523 |
| F5-36 | Positive AI3 Gain | -500.0%～500.0% | 100.0% | ☆ | 0524 |
| F5-37 | AI3 Filter Time | 0.00s～20.00s | 0.01s | ☆ | 0525 |
| F5-38 | Analog frequency negative value reversal | 0: Forward/reverse rotation determined by operation source 1: Forward/reverse rotation determined by bias | 0 | ☆ | 0526 |
| F5-39 | Analog input addition enable | 0: Disabled 1: Enable | 0 | ☆ | 0527 |
| F5-40 | Reserved | 0～65535 | 0 | ☆ | 0528 |
| F5-41 | Reserved | 0～65535 | 0 | ☆ | 0529 |
| F5-42 | 4-20mA open circuit action | 0: Do not detect open circuit 1: Maintain frequency before open circuit 2: Decelerate to 0Hz 3: Immediate stop due to fault | 0 | ☆ | 052A |
| F5-43 | 4-20mA open circuit threshold | 0.00mA～4.00mA | 2.00mA | ☆ | 052B |
| F6 group - output terminals |  |  |  |  |  |
| F6-00 | RLY1 terminal function selection | 0: No Function 1: VFD running 2: Reached set speed 3: Reached frequency 1 4: Reached frequency 2 5: Zero speed command running 6: Zero speed command 7: Over-torque threshold 1 8: Over-torque threshold 2 9: VFD ready 10: Low Voltage Warning 11: Fault Indication 12: Brake Release 13: Overheat Warning 14: Braking Unit Activation 15: PID Deviation Warning 16: Excessive Slip 17: Set Count Value Reached 18: Final Count Value Reached 19: Base Block 20:Warning Indication 21: Overvoltage 22: Overcurrent Stall Prevention 23: Overvoltage Stall Prevention 24: Operation Source Not Operator 25: Forward Command 26: Reverse Command 29: Speed Higher than F6-08 Value 30: Below F6-08 speed value 31: Motor Y connection command output 32: Motor D connection command output 33: Zero operation output frequency 34: Zero output frequency 35: Fault option 1 36: Fault option 2 37: Fault option 3 38: Fault option 4 39: Positioning complete 40: Reached set frequency STP 41: Multi-point position positioning complete 42: Crane brake release output 43: Above F6-09 speed 44: Low current output 45: UVW electromagnetic switch enable 46: dEb action output 47: No function 48: No function 49: Return to origin action completed 50: CANopen control output 51: RS485 control output 52: Communication card control output 66: STO action output A 67: Analog level arrival output 68: STO action output B 73: Over-torque threshold 3 74: Over-torque threshold 4 75: Forward operation status 76: Reverse operation status | 11 | ☆ | 0600 |
| F6-01 | RLY2 terminal function selection |  | 1 | ☆ | 0601 |
| F6-02 | DO1 Terminal Function Selection |  | 0 | ☆ | 0602 |
| F6-03 | DO2 Terminal Function Selection |  | 0 | ☆ | 0603 |
| F6-04 | DO Terminal Valid Logic | 0000H~FFFFH | 0 | ☆ | 0604 |
| F6-05 | Source of DO Output AI | 0: AI1 1: AI2 2: AI3 | 0 | ☆ | 0605 |
| F6-06 | DO Output AI Upper Limit Value | -100.00%～100.00% | 50.00% | ☆ | 0606 |
| F6-07 | DO Output AI Lower Limit Value | -100.00%～100.00% | 10.00% | ☆ | 0607 |
| F6-08 | DO Action Frequency | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 0608 |
| F6-09 | Motor Zero Speed Judgment Threshold | 0rpm～65535rpm | 0rpm | ☆ | 0609 |
| F6-13 | AO1 Signal Type Selection | 0: 0-10V Output Selection 1: 0-20mA Output Selection | 0 | ☆ | 060D |
| F6-14 | AO1 output function selection | 0: Output frequency (Hz) 1: Frequency command (Hz) 2: Motor operating frequency (Hz) 3: Output current (rms) 4: Output voltage 5: DC Bus Voltage 6: Power factor 7: Power 8: Output torque 9: AI1 percentage 10: AI2 percentage 11: AI3 percentage 12: Iq current command 13: Iq current feedback 14: Id current command 15: Id current feedback 16: Vq voltage command 17: Vd voltage command 18: Torque command 19: PG2 frequency command 20: CANopen analog output 21: RS485 analog output 22: Communication card analog output 23: Fixed voltage output | 0 | ☆ | 060E |
| F6-15 | AO1 output gain | 0.0%～500.0% | 100.0% | ☆ | 060F |
| F6-16 | AO1 reverse enable | 0: Absolute value output 1: Negative value outputs as 0V 2: 5V as center point | 0 | ☆ | 0610 |
| F6-17 | AO1 Output Bias | -100.00%～100.00% | 0.00% | ☆ | 0611 |
| F6-18 | AO1 Output Fixed Value | 0.00%～100.00% | 0.00% | ☆ | 0612 |
| F6-19 | AO1 Filter Time | 0.00s～20.00s | 0.01s | ☆ | 0613 |
| F6-20 | AO2 Signal Type Selection | 0: 0-10V Output Selection 1: 0-20mA Output Selection | 0 | ☆ | 0614 |
| F6-21 | AO2 Output Function Selection | 0: Output frequency (Hz) 1: Frequency command (Hz) 2: Motor operating frequency (Hz) 3: Output current (rms) 4: Output voltage 5: DC Bus Voltage 6: Power factor 7: Power 8: Output torque 9: AI1 percentage 10: AI2 percentage 11: AI3 percentage 12: Iq current command 13: Iq current feedback 14: Id current command 15: Id current feedback 16: Vq voltage command 17: Vd voltage command 18: Torque command 19: PG2 frequency command 20: CANopen analog output 21: RS485 analog output 22: Communication card analog output 23: Fixed voltage output | 0 | ☆ | 0615 |
| F6-22 | AO2 Output Gain | 0.0%～500.0% | 100.0% | ☆ | 0616 |
| F6-23 | AO2 Reverse Enable | 0: Absolute value output 1: Negative value outputs as 0V 2: 5V as center point | 0 | ☆ | 0617 |
| F6-24 | AO2 Output Bias | -100.00%～100.00% | 0.00% | ☆ | 0618 |
| F6-25 | AO2 Output Fixed Value | 0.00%～100.00% | 0.00% | ☆ | 0619 |
| F6-26 | AO2 Filter Time | 0.00s～20.00s | 0.01s | ☆ | 061A |
| F6-27 | Frequency Reach 1 Detection Value | 0.00Hz～599.00Hz | 50.00Hz | ☆ | 061B |
| F6-28 | Frequency Reach 1 Amplitude | 0.00Hz～599.00Hz | 2.00Hz | ☆ | 061C |
| F6-29 | Frequency reaches 2 detection value | 0.00Hz～599.00Hz | 50.00Hz | ☆ | 061D |
| F6-30 | Frequency reaches 2 amplitude | 0.00Hz～599.00Hz | 2.00Hz | ☆ | 061E |
| F7 group - auxiliary functions and keyboard display |  |  |  |  |  |
| F7-00 | JOG frequency setting | 0.00Hz～599.00Hz | 6.00Hz | ☆ | 0700 |
| F7-01 | JOG acceleration time | 0.00s～600.00s | 10.00s | ☆ | 0701 |
| F7-02 | JOG deceleration time | 0.00s～600.00s | 10.00s | ☆ | 0702 |
| F7-03 | acceleration time 2 | 0.00s～600.00s | Model Confirmation | ☆ | 0703 |
| F7-04 | deceleration time 2 | 0.00s～600.00s | Model Confirmation | ☆ | 0704 |
| F7-05 | acceleration time 3 | 0.00s～600.00s | Model Confirmation | ☆ | 0705 |
| F7-06 | deceleration time 3 | 0.00s～600.00s | Model Confirmation | ☆ | 0706 |
| F7-07 | acceleration time 4 | 0.00s～600.00s | Model Confirmation | ☆ | 0707 |
| F7-08 | Deceleration Time 4 | 0.00s～600.00s | Model Confirmation | ☆ | 0708 |
| F7-09 | Jump Frequency 1 Upper Limit | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 0709 |
| F7-10 | Jump Frequency 1 Lower Limit | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 070A |
| F7-11 | Jump Frequency 2 Upper Limit | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 070B |
| F7-12 | Jump Frequency 2 Lower Limit | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 070C |
| F7-13 | Jump Frequency 3 Upper Limit | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 070D |
| F7-14 | Jump frequency 3 lower limit | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 070E |
| F7-15 | PMLESS jump frequency upper limit | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 070F |
| F7-16 | PMLESS jump frequency lower limit | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 0710 |
| F7-17 | Fan control method | 0: Fan runs continuously 1: Stop after 1 minute STOP 2: Stops with the inverter 3: Starts when temperature reaches 50 degrees 4: Stops when temperature falls below 50 degrees | 4 | ☆ | 0711 |
| F7-18 | Reserved | 0%～100% | 60% | ☆ | 0712 |
| F7-19 | Keyboard STOP key enable | 0: Disabled 1: Enable | 0 | ☆ | 0713 |
| F7-20 | Startup screen selection | 0: Set frequency display 1: Output frequency display 2: User-defined display 3: Output current display | 0 | ☆ | 0714 |
| F7-21 | Page display selection | 0: Output current 1: PG card feedback frequency 2: Motor actual operating frequency 3: DC Bus Voltage 4: Output voltage 5: Power factorangle 6: Output power 7: Actual motor operating speed 8: Output torque % 9: PG feedback value 10: PID feedback value % 11: AI1 % 12: AI2 % 13: AI3 % 14: IGBT temperature 15: Ambient temperature 16: Digital input terminal status 17: Digital output terminal status 18: Multi-speed status 19: CPU input terminal status 20: CPU output terminal status 21: Actual motor position 22: Pulse input frequency value 23: Pulse input position 24: Position tracking error 25: Overload count value 26: Ground short-circuit current threshold 27: Bus voltage fluctuation value 29: PM motor pole sector 30: User physical quantity 31: H page value multiplied by coefficient K 32: Encoder Z phase count 33: Motor pulse count 34: Reserved 35: Speed/torque mode 36: Current carrier frequency 37: Reserved 38: Inverter status 39: Output torque Nt-m 40: Torque command 41: kWh 42: PID target value 43: PID compensation 44: PID output frequency 45: Reserved 46: Auxiliary frequency 47: Main frequency 48: Set frequency display 49: Reserved 50: Reserved 51: PMVVC torque compensation amount 52: AI10 % 53: AI11 % 54: Reserved 55: Current roll diameter 56: Current line speed 57: Tension reference value 58: MI6 count value 59: U phase current AD value 60: V phase current AD value 61: W-phase current AD value | 3 | ☆ | 0715 |
| F7-22 | H page display gain | 0.00～160.00 | 0.00 | ☆ | 0716 |
| F7-23 | FM terminal function selection | 0: Operating frequency 1: Set frequency 2: Output current 3: Output voltage 4: Output power 5: AI1 value 6: AI2 value 7: AI3 value 8: Output torque absolute value 9: Motor speed 10: Bus Voltage | 0 | ☆ | 0717 |
| F7-24 | FMP Output Maximum Frequency | 1.00%～100.00% | 50.00% | ☆ | 0718 |
| F7-25 | IGBT Temperature Bias | 0.0℃～6553.5℃ | 0.0℃ | ☆ | 0719 |
| F7-26 | Cumulative Start-up Times | 0～65535 | 0 | ● | 071A |
| F7-27 | Cumulative Start-up Days | 0～65535 | 0 | ● | 071B |
| F7-28 | Cumulative Operating Minutes | 0～1439 | 0 | ● | 071C |
| F7-29 | Cumulative Operating Days | 0～65535 | 0 | ● | 071D |
| F7-30 | Cumulative Running Minutes | 0～65535 | 0 | ● | 071E |
| F7-31 | Motor Running Time | 0min～1439min | 0min | ☆ | 071F |
| F7-32 | Motor Running Days | 0～65535 | 0 | ☆ | 0720 |
| F7-33 | Password Input | 0～65535 | 0 | ☆ | 0721 |
| F7-34 | Password Setting | 0～65535 | 0 | ☆ | 0722 |
| F7-35 | Automatic Energy Saving Setting | 0: Disabled 1: Enable | 0 | ☆ | 0723 |
| F7-36 | Energy Saving Gain | 10～1000 | 100 | ☆ | 0724 |
| F7-37 | Automatic Voltage Regulation | 0: Enable AVR Function 1: Disable AVR Function 2: Disable AVR During Deceleration | 0 | ☆ | 0725 |
| F7-38 | Current Display Filter | 0.001s～65.535s | 0.100s | ☆ | 0726 |
| F7-39 | Display Filter Time | 0.001s～65.535s | 0.100s | ☆ | 0727 |
| F7-40 | Soft Start Switch Delay Time | 0～65535 | 7000 | ☆ | 0728 |
| F7-41 | Operating Frequency Deviation Dead Zone | 0.00～599.00 | 0.00 | ☆ | 0729 |
| F7-42 | Output Phase Sequence Switching | 0: Output Phase Sequence Switching 1: Output Phase Sequence Switching | 0 | ☆ | 072A |
| F7-46 | CANopen Quick Stop Time | 0.00s～600.00s | 1.00s | ☆ | 072E |
| F7-47 | Y-D Switching Frequency | 0.00Hz～599.00Hz | 60.00Hz | ☆ | 072F |
| F7-48 | Y-D Switching Enable | 0: Disabled 1: Enable | 0 | ☆ | 0730 |
| F7-49 | Y-D Switching Time | 0.000s～60.000s | 0.200s | ☆ | 0731 |
| F7-50 | ICT Test Dedicated | 0～65535 | 0 | ☆ | 0732 |
| F7-51 | Electromagnetic Switch Check Value | 0～65535 | 200 | ☆ | 0733 |
| F7-52 | W-sec Low Byte | 0.0～6553.5 | 0.0 | ● | 0734 |
| F7-53 | W-sec High Byte | 0.0～6553.5 | 0.0 | ● | 0735 |
| F7-54 | W-hrs | 0.0～6553.5 | 0.0 | ● | 0736 |
| F7-55 | KWh Low Byte | 0.0～6553.5 | 0.0 | ● | 0737 |
| F7-56 | KWh High Byte | 0.0～6553.5 | 0.0 | ● | 0738 |
| F7-57 | Software Version (H) | 0.00～655.35 | \ | ● | 0739 |
| F7-58 | Software Version (L) | 0.00～655.35 | \ | ● | 073A |
| F7-59 | Software Release Date | 0～65535 | \ | ● | 073B |
| F7-60 | PG Card Software Version | 0.00～655.35 | 0.00 | ● | 073C |
| F7-61 | Enhanced Parameter Display Selection | 0～1 | 1 | ☆ | 073D |
| F7-62 | High Four Bits of Serial Code | 0～65535 | \ | ● | 073E |
| F7-63 | Low Three Bits of Serial Code | 0～65535 | \ | ● | 073F |
| F7-64 | Language Setting | 0: Simplified Chinese 1: English | \ | ● | 0740 |
| F7-65 | Page Display Selection 2 | Same as F7-21 | 2 | ☆ | 0741 |
| F8 Group - Communication Parameters |  |  |  |  |  |
| F8-00 | Baud Rate Setting | 4.8kbps～115.2kbps | 115.2kbps | ☆ | 0800 |
| F8-01 | Communication Data Format | 1: 7,N,2 for ASCII 2: 7,E,1 for ASCII 3: 7,O,1 for ASCII 4: 7,E,2 for ASCII 5: 7,O,2 for ASCII 6: 8,N,1 for ASCII 7: 8,N,2 for ASCII 8: 8,E,1 for ASCII 9: 8,O,1 for ASCII 10: 8,E,2 for ASCII 11:8,O,2 for ASCII 12:8,N,1 for RTU 13:8,N,2 for RTU 14:8,E,1 for RTU 15:8,O,1 for RTU 16:8,E,2 for RTU 17:8,O,2 for RTU | 12 | ☆ | 0801 |
| F8-02 | Communication Address | 1～254 | 1 | ☆ | 0802 |
| F8-03 | Response Delay | 0.0ms～200.0ms | 2.0ms | ☆ | 0803 |
| F8-04 | Communication timeout time | 0.0s～100.0s | 0.0s | ☆ | 0804 |
| F8-05 | Communication error handling | 0: Warning and continue running 1: Warning and decelerate to stop 2: Warning and free stop 3: No warning | 3 | ☆ | 0805 |
| F8-06 | Communication given frequency | 0.00Hz～599.00Hz | 0.00Hz | ● | 0806 |
| F8-07 | Communication decoding method | 0: Use 20XX 1: Use 60XX | 1 | ☆ | 0807 |
| F8-09 | Communication Card Type | 0: No Communication Card 1: DeviceNet 2: Profibus-DP 3: CANopen 4: Modbus-TCP 5: EtherNet/IP 6: EtherCAT 7: LonWorks 8: BACNet 9: Reserved 10: 24V Power Supply 11: DMCnet 12: PROFINET | 0 | ● | 0809 |
| F8-10 | Communication Card Version | 0.0～6553.5 | 0.0 | ● | 080A |
| F8-11 | Communication Card Address | 0～65535 | 0 | ● | 080B |
| F8-12 | Communication Card Speed | 0～65535 | 0 | ☆ | 080C |
| F8-13 | Speed Enable Setting | 0～65535 | 0 | ☆ | 080D |
| F8-14 | CANopen Node Address | 0～127 | 0 | ☆ | 080E |
| F8-15 | CAN Bus Communication Speed | 0：1Mbps 1：500kbps 2：250kbps 3：125kbps 4：100kbps 5：50kbps | 0 | ☆ | 080F |
| F8-16 | Reserved | 0～65535 | 2 | ☆ | 0810 |
| F8-17 | Reserved | 0.00～2.00 | 1.00 | ☆ | 0811 |
| F8-18 | CANopen warning record | 0～65535 | 0 | ● | 0812 |
| F8-19 | CiA402 protocol selection | 0: Disabled 1: Enable | 0 | ☆ | 0813 |
| F8-20 | CANopen Communication Status | 0: Node Reset State 1: Communication Reset State 2: Reset Completed State 3: Pre-operational State 4: Operational State 5: Stopped State | 0 | ● | 0814 |
| F8-21 | CiA402 Operating State | 0: Power-up Not Completed State 1: Operation Prohibited State 2: Pre-magnetization State 3: Magnetization State 4: Operation Permitted State 5: No Function 6: No Function 7: Quick Stop State 8: No Function 9: No Function 10: No Function 11: No Function 12: No Function 13: Trigger Error Action Status 14: Error Status | 0 | ● | 0815 |
| F8-22 | CANopen Index Reset | 0000H~FFFFH | 65535 | ☆ | 0816 |
| F8-24 | Product Code | 0～65535 | 0 | ● | 0818 |
| F8-25 | Error Code | 0～65535 | 0 | ● | 0819 |
| F8-26 | Communication Card DHCP Enable | 0～1 | 0 | ☆ | 081A |
| F8-27 | Communication Card IP 1 | 0～255 | 0 | ☆ | 081B |
| F8-28 | Communication Card IP 2 | 0～255 | 0 | ☆ | 081C |
| F8-29 | Communication Card IP 3 | 0～255 | 0 | ☆ | 081D |
| F8-30 | Communication Card IP 4 | 0～255 | 0 | ☆ | 081E |
| F8-31 | Communication Card Subnet Mask 1 | 0～255 | 0 | ☆ | 081F |
| F8-32 | Communication Card Mask 2 | 0～255 | 0 | ☆ | 0820 |
| F8-33 | Communication Card Mask 3 | 0～255 | 0 | ☆ | 0821 |
| F8-34 | Communication Card Mask 4 | 0～255 | 0 | ☆ | 0822 |
| F8-35 | Communication Card Gateway 1 | 0～255 | 0 | ☆ | 0823 |
| F8-36 | Communication Card Gateway 2 | 0～255 | 0 | ☆ | 0824 |
| F8-37 | Communication Card Gateway 3 | 0～255 | 0 | ☆ | 0825 |
| F8-38 | Communication Card Gateway 4 | 0～255 | 0 | ☆ | 0826 |
| F8-39 | Communication Card Low Byte Password | 0～99 | 0 | ☆ | 0827 |
| F8-40 | Communication Card High Byte Password | 0～99 | 0 | ☆ | 0828 |
| F8-41 | Communication Card Reset | 0～65535 | 0 | ☆ | 0829 |
| F8-42 | Communication Card Setup | 0000H~FFFFH | 0 | ☆ | 082A |
| F8-43 | Communication Card Status | 0000H~FFFFH | 0 | ● | 082B |
| F8-44 | Reserved | 0: Disabled 1: Enable | 0 | ☆ | 082C |
| F8-45 | Reserved | 0～127 | 100 | ☆ | 082D |
| F8-46 | Reserved | 0～65535 | 0 | ● | 082E |
| F8-47 | Reserved | 0～65535 | 1 | ☆ | 082F |
| F8-48 | Communication Card Debugging Parameters | 0～65535 | 0 | ☆ | 0830 |
| F9 Group - Faults and Protection |  |  |  |  |  |
| F9-00 | Protection Control Bit | 0000h～FFFFh | 0000h | ☆ | 0900 |
| F9-01 | Motor 1 Overload Selection | 0: Constant Torque Output Motor 1: Variable Torque Output Motor 2: No Motor Overload Protection | 2 | ☆ | 0901 |
| F9-02 | Motor 1 Overload Time | 30.0s～600.0s | 60.0s | ☆ | 0902 |
| F9-03 | Overvoltage Stall Mode | 0: Overvoltage Stall Mode 0 1: Overvoltage Stall Mode 1 2: Overvoltage Stall Mode 2 3: Overvoltage Stall Mode 3 | 1 | ☆ | 0903 |
| F9-04 | overvoltage stall threshold | 0.0V～900.0V | 760.0V | ☆ | 0904 |
| F9-05 | overvoltage stall deceleration time | 0.00s～655.35s | 600.00s | ☆ | 0905 |
| F9-06 | undervoltage fault automatic reset | 0: Disabled 1: Enable | 0 | ☆ | 0906 |
| F9-07 | maximum current limit | 0%～250% | 150% | ☆ | 0907 |
| F9-08 | Acceleration OC Stall Threshold | 0%～200% | 180% | ☆ | 0908 |
| F9-09 | Overcurrent Stall Limit Threshold | 0%～100% | 100% | ☆ | 0909 |
| F9-10 | Running OC Stall Threshold | 0%～200% | 180% | ☆ | 090A |
| F9-11 | Constant Speed OC Acceleration/Deceleration Selection | 0: System Acceleration/Deceleration Time 1: First Acceleration/Deceleration Time 2: Second Acceleration/Deceleration Time 3: Third Acceleration/Deceleration Time 4: Fourth Acceleration/Deceleration Time 5: Automatic Acceleration/Deceleration Time | 0 | ☆ | 090B |
| F9-12 | Input Phase Loss Action Selection | 0: Warning and decelerate to stop 1: Warning and free stop | 0 | ☆ | 090C |
| F9-13 | Input Phase Loss Filter Time | 0.00s～600.00s | 0.20s | ☆ | 090D |
| F9-14 | Input Phase Loss Voltage Threshold | 0.0V～320.0V | 60.0V | ☆ | 090E |
| F9-15 | Output Phase Loss Action Selection | 0: Warning and continue running 1: Warning and decelerate to stop 2: Warning and free stop 3: No warning | 3 | ☆ | 090F |
| F9-16 | Output Phase Loss Detection Time | 0.000s～65.535s | 7.5kw and above 0.500; 5.5kW and below 0.100; | ☆ | 0910 |
| F9-17 | Output Phase Loss Current Threshold | 0.00%～100.00% | 7.5kw and above 2.00; 5.5kW and below 6.00; | ☆ | 0911 |
| F9-18 | Output Phase Loss Braking Time | 0.000s～65.535s | 0.000s | ☆ | 0912 |
| F9-19 | Ground Short Circuit Test Times | 1～20 | 10 | ☆ | 0913 |
| F9-20 | Ground FaultCurrent Threshold | 0.0%～6553.5% | 60.0% | ☆ | 0914 |
| F9-21 | Ground FaultFilter Time | 0.00s～655.35s | 0.10s | ☆ | 0915 |
| F9-22 | Low Current Set Threshold | 0.0%～100.0% | 0.0% | ☆ | 0916 |
| F9-23 | Low Current Detection Time | 0.00s～360.00s | 0.00s | ☆ | 0917 |
| F9-24 | Low Current Action Method | 0: No Function 1: Alarm and Free Stop 2: Alarm for second deceleration stop 3: Alarm and continue running | 0 | ☆ | 0918 |
| F9-25 | Excessive slip detection value | 0.0%～100.0% | 0.0% | ☆ | 0919 |
| F9-26 | Excessive slip detection time | 0.0s～10.0s | 1.0s | ☆ | 091A |
| F9-27 | Excessive slip action selection | 0: Warning and continue running 1: Warning and decelerate to stop 2: Warning and free stop 3: No warning | 0 | ☆ | 091B |
| F9-28 | PG feedback error action | 0: Warning and continue running 1: Warning and deceleration stop 2: Warning and Free Stop | 2 | ☆ | 091C |
| F9-29 | PG Feedback Error Time | 0.0s～10.0s | 1.0s | ☆ | 091D |
| F9-30 | PG Feedback Overspeed Threshold | 0%～120% | 115% | ☆ | 091E |
| F9-31 | PG Feedback Overspeed Time | 0.0s～2.0s | 0.1s | ☆ | 091F |
| F9-32 | PG Feedback Overspeed Action | 0: Warning and continue running 1: Warning and deceleration stop 2: Warning and Free Stop | 2 | ☆ | 0920 |
| F9-33 | PG Feedback Deviation Range | 0%～50% | 50% | ☆ | 0921 |
| F9-34 | PG Feedback Deviation Time | 0.0s～10.0s | 0.5s | ☆ | 0922 |
| F9-35 | PG Feedback Deviation Action | 0: Warning and continue running 1: Warning and deceleration stop 2: Warning and Free Stop | 2 | ☆ | 0923 |
| F9-36 | Over Torque Selection 1 | 0: Not Detected 1: Constant Speed Detection Continue Running 2: Constant Speed Detection Stop Running 3: Running Detection Continue Running 4: Running Detection Stop Running | 0 | ☆ | 0924 |
| F9-37 | Over-torque threshold 1 | 10%～250% | 120% | ☆ | 0925 |
| F9-38 | Over-torque time 1 | 0.1s～60.0s | 0.1s | ☆ | 0926 |
| F9-39 | Reserved | 0～65535 | 0 | ● | 0927 |
| F9-40 | Reserved | 0～65535 | 0 | ★ | 0928 |
| F9-41 | Reserved | 0～65535 | 0 | ☆ | 0929 |
| F9-42 | Reserved | 0～65535 | 0 | ☆ | 092A |
| F9-43 | Reserved | 0～65535 | 0 | ☆ | 092B |
| F9-44 | Reserved | 0～65535 | 0 | ☆ | 092C |
| F9-45 | Reserved | 0～65535 | 0 | ☆ | 092D |
| F9-46 | Abnormal Start Count | 0～10 | 0 | ☆ | 092E |
| F9-47 | Abnormal Restart Reset Time | 0.0s～6000.0s | 60.0s | ☆ | 092F |
| F9-48 | PTC Action Selection | 0: Warning and continue running 1: Warning and decelerate to stop 2: Warning and free stop 3: No warning | 0 | ☆ | 0930 |
| F9-49 | PTC Threshold | 0.0%～100.0% | 50.0% | ☆ | 0931 |
| F9-50 | PT Detection Threshold 1 | 0.000V～10.000V | 5.000V | ☆ | 0932 |
| F9-51 | PT Detection Threshold 2 | 0.000V～10.000V | 7.000V | ☆ | 0933 |
| F9-52 | PT Voltage 1 Protection Frequency | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 0934 |
| F9-53 | PT Action Delay Time | 0s～6000s | 60s | ☆ | 0935 |
| F9-54 | STO Lock Function | 0: Locked 1: Unlocked | 0 | ☆ | 0936 |
| F9-55 | Reserved | 0～65535 | 40 | ☆ | 0937 |
| F9-56 | Reserved | 0～65535 | 0 | ☆ | 0938 |
| F9-57 | Output Phase Loss Threshold 2 | 0.00%～100.00% | 2.00% | ☆ | 0939 |
| F9-58 | Overvoltage Stall Recovery Threshold | 0.0V～900.0V | 630.0V | ☆ | 093A |
| FA Group - PID Function |  |  |  |  |  |
| FA-00 | PID Feedback Type Selection | 0: No Function 1: Negative Feedback Analog Input 2: Negative Feedback Pulse No Direction 3: Negative Feedback Pulse With Direction 4: Positive Feedback Analog Input 5: Positive Feedback Pulse No Direction 6: Positive Feedback Pulse With Direction 7: Negative Feedback Communication Input 8: Positive Feedback Communication Input | 0 | ☆ | 0A00 |
| FA-01 | PID Setpoint Source Selection | 0: Frequency Command 1: Parameter FA-02 2: RS485 Communication 3: Analog Input 4: CANopen 5: Retain 6: Communication Card | 1 | ☆ | 0A01 |
| FA-02 | PID Setpoint | -100.00%～100.00% | 50.00% | ☆ | 0A02 |
| FA-03 | PID Setpoint Change Time | 0.00s～655.35s | 0.00s | ☆ | 0A03 |
| FA-04 | PID Feedback Filter Time | 0.1s～300.0s | 5.0s | ☆ | 0A04 |
| FA-05 | Proportional Coefficient 1 | 0.00%～100.00% | 88.00% | ☆ | 0A05 |
| FA-06 | Integral Time 1 | 0.00s～100.00s | 0.05s | ☆ | 0A06 |
| FA-07 | Differential Time 1 | 0.00s～1.00s | 0.00s | ☆ | 0A07 |
| FA-08 | Proportional Coefficient 2 | 0.00～100.00 | 100.00 | ☆ | 0A08 |
| FA-09 | Integral Time 2 | 0.00s～100.00s | 0.08s | ☆ | 0A09 |
| FA-10 | Differential Time 2 | 0.00s～1.00s | 0.00s | ☆ | 0A0A |
| FA-11 | PID Series/Parallel Selection | 0: Kp, Kp*Ki, Kp*Kd 1: Kp, Ki, Kd | 1 | ☆ | 0A0B |
| FA-12 | PID Control Execution Cycle | 0～1 | 0 | ☆ | 0A0C |
| FA-13 | PID Parameter Switching Conditions | 0: No Function 1: Switch Based on Output Frequency 2: Switch Based on PID Deviation | 0 | ☆ | 0A0D |
| FA-14 | PID Parameter Switching err1 | 0.00%～100.00% | 10.00% | ☆ | 0A0E |
| FA-15 | PID Parameter Switching Error 2 | 0.00%～100.00% | 40.00% | ☆ | 0A0F |
| FA-16 | Allow PID Reverse Delay | 0.0s～6553.5s | 0.0s | ☆ | 0A10 |
| FA-17 | PID Direction Change Selection | 0: Disabled 1: Enable | 0 | ☆ | 0A11 |
| FA-18 | Feedback Suppression Deviation Rate | 0%～65535% | 10% | ☆ | 0A12 |
| FA-19 | Feedback Suppression Gain | 0～1000 | 800 | ☆ | 0A13 |
| FA-20 | PID Compensation Selection | 0: Parameter Setting 1: Analog Input | 0 | ☆ | 0A14 |
| FA-21 | PID Compensation Value | -100.0～100.0 | 0.0 | ☆ | 0A15 |
| FA-22 | PID Deviation Deadband Limit | 0.00%～100.00% | 0.06% | ☆ | 0A16 |
| FA-23 | PID Control Deviation Limit | 0.00%～100.00% | 0.00% | ☆ | 0A17 |
| FA-24 | Integral Separation Level | 0.00%～100.00% | 0.00% | ☆ | 0A18 |
| FA-25 | Integral Upper Limit | 0.00%～100.00% | 100.0% | ☆ | 0A19 |
| FA-26 | Wake-up Integral Limit | 0.0%～200.0% | 50.0% | ☆ | 0A1A |
| FA-27 | Main Auxiliary Reverse Cut-off Frequency | 0.00%～100.00% | 10.0% | ☆ | 0A1B |
| FA-28 | PID Output Positive Limit | 0.00%～100.00% | 100.0% | ☆ | 0A1C |
| FA-29 | PID Output Negative Limit | 0.00%～100.00% | 100.0% | ☆ | 0A1D |
| FA-30 | PID output frequency reference | 0～1 | 0 | ☆ | 0A1E |
| FA-31 | PID output filter time | 0.0s～2.5s | 0.0s | ☆ | 0A1F |
| FA-32 | soft start-PID switch value | 0.00%～100.00% | 5.00% | ☆ | 0A20 |
| FA-33 | soft start frequency | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 0A21 |
| FA-34 | soft start acceleration time | 0.00s～600.00s | 3.00s | ☆ | 0A22 |
| FA-35 | No-load Current | 0.00A～655.35A | 0.00A | ☆ | 0A23 |
| FA-36 | Soft Start Acceleration Step | 0.00s～600.00s | 0.10s | ☆ | 0A24 |
| FA-37 | Fuzzy PID Tuning Enable | 0: Fuzzy PID Tuning Disabled 1: Fuzzy PID Tuning Enabled | 1 | ☆ | 0A25 |
| FA-38 | Deviation Fuzzy Universe of Discourse - NB | 0.00～100.00 | 5.00 | ☆ | 0A26 |
| FA-39 | Deviation Fuzzy Domain-NS | 0.00～100.00 | 2.00 | ☆ | 0A27 |
| FA-40 | Deviation Fuzzy Domain-PS | 0.00～100.00 | 2.00 | ☆ | 0A28 |
| FA-41 | Deviation Fuzzy Domain-PB | 0.00～100.00 | 5.00 | ☆ | 0A29 |
| FA-42 | Deviation Rate Fuzzy Domain-NB | 0.00～100.00 | 10.00 | ☆ | 0A2A |
| FA-43 | Deviation Rate Fuzzy Domain-NS | 0.00～100.00 | 5.00 | ☆ | 0A2B |
| FA-44 | Deviation Rate Fuzzy Domain-PS | 0.00～100.00 | 5.00 | ☆ | 0A2C |
| FA-45 | Fuzzy Set of Deviation Rate PB | 0.00～100.00 | 10.00 | ☆ | 0A2D |
| FA-46 | Fuzzy PID Inference Rules | 0～3 | 2 | ☆ | 0A2E |
| FA-47 | Intermediate Value of Fuzzy Rule KP | 0～100 | 50 | ☆ | 0A2F |
| FA-48 | Intermediate Value of Fuzzy Rule KI | 0～100 | 50 | ☆ | 0A30 |
| FA-49 | Feedback Abnormality Detection Time | 0.0s～3600.0s | 0.0s | ☆ | 0A31 |
| FA-50 | Feedback Disconnection Action Selection | 0: Warning and continue running 1: Warning and deceleration stop 2: Warning and Free Stop 3: Run at the Frequency Before Disconnection | 0 | ☆ | 0A32 |
| FA-51 | PID Feedback Abnormal Deviation | 1.0%～50.0% | 10.0% | ☆ | 0A33 |
| FA-52 | Abnormal Deviation Detection Time | 0.1s～300.0s | 5.0s | ☆ | 0A34 |
| FA-53 | PID Control Flag | 0～65535 | 2 | ☆ | 0A35 |
| FA-54 | PID Feedback Disconnection Threshold | 0.00%～100.00% | 0.00% | ☆ | 0A36 |
| FB Group - Tension Control Parameters |  |  |  |  |  |
| FB-00 | Tension Control Mode Selection | 0～4 | 0 | ☆ | 0B00 |
| FB-01 | Curling Mode | 0～1 | 0 | ☆ | 0B01 |
| FB-02 | Load Side Mechanical Gear A | 1～65535 | 100 | ☆ | 0B02 |
| FB-03 | Load Side Mechanical Gear B | 1～65535 | 100 | ☆ | 0B03 |
| FB-04 | PID Setpoint Source Selection | 0～2 | 0 | ☆ | 0B04 |
| FB-05 | PID Target Setpoint | 0.0%～100.0% | 50.0% | ☆ | 0B05 |
| FB-06 | PID Feedback Source Selection | 0～1 | 0 | ☆ | 0B06 |
| FB-07 | PID Parameter Adjustment Basis | 0～3 | 0 | ★ | 0B07 |
| FB-08 | PID Proportional Coefficient 1 | 0.0～1000.0 | 50.0 | ☆ | 0B08 |
| FB-09 | PID Integral Time 1 | 0.00s～500.00s | 1.00s | ☆ | 0B09 |
| FB-10 | PID Proportional Coefficient 2 | 0.0～1000.0 | 50.0 | ☆ | 0B0A |
| FB-11 | PID Integral Time 2 | 0.00s～500.00s | 1.00s | ☆ | 0B0B |
| FB-12 | PID Output Positive/Negative Selection | 0～1 | 0 | ★ | 0B0C |
| FB-13 | PID Output Positive Limit | 0.00%～100.00% | 20.00% | ★ | 0B0D |
| FB-14 | PID Output Negative Limit | 0.00%～100.00% | 20.00% | ★ | 0B0E |
| FB-15 | PID Feedback Upper Limit Value | 0.0%～100.0% | 100.0% | ★ | 0B0F |
| FB-16 | PID Feedback Lower Limit | 0.0%～100.0% | 0.0% | ★ | 0B10 |
| FB-17 | Linear Speed Input Source | 0～5 | 0 | ★ | 0B11 |
| FB-18 | Maximum Linear Speed | 0.00～650.00 | 100.00 | ★ | 0B12 |
| FB-19 | Minimum Linear Speed | 0.00～650.00 | 0.00 | ★ | 0B13 |
| FB-20 | Pulses per Meter | 0.0～6500.0 | 0.0 | ★ | 0B14 |
| FB-21 | current linear speed | 0.00～650.00 | 0.00 | ★ | 0B15 |
| FB-22 | linear speed filter time | 0.00s～100.00s | 0.10s | ☆ | 0B16 |
| FB-23 | linear speed acceleration time | 0.00s～655.35s | 0.00s | ☆ | 0B17 |
| FB-24 | linear speed deceleration time | 0.00s～655.35s | 0.00s | ☆ | 0B18 |
| FB-25 | winding diameter calculation method selection | 0～5 | 0 | ☆ | 0B19 |
| FB-26 | Maximum Roll Diameter | 1.0mm～6000.0mm | 500.0mm | ☆ | 0B1A |
| FB-27 | Empty Roll Diameter | 1.0mm～6000.0mm | 100.0mm | ☆ | 0B1B |
| FB-28 | Initial Roll Diameter Selection | 0～1 | 0 | ★ | 0B1C |
| FB-29 | Initial Roll Diameter 0 | 0.0mm～6000.0mm | 100.0mm | ★ | 0B1D |
| FB-30 | Initial Roll Diameter 1 | 0.0mm～6000.0mm | 100.0mm | ★ | 0B1E |
| FB-31 | Initial Roll Diameter 2 | 0.0mm～6000.0mm | 100.0mm | ★ | 0B1F |
| FB-32 | Pulses per Revolution | 1ppr～60000ppr | 1ppr | ★ | 0B20 |
| FB-33 | Number of Coils per Layer | 1～10000 | 1 | ★ | 0B21 |
| FB-34 | Material Thickness | 0.001～65.000 | 0.001 | ★ | 0B22 |
| FB-35 | Roll Diameter Filter Time | 0.00s～100.00s | 1.00s | ☆ | 0B23 |
| FB-36 | Roll Diameter Compensation Enable | 0～1 | 0 | ★ | 0B24 |
| FB-37 | Roll Diameter Calculation Delay Time | 0.0s～6553.5s | 0.0s | ☆ | 0B25 |
| FB-38 | Current Roll Diameter | 1.0～6553.5 | 100.0 | ● | 0B26 |
| FB-39 | Minimum Frequency for Roll Diameter Calculation | 0.00Hz～599.00Hz | 1.00Hz | ☆ | 0B27 |
| FB-40 | Pre-drive Mode Selection | 0～2 | 0 | ★ | 0B28 |
| FB-41 | Pre-drive/PID Switching Point | 0.0%～100.0% | 15.0% | ★ | 0B29 |
| FB-42 | soft start frequency | 0.00Hz～599.00Hz | 2.00Hz | ★ | 0B2A |
| FB-43 | soft start acceleration time | 0.00s～600.00s | 3.00s | ☆ | 0B2B |
| FB-44 | Broken Belt Detection Selection | 0～1 | 0 | ☆ | 0B2C |
| FB-45 | Minimum Line Speed for Broken Belt Detection | 0.0～3000.0 | 0.0 | ★ | 0B2D |
| FB-46 | Belt Breakage Detection Roll Diameter Change | 1.0mm～6000.0mm | 100.0mm | ★ | 0B2E |
| FB-47 | Belt Breakage Detection Time | 0.00s～100.00s | 1.00s | ★ | 0B2F |
| FB-48 | Tension Feedback Error Level | 0%～100% | 100% | ★ | 0B30 |
| FB-49 | Tension Error Detection Time | 0.0s～10.0s | 0.5s | ★ | 0B31 |
| FB-50 | Tension Error Abnormal Handling | 0: Warning and Continue Running 1:Faultand Free Stop 2:Faultand Decelerate to Stop | 0 | ★ | 0B32 |
| FB-51 | PID Output Gain | 0.0～200.0 | 100.0 | ★ | 0B33 |
| FB-52 | Tension Reference Source Selection | 0～1 | 0 | ★ | 0B34 |
| FB-53 | Maximum Tension Value | 0～65535 | 0 | ★ | 0B35 |
| FB-54 | Tension Setpoint | 0～65535 | 0 | ☆ | 0B36 |
| FB-55 | Zero-speed Tension Reference Source | 0～2 | 0 | ★ | 0B37 |
| FB-56 | Zero-speed Tension Setpoint | 0～65535 | 0 | ☆ | 0B38 |
| FB-57 | Zero Speed Tension Level | 0.00%～100.00% | 5.00% | ☆ | 0B39 |
| FB-58 | Sliding Friction Compensation Tension | 0.0%～100.0% | 0.0% | ☆ | 0B3A |
| FB-59 | Material Inertia Compensation Coefficient | 0～30000 | 0 | ☆ | 0B3B |
| FB-60 | Acceleration Inertia Compensation Gain | 0.0%～1000.0% | 0.0% | ☆ | 0B3C |
| FB-61 | Inertia Compensation Filter Time | 0.00～100.00 | 5.00 | ☆ | 0B3D |
| FB-62 | Deceleration Inertia Compensation Gain | 0.0%～1000.0% | 0.0% | ★ | 0B3E |
| FB-63 | Tension Taper Curve Selection | 0～4 | 0 | ★ | 0B3F |
| FB-64 | Tension Taper Setting Source | 0～1 | 0 | ★ | 0B40 |
| FB-65 | Taper Setting Value | 0%～100% | 0% | ☆ | 0B41 |
| FB-66 | Taper Curve Compensation Value | 0.0～6000.0 | 0.0 | ★ | 0B42 |
| FB-67 | Taper Calculation Roll Diameter 1 | 1.0～6000.0 | 6000.0 | ★ | 0B43 |
| FB-68 | taper calculation roll diameter 2 | 1.0～6000.0 | 6000.0 | ★ | 0B44 |
| FB-69 | multi-segment taper value 1 | 0～100 | 0 | ☆ | 0B45 |
| FB-70 | multi-segment taper value 2 | 0～100 | 0 | ☆ | 0B46 |
| FB-71 | pre-drive frequency gain | -50.0%～50.0% | 0.0% | ☆ | 0B47 |
| FB-72 | pre-drive acceleration time | 0.00s～655.35s | 0.00s | ☆ | 0B48 |
| FB-73 | pre-drive deceleration time | 0.00s～655.35s | 0.00s | ☆ | 0B49 |
| FB-74 | speed limit gain | 0.0s～6553.5s | 100.0s | ☆ | 0B4A |
| FB-75 | soft start/PI switch LSErr | 0～100 | 5 | ★ | 0B4B |
| FB-76 | soft start/PI switch LS1 | 0.0%～10.0% | 0.4% | ★ | 0B4C |
| FB-77 | soft start/PI switch LS2 | 0.0%～10.0% | 0.7% | ★ | 0B4D |
| FB-78 | Soft Start/PI Switch 1 Setting | 0.0～40.0% | 2.0% | ★ | 0B4E |
| FB-79 | Unwinding Soft Start PI Switch Point | 0.0～40.0% | 5.0% | ★ | 0B4F |
| FB-80 | Tension Control Flag | 0000H~FFFFH | 0 | ☆ | 0B50 |
| FC Group - Position Control Parameters |  |  |  |  |  |
| FC-00 | P2P Position Mode | 0: Relative P2P Position Control 1: Absolute P2P Position Control | 0 | ★ | 0C00 |
| FC-01 | Return-to-Origin Mode Selection | 0000H~FFFFH | 0 | ☆ | 0C01 |
| FC-02 | Position Control Command Source | 0: External Terminal Input 1: Reserved 2: RS485 Communication 3: CANopen 4: Reserved 5: Communication Expansion Card | 0 | ☆ | 0C02 |
| FC-03 | First Segment Return-to-Origin Speed | 0.00Hz～599.00Hz | 8.00Hz | ☆ | 0C03 |
| FC-04 | Second Segment Return-to-Origin Speed | 0.00Hz～599.00Hz | 2.00Hz | ☆ | 0C04 |
| FC-05 | Machine tool application enable | 0: Off 1: Enable | 0 | ☆ | 0C05 |
| FC-06 | Error amount for searching Z point | 0～65535 | 1 | ☆ | 0C06 |
| FC-07 | Position control acceleration time | 0.00s～655.35s | 1.00s | ☆ | 0C07 |
| FC-08 | Position control deceleration time | 0.00s～655.35s | 3.00s | ☆ | 0C08 |
| FC-09 | APR first deceleration frequency | 0.00Hz～655.35Hz | 5.00Hz | ☆ | 0C09 |
| FC-10 | APR Second Deceleration Frequency | 0.00Hz～655.35Hz | 1.00Hz | ☆ | 0C0A |
| FC-11 | PG Disconnection Counter | 0～65535 | 0 | ☆ | 0C0B |
| FC-12 | Positioning Stiffness Adjustment Coefficient | 0.10～10.00 | 1.00 | ★ | 0C0C |
| FC-13 | Positioning Jitter Adjustment Coefficient | 0.00～1.20 | 3.00 | ★ | 0C0D |
| FC-14 | PG Low-Speed Filter | 0Hz～2000Hz | 1Hz | ☆ | 0C0E |
| FC-15 | Position Checkpoint 1 REV | -30000～30000 | 0 | ☆ | 0C0F |
| FC-16 | Position Checkpoint 1 CNT | -32767～32767 | 0 | ☆ | 0C10 |
| FC-17 | Position Checkpoint 2 REV | -30000～30000 | 0 | ☆ | 0C11 |
| FC-18 | Position Checkpoint 2 CNT | -32767～32767 | 0 | ☆ | 0C12 |
| FC-19 | Position Control Error | 0～65535 | 65535 | ☆ | 0C13 |
| FC-20 | Internal Positioning Command | -32767～32767 | 0 | ☆ | 0C14 |
| FC-21 | Feedback Position Arrival Error | 0～65535 | 10 | ☆ | 0C15 |
| FC-22 | Pulse Input Filter | 0.000～65.535 | 0.100 | ☆ | 0C16 |
| FC-23 | Pulse Speed Mode | 0: Electrical Frequency 1: Mechanical Frequency (Number of Pole Pairs) | 0 | ★ | 0C17 |
| FC-24 | Position Loop Proportional Coefficient | 0.00～40.00 | 8.00 | ☆ | 0C18 |
| FC-25 | Position Loop Feedforward Coefficient | 0～100 | 30 | ☆ | 0C19 |
| FC-26 | Position Curve Time | 0.00s～655.35s | 3.00s | ☆ | 0C1A |
| FC-27 | P2P Maximum Frequency | 0.00Hz～599.00Hz | 10.00Hz | ☆ | 0C1B |
| FC-28 | Multi-Segment Position 1 (revs) | -30000～30000 | 0 | ☆ | 0C1C |
| FC-29 | Multi-Segment Position 1 (pulse) | -32767～32767 | 0 | ☆ | 0C1D |
| FC-30 | Multi-segment Position 2 (revs) | -30000～30000 | 0 | ☆ | 0C1E |
| FC-31 | Multi-segment Position 2 (pulse) | -32767～32767 | 0 | ☆ | 0C1F |
| FC-32 | Multi-segment Position 3 (revs) | -30000～30000 | 0 | ☆ | 0C20 |
| FC-33 | Multi-segment Position 3 (pulse) | -32767～32767 | 0 | ☆ | 0C21 |
| FC-34 | Multi-segment Position 4 (revs) | -30000～30000 | 0 | ☆ | 0C22 |
| FC-35 | Multi-segment Position 4 (pulse) | -32767～32767 | 0 | ☆ | 0C23 |
| FC-36 | Multi-segment Position 5 (revs) | -30000～30000 | 0 | ☆ | 0C24 |
| FC-37 | Multi-segment Position 5 (pulse) | -32767～32767 | 0 | ☆ | 0C25 |
| FC-38 | Multi-segment Position 6 (revs) | -30000～30000 | 0 | ☆ | 0C26 |
| FC-39 | Multi-segment Position 6 (pulse) | -32767～32767 | 0 | ☆ | 0C27 |
| FC-40 | Multi-segment Position 7 (revs) | -30000～30000 | 0 | ☆ | 0C28 |
| FC-41 | Multi-segment Position 7 (pulse) | -32767～32767 | 0 | ☆ | 0C29 |
| FC-42 | Multi-segment Position 8 (revs) | -30000～30000 | 0 | ☆ | 0C2A |
| FC-43 | Multi-segment Position 8 (pulse) | -32767～32767 | 0 | ☆ | 0C2B |
| FC-44 | Multi-segment Position 9 (revs) | -30000～30000 | 0 | ☆ | 0C2C |
| FC-45 | Multi-segment Position 9 (pulse) | -32767～32767 | 0 | ☆ | 0C2D |
| FC-46 | Multi-segment Position 10 (revs) | -30000～30000 | 0 | ☆ | 0C2E |
| FC-47 | Multi-segment Position 10 (pulse) | -32767～32767 | 0 | ☆ | 0C2F |
| FC-48 | Multi-segment Position 11(revs) | -30000～30000 | 0 | ☆ | 0C30 |
| FC-49 | Multi-segment Position 11(pulse) | -32767～32767 | 0 | ☆ | 0C31 |
| FC-50 | Multi-segment Position 12(revs) | -30000～30000 | 0 | ☆ | 0C32 |
| FC-51 | Multi-segment Position 12(pulse) | -32767～32767 | 0 | ☆ | 0C33 |
| FC-52 | Multi-segment Position 13(revs) | -30000～30000 | 0 | ☆ | 0C34 |
| FC-53 | Multi-segment Position 13(pulse) | -32767～32767 | 0 | ☆ | 0C35 |
| FC-54 | Multi-segment Position 14(revs) | -30000～30000 | 0 | ☆ | 0C36 |
| FC-55 | Multi-segment Position 14(pulse) | -32767～32767 | 0 | ☆ | 0C37 |
| FC-56 | Multi-segment Position 15(revs) | -30000～30000 | 0 | ☆ | 0C38 |
| FC-57 | Multi-segment Position 15(pulse) | -32767～32767 | 0 | ☆ | 0C39 |
| FC-58 | Multi-segment Position0(revs) | -30000～30000 | 0 | ☆ | 0C40 |
| FC-59 | Multi-segment Position0(pulse) | -32767～32767 | 0 | ☆ | 0C41 |
| FD Group - Multi-speed and Simple PLC Function |  |  |  |  |  |
| FD-00 | Multi-Speed 0 | -100.00～100.00% | 0.00% | ☆ | 0D00 |
| FD-01 | Multi-Speed 1 | -100.00～100.00% | 0.00% | ☆ | 0D01 |
| FD-02 | Multi-Speed 2 | -100.00～100.00% | 0.00% | ☆ | 0D02 |
| FD-03 | Multi-Speed 3 | -100.00～100.00% | 0.00% | ☆ | 0D03 |
| FD-04 | Multi-Speed 4 | -100.00～100.00% | 0.00% | ☆ | 0D04 |
| FD-05 | Multi-Speed 5 | -100.00～100.00% | 0.00% | ☆ | 0D05 |
| FD-06 | Multi-speed 6 | -100.00～100.00% | 0.00% | ☆ | 0D06 |
| FD-07 | Multi-speed 7 | -100.00～100.00% | 0.00% | ☆ | 0D07 |
| FD-08 | Multi-speed 8 | -100.00～100.00% | 0.00% | ☆ | 0D08 |
| FD-09 | Multi-speed 9 | -100.00～100.00% | 0.00% | ☆ | 0D09 |
| FD-10 | Multi-speed 10 | -100.00～100.00% | 0.00% | ☆ | 0D0A |
| FD-11 | Multi-speed 11 | -100.00～100.00% | 0.00% | ☆ | 0D0B |
| FD-12 | Multi-Speed 12 | -100.00～100.00% | 0.00% | ☆ | 0D0C |
| FD-13 | Multi-Speed 13 | -100.00～100.00% | 0.00% | ☆ | 0D0D |
| FD-14 | Multi-Speed 14 | -100.00～100.00% | 0.00% | ☆ | 0D0E |
| FD-15 | Multi-Speed 15 | -100.00～100.00% | 0.00% | ☆ | 0D0F |
| FD-16 | PLC Operation Mode | 0: Single Run Stop at End 1: Single Run Save at End 2: Continuous Run | 0 | ☆ | 0D10 |
| FD-17 | PLC Power Failure Memory Selection | 0: No Memory on Power Failure/Shutdown 1: Memory on Power Failure 2: Memory on Shutdown 3: Memory on Power Failure/Shutdown | 0 | ☆ | 0D11 |
| FD-18 | PLC Operation Time Unit | 0: Second 1: Hour | 0 | ☆ | 0D12 |
| FD-19 | Multi-Speed 0 Command Setting | 0: Function Code FD-00 Setting 1: AI 2: VR 3: Function Code F0-19 Given | 0 | ☆ | 0D13 |
| FD-20 | PLC Segment 00 Running Time | 0.0~6553.5 | 0.0 | ☆ | 0D14 |
| FD-21 | PLC Segment 00 Acceleration/Deceleration Settings | 0~3 | 0 | ☆ | 0D15 |
| FD-22 | PLC Segment 01 Running Time | 0.0~6553.5 | 0.0 | ☆ | 0D16 |
| FD-23 | PLC Segment 01 Acceleration/Deceleration Settings | 0~3 | 0 | ☆ | 0D17 |
| FD-24 | PLC Segment 02 Running Time | 0.0~6553.5 | 0.0 | ☆ | 0D18 |
| FD-25 | PLC Segment 02 Acceleration/Deceleration Settings | 0~3 | 0 | ☆ | 0D19 |
| FD-26 | PLC Segment 03 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D1A |
| FD-27 | PLC Segment 03 Acceleration/Deceleration Settings | 0~3 | 0 | ☆ | 0D1B |
| FD-28 | PLC Segment 04 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D1C |
| FD-29 | PLC Segment 04 Acceleration/Deceleration Settings | 0~3 | 0 | ☆ | 0D1D |
| FD-30 | PLC Segment 05 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D1E |
| FD-31 | PLC Segment 05 Acceleration/Deceleration Settings | 0~3 | 0 | ☆ | 0D1F |
| FD-32 | PLC Segment 06 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D20 |
| FD-33 | PLC Segment 06 Acceleration/Deceleration Settings | 0~3 | 0 | ☆ | 0D21 |
| FD-34 | PLC Segment 07 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D22 |
| FD-35 | PLC Segment 07 Acceleration/Deceleration Settings | 0~3 | 0 | ☆ | 0D23 |
| FD-36 | PLC Segment 08 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D24 |
| FD-37 | PLC Segment 08 Acceleration/Deceleration Setting | 0~3 | 0 | ☆ | 0D25 |
| FD-38 | PLC Segment 09 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D26 |
| FD-39 | PLC Segment 09 Acceleration/Deceleration Setting | 0~3 | 0 | ☆ | 0D27 |
| FD-40 | PLC Segment 10 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D28 |
| FD-41 | PLC Segment 10 Acceleration/Deceleration Setting | 0~3 | 0 | ☆ | 0D29 |
| FD-42 | PLC Segment 11 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D2A |
| FD-43 | PLC Segment 11 Acceleration/Deceleration Setting | 0~3 | 0 | ☆ | 0D2B |
| FD-44 | PLC Segment 12 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D2C |
| FD-45 | PLC Segment 12 Acceleration/Deceleration Setting | 0~3 | 0 | ☆ | 0D2D |
| FD-46 | PLC Segment 13 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D2E |
| FD-47 | PLC Segment 13 Acceleration/Deceleration Setting | 0~3 | 0 | ☆ | 0D2F |
| FD-48 | PLC Segment 14 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D30 |
| FD-49 | PLC Segment 14 Acceleration/Deceleration Setting | 0~3 | 0 | ☆ | 0D31 |
| FD-50 | PLC Segment 15 Operation Time | 0.0~6553.5 | 0.0 | ☆ | 0D32 |
| FD-51 | PLC Segment 15 Acceleration/Deceleration Setting | 0~3 | 0 | ☆ | 0D33 |
| FD-52 | PLC Current Work Step | 0~15 | 0 | ● | 0D34 |
| FD-53 | PLC Operation Time High Position | 0000H~FFFFH | 0 | ● | 0D35 |
| FD-54 | PLC runtime low position | 0000H~FFFFH | 0 | ● | 0D36 |
| FE group - torque control parameters |  |  |  |  |  |
| FE-00 | Torque mode selection | 0: TQCPG torque control IM 1: TQCPG torque control PM 2: IM open-loop torque control 3: SVC open-loop torque control | 0 | ★ | 0E00 |
| FE-01 | Torque command source selection | 0: Digital Operator 1: RS485 Communication 2: Analog signal input 3: CANopen 4: Reserved 5: Communication Expansion Card | 0 | ☆ | 0E01 |
| FE-02 | Torque digital setting | -100.0%～100.0% | 0.0% | ☆ | 0E02 |
| FE-03 | Forward Torque Frequency Limit | 0%～120% | 110% | ☆ | 0E03 |
| FE-04 | Reverse Torque Frequency Limit | 0%～120% | 110% | ☆ | 0E04 |
| FE-05 | Torque Bias Selection | 0: No Function 1: Analog Input 2: Parameter PE-06 Input 3: External Terminal Control | 0 | ☆ | 0E05 |
| FE-06 | Torque Bias Value | -100.0%～100.0% | 0.0% | ☆ | 0E06 |
| FE-07 | High Torque Compensation | -100.0%～100.0% | 30.0% | ☆ | 0E07 |
| FE-08 | Intermediate torque compensation | -100.0%～100.0% | 20.0% | ☆ | 0E08 |
| FE-09 | Low torque compensation | -100.0%～100.0% | 10.0% | ☆ | 0E09 |
| FE-10 | Maximum torque command | 0～500 | 100 | ☆ | 0E0A |
| FE-11 | Forward Motor Torque Upper Limit | 0～500 | 500 | ● | 0E0B |
| FE-12 | Forward Braking Torque Upper Limit | 0～500 | 500 | ● | 0E0C |
| FE-13 | Reverse Electric Torque Upper Limit | 0～500 | 500 | ● | 0E0D |
| FE-14 | Reverse Braking Torque Upper Limit | 0～500 | 500 | ● | 0E0E |
| FE-15 | Torque Filter Time | 0.000s～1.000s | 0.000s | ☆ | 0E0F |
| FE-16 | Torque Output Filter Gain | 0.000s～65.535s | 0.200s | ☆ | 0E10 |
| FE-17 | Zero Torque Mode Selection | 0: Torque Mode 1: Speed Mode | 0 | ★ | 0E11 |
| FF Group - Manufacturer Parameters |  |  |  |  |  |
| FF-00 | Reserved | 0～65535 | 0 | ○ | 0F00 |
| U0 Group - Fault Record Parameters |  |  |  |  |  |
| U0-00 | Fault Record 1 | 0～65535 | 0 | ● | 1000 |
| U0-01 | Fault Record 2 | 0～65535 | 0 | ● | 1001 |
| U0-02 | Fault Record 3 | 0～65535 | 0 | ● | 1002 |
| U0-03 | Fault Record 4 | 0～65535 | 0 | ● | 1003 |
| U0-04 | Fault Record 5 | 0～65535 | 0 | ● | 1004 |
| U0-05 | Fault Record 6 | 0～65535 | 0 | ● | 1005 |
| U0-06 | Fault Record 7 | 0～65535 | 0 | ● | 1006 |
| U0-07 | Fault Record 8 | 0～65535 | 0 | ● | 1007 |
| U0-08 | Fault Record 9 | 0～65535 | 0 | ● | 1008 |
| U0-09 | Fault Record 10 | 0～65535 | 0 | ● | 1009 |
| U0-10 | Fault Output 1 | 0～65535 | 0 | ☆ | 100A |
| U0-11 | Fault Output 2 | 0～65535 | 0 | ☆ | 100B |
| U0-12 | Fault Output 3 | 0～65535 | 0 | ☆ | 100C |
| U0-13 | Fault output 4 | 0～65535 | 0 | ☆ | 100D |
| U0-14 | Fault 1 - Motor speed | -32767rpm～32767rpm | 0rpm | ● | 100E |
| U0-15 | Fault 1 - Torque command | -3276.7～3276.7 | 0.0 | ● | 100F |
| U0-16 | Fault 1 - Input terminals | 0000H~FFFFH | 0 | ● | 1010 |
| U0-17 | Fault 1 - Output terminals | 0000H~FFFFH | 0 | ● | 1011 |
| U0-18 | Fault 1 - Inverter Status | 0～65535 | 0 | ● | 1012 |
| U0-19 | Fault 1 - Frequency Command Hz | 0.00Hz～655.35Hz | 0.00Hz | ● | 1013 |
| U0-20 | Fault 1 - Output Frequency | 0.00Hz～599.00Hz | 0.00Hz | ● | 1014 |
| U0-21 | Fault 1 - Output Voltage | 0.0V～6553.5V | 0.0V | ● | 1015 |
| U0-22 | Fault 1 - DC Voltage | 0.0V～6553.5V | 0.0V | ● | 1016 |
| U0-23 | Fault 1-Output Current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | 0.00A or 0.0A | ● | 1017 |
| U0-24 | Fault 1-IGBT Temperature | -3276.7°C to 3276.7°C | 0.0℃ | ● | 1018 |
| U0-25 | Fault 1-Capacitor Temperature | -3276.7°C to 3276.7°C | 0.0℃ | ● | 1019 |
| U0-26 | Fault 2-Output Frequency | 0.00Hz～599.00Hz | 0.00Hz | ● | 101A |
| U0-27 | Fault 2-Direct Current Voltage | 0.0V～6553.5V | 0.0V | ● | 101B |
| U0-28 | Fault 2-Output Current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | 0.00A or 0.0A | ● | 101C |
| U0-29 | Fault 2 - IGBT Temperature | -3276.7°C to 3276.7°C | 0.0℃ | ● | 101D |
| U0-30 | Fault 3 - Output Frequency | 0.00Hz～599.00Hz | 0.00Hz | ● | 101E |
| U0-31 | Fault 3 - DC Voltage | 0.0V～6553.5V | 0.0V | ● | 101F |
| U0-32 | Fault 3 - Output Current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | 0.00A or 0.0A | ● | 1020 |
| U0-33 | Fault 3 - IGBT Temperature | -3276.7°C to 3276.7°C | 0.0℃ | ● | 1021 |
| U0-34 | Fault 4 - Output Frequency | 0.00Hz～599.00Hz | 0.00Hz | ● | 1022 |
| U0-35 | Fault 4-Direct Current Voltage | 0.0V～6553.5V | 0.0V | ● | 1023 |
| U0-36 | Fault 4-Output Current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | 0.00A or 0.0A | ● | 1024 |
| U0-37 | Fault 4-IGBT Temperature | -3276.7°C to 3276.7°C | 0.0℃ | ● | 1025 |
| U0-38 | Fault 5-Output Frequency | 0.00Hz～599.00Hz | 0.00Hz | ● | 1026 |
| U0-39 | Fault 5-Direct Current Voltage | 0.0V～6553.5V | 0.0V | ● | 1027 |
| U0-40 | Fault 5-Output Current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | 0.00A or 0.0A | ● | 1028 |
| U0-41 | Fault 5-IGBT Temperature | -3276.7°C to 3276.7°C | 0.0℃ | ● | 1029 |
| U0-42 | Fault 6-Output Frequency | 0.00Hz～599.00Hz | 0.00Hz | ● | 102A |
| U0-43 | Fault 6-Direct Current Voltage | 0.0V～6553.5V | 0.0V | ● | 102B |
| U0-44 | Fault 6-Output Current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | 0.00A or 0.0A | ● | 102C |
| U0-45 | Fault 6-IGBT Temperature | -3276.7°C to 3276.7°C | 0.0℃ | ● | 102D |
| U0-46 | Number of Days Since Fault 1 Occurred | 0～65535 | 0 | ● | 102E |
| U0-47 | Fault 1 Occurrence Minutes | 0～1439 | 0 | ● | 102F |
| U0-48 | Fault 2 Occurrence Days | 0～65535 | 0 | ● | 1030 |
| U0-49 | Fault 2 Occurrence Minutes | 0～1439 | 0 | ● | 1031 |
| U0-50 | Fault 3 Occurrence Days | 0～65535 | 0 | ● | 1032 |
| U0-51 | Fault 3 Occurrence Minutes | 0～1439 | 0 | ● | 1033 |
| U0-52 | Fault 4 Occurrence Days | 0～65535 | 0 | ● | 1034 |
| U0-53 | Minutes since Fault 4 occurred | 0～1439 | 0 | ● | 1035 |
| U0-54 | Days since Fault 5 occurred | 0～65535 | 0 | ● | 1036 |
| U0-55 | Minutes since Fault 5 occurred | 0～1439 | 0 | ● | 1037 |
| U0-56 | Days since Fault 6 occurred | 0～65535 | 0 | ● | 1038 |
| U0-57 | Minutes since Fault 6 occurred | 0～1439 | 0 | ● | 1039 |
| U1 Group - Status Monitoring Parameters |  |  |  |  |  |
| U1-00 | DI terminal status | 0000H~FFFFH | 0 | ● | 1100 |
| U1-01 | DO terminal status | 0000H~FFFFH | 0 | ● | 1101 |
| U1-02 | Set frequency | 0.00Hz～599.00Hz | 50.00Hz | ● | 1102 |
| U1-03 | External frequency record | 0.00Hz～599.00Hz | 60.00Hz | ● | 1103 |
| U1-04 | PID feedback value | -200.00～200.00% | 0.00% | ● | 1104 |
| U1-05 | KP gain monitoring value | 0.00%～100.00% | 88.00% | ● | 1105 |
| U1-06 | KI Gain Monitoring Value | 0.00s～100.00s | 0.05s | ● | 1106 |
| U1-07 | KD Gain Monitoring Value | 0.00～1.00 | 0.00 | ● | 1107 |
| U1-08 | Model Change Record | 0～65535 | 0 | ☆ | 1108 |
| U1-09 | IO Card ID | 0～13 | 0 | ● | 1109 |
| H0 Group - Other Motor Parameters |  |  |  |  |  |
| H0-00 | IM Motor Selection | 1: Induction Motor 1 2: Induction Motor 2 3: Induction Motor 3 4: Induction Motor 4 | 1 | ★ | 1200 |
| H0-01 | M2 Maximum Frequency | 0.00Hz～599.00Hz | 50.00Hz | ★ | 1201 |
| H0-02 | M2 Rated Frequency | 0.00Hz～599.00Hz | 50.00Hz | ★ | 1202 |
| H0-03 | M2 Rated Voltage | 0.0～510.0V | 380.0 | ★ | 1203 |
| H0-04 | IM2 Rated Power | 0.00kW～655.35kW | Model Confirmation | ★ | 1204 |
| H0-05 | IM2 Number of Poles | 2～20 | Model Confirmation | ● | 1205 |
| H0-06 | IM2 Rated Current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | H0-04 Confirm | ★ | 1206 |
| H0-07 | IM2 Rated Speed | 0rpm～65535rpm | H0-04 Confirm | ★ | 1207 |
| H0-08 | IM2 No-load Current | 0.00A～H0-06 | Model Confirmation | ★ | 1208 |
| H0-09 | IM2 Stator Resistance | 0.000Ω～65.535Ω | Model Confirmation | ★ | 1209 |
| H0-10 | IM2 Rotor Resistance | 0.000Ω～65.535Ω | Model Confirmation | ★ | 120A |
| H0-11 | IM2 Mutual Inductance | 0.0mH～6553.5mH | Model Confirmation | ★ | 120B |
| H0-12 | IM2 Leakage Inductance | 0.00mH～655.35mH | Model Confirmation | ★ | 120C |
| H0-13 | M3 Maximum Frequency | 0.00Hz～599.00Hz | 50.00Hz | ★ | 120D |
| H0-14 | M3 Rated Frequency | 0.00Hz～599.00Hz | 50.00Hz | ★ | 120E |
| H0-15 | M3 Rated Voltage | 0.0V～510.0V | 380.0V | ★ | 120F |
| H0-16 | IM3 rated power | 0.00kW～655.35kW | Model Confirmation | ★ | 1210 |
| H0-17 | IM3 number of poles | 2～20 | Model Confirmation | ● | 1211 |
| H0-18 | IM3 rated current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | H0-16 confirm | ★ | 1212 |
| H0-19 | IM3 rated speed | 0rpm～65535rpm | H0-16 confirm | ★ | 1213 |
| H0-20 | IM3 no-load current | 0.00A～H0-18 | Model Confirmation | ★ | 1214 |
| H0-21 | IM3 stator resistance | 0.000Ω～65.535Ω | Model Confirmation | ★ | 1215 |
| H0-22 | M4 Maximum Frequency | 0.00Hz～599.00Hz | 50.00Hz | ★ | 1216 |
| H0-23 | M4 Rated Frequency | 0.00Hz～599.00Hz | 50.00Hz | ★ | 1217 |
| H0-24 | M4 Rated Voltage | 0.0V～510.0V | 380.0V | ★ | 1218 |
| H0-25 | IM4 Rated Power | 0.00kW～655.35kW | Model Confirmation | ★ | 1219 |
| H0-26 | IM4 Number of Poles | 2～20 | Model Confirmation | ● | 121A |
| H0-27 | IM4 Rated Current | 0.00A～655.35A (93kW and below) 0.0A～6553.5A (110kW and above) | H0-25 Confirm | ★ | 121B |
| H0-28 | IM4 Rated Speed | 0rpm～65535rpm | H0-25 Confirm | ★ | 121C |
| H0-29 | IM4 No-load Current | 0.00A～H0-27 | Model Confirmation | ★ | 121D |
| H0-30 | IM4 Stator Resistance | 0.000Ω～65.535Ω | Model Confirmation | ★ | 121E |
| H1 Group - Other Motor VF Control |  |  |  |  |  |
| H1-00 | M2 Multi-point VF Frequency Point 1 | 0.00Hz～599.00Hz | 0.50Hz | ★ | 1300 |
| H1-01 | M2 Multi-point VF Voltage Point 1 | 0.0V～480.0V | 2.0V | ☆ | 1301 |
| H1-02 | M2 Multi-point VF Frequency Point 2 | 0.00Hz～599.00Hz | 1.50Hz | ★ | 1302 |
| H1-03 | M2 Multi-point VF Voltage Point 2 | 0.0V～480.0V | 10.0V | ☆ | 1303 |
| H1-04 | M2 Multi-point VF Frequency Point 3 | 0.00Hz～599.00Hz | 3.00Hz | ★ | 1304 |
| H1-05 | M2 Multi-point VF Voltage Point 3 | 0.0V～480.0V | 22.0V | ☆ | 1305 |
| H1-06 | M2 Torque Compensation Gain | 0～10 | 1 | ☆ | 1306 |
| H1-07 | M2 Slip Compensation Gain | 0.00～10.00 | 0.00 | ☆ | 1307 |
| H1-08 | M3 Multi-point VF Frequency Point 1 | 0.00Hz～599.00Hz | 0.50Hz | ★ | 1308 |
| H1-09 | M3 Multi-point VF Voltage Point 1 | 0.0V～480.0V | 2.0V | ☆ | 1309 |
| H1-10 | M3 Multi-point VF Frequency Point 2 | 0.00Hz～599.00Hz | 1.50Hz | ★ | 130A |
| H1-11 | M3 Multi-point VF Voltage Point 2 | 0.0V～480.0V | 10.0V | ☆ | 130B |
| H1-12 | M3 Multi-point VF Frequency Point 3 | 0.00Hz～599.00Hz | 3.00Hz | ★ | 130C |
| H1-13 | M3 Multi-point VF Voltage Point 3 | 0.0V～480.0V | 22.0V | ☆ | 130D |
| H1-14 | M3 Torque Compensation Gain | 0～10 | 1 | ☆ | 130E |
| H1-15 | M3 Slip Compensation Gain | 0.00～10.00 | 0.00 | ☆ | 130F |
| H1-16 | M4 Multi-point VF Frequency Point 1 | 0.00Hz～599.00Hz | 0.50Hz | ★ | 1310 |
| H1-17 | M4 Multi-point VF Voltage Point 1 | 0.0V～480.0V | 2.0V | ☆ | 1311 |
| H1-18 | M4 Multi-point VF Frequency Point 2 | 0.00Hz～599.00Hz | 1.50Hz | ★ | 1312 |
| H1-19 | M4 Multi-point VF Voltage Point 2 | 0.0V～480.0V | 10.0V | ☆ | 1313 |
| H1-20 | M4 Multi-point VF Frequency Point 3 | 0.00Hz～599.00Hz | 3.00Hz | ★ | 1314 |
| H1-21 | M4 Multi-point VF Voltage Point 3 | 0.0V～480.0V | 22.0V | ☆ | 1315 |
| H1-22 | M4 Torque Compensation Gain | 0～10 | 1 | ☆ | 1316 |
| H1-23 | M4 Slip Compensation Gain | 0.00～10.00 | 0.00 | ☆ | 1317 |
| H2 Group - Other Motor Vector Control Parameters |  |  |  |  |  |
| H2-00 | Reserved | 0～65535 | 0 | ● | 1400 |
| H3 Group - Other Motor Fault Parameters |  |  |  |  |  |
| H3-00 | Over-torque selection 2 | 0: Not Detected 1: Constant Speed Detection Continue Running 2: Constant Speed Detection Stop Running 3: Running Detection Continue Running 4: Running Detection Stop Running | 0 | ☆ | 1500 |
| H3-01 | Over-torque threshold 2 | 10%～250% | 120% | ☆ | 1501 |
| H3-02 | Over-torque time 2 | 0.1s～60.0s | 0.1s | ☆ | 1502 |
| H3-03 | Motor 2 overload selection | 0: Constant Torque Output Motor 1: Variable Torque Output Motor 2: No Motor Overload Protection | 2 | ☆ | 1503 |
| H3-04 | Motor 2 overload time | 30.0s～600.0s | 60.0s | ☆ | 1504 |
| H3-05 | Over-torque selection 3 | 0: Not Detected 1: Constant Speed Detection Continue Running 2: Constant Speed Detection Stop Running 3: Running Detection Continue Running 4: Running Detection Stop Running | 0 | ☆ | 1505 |
| H3-06 | Over-torque threshold 3 | 10%～250% | 120% | ☆ | 1506 |
| H3-07 | Over-torque time 3 | 0.1s～60.0s | 0.1s | ☆ | 1507 |
| H3-08 | Motor 3 overload selection | 0: Constant Torque Output Motor 1: Variable Torque Output Motor 2: No Motor Overload Protection | 2 | ☆ | 1508 |
| H3-09 | Motor 3 overload time | 30.0s～600.0s | 60.0s | ☆ | 1509 |
| H3-10 | Over-torque selection 4 | 0: Not Detected 1: Constant Speed Detection Continue Running 2: Constant Speed Detection Stop Running 3: Running Detection Continue Running 4: Running Detection Stop Running | 0 | ☆ | 150A |
| H3-11 | Over-torque threshold 4 | 10%～250% | 120% | ☆ | 150B |
| H3-12 | Over Torque Time 4 | 30.0s～600.0s | 0.1s | ☆ | 150C |
| H3-13 | Motor 4 Overload Selection | 0: Constant Torque Output Motor 1: Variable Torque Output Motor 2: No Motor Overload Protection | 2 | ☆ | 150D |
| H3-14 | Motor 4 Overload Time | 30.0s～600.0s | 60.0s | ☆ | 150E |
| L0 Group - System Control Parameters |  |  |  |  |  |
| L0-00 | APP MACRO | 0～65535 | 0 | ☆ | 1600 |
| L0-01 | System Control Flag | 0～65535 | 0 | ☆ | 1601 |
| L0-02 | Debugging Flag | 0～65535 | 0 | ☆ | 1602 |
| L0-03 | Debug Flag 1 | 0～65535 | 64 | ☆ | 1603 |
| L0-04 | Reserved | 0～65535 | 0 | ☆ | 1604 |
| L0-05 | System Control Parameters | 0～65535 | 0 | ☆ | 1605 |
| L0-06 | Special Control Flags | 0～65535 | 0 | ☆ | 1606 |
| L0-07 | Reserved | 0～65535 | 0 | ★ | 1607 |
| L0-08 | Reserved | 0～65535 | 0 | ☆ | 1608 |
| L0-09 | KPD Automatic Screen Refresh Enable | 0～1 | 0 | ☆ | 1609 |
| L0-10 | KPD Screen Refresh Time Setting | 0.00s～6.00s | 0.15s | ☆ | 160A |
| L0-11 | Low Speed Debugging Position | 0～65535 | 1 | ☆ | 160B |
| L1 Group - User Function Code Customization |  |  |  |  |  |
| L1-00 | Frequency Source Selection (HAND) | 0: Digital Operator 1: RS485 Communication 2: Analog Input 3: External Up/Down Input 4: Pulse Input Without Direction 5: Pulse Input With Direction 6: CANopen Input 7: Reserved 8: Communication Card Input 9: PID | 0 | ★ | 1700 |
| L1-01 | Operation Command Source (HAND) | 0: Digital Operator 1: External Terminal Input 2: RS485 Communication Input 3: CANopen Input 4: Reserved 5: Communication Card Input | 0 | ★ | 1701 |
| L1-02 | LOC/REM Action Selection | 0: Standard HOA Operation 1: L/R Does Not Maintain State 2: Maintain REM State 3: Maintain LOC State 4: Maintain REM/LOC State | 0 | ★ | 1702 |
| L2 Group - Optimized Control Parameters |  |  |  |  |  |
| L2-00 | PWM Mode Selection | 0: DPWM Modulation Mode 1: Reserved 2: SVPWM Modulation Mode | 2 | ★ | 1800 |
| L2-01 | PWM Seven/Five Segment Switching Point | 0.00Hz～655.35Hz | 15.00Hz | ☆ | 1801 |
| L2-02 | PWM Mode Selection | 0: SVPWM 66% DPWM 1 1: SVPWM 100% 2: SVPWM 66% 3: SPWM-DPWM 100% 4: SVPWM-DPWM 100% 5: SPWM 100% 6: SVPWM 66%-100% | 0 | ★ | 1802 |
| L2-03 | PWM dead time value | 0～666 | 133 | ☆ | 1803 |
| L2-04 | PWM narrow pulse width | 0～333 | 66 | ☆ | 1804 |
| L2-05 | Reserved | 0～1440 | 180 | ☆ | 1805 |
| L2-06 | Dead Time Compensation Mode | 0～65535 | 5 | ☆ | 1806 |
| L2-07 | Dead Time Compensation Value | 0～65535 | 180 | ☆ | 1807 |
| L2-08 | Tube Voltage Drop Compensation Value | 0～65535 | 5 | ☆ | 1808 |
| L2-09 | Dead Time Compensation 4 Slope | 0～65535 | 200 | ☆ | 1809 |
| L2-10 | Dead Time Compensation 5 Slope | 0～65535 | 5000 | ☆ | 180A |
| L2-11 | Dead-time Compensation Bias Value | -100.0%～100.0% | 0.0% | ☆ | 180B |
| L2-12 | Dead-time Compensation Width | 0～65535 | 512 | ☆ | 180C |
| L2-13 | Dead-time Compensation Switching Point | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 180D |
| L2-14 | Dead-time Compensation Filter | 0～65535 | 50000 | ☆ | 180E |
| L2-15 | Dead-time Compensation Identification Id | 2000pu～65535pu | 8000pu | ☆ | 180F |
| L2-17 | BrakingUnitStart Voltage | 700.0V～900.0V | 740.0V | ☆ | 1811 |
| L2-18 | Undervoltage Protection Value | 250.0V～440.0V | 360.0V | ☆ | 1812 |
| L2-19 | Zero Speed Operation Selection | 0: Wait for Output 1: Zero Speed Position Control Output 2: Output at Minimum Frequency | 0 | ★ | 1813 |
| L2-20 | Reserved | 0～65535 | 0 | ● | 1814 |
| L2-21 | Reserved | 0～65535 | 0 | ● | 1815 |
| L2-22 | Random PWM | 0: Disable 1: Enable | 0 | ★ | 1816 |
| L2-23 | Overmodulation Gain | 80～120 | 100 | ☆ | 1817 |
| L2-24 | Back EMF Identification Frequency | 0～1000% | 50 | ● | 1818 |
| L3 Group - Master/Slave Control Parameters |  |  |  |  |  |
| L3-00 | Reserved | 0～65535 | 0 | ● | 1900 |
| L4 Group - Brake Function Parameters |  |  |  |  |  |
| L4-00 | Brake Frequency | 0.00～599.00Hz | 0.00 | ☆ | 1A00 |
| L4-01 | Brake Control Enable Selection | 0～1 | 0 | ☆ | 1A01 |
| L4-02 | Brake Release Frequency | 0.00～20.00Hz | 2.50 | ☆ | 1A02 |
| L4-03 | Brake Release Frequency Maintenance | 0.0～20.0 seconds | 1.0 | ☆ | 1A03 |
| L4-04 | Brake Current Limit Value | 50～200% | 120 | ☆ | 1A04 |
| L4-05 | Brake Close Frequency | 0～20.00Hz | 1.50 | ☆ | 1A05 |
| L4-06 | Brake Close Delay Time | 0～20.0s | 0.0 | ☆ | 1A06 |
| L4-07 | Brake Close Frequency Maintenance | 0～20.0s | 1.0 | ☆ | 1A07 |
| L5 Group - Sleep Wake Function Parameters |  |  |  |  |  |
| L5-00 | Sleep Method Reference Selection | 0: PID Command Arrival 1: PID Feedback Arrival | 0 | ★ | 1B00 |
| L5-01 | Sleep Threshold | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 1B01 |
| L5-02 | Wake Threshold | 0.00Hz～599.00Hz | 0.00Hz | ☆ | 1B02 |
| L5-03 | Sleep Delay | 0.0s～6000.0s | 0.0s | ☆ | 1B03 |
| L5-04 | Wake Delay | 0.00s～600.00s | 0.00s | ☆ | 1B04 |
| L6 Group - Frequency Swinging, Fixed Length, and Counting |  |  |  |  |  |
| L6-00 | Final Count Value Setting | 0～65500 | 0 | ☆ | 1C00 |
| L6-01 | Intermediate Count Value Setting | 0～65500 | 0 | ☆ | 1C01 |
| L6-02 | Count Reached E.F Enable | 0: Count Reached, No EF 1: Count Reached, EF | 0 | ☆ | 1C02 |
| L7 Group - AI Multi-point Curve Setting |  |  |  |  |  |
| L7-00 | AI Curve Selection | 0: General Curve 1: AI1 Three-point Curve 2: AI2 Three-point Curve 3: AI1&AI2 Three-point Curve 4: AI3 Three-point Curve 5: AI1&AI3 Three-point Curve 6: AI2&AI3 Three-point Curve 7: AI123 Three-point Curve | 0 | ☆ | 1D00 |
| L7-01 | AI1 Minimum Point Input Value | 0.00～20.00 | 0.00 | ☆ | 1D01 |
| L7-02 | AI1 Minimum Point Percentage | 0.00%～100.00% | 0.00% | ☆ | 1D02 |
| L7-03 | AI1 Midpoint Input Value | 0.00～20.00 | 5.00 | ☆ | 1D03 |
| L7-04 | AI1 Midpoint Percentage | 0.00%～100.00% | 50.00% | ☆ | 1D04 |
| L7-05 | AI1 Maximum Point Input Value | 0.00～20.00 | 10.00 | ☆ | 1D05 |
| L7-06 | AI1 Maximum Point Percentage | 0.00%～100.00% | 100.00% | ☆ | 1D06 |
| L7-07 | AI2 Minimum Point Input Value | 0.00～20.00 | 0.00 | ☆ | 1D07 |
| L7-08 | AI2 Minimum Point Percentage | 0.00%～100.00% | 0.00% | ☆ | 1D08 |
| L7-09 | AI2 Midpoint Input Value | 0.00～20.00 | 5.00 | ☆ | 1D09 |
| L7-10 | AI2 Midpoint Percentage | 0.00%～100.00% | 50.00% | ☆ | 1D0A |
| L7-11 | AI2 Maximum Point Input Value | 0.00～20.00 | 10.00 | ☆ | 1D0B |
| L7-12 | AI2 Maximum Point Percentage | 0.00%～100.00% | 100.00% | ☆ | 1D0C |
| L7-13 | AI3 Minimum Point Input Value | 0.00～20.00 | 0.00 | ☆ | 1D0D |
| L7-14 | AI3 Minimum Point Percentage | 0.00%～100.00% | 0.00% | ☆ | 1D0E |
| L7-15 | AI3 Midpoint Input Value | 0.00～20.00 | 5.00 | ☆ | 1D0F |
| L7-16 | AI3 Intermediate Point Percentage | 0.00%～100.00% | 50.00% | ☆ | 1D10 |
| L7-17 | AI3 Maximum Point Input Value | 0.00～20.00 | 10.00 | ☆ | 1D11 |
| L7-18 | AI3 Maximum Point Percentage | 0.00%～100.00% | 100.00% | ☆ | 1D12 |
| L8 Group - Application Macro Parameters |  |  |  |  |  |
| L8-00 | Industry Application Macro Selection | 0: No Function 1: User Defined 2: Air Compressor 3: Fan 4: Water Pump 5: Conveyor Belt 6: Machine Tool Application 7: Packaging 8: Textile Application 9: High-Speed Drill Application 10:Production Testing Dedicated 11: PID 12: PID + Auxiliary Frequency | 0 | ★ | 1E00 |
| L8-01 | Application Macro Parameter 1 | 0.00～29.00 | 0.00 | ☆ | 1E01 |
| L8-02 | Application Macro Parameter 2 | 0.00～29.00 | 0.00 | ☆ | 1E02 |
| L8-03 | Application Macro Parameter 3 | 0.00～29.00 | 0.00 | ☆ | 1E03 |
| L8-04 | Application Macro Parameter 4 | 0.00～29.00 | 0.00 | ☆ | 1E04 |
| L8-05 | Application Macro Parameter 5 | 0.00～29.00 | 0.00 | ☆ | 1E05 |
| L8-06 | Application Macro Parameter 6 | 0.00～29.00 | 0.00 | ☆ | 1E06 |
| L8-07 | Application Macro Parameter 7 | 0.00～29.00 | 0.00 | ☆ | 1E07 |
| L8-08 | Application Macro Parameter 8 | 0.00～29.00 | 0.00 | ☆ | 1E08 |
| L8-09 | Application Macro Parameter 9 | 0.00～29.00 | 0.00 | ☆ | 1E09 |
| L8-10 | Application Macro Parameter 10 | 0.00～29.00 | 0.00 | ☆ | 1E0A |
| L8-11 | Application Macro Parameter 11 | 0.00～29.00 | 0.00 | ☆ | 1E0B |
| L8-12 | Application Macro Parameter 12 | 0.00～29.00 | 0.00 | ☆ | 1E0C |
| L8-13 | Application Macro Parameter 13 | 0.00～29.00 | 0.00 | ☆ | 1E0D |
| L8-14 | Application Macro Parameter 14 | 0.00～29.00 | 0.00 | ☆ | 1E0E |
| L8-15 | Application Macro Parameter 15 | 0.00～29.00 | 0.00 | ☆ | 1E0F |
| L8-16 | Application Macro Parameter 16 | 0.00～29.00 | 0.00 | ☆ | 1E10 |
| L8-17 | Application Macro Parameter 17 | 0.00～29.00 | 0.00 | ☆ | 1E11 |
| L8-18 | Application Macro Parameter 18 | 0.00～29.00 | 0.00 | ☆ | 1E12 |
| L8-19 | Application Macro Parameter 19 | 0.00～29.00 | 0.00 | ☆ | 1E13 |
| L8-20 | Application Macro Parameter 20 | 0.00～29.00 | 0.00 | ☆ | 1E14 |
| L8-21 | Application Macro Parameter 21 | 0.00～29.00 | 0.00 | ☆ | 1E15 |
| L8-22 | Application Macro Parameter 22 | 0.00～29.00 | 0.00 | ☆ | 1E16 |
| L8-23 | Application Macro Parameter 23 | 0.00～29.00 | 0.00 | ☆ | 1E17 |
| L8-24 | Application Macro Parameter 24 | 0.00～29.00 | 0.00 | ☆ | 1E18 |
| L8-25 | Application Macro Parameter 25 | 0.00～29.00 | 0.00 | ☆ | 1E19 |
| L8-26 | Application Macro Parameter 26 | 0.00～29.00 | 0.00 | ☆ | 1E1A |
| L8-27 | Application Macro Parameter 27 | 0.00～29.00 | 0.00 | ☆ | 1E1B |
| L8-28 | Application Macro Parameter 28 | 0.00～29.00 | 0.00 | ☆ | 1E1C |
| L8-29 | Application Macro Parameter 29 | 0.00～29.00 | 0.00 | ☆ | 1E1D |
| L8-30 | Application Macro Parameter 30 | 0.00～29.00 | 0.00 | ☆ | 1E1E |
| L8-31 | Application Macro Parameter 31 | 0.00～29.00 | 0.00 | ☆ | 1E1F |
| L8-32 | Application Macro Parameter 32 | 0.00～29.00 | 0.00 | ☆ | 1E20 |
| L8-33 | Application Macro Parameter 33 | 0.00～29.00 | 0.00 | ☆ | 1E21 |
| L8-34 | Application Macro Parameter 34 | 0.00～29.00 | 0.00 | ☆ | 1E22 |
| L8-35 | Application Macro Parameter 35 | 0.00～29.00 | 0.00 | ☆ | 1E23 |
| L8-36 | Application Macro Parameter 36 | 0.00～29.00 | 0.00 | ☆ | 1E24 |
| L8-37 | Application Macro Parameter 37 | 0.00～29.00 | 0.00 | ☆ | 1E25 |
| L8-38 | Application Macro Parameter 38 | 0.00～29.00 | 0.00 | ☆ | 1E26 |
| L8-39 | Application Macro Parameter 39 | 0.00～29.00 | 0.00 | ☆ | 1E27 |
| L8-40 | Application Macro Parameter 40 | 0.00～29.00 | 0.00 | ☆ | 1E28 |
| L8-41 | Application Macro Parameter 41 | 0.00～29.00 | 0.00 | ☆ | 1E29 |
| L8-42 | Application Macro Parameter 42 | 0.00～29.00 | 0.00 | ☆ | 1E2A |
| L8-43 | Application Macro Parameter 43 | 0.00～29.00 | 0.00 | ☆ | 1E2B |
| L8-44 | Application Macro Parameter 44 | 0.00～29.00 | 0.00 | ☆ | 1E2C |
| L8-45 | Application Macro Parameter 45 | 0.00～29.00 | 0.00 | ☆ | 1E2D |
| L8-46 | Application Macro Parameter 46 | 0.00～29.00 | 0.00 | ☆ | 1E2E |
| L8-47 | Application Macro Parameter 47 | 0.00～29.00 | 0.00 | ☆ | 1E2F |
| L8-48 | Application Macro Parameter 48 | 0.00～29.00 | 0.00 | ☆ | 1E30 |
| L8-49 | Application Macro Parameter 49 | 0.00～29.00 | 0.00 | ☆ | 1E31 |
| L8-50 | Application Macro Parameter 50 | 0.00～29.00 | 0.00 | ☆ | 1E32 |

2 Parameter Group Description

2.1 F0 Group Basic Function Codes

| F0-00 | Model Number | Range: 0～65535 | Factory value: Model determined |
| --- | --- | --- | --- |

For user viewing only, cannot be modified.

The model code meanings are shown in Table2-1.

Table 2-1 Product Model List

| Model Number | Inverter Status | Model Number | Meaning |
| --- | --- | --- | --- |
| 4301 | 380V～480V three-phase input, 0.75kW | 4315 | 380V～480V three-phase input, 75kW |
| 4302 | 380V～480V three-phase input, 1.5kW | 4316 | 380V～480V three-phase input, 93kW |
| 4303 | 380V～480V three-phase input, 2.2kW | 4317 | 380V～480V three-phase input, 110kW |
| 4304 | 380V～480V three-phase input, 4kW | 4318 | 380V～480V three-phase input, 132kW |
| 4305 | 380V～480V three-phase input, 5.5kW | 4319 | 380V～480V three-phase input, 160kW |
| 4306 | 380V～480V three-phase input, 7.5kW | 4320 | 380V～480V three-phase input, 185kW |
| 4307 | 380V～480V three-phase input, 11kW | 4321 | 380V～480V three-phase input, 200kW |
| 4308 | 380V～480V three-phase input, 15kW | 4322 | 380V～480V three-phase input, 220kW |
| 4309 | 380V～480V three-phase input, 18.5kW | 4323 | 380V～480V three-phase input, 250kW |
| 4310 | 380V～480V three-phase input, 22kW | 4324 | 380V～480V three-phase input, 280kW |
| 4311 | 380V～480V three-phase input, 30kW | 4325 | 380V～480V three-phase input, 315kW |
| 4312 | 380V～480V three-phase input, 37kW | 4326 | 380V～480V three-phase input, 355kW |
| 4313 | 380V～480V three-phase input, 45kW | 4327 | 380V～480V three-phase input, 400kW |
| 4314 | 380V～480V three-phase input, 55kW | 4328 | 380V～480V three-phase input, 450kW |

| F0-01 | Light/Heavy Load Selection | Range: 1～1 | Factory value: 1 |
| --- | --- | --- | --- |

Default value 1: Heavy load, non-modifiable.

This series of inverters only provides heavy load models.

| F0-02 | Rated Current | Range: 0.00～655.35 or 0.0～6553.5A | Factory value: Model determined |
| --- | --- | --- | --- |

For user viewing only, cannot be modified.

For inverters up to and including 93kW, the rated current is given to two decimal places; for 110kW and above, the rated current is given to one decimal place.

| F0-03 | Control Mode | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

0: Speed Mode

1: Point-to-point position control

2: Torque Mode

3: Homing Mode

This function code is used to set the operating mode of the inverter, allowing it to operate in speed mode, torque mode, or position mode (including point-to-point and homing).

| F0-04 | Speed Mode Selection | Range: 0～7 | Factory value: 0 |
| --- | --- | --- | --- |

0: V/F Control

2: VVC Voltage Vector Control

3: Induction Motor Closed-loop Control

4: Permanent Magnet Motor Closed-loop Control

6: SVC Open-loop Vector Control

When the inverter operates in speed mode, this function code is used to select the required control method. When driving an asynchronous motor, if there is an encoder, induction motor closed-loop control can be selected; if there is no encoder, VF control, VVC voltage vector control, or SVC open-loop vector control can be selected; When driving a permanent magnet synchronous motor, if there is an encoder, permanent magnet motor closed-loop control can be selected; if there is no encoder, VVC voltage vector control or SVC open-loop vector control can be selected; When driving a synchronous reluctance motor, VVC voltage vector control or SVC open-loop vector control can be selected.

| F0-05 | Run Command Source Selection | Range: 0～5 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to select the input channel for the inverter control command.

Inverter control commands include: start, stop, forward, reverse, jog, etc.

0: Digital Operator

Selecting this command channel allows the inverter to be controlled via the RUN, STOP, JOG, etc., buttons on the keypad to operate and stop the inverter.

1: External Terminal Input

Selecting this command channel allows the inverter to be controlled through digital input terminals.

2: RS485 Communication Input

Selecting this command channel allows commands to be sent to the inverter via RS485 communication to control the inverter to perform actions such as start and stop.

3: CANopen Input

Selecting this command channel requires the installation of a CANopen communication card in the inverter, allowing the master device to send control commands to the inverter using the CANopen protocol.

4: Reserved

5: Communication Card Input

Select this command channel, the inverter needs to be equipped with other communication cards (such as Profinet, Profibus-DP, EtherCAT communication cards), and the host computer can use the corresponding bus communication protocol to send control commands to the inverter.

| F0-06 | Frequency Source Selection | Range: 0～10 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the main frequency source.

0: Digital Operator

Set the frequency value via the keyboard; set the frequency value at the 'F xx.xx Hz' display on the main interface. Use the up and down buttons to move the arrow icon to the 'F xx.xx Hz' position, then press the 'OK' button, the frequency value will flash. Modify the frequency value using the left and right arrows and the up and down arrows, and press the 'ESC' key to exit after modification.

1: RS485 Communication

Set the frequency value through RS485 communication; the host computer needs to connect to the inverter's 485+, 485- terminals, and then the frequency can be set via communication. For details, refer to Appendix A MODBUS Communication Protocol.

2: Analog Input

Set the frequency value through an analog signal. The frequency value can be set through three analog input channels on the inverter; it is necessary to set the analog input function of the corresponding channel to “frequency setting” (set through F5-21, F5-27, or F5-33).

3: External Up/Down Input

Set the frequency value through DI terminals by setting the DI terminal function in group F5 to 19, 20 (external command for frequency increase, external command for frequency decrease), then adjust the frequency value using the corresponding DI terminals.

4: Pulse Input Without Direction

Select the reference pulse input type (F4-30).

5: Pulse Input With Direction

Select the reference pulse input type (F4-30).

6: CANopen Input

Set the frequency value through CANopen communication; the master device uses the CANopen protocol to communicate with the inverter and set its frequency value.

7: Reserved

8: Communication Card Input

Set the frequency value through a communication card; the inverter must have a communication card installed to communicate with the master device, which can use the corresponding bus communication protocol to set the inverter's frequency.

9: PID

The frequency setpoint comes from process PID control. The PID function can be configured through the FA group function codes, where the output of the PID controller serves as the inverter's frequency setpoint. For details, refer to the introduction of the 'PID function' in the FA group.

10: Digital Terminal Multi-Speed

The frequency value is set through the DI terminals. The F5 group 'Input Terminals' and FD group 'Multi-Speed and Simple PLC Function' function codes can be used to set multi-speeds and the correspondence between DI input signals and multi-speeds. For details, refer to the introductions of these two groups of function codes.

| F0-07 | Auxiliary Frequency Source | Range: 0～8 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to select the auxiliary frequency source.

0: Disabled

Disable the auxiliary frequency function.

1: Digital Operator

Set the auxiliary frequency via the keypad; the frequency value is set in the 'F xx.xx Hz' display on the main screen. Use the up and down buttons to move the arrow icon to the 'F xx.xx Hz' position, then press the 'OK' button, the frequency value will flash. Modify the frequency value using the left and right arrows and the up and down arrows, and press the 'ESC' key to exit after modification.

2: RS485 Communication

Set the frequency value through RS485 communication; the host computer needs to connect to the inverter's 485+, 485- terminals, and then the frequency can be set via communication. For details, refer to Appendix A MODBUS Communication Protocol.

3: Analog Input

Set the frequency value through an analog signal. The frequency value can be set through three analog input channels on the inverter. The corresponding channel's analog input function must be set to 'Auxiliary Frequency Setting' (set through F5-21, F5-27, or F5-33).

4: External Up/Down Input

Set the frequency value through DI terminals by setting the DI terminal function in group F5 to 19, 20 (external command for frequency increase, external command for frequency decrease), then adjust the frequency value using the corresponding DI terminals.

5: Pulse Input

Select the reference pulse input type (F4-30).

6: CANopen Input

Selecting this command channel requires the installation of a CANopen communication card in the inverter, allowing the master device to send control commands to the inverter using the CANopen protocol.

7: Reserved

8: Communication Card Input

Set the frequency value through a communication card; the inverter must have a communication card installed to communicate with the master device, which can use the corresponding bus communication protocol to set the inverter's frequency.

Note:

1. The auxiliary frequency source can only be modified when the inverter is stopped.

2. The main and auxiliary frequency function selection can only be modified when the inverter is stopped.

3. Some command source methods require additional accessories, such as communication cards.

4. The main frequency and auxiliary frequency cannot be set to the same source.

5. When digital inputs (F5-00 ~ F5-06, LC-00 ~ LC-02) are configured to force the auxiliary frequency to 0, the auxiliary frequency output function will be disabled.

6. If the result of subtracting the auxiliary frequency from the main frequency is negative, negative frequency input must be allowed (set F5-38 to 1) for reversal; otherwise, the output frequency will be 0. Additionally, pay attention to the setting of the main-auxiliary reverse stop frequency (FA-27) to avoid limiting the desired reverse output.

| F0-08 | Frequency Source Superposition Selection | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

0: Main Freq + Aux Freq

The actual set frequency of the inverter is the sum of the main frequency and the auxiliary frequency.

1：Main Frequency-Auxiliary Frequency

The actual set frequency of the inverter is the main frequency minus the auxiliary frequency.

2: Aux Freq - Main Freq

The actual set frequency of the inverter is the auxiliary frequency minus the main frequency.

3：Auxiliary Frequency

The actual set frequency of the inverter isthe auxiliary frequency.

| F0-09 | Forward/Reverse Prohibited Selection | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

0: Forward/Reverse Enabled

Allows the inverter to run forward and reverse.

1: Reverse Prohibited

The inverter can only run forward, not reverse.

2: Forward Prohibited

The inverter can only run reverse, not forward.

| F0-10 | Upper Limit Frequency | Range: 0.00～599.00Hz | Factory setting: 599.00Hz |
| --- | --- | --- | --- |

This function code is used to set the maximum frequency that the inverter can output.

| F0-11 | Lower Limit Frequency | Range: 0.00～F0-10 | Factory Value: 0.00Hz |
| --- | --- | --- | --- |

This function code is used to set the lower limit frequency for the variable speed drive. When the inverter starts running, it begins at the start frequency. During operation, if the set frequency is less than the lower limit frequency, the inverter will continue to run at the lower limit frequency until the inverter stops or the set frequency is 0Hz or greater than the lower limit frequency.

| F0-12 | Speed curve time unit | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

Used to set the precision of the acceleration and deceleration time units.

0: Acceleration and deceleration unit is 0.01 seconds

1: Acceleration and deceleration unit is 0.1 seconds

By default, this function code is 0, at which point the acceleration and deceleration time is two decimal places, with a maximum value of 600.00 seconds. If longer acceleration and deceleration times are required, this function code can be changed to 1, at which point the acceleration and deceleration time becomes one decimal place, with a maximum value of 6000.0 seconds, to meet the demand for longer acceleration and deceleration times.

| F0-13 | Acceleration time 1 | Range: 0.00～600.00 seconds or 0.0～6000.0 seconds | Factory value: Model determined |
| --- | --- | --- | --- |
| F0-14 | Deceleration time 1 | Range: 0.00～600.00 seconds or 0.0～6000.0 seconds | Factory value: Model determined |

Acceleration time refers to the time required for the inverter to accelerate from zero frequency to the acceleration/deceleration reference frequency (determined by F4-02), see t1 in Fig.2-1.

Deceleration time refers to the time required for the inverter to decelerate from the acceleration/deceleration reference frequency (determined by F4-02) to zero frequency, see t2 in Fig.2-1.

This series of inverters provides 4 sets of acceleration/deceleration times, which users can switch and select using digital input terminals DI.

Fig. 2-1 Acceleration/Deceleration Time

| F0-15 | Carrier frequency | Range: 2～15kHz | Factory setting: 6kHz |
| --- | --- | --- | --- |

This function code is used to adjust the carrier frequency of the inverter. By adjusting the carrier frequency, motor noise can be reduced, mechanical system resonance points can be avoided, ground leakage current can be reduced, and interference generated by the inverter can be minimized. When the carrier frequency is low, the high-order harmonic components of the output current increase, leading to increased motor losses and motor temperature rise. When the carrier frequency is high, motor losses decrease and motor temperature rise decreases, but inverter losses increase, inverter temperature rise increases, and interference increases. Adjusting the carrier frequency will affect the performance shown inTable 2-2:

Table 2-2 Effects of High and Low Carrier Frequencies

| Carrier frequency | Low -> High |
| --- | --- |
| Motor noise | High -> Low |
| Output current waveform | Poor -> Good |
| Motor temperature rise | High -> Low |
| Inverter Temperature Rise | Low -> High |
| Leakage Current | Small -> Large |
| External Radiated Interference | Small -> Large |

The carrier frequency factory settings for inverters of different power ratingsare shown in Table 2-3. Although users can modify the settings according to their needs, it should be noted that if the carrier frequency is set higher than the factory value, it will lead to an increase in the temperature rise of the inverter's heat sink. At this time, users need to derate the inverter, otherwise there is a risk of overheating alarms.

Table 2-3 Factory Carrier Frequency Settings for Different Models

| Inverter Power | Carrier Frequency Range | Factory Set Carrier Frequency |
| --- | --- | --- |
| 0.75kW～7.5kW | 2～15kHz | 6kHz |
| 11kW～93kW | 2～10kHz | 4kHz |
| 110kW～450kW | 2～6kHz | 2kHz |

| F0-16 | Reduced Carrier Method | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the inverter's carrier frequency reduction method.

0: Reduce carrier based on current temperature

When the inverter output current is too high or the module temperature is too high, it will automatically reduce the inverter's carrier frequency, and simultaneously lower the overcurrent stall prevention level.

1: Fixed carrier and limit current

When the inverter output current is too high or the module temperature is too high, the carrier frequency remains unchanged, but it will automatically reduce the overload protection threshold and the overcurrent stall prevention threshold.

2: Same as setting 0, but the overcurrent stall prevention threshold remains unchanged

When the inverter output current is too high or the module temperature is too high, it will automatically reduce the inverter's carrier frequency, but the overcurrent stall prevention level remains unchanged.

| F0-17 | High-speed Machine Mode | Range: 0～65535 | Factory value: 0 |
| --- | --- | --- | --- |

This function code can set whether the inverter operates in high-speed machine mode.

By default, the maximum output frequency of this series of inverters is 599Hz. If a higher output frequency is required, this function code can be modified. Set F0-17 to 1 first, then set it to 2, which will enable the inverter to operate in high-speed machine mode. At this point, the maximum output frequency of the inverter becomes 599.00Hz, and the upper limits of function codes such as the motor's maximum frequency and rated frequency will change to 2000.0Hz to meet the control requirements of high-speed machines. To return to normal mode, set F0-17 to 1 first, then set it to zero.

Note: When switching to high-speed machine mode or switching back from high-speed machine mode to normal mode, the inverter will reset to factory settings.

| F0-18 | Parameter Management Settings | Range: 0～13 | Factory value: 0 |
| --- | --- | --- | --- |

0: No Function

1: Parameter Write Protection

After entering 1, most function codes will be locked and cannot be modified; only parameter management (F0-18) and password input (F7-33) can be modified. This value will be written to EEPROM and remains effective even after power loss. Only by setting parameter management (F0-18) to 0 can other parameter values be modified.

5: Reset kWH Display to Zero

Can reset the inverter's energy statistics to zero.

7: Reset CAN Slave

Reset CAN slave

8: No Function

9: Reset to 50Hz Factory Values

Restore factory settings for function codes, which resets the function codes to their default values. If a password has been set for the inverter (via F7-34), the password must be removed (via F7-33) before restoring factory settings.

1. When the parameter management setting is 6 or 7, the control board needs to be repowered after configuration.

2. If the frequency command source isDigital Operator, set parameter management to 7 to achieve reverse function, it is necessary to simultaneously reverse the analog frequency negative value.

(F5-38) Set to 1 (allow negative frequency input, positive frequency forward rotation, negative frequency reverse rotation, digital operator and external terminals cannot control forward/reverse rotation).

| F0-19 | Keyboard Set Frequency | Range: 0.00～599.00Hz | Factory Value: 50.00Hz |
| --- | --- | --- | --- |

This function code is used to set the keyboard frequency command.

2.2 F1 Group Start/Stop Control Parameters

| F1-00 | Startup Speed Tracking | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

This function code sets the speed tracking method.

Speed tracking is applicable for punch presses, fans, and otherlargeinertia load applications. In control modes with an encoder, when this parameter is non-zero, the inverter will automatically start speed tracking based on the speed feedback from the encoder.

When using free stop (that is, inertial stop) for parking, or when the inverter suddenly stops due to a fault, the motor will decelerate under the effect of friction. If the motor inertia is large, the time required for the motor to come to a complete stop will be longer. If the inverter starts directly while the motor has not completely stoppedrotating, it may cause significant impact. The speed tracking function can obtain the motor's speed and then control the motor to run to the set frequency based on this speed. On one hand, this shortens the time required for restarting, and on the otherhand, it can avoid excessive impact.

0: No Action

Speed tracking function is off.

1: Track from the maximum output frequency

Upon restart, track the speed downward from the maximum output frequency until the motor speed is detected, then control the motor to run at the set frequency.

2: Track Frequency at Startup

Upon restart, track the speed from the given frequency at startup, ultimately controlling the motor to run at the set frequency.

3: Track from the minimum output frequency

Upon restart, track the speed from the minimum frequency until the motor speed is detected, then control the motor to run at the set frequency.

| F1-01 | Initial Position Identification Method | Range: 0～5 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the initial position identification method.

0: Do Not Perform Initial Angle Identification

Do not perform initial position identification.

1: Attraction Method

By injecting DC current into the windings, the motor rotor is rotated to the specified position.

3: Pulse Injection Method 1

4: Pulse Injection Method 2

Determine the position of the motor rotor by injecting voltage pulses, giving priority to Pulse Injection Method 1. If Pulse Injection Method 1 is ineffective, Pulse Injection Method 2 can be tried.

| F1-02 | Abnormal Restart Method | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

This function code sets the abnormal restart method.

When anexceptionoccurs, the invertergenerally stops running until the fault is reset and a runcommand is received, after which it will restart. The abnormal restart function can automatically clear faults and control the inverter to restart when afaultoccurs, achieving the goal of continuous motor operation. When the number of automatic fault clearances reaches the set value, it will not be possible to automatically clear the fault and restart. The abnormal start-up times can be set through function codeF9-46.

0: Stop Operation

In the event of an anomaly, the system stops; after the fault is reset, the inverter does not automatically restart.

1: Current Speed Tracking

When restarting after an anomaly, the speed is tracked from the current speed, and once the speed is tracked, the motor is controlled to run to the set frequency.

2: Minimum Frequency Tracking

When restarting after an anomaly, the speed is tracked from the minimum frequency, and once the speed is tracked, the motor is controlled to run to the set frequency.

| F1-03 | Speed tracking maximum current% | Range: 20%～200% | Factory setting: 100% |
| --- | --- | --- | --- |

The inverter only begins speed tracking when the output current exceeds the value set by this function code. The larger the setting value of this function code, the faster the tracking speed, but if it is too large, it may cause overcurrent or overload faults.

| F1-04 | Voltage Increase Rate | Range: 1～200 | Factory Value: 100 |
| --- | --- | --- | --- |

This function code sets the voltage increase rate during speed tracking. Under normal circumstances, this function code does not need to be changed and should remain at the default setting.

| F1-05 | Start Frequency | Range: 0～599.00Hz | Factory Value: 0.50Hz |
| --- | --- | --- | --- |

This function code is used to set the start frequency. If the start frequency is higher than the lower limit frequency, the inverter starts running from the start frequency.

| F1-06 | Startup Hold Time | Range: 0.00～600.00 seconds | Factory Value: 0.00 seconds |
| --- | --- | --- | --- |
| F1-07 | Startup Hold Frequency | Range: 0～599.00Hz | Factory Value: 0.00Hz |

In applications such as cranes and elevators, to improve reliability or positioning accuracy, it is often necessary to run at a certain frequency for a period of time before continuing to accelerate to the set frequency, and when stopping, to run at a certain frequency for a period of time before decelerating to zero speed stop. The frequency hold function can achieve these purposes. The frequency holding function includes two types: start-up frequency holding and stop frequency holding.  The start-up frequency holding function is shown in Figure 2-2, where F1-06 start-up holding time and F6-07 start-up holding frequency can be set for the start-up frequency holding function.

Figure 2-2 Start-up Frequency Holding Function

| F1-08 | Braking Current Level | Range: 0～100% | Factory Value: 0% |
| --- | --- | --- | --- |
| F1-09 | Startup Braking Time | Range: 0.0～60.0 seconds | Factory Value: 0.0 seconds |
| F1-10 | DC Braking Ratio Coefficient | Range: 0～65535 | Factory Value: 2000 |
| F1-11 | DC Braking Integral Coefficient | Range: 0～65535 | Factory Value: 100 |

When the inverter starts, if the motor is still rotating, it may cause significant impact. To avoid this situation, the motor can be braked first, and then started from zero speed after the motor has stopped rotating. To activate the DC braking function, inject DC current into the motor windings to produce a braking torque that stops the motor from rotating, as shown in Figure 2-3.

Figure 2-3 DC/Stop Braking Function

If the braking time is not zero, the DC braking function during startup is effective. When the inverter starts, it first applies the set braking current for DC braking, then after the set startup braking time, it begins to run up to the set frequency. The greater the braking current, the greater the braking force.

F1-08 Braking Current Size, used to set the magnitude of the DC braking current during startup and stop, the unit is %, based on the rated current of the motor.

F1-09 Startup Braking Time, used to set the duration of the DC braking during startup. If the startup braking time is set to zero, the DC braking during startup is ineffective.

When using VF or induction motor VVCcontrol, the current PI regulator parameters can be adjusted using F1-10 DC braking ratio coefficient and F1-11 DC braking integral coefficient.

| F1-12 | Stopping Method | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: Deceleration Stop

During deceleration stop, the inverter decelerates according to the set deceleration time, down to 0 or M1 multi-point VF frequency point 1 (F2-04) before stopping output.

1: Free Stop

After the stop command becomes effective, the inverter immediately terminates output.

| F1-13 | Deceleration Method | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the deceleration method. When the motor decelerates, it may feed energy back to the inverter, thereby increasing the bus voltage. If the bus voltage becomes too high, it can lead to overvoltage faults. Choosing an appropriate deceleration method can reduce the probability of overvoltage faults.

0:No function

Normal deceleration or stop, without performing regenerative energy suppression.

1: Deceleration method 1

During deceleration, the inverter will automatically adjust the deceleration curve based on the F9-04 overvoltage stall threshold and the bus voltage.

2: Deceleration Method 2

During deceleration, when the bus voltage is too high, deceleration method 2 will be activated, and the inverter will automatically adjust the output frequency and output voltage to achieve the purpose of accelerating the consumption of regenerative energy.

| F1-15 | 1-4 Segment Acceleration/Deceleration Switching Points | Range: 0～599.00Hz | Factory Value: 0.00Hz |
| --- | --- | --- | --- |

If it is necessary to switch acceleration and deceleration times during operation, this can be achieved by modifying this function code. When the current frequency is less than the switching frequency, the motor adjusts speed according to acceleration/deceleration time 4; when the current frequency is greater than the switching frequency, the motor adjusts speed according to acceleration\deceleration time 1, the 1-4 segment acceleration/deceleration switching function is shown in Figure 2-4.

Figure 2-4 1-4 Segment Acceleration/Deceleration Frequency Switching Points

| F1-16 | S Acceleration Time 1 | Range: 0.00～25.00 seconds | Factory Value: 0.20 seconds |
| --- | --- | --- | --- |
| F1-17 | S Acceleration Time 2 | Range: 0.00～25.00 seconds | Factory Value: 0.20 seconds |
| F1-18 | S Deceleration Time 1 | Range: 0.00～25.00 seconds | Factory Value: 0.20 seconds |
| F1-19 | S Deceleration Time 2 | Range: 0.00～25.00 seconds | Factory Value: 0.20 seconds |

There are two modes for the speed curve: normal acceleration/deceleration mode and S-curve acceleration/deceleration mode. When all four function codes are zero, the speed curve operates in normal acceleration/deceleration mode; otherwise, it operates in S-curve acceleration/deceleration mode.

In general acceleration and deceleration modes, the slope of acceleration and deceleration will undergo sudden changes at the start or end of the process, causing significant variations in motor torque within a short period, which can impact both the motor and the load. To reduce this impact, S-curve acceleration and deceleration mode can be used. Under this mode, the slope of the speed curve is continuous, thereby reducing torque shock. Figure2-5shows the frequency and acceleration waveforms during general acceleration and deceleration. It can be seen that the acceleration is discontinuous and sudden changes occur. In contrast, the acceleration in Figure2-6, which represents S-curve acceleration and deceleration, changes continuously. Therefore, using an S-curve for acceleration and deceleration can mitigate the impact caused by sudden changes in torque.

Figure 2-5 General Acceleration and Deceleration Mode

Figure 2-6 S Acceleration and Deceleration Mode

In S acceleration and deceleration mode, the total acceleration and deceleration time will belonger. When acceleratingfromzerotothemaximumoperatingfrequency, the total acceleration time= set acceleration time + (S acceleration time 1)/2 + (S acceleration time 2)/2, the total deceleration time = set deceleration time + (S deceleration time 3)/2 + (S deceleration time 4)/2. As shown inFigure2-7, in the figure below, S1 represents S acceleration time 1, S2 represents S acceleration time 2, S3 represents S deceleration time 3, S4 represents S deceleration time 4.

Figure 2-7 S Acceleration and Deceleration Mode Time

| F1-20 | Automatic Acceleration and Deceleration Selection | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the automatic acceleration/deceleration mode. In practical applications, the setting of acceleration and deceleration times is influenced by factors such as load conditions and motor inertia, and may require multiple adjustments to determine. If the acceleration time is too short, it may cause excessive acceleration current, leading to overcurrent; If the deceleration time is too short, it may result in excessively high bus voltage, causing overvoltage. Theautomatic acceleration/deceleration function can automatically adjust the acceleration and deceleration times based on actual conditions,simplifying the debugging process.

0: Linear Acceleration and Deceleration

Both automatic acceleration and automatic deceleration functions are disabled.

1: Automatic Acceleration and Linear Deceleration

The automatic acceleration function is enabled, and the automatic deceleration function is disabled.

2: Linear Acceleration and Automatic Deceleration

Automatic acceleration function off, automatic deceleration function on.

3: Automatic Acceleration and Deceleration

Both automatic acceleration and automatic deceleration functions are on.

4: Automatic Acceleration and Deceleration Suppression

When the current is too high, the automatic acceleration function is enabled; when the bus voltage is too high, the automatic deceleration function is enabled; in other cases, both automatic acceleration and automatic deceleration functions are off.

| F1-21 | Automatic Acceleration and Deceleration Kp | Range: 0～65535 | Factory setting: 20 |
| --- | --- | --- | --- |
| F1-22 | Automatic Acceleration and Deceleration Ki | Range: 0～655.35 | Factory setting: 0.400 |

When the automatic acceleration/deceleration function is enabled, the acceleration/deceleration slope is adjusted via a PI controller, and the above two parameters are used to set the PI controller parameters.

| F1-23 | Abnormal Shutdown Method | Range: 0～6 | Factory value: 0 |
| --- | --- | --- | --- |

This parameter is used to set the abnormal stop method. When the digital input terminal is set to external fault or deceleration stop function, if this terminal receives an effective level, the inverter will stop according to the method set by this parameter.

0: Free Stop

The inverter immediately stops output, allowing for free stop.

1: According to First Deceleration Time

Decelerate according to deceleration time 1.

2: According to Second Deceleration Time

Decelerate according to deceleration time 2.

3: According to Third Deceleration Time

Decelerate according to deceleration time 3.

4: According to Fourth Deceleration Time

Decelerate according to deceleration time 4.

5: According to System Deceleration Time

Decelerate according to the currently set deceleration time.

6: Automatic Deceleration

The inverter automatically adjusts the deceleration time.

| F1-24 | Braking Time | Range: 0.0～60.0 seconds | Factory Value: 0.0 seconds |
| --- | --- | --- | --- |
| F1-25 | Braking Start Frequency | Range: 0.00～599.00Hz | Factory Value: 0.00Hz |

The above function codes are used to set the stop DC braking function. When the inverter stops, the motor may not come to a complete stop. To prevent this, the stop DC braking function can be used to apply DC braking to the motor after it stops, ensuring that the motor does not rotate after shutdown.

If the stop braking time is not zero, the stop DC braking function is effective. The inverter decelerates to the braking start frequency and then begins DC braking, stopping output after the set stop braking time. The greater the braking current, the greater the braking force.

F1-08 Braking Current Size, used to set the magnitude of the DC braking current during startup and stop, the unit is %, based on the rated current of the motor. F1-24 Stop Braking Time, used to set the duration for stopping DC braking. When stopping, if DC braking is required, F1-12 Motor Stopping Method must be set to deceleration stop for the DC braking function to be effective. When the stop braking time is set to zero, the DC braking function is disabled. F1-25 Braking Start Frequency, this function code sets the starting frequency for stopping DC braking. When the inverter decelerates to the frequency value set by this function code, DC braking begins.

| F1-26 | Stop Holding Time | Range: 0.00～600.0 seconds | Factory Value: 0.00 seconds |
| --- | --- | --- | --- |
| F1-27 | Parking Hold Frequency | Range: 0.00～599.00Hz | Factory Value: 0.00Hz |

In applications such as elevators and cranes, to improve reliability or positioning accuracy, it is necessary to maintain a certain frequency for a period of time before reducing to zero speed during stopping. This is the stop frequency hold function. The stop frequency holding function is shown in Figure 2-8, F1-26 stop holding time and F1-27 stop holding frequency can be used to set the stop frequency holding function.

Figure 2-8 Stop Holding Frequency and Time

| F1-28 | Speed Tracking Demagnetization Time | Range: 0～65535 | Factory Value: 50 |
| --- | --- | --- | --- |

This function code is used for speed tracking and generally does not need to be changed.

| F1-29 | Instant Power Failure Restart Method | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

In general, when power is lost, the inverter will shut down due to undervoltage, and even if the power returns to normal within a short period, the inverter will not restart automatically. The instantaneous power failure restart function allows the inverter to automatically restart after a brief power outage and recovery, achieving continuous motor operation.

0: Stop Operation

An instantaneous power outage has occurred, causing the operation to stop and preventing automatic restart.

1: Current Speed Tracking

The inverter starts from the frequency before the power failure and performs speed tracking downwards. Once the motor frequency is detected, it accelerates from the detected motor frequency to the set frequency. This option is recommended when the motor inertia is large.

2: Minimum Frequency Tracking

The inverter starts tracking upwards from the lowest frequency. Once the motor frequency is detected, it accelerates from the detected motor frequency to the set frequency. This option is recommended when the motor inertia is small.

| F1-30 | Allowed Power Failure Time | Range: 0.0～20.0 seconds | Factory setting: 2.0 seconds |
| --- | --- | --- | --- |

When the power outage duration is less than the value set for this function code, automatic restart according to the settings of F1-29 is allowed. If the power outage duration exceeds the time set for this function code, automatic restart will not be performed.

| F1-31 | Base Blockade Interruption Time | Range: 0.0～5.0 seconds | Factory Value: 0.5 seconds |
| --- | --- | --- | --- |

When an external controller detects an abnormal situation, it may be necessary for the inverter to immediately stop output; at this time, the base block function of the inverter can be used. Upon receiving the base block signal, the inverter immediately stops PWM output. After the base block signal is removed, the inverter will restart after waiting for the time set by this function code. Additionally, if the inverter's stopping method is free stop, it must wait for the time set by this function code before it can run again after stopping.

| F1-32 | dEb recovery threshold | Range: 0.0～200.0 | Factory Value: 40.0 |
| --- | --- | --- | --- |
| F1-33 | dEb action bias threshold | Range: 0.0～200.0 | Factory Value: 40.0 |
| F1-34 | dEb deceleration selection | Range: 0～2 | Factory value: 0 |
| F1-35 | dEb recovery time | Range: 0.0～25.0 seconds | Factory value: 3.0 seconds |

When the grid voltage drops, the inverter may shut down due to undervoltage. Uncontrolled abnormal shutdowns can cause significant impacts, leading to system damage and serious consequences. The ride-through function (also known as KEB, Kinetic Energy Backup, or dEb, Deceleration Energy Backup) can, in such cases, allow the motor to decelerate in a controlled manner, reducing the impact on the system.

F1-32 dEb recovery threshold

This function code is used to set the dEb recovery voltage. By default, dEb recovery voltage = dEb action voltage + (F1-32 setting value).

F1-33 dEb action bias threshold

This function code is used to set the dEb action voltage. By default, dEb action voltage = undervoltage protection value+60V+(F1-33 setting)

F1-34 dEb deceleration selection

This function code is used to set the deceleration mode when the instantaneous stop does not occur.

0: No Action

When the grid voltage drops, the inverter does not decelerate, and the inverter may shut down due to undervoltage.

1: Enable, no recovery

When the grid voltage drops and reaches the dEb action voltage, the inverter controls the motor to decelerate. When the grid voltage returns to normal, it still controls the motor to decelerate to zero, then shuts down.

2: Enable, recovery

When the grid voltage drops and reaches the dEb action voltage, the inverter controls the motor to decelerate. When the grid voltage returns to normal, it maintains the current frequency operation for a period (F1-35) before accelerating to the target frequency.

F1-35 dEb recovery time

This parameter is used to set the time during which the inverter maintains operation at the current frequency when power is restored. After this time ends, the inverter accelerates back to the given frequency.

| F1-36 | PM Voltage Pulse Width | Range: 0.0～3.0 milliseconds | Factory setting: 1.0 millisecond |
| --- | --- | --- | --- |

This function code is used for initial position identification, adjusting the pulse width of the voltage.

| F1-41 | Flux Zero Position Alignment Time | Range: 0.0～60.0 seconds | Factory setting: 2.0 seconds |
| --- | --- | --- | --- |

This function code is used for initial position identification, adjustingzero alignment time.

2.3 F2 Group VF Control Parameters

The function codes in this group can be used to adjust VF and VVC control modes.

| F2-00 | V/F Voltage Selection | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the type of voltage curve. This inverter provides multiple voltage curves and allows forcustom-defined voltage curvesto meet different operating conditions.

0: General V/F Curve

Multi-segment (custom) V/F curveas shown in Figure 2-9, define multi-segment V/F curves according to F2-04～F2-09 and F4-03 and F4-04, as shown in the figure below. InFigure2-9, f1, f2, f3, and fb are the M1 multi-point VF frequency point 1, M1 multi-point VF frequency point 2, M1 multi-point VF frequency point 3, and M1 rated frequency, respectively; V1, V2, V3, and Vb are the M1 multi-point VF voltage point 1, M1 multi-point VF voltage point 2, M1 multi-point VF voltage point 3, and M1 rated voltage, respectively.

Note: Generally, V1<V2<V3<Vb, f1<f2<f3<fb. Setting the voltage too high at low frequencies may cause the motor to overheat or even burn out, and the inverter may experience overcurrent or overvoltage protection.

Figure 2-9 Typical V/F Curve

1: 1.5 Power V/F Curve

The output voltage of the inverter is proportional to the 1.5 power of the frequency.

2: 2 Power V/F Curve

The output voltage of the inverter is proportional to the square of the frequency.

| F2-01 | Torque Compensation Gain | Range: 0～10 | Factory value: 1 |
| --- | --- | --- | --- |
| F2-02 | Torque Filter Time | Range: 0.001～10.000 seconds | Factory setting: 0.500 |

The above two function codes apply only to VF and VVC control modes.

F2-01 Torque compensation gain, used to adjust the torque compensation level. By adjusting the output voltage, the motor's load-carrying capacity can be adjusted. The greater the torque compensation gain, the better the load-carrying capacity, but the output current will increase. If set too high, it may lead to overcurrent faults; reducing the torque compensation gain will decrease the load-carrying capacity accordingly.

F2-02 Torque filter time, used to adjust the filtering time for torque compensation. If the filter time is set too high, control stability improves, but response becomes slower. When the filter time is tooshort, the response is faster, but it may lead to instability. Adjust according to actual conditions.

| F2-04 | M1 Multi-point VF Frequency Point 1 | Range: 0.00～599.00Hz | Factory Value: 0.50Hz |
| --- | --- | --- | --- |
| F2-05 | Multi-point VF Voltage Point 1 | Range: 0.0～480.0V | Factory setting: 2.0V |
| F2-06 | Multi-point VF Frequency Point 2 | Range: 0.00～599.00Hz | Factory setting: 1.50Hz |
| F2-07 | Multi-point VF Voltage Point 2 | Range: 0.0～480.0V | Factory setting: 10.0V |
| F2-08 | Multi-point VF Frequency Point 3 | Range: 0.00～599.00Hz | Factory setting: 3.00Hz |
| F2-09 | Multi-point VF Voltage Point 3 | Range: 0.0～480.0V | Factory setting: 22.0V |

The above function codes are used to set the voltage curve, please refer to function code F2-00.

| F2-10 | Slip Filter Time | Range: 0.001～10.000 seconds | Factory setting: 0.100 seconds |
| --- | --- | --- | --- |
| F2-11 | Slip Compensation Gain | Range: 0.00～10.00 | Factory Value: 0.00 |

F2-10 Slip Filter Time, used to set the filter time for slip compensation. The larger the value of this function code, the slower the compensation response; the smaller the value, the faster the response, but it may lead to system instability. Adjust according to actual conditions.

F2-11 Slip Compensation Gain, used to set the gain for slip compensation. By modifying the slip compensation gain, the motor speed accuracy can be improved.

When using VF control: Under electric load conditions, if the motor speed is lower than the actual speed, the slip compensation gain can be appropriately increased; otherwise, reduce the slip compensation gain; In power generation load conditions, if the motor speed is higher than the actual speed, the slip compensation gain can be appropriately increased; conversely, it should be decreased.

When using induction motor VVC control: In electric load conditions, if the motor speed is lower than the actual speed, the slip compensation gain can be appropriately increased; conversely, it should be decreased.

| F2-12 | Power Generation Slip Compensation Gain | Range: 0.00～1.00 | Factory Value: 1.00 |
| --- | --- | --- | --- |

This function code is only applicable to the VVC control mode for induction motors, used to adjust the slip compensation gain in power generation mode.

Whenusinginduction motorVVC control, in power generation load conditions, if the motor speed is higher than the actual speed, the slip compensation gain for power generation can be appropriately increased; conversely, it should be decreased.

| F2-13 | Maximum Slip Limit Value | Range: 0.00～200.00Hz | Factory Value: 20.00Hz |
| --- | --- | --- | --- |

This function code is used to set the maximum slip.

| F2-14 | Oscillation Suppression Gain | Range: 0～10000 | Factory Value: 1000 |
| --- | --- | --- | --- |

When using VF control to drive an asynchronous motor, there may be vibration issues. If the vibration is severe, it may cause a fault shutdown, thusunable to operate normally. Adjusting this function code can improve the oscillation condition. The greater the oscillation suppression gain, the better the oscillation suppression effect, but the dynamic performance of the motor will decrease. Conversely, the poorer the vibration suppression effect, the better the dynamic performance of the motor. In actual use, the function code can be adjusted according to specific requirements.

| F2-15 | Overexcitation Braking Gain | Range: 0～65535 | Factory value: 30 |
| --- | --- | --- | --- |

This parameter can suppress the increase in bus voltage during the deceleration process of the inverter; the higher the value, the better the suppression effect.

Flux braking increases the output voltage of the inverter to boost the current at the motor terminals, thereby enhancing the consumption of regenerative energy to suppress the rise in bus voltage. The greater the gain, the larger the motor current, so please pay attention when applying it. When a braking resistor is also present, it is recommended to set this value to 0; otherwise, excessive deceleration current during deceleration may cause abnormalities.

| F2-16 | Voltage Stability Gain | Range: 0～200Hz | Factory value: 10 |
| --- | --- | --- | --- |

This parameter can effectively improve the current control performance and stability during VF operation. Increasing this parameter can effectively enhance the current stability throughout the VF and VVC operation, but increasing this parameter under VF will reduce the load-carrying capacity at startup.

2.4 F3 Group Vector Control Parameters

| F3-00 | System Control | Range: 0000H~FFFFH | Factory value: 0 |
| --- | --- | --- | --- |

This parameter needs to be set bit by bit, as shown in Table 2-4:

Table 2-4 System Control Bit Description

| bit | Function | Description |
| --- | --- | --- |
| bit0 | Speed Loop Gain Auto Design | 0: Speed loop gain auto design off, speed loop response is determined by PI parameters in F3-05 to F3-10. |
|  |  | 1: Speed loop gain auto design on, speed loop response is determined by bandwidth parameters in F3-02 to F3-04. |
| bit3 | Dead time compensation off | 0: Dead time compensation on |
|  |  | 1: Dead time compensation off |

| F3-01 | ASR1/2 Switching Frequency | Range: 5.00～599.00 Hz | Factory setting: 7.00 Hz |
| --- | --- | --- | --- |

This parameter sets the switching frequency between ASR1 low speed and ASR2 high speed.

| F3-02 | Zero Speed Bandwidth | Range: 1～100 Hz | Factory setting: 10 Hz |
| --- | --- | --- | --- |
| F3-03 | ASR1 Low-Speed Bandwidth | Range: 1～100 Hz | Factory setting: 10 Hz |
| F3-04 | ASR2 High-Speed Bandwidth | Range: 1～100 Hz | Factory setting: 10 Hz |

When bit 0 of parameter F3-00 is set to 1, F3-02, F3-03, and F3-04 are used to set the speed loop bandwidth for the motor at zero speed, low speed, and high speed, respectively. By increasing the bandwidth value, the dynamic response of the speed loop can be accelerated, but an excessively high bandwidth may cause system oscillation.

Note: This parameter can only be modified when bit0=1 in F3-00.

| F3-05 | ASR Zero Speed Gain | Range: 0～100 Hz | Factory Value: 20 Hz |
| --- | --- | --- | --- |
| F3-06 | ASR Zero Speed Integration Time | Range: 0.000～10.000 s | Factory Value: 0.100 s |
| F3-07 | ASR1 Low-Speed Gain | Range: 0～100 Hz | Factory setting: 10 Hz |
| F3-08 | ASR1 Low-Speed Integral Time | Range: 0.000～10.000 s | Factory Value: 0.100 s |
| F3-09 | ASR2 High-Speed Gain | Range: 0～100 Hz | Factory setting: 10 Hz |
| F3-10 | ASR2 High-Speed Integral Time | Range: 0.000～10.000 s | Factory Value: 0.100 s |

When the bit 0 of parameter F3-00 is set to 0, F3-05～F3-06, F3-07～F3-08, and F3-09～F3-10 are used to set the speed loop PI regulator parameters for the motor at zero speed, low speed, and high speed, respectively. Increasing the gain value or decreasing the integration time can both accelerate the dynamic response of the speed loop, but an excessively large gain value or too small an integration time will cause system oscillation. ASR Adjustment - Manual Design and Automatic Design as Shown in Figure 2-10.

Figure 2-10 ASR Adjustment - Manual Design and Automatic Design

| F3-11 | ASR Speed Feedforward Coefficient | Range: 0%～200% | Factory Value: 0% |
| --- | --- | --- | --- |

This parameter is used to set the torque feedforward gain of the speed loop. Increasing this parameter helps improve the speed loop response during sudden load changes. The default value is zero, corresponding to the feedforward function being disabled. If you need to enable it, it is recommended to contact a professional technician to adjust this parameter.

| F3-12 | PDF Coefficient | Range: 0～200 | Factory value: 30 |
| --- | --- | --- | --- |

When bit 0 of parameter F3-00 is set to 1, the pseudo-differential feedforward function is enabled, which helps improve the speed loop response. It is recommended to contact a professional technician to adjust this parameter.

| F3-13 | DOB Compensation Gain | Range: 0～5000Hz | Factory value: 0 |
| --- | --- | --- | --- |

In inverters, PI controllers are commonly used to control speed; if a higher system speed response is required, the bandwidth of the PI controller needs to be increased. However, in practical applications, the bandwidth of the PI controller is subject to many constraints and maynotbe adjustabletothe required value. At this point, if it is still necessary to further improve the system speed response, load torque compensation can be added on top of thePI controller.

DOB: Disturbance Observer.

F3-13 DOB Compensation Gain

This parameter sets the size of the load torque compensation gain.

| F3-14 | ASR Output Filter Time | Range: 0.000 to 0.350 s | Factory Value: 0.008 s |
| --- | --- | --- | --- |

When the speed loop control bandwidth is high, the torque setpoint output by the speed regulator (ASR) usually contains some high-frequency noise. F3-14 is used to set the low-pass filter time constant for the torque setpoint. Increasing this parameter helps improve speed stability. Generally, no modification is required.

| F3-15 | Notch Depth | Range: 0 to 20 dB | Factory Value: 0 dB |
| --- | --- | --- | --- |
| F3-16 | Notch Filter Frequency | Range: 0.00 to 200.00 Hz | Factory Value: 0 Hz |

When resonance occurs in the mechanical system connected to the controlled motor, oscillations at that frequency will also appear in the motor speed, even though the speed loop control is stable. As shown in Fig. 2-11, setting F3-16 to this resonant frequency can effectively suppress oscillation in the torque setpoint; increasing F3-15 can enhance the suppression of oscillation at the resonant frequency.

Fig. 2-11 Notch filter frequency and depth

| F3-17 | Speed Limit Selection | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

This parameter is used to set the speed limit value in torque control mode.

0: Parameters FE-03&FE-04

Forward speed limit is set by FE-03, and reverse speed limit is set by FE-04.

1: Parameter & frequency source given

In closed-loop torque control mode with PG

When torque command≥0, the forward speed limit is the set value of the specified frequency source (F0-06), and the reverse speed limit is FE-04;

When torque command<0, forward speed limit is FE-03, reverse speed limit is the set value of the specified frequency source (F0-06).

In open-loop torque control mode

When torque command≥0, forward speed limit is FE-03, reverse speed is not limited;

When torque command<0, forward speed is not limited, reverse speed limit is FE-04.

2: Frequency source setting

Both forward and reverse speed limits are determined by the set value of the specified frequency source (F0-06).

3: Tension Control Line Speed

Both forward and reverse speed limits are determined by the line speed controlled by tension control. Refer to the detailed tension control description for more information.

| F3-18 | Current Loop Bandwidth | Range: 0～65535 Hz | Factory Value: 290 Hz |
| --- | --- | --- | --- |
| F3-19 | Iq Current Loop Proportional Gain | Range: 0%～65535% | Factory setting: 100% |
| F3-20 | Iq Current Loop Integral Gain | Range: 0%～65535% | Factory setting: 100% |
| F3-21 | Id Current Loop Proportional Gain | Range: 0%～65535% | Factory setting: 100% |
| F3-22 | Id Current Loop Integral Gain | Range: 0%～65535% | Factory setting: 100% |

F3-19～F3-22: Scale vector control current loop PI regulator parameters by percentage. Generally, no modification is required after successful motor parameter identification. If oscillation is detected in the current control loop during use, the proportional gain and integral gain here can be appropriately reduced. Conversely, if poor current tracking performance is observed, the proportional gain and integral gain here can be appropriately increased.

| F3-23 | I/F Mode Current Command | Range: 0%～250% | Factory value: 40% |
| --- | --- | --- | --- |

When F1-00 is 1, this parameter sets the percentage of the DC drive current relative to the motor rated current during the synchronous motor flux zero position alignment period;

When F3-24 is zero, this parameter sets the percentage of the low-speed magnetization current relative to the motor rated current when the SVC observer mode is running;

When F3-24 is non-zero, this parameter sets the percentage of the current command relative to the motor rated current when the motor I/F mode is running.

| F3-24 | IF Switching Frequency 1 | Range: 0.00～599.00 Hz | Factory value: 20.00 Hz |
| --- | --- | --- | --- |
| F3-25 | IF Switching Frequency 2 | Range: 0.00～599.00 Hz | Factory value: 20.00 Hz |

F3-24: When this parameter is non-zero, the synchronous motor is forced to operate in I/F mode below this frequency.

F3-25: This parameter is default equal to F3-25, generally no need for modification.

| F3-26 | Weak Magnetism Proportion Coefficient | Range: 0.00～655.35 | Factory Value: 1.00 |
| --- | --- | --- | --- |
| F3-27 | Weak Magnetism Integral Coefficient | Range: 0.00～655.35 | Factory Value: 0.20 |
| F3-28 | Output Voltage Limit Value | Range: 0%～115% | Factory setting: 100% |

The inverter can output AC voltage to the motor under specific DC bus voltage, which is limited by the PWM modulation degree. Therefore, after the motor operating speed exceeds the rated speed, it is usually necessary to adjust the demagnetization current in real time to control the motor voltage within the range achievable by PWM modulation.

F3-26, F3-27: These parameters are used to set the proportional gain and integral gain of the voltage amplitude PI regulator, respectively.

F3-28: Increasing this parameter can increase the modulation limit of the output voltage, generally set not to exceed 105%.

| F3-29 | Torque Filter Time | Range: 0.001～10.000 s | Factory value: 0.500 s |
| --- | --- | --- | --- |

This parameter is used to set the high-pass filter time constant for torque (voltage) compensation under VVC control mode for permanent magnet motors. It generally does not need to be modified.

| F3-30 | VVC Voltage Feedforward Gain | Range: 0.00～2.00 | Factory Value: 1.00 |
| --- | --- | --- | --- |

This parameter is used to adjust the magnitude of the output voltage feedforward under VVC control mode for permanent magnet motors, to meet the demand for rapid response under certain operating conditions.

Parameter F3-30 = 1.00 represents feedforward = back EMF × motor speed / motor rated speed.

When using VVC for induction motors, if L0-04 & 512 is not zero, this parameter is also effective for VVC of induction motors.

| F3-31 | VVC Compensation Low Pass | Range: 0.00～600.00 s | Factory Value: 0.05 s |
| --- | --- | --- | --- |

The time constant for high-pass filtering of frequency compensation under VVC mode. This value is typically 1% of F3-41 and generally does not require adjustment. If the actual motor speed oscillation frequency exceeds the filter frequency range corresponding to F3-31 and F3-41, appropriate adjustments can be made.

| F3-41 | VVC Compensation High Pass | Range: 0.00～655.35 s | Factory Value: 5.00 s |
| --- | --- | --- | --- |

The time constant for high-pass filtering of frequency compensation under VVC mode. This value is usually 100 times that of F3-31.

| F3-42 | VVC Compensation Gain | Range: 0.00～655.35 | Factory Value: 1.00 |
| --- | --- | --- | --- |

The gain value for frequency compensation under VVC mode; increasing this gain helps improve speed oscillation under VVC mode.

| F3-43 | PM Sensorless Control Bit | Range: 0000H~FFFFH | Factory value: 0 |
| --- | --- | --- | --- |

This parameter needs to be set bit by bit, as shown in Table 2-5:

Table 2-5 PM Sensorless Control Bit Description

| bit (bit) | Function | Description |
| --- | --- | --- |
| 0 | Current Loop Feedforward | 0: Feedforward Enabled 1: Feedforward Disabled |

| F3-45 | MTPA Low-pass Filter Coefficient | Range: 0～1000 | Factory value: 10 |
| --- | --- | --- | --- |

MTPA low-pass filter coefficient, generally does not need to be modified.

2.5 Parameters for the First Motor in Group F4

| F4-00 | Motor Type Selection | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

This parameter is used to set the motor type:

0: Induction Motor

1: Surface-Mounted Permanent Magnet Synchronous Motor

Corresponds to equal d-axis and q-axis inductances.

2: Interior Permanent Magnet Synchronous Motor

Corresponds to unequal d-axis and q-axis inductances.

3: Synchronous Reluctance Motor

| F4-01 | Motor Parameter Identification | Range: 0～13 | Factory value: 0 |
| --- | --- | --- | --- |

0: No Function

1: IM Advanced Rotary Identification

Includes static parameter identification, rotary parameter identification, and mutual inductance saturation curve identification of induction motors.

2: IM Static Identification

Used in scenarios where the induction motor cannot rotate initially.

4: PM Motor Pole Identification

5: PM Rotation Identification

Includes static identification and back-EMF parameter identification for permanent magnet motors.

6: IM Rotation Identification

Used in scenarios where the induction motor can rotate; it is recommended that the motor be lightly loaded or unloaded.

12: FOC Inertia Identification

13: PM Static Identification

Includes only the stator resistance and inductance identification for permanent magnet motors, does not include back-EMF parameter identification.

| F4-02 | M1 Maximum Frequency | Range: 0.00～599.00 Hz | Factory Value: 50.00 Hz |
| --- | --- | --- | --- |
| F4-03 | M1 Rated Frequency | Range: 0.00～599.00 Hz | Factory Value: 50.00 Hz |
| F4-04 | M1 Rated Voltage | Range: 0.0～510.0 V | Factory value: 380.0 V |

When F4-00 motor type is set to 0 (induction motor), the above parameters are used to set induction motor 1; otherwise, they are used to set permanent magnet synchronous motors or synchronous reluctance motors. Note that changing the F4-02 parameter will affect the motor's acceleration and deceleration speed. The current acceleration/deceleration time parameter corresponds to the time it takes for the motor to accelerate/decelerate from zero speed to the highest frequency.

| F4-05 | IM1 Rated Power | Range: 0.00～655.35 kW | Factory value: Model determined |
| --- | --- | --- | --- |

This parameter is used to set the rated power of induction motor 1.

| F4-06 | IM1 Number of Poles | Range: 2～20 | Factory value: 4 |
| --- | --- | --- | --- |

This parameter is a read-only parameter used to display the number of poles of induction motor 1. Based on the user input of M1 rated frequency (F4-03) and IM1 rated speed (F4-08), this parameter value is automatically calculated. This parameter equals twice the number of motor pole pairs.

| F4-07 | IM1 Rated Current | Range: 0.00A～655.35A (Model ≤93kW) 0.0A～6553.5A (Model ≥110kW) | Factory value: Model determined |
| --- | --- | --- | --- |

This parameter is used to set the rated current of induction motor 1. When the inverter model is ≤93kW, this parameter has two decimal places; for models with higher power, this parameter has one decimal place.

| F4-08 | IM1 Rated Speed | Range: 0～65535 rpm | Factory Value: 1410 rpm |
| --- | --- | --- | --- |

This parameter is used to set the rated speed of induction motor 1, measured in rpm (revolutions per minute). The inverter software will automatically calculate the number of poles of induction motor 1 based on this parameter and the M1 rated frequency (F4-03).

| F4-09 | Inertia Per Unit Value | Range: 1～65535 pu | Factory Value: 256 pu |
| --- | --- | --- | --- |

This parameter is used to set the per unit value of inertia for induction motor 1 and synchronous motors. For a small number of motors with abnormally high inertia, if the speed loop proportional parameter is set to the maximum value of 100 and the speed response is still too slow, this parameter can be appropriately increased to further speed up the speed loop response.

| F4-10 | IM1 No-load Current | Range: 0.00A～F4-07 | Factory value: Model determined |
| --- | --- | --- | --- |

This parameter is used to set the no-load current value of induction motor 1, which is typically obtained through induction motor parameter identification.

When the inverter model ≤93kW, this parameter has two decimal places; for higher power models, this parameter has one decimal place.

| F4-11 | IM1 Stator Resistance | Range: 0.000～655.35Ω or mΩ | Factory value: Model determined |
| --- | --- | --- | --- |
| F4-12 | IM1 Rotor Resistance | Range: 0.000～655.35Ω or mΩ | Factory Value: 0.000 Ω or mΩ |

This parameter is used to set the stator resistance and rotor resistance of Induction Motor 1, typically obtained through induction motor parameter identification. When the inverter model ≤185kW, the unit of this parameter is Ω; for models with higher power, the unit of this parameter is mΩ.

| F4-13 | IM1 Mutual Inductance | Range: 0.0～6553.5mH (models ≤185kW) 0.00～655.35mH (models ≥200kW) | Factory Value: 0.0 or 0.00 |
| --- | --- | --- | --- |

This parameter is used to set the mutual inductance value of Induction Motor 1, typically obtained through induction motor parameter identification. For inverter models ≤185kW, this parameter has one decimal place; for models with higher power, this parameter has two decimal places.

| F4-14 | IM1 leakage inductance | Range: 0.00～655.35mH (models ≤185kW) 0.000～65.535mH (models ≥200kW) | Factory value: 0.00 or 0.000 |
| --- | --- | --- | --- |

This parameter is used to set the total leakage inductance value of induction motor 1, which is usually obtained through induction motor parameter identification. For models ≤185kW, this parameter has two decimal places; for higher power models, this parameter has three decimal places.

| F4-15 | PM rated power | Range: 0.00～655.35 kW | Factory value: Model determined |
| --- | --- | --- | --- |

This parameter is used to set the rated power of permanent magnet synchronous motors or synchronous reluctance motors.

| F4-16 | PM pole number | Range: 0～65535 | Factory value: 10 |
| --- | --- | --- | --- |

This parameter is a read-only parameter used to display the number of poles of the synchronous motor. Based on the user-input M1 rated frequency (F4-03) and PM rated speed (F4-18), this parameter value is automatically calculated. This parameter equals twice the number of motor pole pairs.

| F4-17 | PM rated current | Range: 0.00A～655.35A (Model ≤93kW) 0.0A～6553.5A (Model ≥110kW) | Model Confirmation |
| --- | --- | --- | --- |

This parameter is used to set the rated current for permanent magnet synchronous motors or synchronous reluctance motors. When the inverter model ≤ 93kW, this parameter has two decimal places; for higher power models, this parameter has one decimal place.

| F4-18 | PM rated speed | Range: 0～65535 rpm | 2000 rpm |
| --- | --- | --- | --- |

This parameter is used to set the rated speed of the synchronous motor, with units of rpm (revolutions per minute). The inverter software will automatically calculate the number of poles of the synchronous motor based on this parameter and the M1 rated frequency (F4-03).

| F4-19 | PM motor inertia | Range: 0.0～6553.5 kg·m2 | Model Confirmation |
| --- | --- | --- | --- |

This parameter is used to set the moment of inertia of the synchronous motor, and it is generally recommended not to modify it.

| F4-20 | PM Stator Resistance | Range: 0.000～655.35Ω or mΩ | Factory value: Model determined |
| --- | --- | --- | --- |

This parameter is used to set the stator resistance and rotor resistance of the synchronous motor, which are usually obtained through synchronous motor parameter identification.

When the inverter model is ≤185kW, the unit of this parameter is Ω; for models with greater power, the unit of this parameter is mΩ.

| F4-21 | PM D-axis Inductance | Range: 0.00～655.35mH (models ≤185kW) 0.000～65.535mH (models ≥200kW) | Factory value: 0.00 or 0.000 |
| --- | --- | --- | --- |
| F4-22 | PM Q-axis Inductance |  |  |

This parameter is used to set the D-axis inductance and Q-axis inductance of the synchronous motor, which usually needs to be obtained through synchronous motor parameter identification.

When the inverter model is ≤185kW, this parameter has 2 decimal places; for models with greater power, this parameter has 3 decimal places.

| F4-23 | PM Ke Parameter | Range: 0～65535 V/krpm | Factory value: 0 V/krpm |
| --- | --- | --- | --- |

This parameter is used to set the back EMF constant of the synchronous motor, with units of V/krpm, which usually needs to be obtained through synchronous motor parameter identification.

| F4-26 | PM Magnetic Pole Angle | Range: 0.0～6553.5 degrees | Factory value: 0.0 degrees |
| --- | --- | --- | --- |

This parameter is used to set the initial position angle of the synchronous motor. It usually needs to be obtained through initial position identification of the synchronous motor and does not require modification.

| F4-27 | Encoder Type Selection | Range: 0～5 | Factory value: 0 |
| --- | --- | --- | --- |

0: No PG card

1: ABZ encoder

2: Reserved

3: Resolver

| F4-28 | Encoder Pulse Count | Range: 6～20000 ppr | Factory value: 1024 ppr |
| --- | --- | --- | --- |

This parameter is used to set the number of lines of the selected incremental encoder, indicating the number of A/B signal pulses output by the encoder per rotation. The higher this value, the higher the resolution of the encoder, and the higher the accuracy of speed and position measurement.

If this parameter is set incorrectly, it can cause overcurrent in the inverter output and motor overspeed during closed-loop speed control. When using a permanent magnet motor, it can also result in incorrect initial position identification of the rotor. When changing the encoder results in a change in the number of encoder lines, it is necessary to perform the rotor position dynamic identification again (parameter F4-01=4).

| F4-29 |  | Encoder Pulse Type | Range: 0～5 | Factory value: 1 |
| --- | --- | --- | --- | --- |

0: No Function

1: A phase leading B phase indicates forward rotation

2: B phase leading A phase indicates forward rotation

The pulse signal diagram for forward rotation where A pulse leads B pulse is shown inFigure 2-12.

Figure 2-12 A phase pulse leading B phase pulse

The pulse signal diagram for forward rotation where B pulse leads A pulse is shown inFigure 2-13.

Figure 2-13 B phase pulse leading A phase pulse

| F4-30 | Reference Pulse Input Type | Range: 0～5 | Factory value: 0 |
| --- | --- | --- | --- |

0: No Function

1: A phase leading B phase indicates forward rotation

2: B phase leading A phase indicates forward rotation

The pulse signal diagram for forward rotation where A pulse leads B pulse is shown inFigure 2-14.

Figure 2-14 A-phase pulse leads B-phase pulse

The diagram showing B-phase pulse leading A-phase pulse for positive rotation isFigure 2-15:

Figure 2-15 B-phase pulse leads A-phase pulse

| F4-32 | Rotary Encoder Pole Pairs | Range: 1～50 | Factory value: 1 |
| --- | --- | --- | --- |

This parameter is used to set the number of pole pairs of the rotary encoder.

Note: This can only be modified when parameter F3-27=3

| F4-34 | PG feedback filter | Range: 0～2000 Hz | Factory value: 100 Hz |
| --- | --- | --- | --- |

At normal speed,

PG feedback speed digital low-pass filter cutoff frequency.

| F4-35 | PG feedback zero-speed filter | Range: 0～2000 Hz | Factory value: 50 Hz |
| --- | --- | --- | --- |

Near zero speed,PG feedback speed digital low-pass filter cutoff frequency.

| F4-36 | Load-side Gear A1 | Range: 1～65535 | Factory Value: 100 |
| --- | --- | --- | --- |
| F4-37 | Motor side gear B1 | Range: 1～65535 | Factory Value: 100 |
| F4-38 | Load-side Gear A2 | Range: 1～65535 | Factory Value: 100 |
| F4-39 | Motor side gear B2 | Range: 1～65535 | Factory Value: 100 |

The two sets of gear ratios can be switched via digital input terminals, terminal function set to 48, and when the terminal status is valid, the A2 and B2 gear ratios are effective; otherwise,theA1 and B1 gear ratios are effective. An encoder-equipped variable frequency drive system with mechanical gears is shown in Figure 2-16.

Figure 2-16Encoder-equipped variable frequency drive system with mechanical gears

| F4-40 | Electronic Gear A | Range: 1～65535 | Factory Value: 100 |
| --- | --- | --- | --- |
| F4-41 | Electronic Gear B | Range: 1～65535 | Factory Value: 100 |
| F4-42 | Synchronous reluctance no-load current | Range: 0.00～13.00A | Factory Value: 5.20A |

The electronic gear is used to set the ratio for amplifying or reducing position control commands; actual position command = position command * F4-40/F4-41. When PG2 is used as a frequency command, actual frequency command = PG2 frequency command × F4-40/F4-41.

2.6 F5 Group Input Terminals

The CM680 series inverter is equipped with 8 multifunctional digital input terminals, among which HDI8 can be used as a high-speed pulse input terminal, and there are also 3 analog input terminals.

| F5-00 | DI1 Terminal Function Selection | Range: 0～94 | Factory value: 0 |
| --- | --- | --- | --- |
| F5-01 | DI2 Terminal Function Selection | Range: 0～94 | Factory value: 0 |
| F5-02 | DI3 Terminal Function Selection | Range: 0～94 | Factory value: 1 |
| F5-03 | DI4 Terminal Function Selection | Range: 0～94 | Factory Value: 2 |
| F5-04 | DI5 Terminal Function Selection | Range: 0～94 | Factory Value: 3 |
| F5-05 | DI6 Terminal Function Selection | Range: 0～94 | Factory value: 4 |
| F5-06 | DI7 Terminal Function Selection | Range: 0～94 | Factory value: 0 |
| F5-07 | HDI8 Terminal Function Selection | Range: 0～94 | Factory value: 0 |

The above function codes are used to set the functions of the multifunctional digital input terminals, as shown in Table2-6.

Note: Only when parameter F5-08is equal to 0 can F5-00, F5-01, and F5-02be normally modified (set function). When F5-08is a non-zero parameter, the functions of F5-00, F5-01, and F5-02are affected by F5-08. (F5-02is only controlled by F5-08when F5-08=3or F5-08=6; at other times, it can be set to normal MI function), refer to the description section of Table 2-6 for details.

Table 2-6 Functions Corresponding to Digital Multi-function Input Terminals

| Set Value | Function | Description |
| --- | --- | --- |
| 0 | No function | The inverter does not process digital terminal signals. |
| 1 | Multi-speed/Multi-point Position 1 | A total of 16 segments can be set through the combination of states of four digital input terminals, detailed combinations are shown below. |
| 2 | Multi-speed/Multi-point Position 2 |  |
| 3 | Multi-speed/Multi-point Position 3 |  |
| 4 | Multi-speed/Multi-point Position 4 |  |
| 5 | Fault Reset | External fault reset function. It has the same function as the STOP key on the digital operator. |
| 6 | Jog Operation | Jog operation, which has the same function as the JOG key on the digital operator. The jog operation set frequency, acceleration and deceleration times refer to F7-00, F7-01, F7-02. |
| 7 | Speed Hold | During the inverter acceleration or deceleration process, if the external terminal function is selected as speed hold and the terminal status is valid, the inverter maintains the current speed operation; when the terminal status is invalid, the inverter continues to accelerate or decelerate to the set value. |
| 8 | 1-2 segment acceleration/deceleration switching | Firstandsecondacceleration/deceleration time switching, default usethefirstand fourth acceleration/deceleration times, both switch through thefirstand fourth acceleration/deceleration switching frequency (F1-15), select multi-function input terminal, can switch terminal status to achieve firstand secondacceleration/deceleration time switching. |
| 9 | 3-4 segment acceleration/deceleration switching | Third and fourth acceleration/deceleration time switching, select multi-function input terminal, can switch terminal status to achieve third and fourth acceleration/deceleration time switching. |
| 10 | External Fault | External Fault Input (External Fault), when the status of the multifunctional digitalterminal is valid, the inverter decelerates and stops according to the emergency or forced shutdown decelerationmethod (F1-23). The Digital Operator displaysEF until the external terminal status returns to normal. After fault reset (RESET), the inverter can continue to operate. |
| 11 | Base Block | Base Block (Base Block) Input, when the status of the multifunctional digitalterminal is valid, the inverter immediately stopsoutput, and the motorfreely decelerates and stops.The Digital Operator displaysB.B. |
| 12 | Stop Output | Output Stop, when the set multifunctional digital terminalisvalid, the inverter will immediately stopoutput, the motorwillfreely decelerate to a stop, and the inverter enters output wait mode until the input IO signal returns to normal, at which point the inverter restartstothe current set frequency. |
| 13 | Automatic Acceleration/Deceleration Prohibited | CancelAutoAcceleration/Deceleration Setting, you need to set the Auto Acceleration/Deceleration setting (F1-20) to non-linear acceleration/deceleration. By switching the state of the multifunctional input IOport, you can switch between Auto Acceleration/Deceleration and linear acceleration/deceleration modes. |
| 15 | AI1 Input Frequency Command | Speed command comesfromAI1; when the set multifunctional input terminalstatusis valid, the inverter's frequency command source is forced to AI1. When setting other AI command sourcesatthe same time, AI1 has the highest priority. |
| 16 | AI2 Input Frequency Command | Speed Command comesfromAI2, when the set multi-function input terminalstatus is valid, the inverter frequency command source is forcibly set toAI2. |
| 17 | AI3 Input Frequency Command | Speed Command comesfromAI3, when the set multi-function input terminalstatus is valid, the inverter frequency command source is forcibly set toAI3. |
| 18 | Deceleration Stop | Forced Shutdown, when the set multi-function input terminalstatus is valid, the inverter will decelerate and stop according to the emergency or forced shutdown deceleration method(F1-23). |
| 19 | Frequency Increase External Command | Frequency increment command, requires setting the frequency command source (F0-06 = 3) to external UP/DOWN terminals, when the configured multi-function input terminal status is active, the inverter's frequency setting will increase one unit. If the input terminal remains active, the frequency command will increment according to the settings for the external terminal UP/DOWN key mode (F5-09) and the external terminal UP/DOWN key acceleration/deceleration rate (F5-10) until it reaches the maximum frequency command value. |
| 20 | External frequency decrease command | Frequency decrement command, requires setting the frequency command source (F0-06 = 3) to external UP/DOWN terminals, when the configured multi-function input terminal status is active, the inverter's frequency setting will decrement one unit. If the input terminal remains active, the frequency command will decrement according to the settings for the external terminal UP/DOWN key mode (F5-09) and the external terminal UP/DOWN key acceleration/deceleration rate (F5-10) until it reaches the minimum frequency command value. |
| 21 | PID function disabled | PID function cancellation, when the set multifunctional input terminalstatus is valid, thePID function becomes invalid. |
| 22 | Clear counter | Counter reset, when the set multifunctional input terminalstatus is valid, the inverter will reset the current count display value to zero. When the input terminalstatus switches to invalid, the inverter will resume counting. |
| 23 | Input count | Each time the input terminalstatus is valid,the digital operatorwill automatically increment the displayed count value by 1. The count value reset judgment is determined by the last count value reaching the set (L6-00). |
| 24 | External Forward Jog | Need to set the operation command source to external terminals (F0-05 = 1), when the configured multi-function input terminal status is valid, the inverter will perform forward jog operation. When executing the JOG command in torque mode, the inverter forcibly switches to speed mode, and after the JOG command disappears, it automatically returns to speed mode. |
| 25 | External Reverse Jog | Need to set the operation command source to external terminals (F0-05 = 1), when the configured multi-function input terminal status is valid, the inverter will perform reverse jog operation. When executing the JOG command in torque mode, the inverter forcibly switches to speed mode, and after the JOG command disappears, it automatically returns to speed mode. |
| 26 | Torque/Speed Mode Switching | Torque Speed Mode Switching, when the set multifunctional input terminalstatus is valid, the inverter will operate in torque mode,or otherwise, it will operate in speed mode. |
| 27 | Speed Loop 1/2 Switching | ASR1/ASR2 Switching, low-speed andhigh-speed zone speed loop control parameter switching, when the set multifunctional input terminalstatus is valid, the inverter will perform speed control with high-speed zone control parameters (ASR2),or otherwise, it will use low-speed zone control parameters (ASR1). |
| 28 | External Fault Free Stop | EmergencyStop, when the set multifunctional input terminalstatus is valid, the inverter immediately stopsoutput and displays EF1 on the digital operator, the motorfreely decelerates to a stop, when the external input terminalstatus returns to normal, fault reset must be performed throughRESET before it can continue to run. |
| 31 | High torque compensation amount | Torque command bias, torque command bias source (FE-05) set to 3 (torque command bias source is external terminal), multi-function input terminal set to31, when the multi-function terminal input status is valid, the high torque command bias compensation value is high torque command compensation (FE-07), multi-function input terminal set to32, when the multi-function terminal input status is valid, the medium torque command bias compensation value is medium torque command compensation (FE-08), multi-function input terminal set to33, when the multi-function terminal input status is valid, the low torque command bias compensation value is low torque command compensation (FE-09). |
| 32 | Intermediate Torque Compensation | Please see the setting value is 31. |
| 33 | Low Torque Compensation | Please see the setting value is 31. |
| 34 | Multi-speed/Position Switching | Terminal selection for multi-speed and multi-position function switching,the set multifunctional input terminalstatus is valid when selecting multi-position command terminals, otherwise, they are used to select multi-speed command terminals. |
| 35 | Internal Positioning Enable | Enable single-point positioning, when the set multifunctional input terminalstatus is valid, the inverter performs internal single-point positioning according to the encoder's internal positioning position (FC-20). Only supports FVCcontrol mode. |
| 36 | Multi-point Position Input | When the set multifunctional input terminalstatus is valid, the inverter writes the current position information into the corresponding multi-segment position command parameter for manual calibration of the multi-segment position command parameter setting. |
| 37 | Pulse Input Position Control | Enable full-range position control pulse command input, select the inverter frequency command source as pulse input (F0-06 = 4,5). When the set multifunctional input terminalstatus is valid, the pulse input on the PG card serves as the position command. It is recommended to set the APR feedforward gain (FC-25) to 0. |
| 38 | Prohibit Writing to EEPROM | Prohibit writing to EEPROM,when the set multifunctional input terminalstatus is valid, some parameter modifications will not be saved toEEPROM, and will not be retained after power loss. |
| 39 | Torque Command Direction Switching | Torque Command Direction, Applicable for Torque Control Mode, when the torque command source is AI1/AI2/AI3, if the set multifunctional input terminal status is valid, the torque command automatically converts to a negative value. |
| 40 | Motor Free Stop | Forced Free Run Stop, during the operation of the inverter, when the set multifunctional input terminal status is valid, the inverter immediately stops output, and the motor freewheels to a stop. |
| 41 | Manual Mode Enable | HAND Switch, when the set multifunctional input terminal status is valid, the inverter switches to HAND mode, and the frequency command source and operation command source switch to being determined by parameters L1-00 and L1-01. When the multi-function input terminalstatus isinvalid, the inverter will stop (it will be unable to start in standby mode). |
| 42 | Automatic mode enable | When the multi-function input terminal status set for AUTO switching is valid, the inverter switches toAUTO mode, and the frequency command source and operationcommand source switch according to parameters F0-06 and F0-05. When the multi-function input terminalstatus isinvalid, the inverter will stop (it will beunable to start in standby mode). |
| 48 | Mechanical gear ratio switching | When the multi-function input terminalstatus set for mechanical gear ratio switching is valid, the mechanical gear ratio switches to thesecondset (from F4-36,F4-37 to F4-38, F4-39). |
| 49 | Inverter Enable | Inverter Enable, by default, the inverter enable is effective, allowing the inverter to control start and stop. When the inverter is not enabled, the operation command is invalid. If the inverter enable fails during operation, the motor will freely decelerate and stop. Switching the status of the multi-function input terminal can change the inverter enable status. Once the multi-function input terminal is configured as the inverter enable, the inverter can only be started by setting the multi-function input terminal status to effective. |
| 50 | Slave dEB (instant stop without stopping) action input | Slave dEB action input, when the master station experiences a dEB action, this information is input to notify the slave station to also perform a dEB action, ensuring that the slave station can stop simultaneously. |
| 53 | CANopen Quick Stop Trigger | CANopen Quick Stop Trigger,When CANopen control is set, if the designated multifunctional input terminalisvalid, the inverter operation state is forcibly switched to quick stop. |
| 56 | Local/Remote Switching | LOCAL/REMOTE Switch Selection,When LOCAL/REMOTE action selection is set to LOCAL/REMOTE switching (L1-02 is not 0),at this time, the higher-level machine will displayLOC/REM status. If the designated multifunctional input terminalisvalid, the inverter operates inLOCAL mode; otherwise, REMOTE mode. |
| 70 | Auxiliary Frequency Forced to 0 | Auxiliary Frequency Forced to 0,When the auxiliary frequency function is enabled (auxiliary frequency source F0-07 is not 0), if the designated multifunctional input terminalisvalid, the auxiliary frequency is forced to0. If PID is the main frequency, PID will continue to operate. |
| 71 | PID function disabled, PID output forced to 0 | PID function disabled,PID output forced to 0, auxiliary frequency function enabled (auxiliary frequency source F0-07 not 0), and when using the PID function, if the set multifunctional input terminalisvalid, thePID function stops, integral value is reset, andPID output is reset. |
| 72 | PID function disabled, PID maintains current output level | PID function disabled,PID maintains previous output value, auxiliary frequency function enabled (auxiliary frequency source F0-07 not 0), and when using the PID function, if the set multifunctional input terminalisvalid, thePID function stops, andPID maintains previous output. |
| 73 | Force PID integral gain to 0 | Force PIDintegral gain to 0, integral action is disabled, when the set multifunctional input terminaliseffective,PID continuestowork, but the integrator stopsworking, and the integrator output is cleared. |
| 74 | Invert PID feedback | Invert PID feedback, when the set multifunctional input terminaliseffective,the sign of the PID feedback value is reversed; if the PID feedback is positive, it is converted to negative, and if the PID feedback is negative, it is converted to positive. |
| 83 | Select bit0 for multiple induction motors | Select multiple (induction) motors, when the set multifunctional input terminaliseffective,switch different motor parameters based on the terminalstate combination, the working frequency and V/F curve parameters corresponding to different motor parameters need to refer to the basic parameter group. The combination relationship is shown in Table 2-9. |
| 84 | Multiple Induction Motor Selection Bit 1 | Same as Above. |
| 86 | Diameter Reset | Diameter Reset, under Tension Control Mode (Tension Control Mode FB-00 not 0), when the set multifunctional input terminal is effective, the current diameter (FB-38) resets to the initial diameter or empty roll diameter. |
| 87 | Initial Diameter Selection 1 | Initial Diameter Selection, under Tension Control Mode (Tension Control Mode FB-00 not 0), when the set multifunctional input terminal is effective, the current diameter (FB-38) can reset to the initial diameter (FB-30, FB-31). The terminal selection relationship is shown in Table 2-10. |
| 88 | Initial Diameter Selection 2 | Same as Above. |
| 89 | Tension PID Control Integral Reset | Tension PIDControl Integral Reset, in TensionControl Mode (TensionControl Mode FB-00 is not 0), when the designated multi-function input terminalis effective, the integrator output is reset. |
| 90 | Diameter Calculation Hold | Diameter Calculation Hold, when the designated multi-function input terminalis effective, the diameter calculation update stops. |
| 91 | Diameter Mode Selection (0: Winding In 1: Winding Out) | Drum Mode Selection, when the designated multi-function input terminalis effective, the drum mode switches to winding out; otherwise, it is winding in. |
| 92 | Disable Tension PID | DisableTensionPID, when the set multifunctional input terminalisvalid, disablethetensionPID function. |
| 93 | Pause Tension PID | Pause Tension PID, when the set multifunctional input terminalisvalid, pause the tensionPID function. |
| 94 | Automatic Diameter Switching | AutoDiameter Switching, when the set multifunctional input terminalisvalid, autoswitch the diameter. |

Note:
    1. All digital input terminals are trigger-effective, meaning that after each power-on, the terminal status needs to switch from invalid to valid for the inverter to recognize the terminal input status as valid. If the terminal input status remains valid before and after power-on, the inverter will consider the terminal input status as invalid. If the terminal function is selected as2/3wire or jog operation, you can use function codeF5-19to choose whether the inverter runs directly when the power is turned on or after a fault reset, if the terminal status for the selected terminal operation function is valid.
    2. When 2/3 wire control is selected (F5-08 is not 0), digital input terminals DI1 and DI2 are forced to forward and reverse input, and other functions cannot be selected. To select other functions for DI1 and DI2, F5-08 must be set to 0.

- Before changing the digital terminal function, ensure that the corresponding terminal status has been reset to an invalid state to guarantee that the previously selected function is no longer active when selecting the next function.

Multi-speed/Multi-position 1, 2, 3, 4 function description (set value as 1/2/3/4):

Four multi-segment command terminals can be combined into 16 states, these 16 states correspond to 16 command set values. Taking the default configuration as an example (F5-02 = 1, F5-03 = 2, F5-04 = 3, F5-05 = 4, F5-16 = 0 low level effective), the relationship between DI status and multi-segment speed mapping is shown in Table 2-7 or Figure 2-17.

Table 2-7 DI Status and Multi-Segment Speed Mapping Relationship

| DI6 | DI5 | DI4 | DI3 | Frequency setting | Corresponding Function Code |
| --- | --- | --- | --- | --- | --- |
| H | H | H | H | Multi-Speed 0 | FD-00 |
| H | H | H | L | Multi-Speed 1 | FD-01 |
| H | H | L | H | Multi-Speed 2 | FD-02 |
| H | H | L | L | Multi-Speed 3 | FD-03 |
| H | L | H | H | Multi-Speed 4 | FD-04 |
| H | L | H | L | Multi-Speed 5 | FD-05 |
| H | L | L | H | Multi-speed 6 | FD-06 |
| H | L | L | L | Multi-speed 7 | FD-07 |
| L | H | H | H | Multi-speed 8 | FD-08 |
| L | H | H | L | Multi-speed 9 | FD-09 |
| L | H | L | H | Multi-speed 10 | FD-10 |
| L | H | L | L | Multi-speed 11 | FD-11 |
| L | L | H | H | Multi-Speed 12 | FD-12 |
| L | L | H | L | Multi-Speed 13 | FD-13 |
| L | L | L | H | Multi-Speed 14 | FD-14 |
| L | L | L | L | Multi-Speed 15 | FD-15 |

Figure 2-17 DI Corresponding Multi-Segment Speed

Speed Hold Function Description (Set Value is 7):

When this function is invalid, normal acceleration and deceleration occur; When this function is valid, the current frequency remains unchanged, as shown in Figure2-18.

Figure 2-18 Speed Holding Function Schematic

Torque/Speed Mode Switching Function (Setting Value 26) Explanation:

When this function is invalid, the inverter operates in speed mode, running according to the frequency command, with the torque command serving as torque limitation; When this function is valid, the inverter operates in torque mode, running according to the torque command, with the frequency command serving as frequency limiting,as shown in Figure2-19.

Figure 2-19 Torque/Speed Mode Switching Function

External Fault Free Stop Function (Setting Value 28) Explanation:

When this function is invalid, the inverter operates normally; When this function is effective, the inverter reports an external fault and immediately stops operation; it needs to be reset before it can be restarted, as shown in Figure 2-20.

Figure 2-20 External Fault Free Stop Function

Internal Positioning Enable (terminal function selection setting value 35) Explanation:

When this function is ineffective, the inverter operates in normal speed mode; When this function is effective, the inverter will stop at the position set by FC-20, the internal positioning logic is shown in Figure 2-21.

Figure 2-21 Internal Positioning EnableExplanation

Pulse Input Position Control Function (terminal function selection setting value 37) Explanation:

When this function is effective, the inverter performs position control based on the pulse input. If the internal positioning enable is also effective at this time, it will perform home position control, as shown in Figure 2-22.

Figure 2-22 pulse input position control function

Manual and automatic mode enable functions (set values 41, 42) description:

The manual and automatic mode enable functions can switch between manual and automatic modes, as shown in Table2-8.

Table 2-8 manual and automatic mode enable functions

| Manual mode enable function | Automatic mode enable function | Inverter operating mode |
| --- | --- | --- |
| Invalid | Invalid | stop operation |
| Valid | Invalid | Manual mode |
| Invalid | Valid | Automatic mode |
| Valid | Valid | stop operation |

Multiple induction motor selection function (set values 83 and 84) description:

The multiple induction motor selection function can select induction motor parameters, as shown in Table2-9.

Table 2-9 multiple induction motor selection function

| Multiple Induction Motor Selection Bit 1 | Select bit0 for multiple induction motors | Induction Motor |
| --- | --- | --- |
| Invalid | Invalid | Motor 1 |
| Invalid | Valid | Motor 2 |
| Valid | Invalid | Motor 3 |
| Valid | Valid | Motor 4 |

Initial Roll Diameter Selection Function (Set Values 87 and 88) Description:

This function can select the initial roll diameter, as shown in Table2-10.

Table 2-10Initial Roll Diameter Selection Function

| Initial Diameter Selection 2 | Initial Diameter Selection 1 | Initial Roll Diameter Selection |
| --- | --- | --- |
| Invalid | Invalid | Initial Roll Diameter 0 (FB-29) |
| Invalid | Valid | Initial Roll Diameter 1 (FB-30) |
| Valid | Invalid | Initial Roll Diameter 2 (FB-31) |

| F5-08 | Terminal Command Mode | Range: 0～6 | Factory value: 1 |
| --- | --- | --- | --- |

This function code is used to set the terminal command mode.

0: No Function

1: 2-Wire Mode 1

2: 2-wire Mode 2

3: 3-wire

4: 2-wire Mode 1/Fast Start

5: 2-wire Mode 2/Fast Start

6: 3-wire Fast Start

Table 2-11 provides the wiring diagram for the terminal command mode.

Table 2-11 Wiring Diagram for Terminal Command Mode

| Terminal Command Mode (F5-08) | External Terminal Wiring Schematic |
| --- | --- |
| 1: 2-Wire Mode 1 |  |
| 2: 2-wire Mode 2 |  |
| 3: 3-wire |  |
| 4: 2-wire Mode 1/Fast Start | Same as Mode 1 |
| 5: 2-wire Mode 2/Fast Start | Same as Mode 2 |
| 6: 3-wire Fast Start | Same as Mode 3 |

Note: When set to Quick Start mode, the inverter will immediately respond to the start command. During shutdown, there will be a relativelylargevoltage at the inverter output terminals.

| F5-09 | UP/DOWN Mode Selection | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |
| F5-10 | UP/DOWN Rate of Change | Range: 0.001～1.000Hz/ms | Factory setting: 0.001 |

This function code is used to set the UP/DOWN mode.

0: System Acceleration/Deceleration Time

When the external UP/DOWN signal is valid, the inverter accelerates and decelerates according to the normal acceleration and deceleration times (F0-13, F0-14, F7-03～F7-08).

1: Constant Speed

When the external UP/DOWN signal is valid, the inverter accelerates and decelerates according to the UP/DOWN rate of change (F5-10).

2: Step Acceleration/Deceleration (1step/pulse)

When an external UP/DOWN signal produces a valid pulse, the inverter performs a step acceleration or deceleration according to the UP/DOWN change rate (F5-10).

3: Exponential Curve

When the external UP/DOWN signal is valid, the inverter performs acceleration or deceleration according to the exponential curve.

4: Step Acceleration/Deceleration (1step/100ms)

When the external UP/DOWN signal is valid, the inverter performs a step acceleration or deceleration every 100ms according to the UP/DOWN change rate (F5-10).

| F5-15 | DI Terminal Valid Logic | Range: 0000H~FFFFH | Factory value: 0 |
| --- | --- | --- | --- |

This function code can be used to set the effective logic level of the DI terminal. The table below lists the effective logic control bits for each DI terminal. If the corresponding control bit is 0, the effective logic level remains unchanged; if the corresponding control bit is 1, the effective logic level is inverted. The effective logic control bits for DI terminals are shown in Table 2-12.

Table 2-12 Valid Logic Control Bits for DI Terminals

| Second digit from the right: Hexadecimal 0～F |  |  |  | First digit from the right: Hexadecimal 0～F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HDI8 | DI7 | DI6 | DI5 | DI4 | DI3 | DI2 | DI1 |
| Fourth digit from the right: Hexadecimal 0～F |  |  |  | Third digit from the right: Hexadecimal 0～F |  |  |  |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| F5-16 | DI Terminal Response Time | Range: 0.000s～30.000s | Factory Value: 0.005s |
| --- | --- | --- | --- |

This function code is used to set the sensitivity of the DI terminals. If the DI terminals experience false triggering due to interference, this parameter can be increased to enhance the DI terminal's resistance to interference, but the sensitivity of the DI terminals will also decrease accordingly.

| F5-17 | Virtual/Real DI Terminal Selection | Range: 0000H~FFFFH | Factory value: 0 |
| --- | --- | --- | --- |

This function code can be used to set whether the DI terminal signals come from actual terminals or virtual terminals. Table2-13 lists the virtual/actual control bits for each DI terminal. If the corresponding control bit is 0, it selects the actual terminal; if the corresponding control bit is 1, it selects the virtual terminal. Virtual terminal signals are set through F5-18.

Table 2-13 Virtual/Actual DI Terminal Selection

| Second digit from the right: Hexadecimal 0～F |  |  |  | First digit from the right: Hexadecimal 0～F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HDI8 | DI7 | DI6 | DI5 | DI4 | DI3 | DI2 | DI1 |
| Fourth digit from the right: Hexadecimal 0～F |  |  |  | Third digit from the right: Hexadecimal 0～F |  |  |  |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| F5-18 | Virtual Terminal Status Setting | Range: 0～65535 | Factory value: 0 |
| --- | --- | --- | --- |

This function code can be used to set the virtual DI terminal signals. Table2-14 lists the signal control bits for each virtual DI terminal. If the corresponding signal control bit is 0, the corresponding virtual terminal signal is invalid; if the corresponding signal control bit is 1, the corresponding virtual terminal signal is valid.

Table 2-14 Virtual Terminal Status Setting

| Second digit from the right: Hexadecimal 0～F |  |  |  | First digit from the right: Hexadecimal 0～F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HDI8 | DI7 | DI6 | DI5 | DI4 | DI3 | DI2 | DI1 |
| Fourth digit from the right: Hexadecimal 0～F |  |  |  | Third digit from the right: Hexadecimal 0～F |  |  |  |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| F5-19 | External Operation Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the response of the inverter to external terminal command signals when powered on and after fault reset.

0: Invalid

When powered on or after a fault reset, if there is a terminal operation command, the inverter does not run until the terminal command signal is canceled. The inverter will only run again upon receiving another terminal operation command.

1: Run if there is an operation command

When powered on or after a fault reset, if there is a terminal operation command, the inverter runs.

| F5-20 | AI1 signal type selection | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the AI1 signal type.

0: 0-10V input selection

Voltage signal input, with an input range of 0-10V.

1: 0-20mA input selection

Current signal input, with an input range of 0-20mA.

2: 4-20mA input selection

Current signal input, input range is4-20mA.

| F5-21 | AI1 function selection | Range: 0～20 | Factory value: 1 |
| --- | --- | --- | --- |

This function code is used to set the AI1 function, the analog input function selection is shown inTable 2-15:

Table 2-15 Analog InputFunction Selection

| Set Value | Function | Description |
| --- | --- | --- |
| 0 | No function | AI terminal has no function |
| 1 | Frequency setting | When used as frequency setting, the frequency source (F0-06) must be set to 2 |
| 2 | Torque setting | When used as torque setting, the inverter control mode (F0-03) must be set to 2 (torque mode), the torque command source selection (FE-01) must be set to 2, and parameters F5-21, F5-27, or F5-33 must be set to 2. |
| 3 | Torque compensation setting | When used as torque compensation, the inverter must set the torque command bias source (FE-05) to 1 (from external analog input). |
| 4 | PID target value | When used as a PID target value, the PID feedback terminal (FA-00 not equal to 0) must be set, and there are two other cases: 1. The PID target value source (FA-01) is set to 0 (from frequency setting), parameters F5-21 or F5-27 or F5-33 are set to 1 or 4; if both 1 and 4 are set in the analog input simultaneously, AI1 is prioritized as the PID target value. 2. The PID target value source (FA-01) is set to 3 (from external analog input). The PID target value will be displayed in percentage form via parameter FA-02 (PID target value setting) (precision 0.01%). |
| 5 | PID feedback value | When used as PID feedback, PID feedback selection (FA-00) must be set to 1 or 4, and the feedback value is displayed in percentage form via U1-04 (Communication setting PID feedback value)（precision0.01%）. |
| 6 | Thermistor PTC Input | An external PTC thermistor must be connected to detect whether the motor is overheating. |
| 7 | Forward Torque Limit | When the analog input is used as torque limit for four-quadrant operation, parameters F5-21, F5-27, or F5-33 should be set to 7, 8, 9, or 10. The torque limit values are displayed through parameters FE-11 to FE-14 (shared variables with torque limits received via communication), and the diagram below illustrates the torque limit for four-quadrant operation: |
| 8 | reverse torque limit |  |
| 9 | regenerative torque limit |  |
| 10 | Forward/Reverse Torque Limit |  |
| 11 | Thermistor PT100 value | An external PT100 thermistor is required to detect whether the motor is overheated. |
| 12 | Auxiliary frequency setting | When used as an auxiliary frequency input, the auxiliary frequency source (F0-07) must be set to 3 (external analog input). |
| 13 | PID offset | When used as a PID compensation amount, the PID feedback selection (FA-00 not equal to 0) must be valid, and the PID compensation amount selection (FA-20) must be set to 1 (external analog input), the intelligent start frequency command (FA-33) to 0, and the application macro parameter (L0-00) to 6 (for air compressor industry applications). The change in the analog input compensation amount can be displayed in parameter FA-21 (PID compensation) as a percentage (precision 0.1%). |
| 14 | Tension PID Feedback Value | When used as tension PID feedback, the tension control mode selection (FB-00) must be set to 1 (speed closed-loop mode). |
| 15 | Line Speed Feedback | As tension control line speed feedback |
| 16 | roll diameter feedback | When used as roll diameter feedback (including initial roll diameter and real-time roll diameter calculation feedback), the tension control selection (FB-00 not zero) must be enabled, the roll diameter source (FB-25) must be set to 1 for analog input as roll diameter feedback (FB-38), and the initial roll diameter source (FB-28) must be set to 1 for analog input as the initial roll diameter (FB-29). |
| 17 | Tension PID Setpoint | When used as tension PID setpoint, the tension control mode selection (FB-00) must be set to 1 (speed closed-loop mode), and the PID target source setting (FB-04) must be set to 2 (analog input). When the inverter is in operation, the PID target value setting (FB-05) isdisplayed. |
| 18 | Tension Setpoint | When setting the tension command, the tension control mode selection (FB-00) must be set to 3/4 (torque mode), the tension command source selection (FB-52) to 1 (analog input), and the maximum tension value (FC-53) should be set according to the site conditions (default value is 0). Parameters F5-21, F5-27, or F5-33 should be set to 18. When the inverter is in operation, the tension setting is displayed through parameter (FB-54). |
| 19 | Zero-speed tension setting | When setting the zero-speed tension command, the tension control mode selection (FB-00) must be set to 3/4 (torque mode), the zero-speed tension command source selection (FB-55) to 2 (analog input), and the maximum tension value (FB-53) should be set according to the site conditions (default value is 0). When the inverter is in operation, the zero-speed tension setting is displayed through parameter (FB-56). |
| 20 | Tension Taper Setting | When setting the tension taper, the tension control mode selection (FB-00) must be set to 3/4 (torque mode), the tension taper setting source selection (FC-64) must be set to 1 (analog input), and parameters F5-21, F5-27, or F5-33 must be set to 20. During the inverter operation, the tension taper setting is displayed through parameter (FC-65). |

| F5-22 | AI1 Input Bias | Range: -100.0% to 100.0% | Factory Value: 0.0 |
| --- | --- | --- | --- |

This function code is used to set the analog input value corresponding to the 0 point of the AI1 input signal.

| F5-23 | AI1 Bias Mode Selection | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the AI1 bias mode. Properly setting the bias mode can effectively process the input signal to reduce the impact of noise.

0: No Bias

1: Below Bias = Bias

2: Above Bias = Bias

3: Absolute Value Bias as Center

4: Bias as Center

AI1 variousbias modesareillustrated in Figures 2-23 to 2-27, whereBias represents bias, set through F5-22, Gain represents gain, set through F5-24,the horizontal axis is the input signal, and the vertical axis is the signal after bias processing. Bias modes can be selected according to requirements.

Figure 2-23 No Bias

Figure 2-24 Below Bias = Bias

Figure 2-25 AboveBias = Bias

Figure 2-26 Biasas Center for Absolute Value

Figure 2-27 Biasas Center

| F5-24 | AI1 Gain | Range: -500.0%～500.0% | Factory value: 100.0% |
| --- | --- | --- | --- |

This function code is used to set the AI1 signal gain. The AI1 signal gain can proportionally adjust the analog input signal; the gain setting can refer to F5-23.

| F5-25 | AI1 Filter Time | Range: 0.00～20.00 seconds | Factory value: 0.01 second |
| --- | --- | --- | --- |

The inverter software filters out noise signals contained in the analog input signal at the control terminal through a digital low-pass filter; this parameterisused to set the time constant of the digital low-pass filter.  The larger this parameter value, the better the filtering effect, but the greater the signal delay, and the control response performance deteriorates; the smaller the setting value, the faster the control response, but the worse the noise filtering effect, which may lead to unstable control. When selecting the optimal setting value, it can be determined by balancing control stability and response delay.

| F5-26 | AI2 Signal Type Selection | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |
| F5-27 | AI2 Function Selection | Range: 0～20 | Factory value: 0 |
| F5-28 | AI2 Input Bias | Range: -100.0% to 100.0% | Factory Value: 0.0 |
| F5-29 | AI2 Bias Mode Selection | Range: 0～4 | Factory value: 0 |
| F5-30 | AI2 Gain | Range: -500.0%～500.0% | Factory value: 100.0% |
| F5-31 | AI2 Filter Time | Range: 0.00～20.00 seconds | Factory value: 0.01 second |

The above function codes are used for setting AI2; refer to F5-20 to F5-25.

| F5-32 | AI3 Signal Type Selection | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |
| F5-33 | AI3 Function Selection | Range: 0～20 | Factory value: 0 |
| F5-34 | AI3 Input Bias | Range: -100.0% to 100.0% | Factory Value: 0.0 |
| F5-35 | AI3 Bias Mode Selection | Range: 0～4 | Factory value: 0 |
| F5-36 | AI3 Gain | Range: -500.0%～500.0% | Factory value: 100.0% |
| F5-37 | AI3 Filter Time | Range: 0.00～20.00 seconds | Factory value: 0.01 second |

The above function codes are used for setting AI3; refer to F5-20 to F5-25.

| F5-38 | Analog frequency negative value reversal | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set whether the analog signal can control the inverter direction.

0: Forward/reverse rotation determined by operation source

Negative frequency input from the analog input signal is not allowed; forward and reverse actions are controlled by the keyboard or external terminals.

1: Forward/reverse rotation determined by bias

Negative frequency input from the analog input signal is allowed; positive frequency results in forward rotation, and negative frequency results in reverse rotation. At this time, the keyboard and external terminalscannot control forward and reverse rotation.

The condition for analog signals to input negative frequency is:

1. Set F5-38 to 1;

2. Bias mode setting (F5-23, F5-29, or F5-35) to 4 (centered on bias);

3. The corresponding analog input gain is less than 0, making the calculated input frequency negative. When using the analog input addition function (F5-39), if the added analog signal is negative, this parameter can control whether the inverter reverses.

| F5-39 | Analog input addition enable | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set whether the analog input addition function is enabled.

0: Disabled

The analog input addition function is disabled.

1: Enable

The analog input addition function is enabled.

When F5-21 = F5-27 = F5-33 = 1, Frequency Command = AI1 + AI2 + AI3; When F5-21 = F5-27 = 1, and F5-33 ≠ 1, frequency command = AI1 + AI2; When F5-21 = F5-33 = 1, and F5-27 ≠ 1, frequency command = AI1 + AI3; When F5-27 = F5-33 = 1, and F5-21 ≠ 1, frequency command = AI2 + AI3.

When the analog input addition function is invalid, if the analog input functions are selected the same, the effective signal is determined based on the analog input priority (AI1 > AI2 > AI3).

| F5-42 | 4-20mA Wire Break Action | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the handling method when an analog input current signal wire break is detected, which is valid when the analog type is selected as 4-20mA input.

0: Do not detect open circuit

Do not perform wire break detection.

1: Maintain frequency before open circuit

After detecting a wire break, continue running and display a wire break warning.

2: Decelerate to 0Hz

After detecting a wire break, reduce the frequency to zero speed while displaying a wire break warning.

3: Immediate stop due to fault

After detecting a wire break, immediately stop operation and report a wire break fault.

| F5-43 | 4-20mA Wire Break Threshold | Range: 0.00mA～4.00mA | Factory Value: 2.00mA |
| --- | --- | --- | --- |

This function code is used to set the threshold for analog input current signal wire break detection; if the value is below this threshold, a wire break is considered to have occurred.

2.7 F6 Group Output Terminals

This series of inverters is equipped with 2 multifunctional digital output terminals and two multifunctional relay output terminals.

| F6-00 | RLY1 terminal function selection | Range: 0～76 | Factory value: 11 |
| --- | --- | --- | --- |
| F6-01 | RLY2 terminal function selection | Range: 0～76 | Factory value: 1 |
| F6-02 | DO1 Terminal Function Selection | Range: 0～76 | Factory value: 0 |
| F6-03 | DO2 Terminal Function Selection | Range: 0～76 | Factory value: 0 |

Multi-function output terminal function selection asTable 2-16 shown:

Table 2-16 Multi-function output terminal function selection

| Set Value | Function | Description |
| --- | --- | --- |
| 0 | No function | Disable multi-function digital output function |
| 1 | Inverter running | Inverter running, this terminal output is valid. |
| 2 | Reach set frequency | Reach set frequency, when the inverter output frequency deviation from the set frequency is within a certain range (±2Hz), this terminal output is valid. |
| 3 | Reach frequency 1 detection value | Reach frequency 1 detection value, when the inverter output frequency deviation from the frequency 1 detection value (F6-27) is less than the frequency 1 amplitude (F6-28), this terminal output is valid. |
| 4 | Reach frequency 2 detection value | When the inverter output frequency deviates from the frequency arrival 2 detection value (F6-29) by less than the frequency arrival 2 amplitude (F6-30), this terminal is valid. |
| 5 | Zero Speed Command Operation | This terminal is valid when the frequency command is 0 during inverter operation. |
| 6 | Zero Speed Command Operation (Including STOP) | This terminal is valid when the inverter frequency command is 0 (including when the inverter is stopped). |
| 7 | Over Torque 1 | When the inverter detects an over torque condition (over torque action method, detection threshold, and detection time refer to parameters F9-36 ~ F9-38), this terminal is valid. |
| 8 | Over Torque 2 | When the inverter detects an over torque (for over torque action method, detection threshold, and detection time, refer to parameters H3-00 ~ H3-02), this terminal is active. |
| 9 | Inverter Ready | When the inverter has no fault (including during stop and operation), this terminal is active. |
| 10 | Low Voltage Warning | When the inverter experiences DC bus under-voltage (including during stop and operation, and when the DC bus voltage is below the low voltage threshold L2-18), this terminal is active. |
| 11 | Fault Indication | When the inverter experiences a fault (excluding under-voltage during stop, base block fault, and instantaneous power failure without stop fault), this terminal is active. |
| 12 | Brake release | This terminal is effective when the inverter operating frequency is not 0 or greater than the motor zero-speed threshold (F6-09). |
| 13 | Overtemperature warning | This terminal is effective when the inverter detects an overtemperature warning for the IGBT or bus capacitor. |
| 14 | Software brake action indication | This terminal is effective when the inverter's discharge resistor switch actuates. |
| 15 | PID feedback abnormality | This terminal is effective when the inverter detects that the deviation between the PID setpoint and feedback exceeds the set threshold (FA-51) and persists for the duration (FA-52). |
| 16 | Excessive slip | This terminal is effective when the inverter detects an excessive slip fault. |
| 17 | Count Value Does Not Reset to Zero | When the inverter executes the external counter, if the count value equals the set value of parameter L6-01, this terminal outputs effectively. If the set value of parameter L6-01 > the set value of parameter L6-00, this terminal output is invalid. |
| 18 | Count Value Resets to Zero | When the inverter executes the external counter, if the count value equals the set value of parameter L6-00, this terminal outputs effectively. |
| 19 | Base Block State | When the inverter stops output due to base block (B.B.), this terminal outputs effectively. |
| 20 | Warning Indication | When the inverter detects a warning, this terminal outputs effectively. |
| 21 | Overvoltage Warning | When the inverter detects that the bus voltage exceeds the set overvoltage warning threshold (0.9 times the overvoltage threshold, no warning state output), this terminal outputs effectively. |
| 22 | Overcurrent Slip Warning | When the inverter detects an overcurrent slip prevention warning, this terminal output is valid. |
| 23 | Overvoltage Slip Warning | When the inverter detects an overvoltage slip prevention warning, this terminal output is valid. |
| 24 | Inverter Operation Source | When the inverter operation command source is not the keyboard (F0-05 is not 0), this terminal output is valid. |
| 25 | Forward Command | When the inverter's direction command is forward, this terminal output is valid. |
| 26 | Reverse Command | When the inverter's direction command is reverse, this terminal output is valid. |
| 29 | Over Frequency | When the actual output frequency of the inverter exceeds the DO action frequency (F6-08), this terminal output is valid. |
| 30 | Under Frequency | When the actual output frequency of the inverter is less than the DO action frequency (F6-08), this terminal outputs a valid signal. |
| 31 | Motor Star Connection Switch Command | Motor Star Connection Switch Command, set the multi-function digital input terminal for star connection confirmation signal (DIx = 29) and delta connection confirmation signal (DIx = 30), set the motor star-delta switching enable (F7-48 = 1). When the inverter output frequency is below the motor star-delta switching frequency (F7-47) setting minus 2Hz and the duration is greater than the motor star-delta switching delay (F7-49), this terminal outputs a valid signal. |
| 32 | Motor Delta Connection Switching Command | To switch the motor connection from star to delta, set the multifunction digital input terminal for star connection confirmation signal (DIx = 29) and delta connection confirmation signal (DIx = 30), enable the motor star-delta switching (F7-48 = 1). When the inverter output frequency is higher than the motor star-delta switching frequency (F7-47) setting plus 2Hz and the duration exceeds the motor star-delta switching delay (F7-49), this terminal becomes active. |
| 33 | Zero Frequency Operation | When the inverter is running and the output frequency is zero, this terminal becomes active. |
| 34 | Output zero frequency (including STOP) | This terminal is valid when the inverter output frequency is zero or stopped. |
| 35 | Fault output selection 1 | This terminal is valid when the inverter encounters a fault set by U0-10. |
| 36 | Fault output selection 2 | This terminal is valid when the inverter encounters a fault set by U0-11. |
| 37 | Fault output selection 3 | This terminal is valid when the inverter encounters a fault set by U0-12. |
| 38 | Fault output selection 4 | This terminal is valid when the inverter encounters a fault set by U0-13. |
| 39 | Position reached | This terminal is valid when the position is reached in the inverter's positioning mode. |
| 40 | Target frequency reached | This terminal is valid when the inverter output frequency reaches the set frequency (including stop and frequency command of 0). |
| 41 | Multi-point Positioning Arrival | When the position error of multi-point positioning of the inverter is less than the allowable position arrival error range of the encoder (FC-21), this terminal output is valid. |
| 42 | Mechanical Brake Release | It is necessary to set the DO action frequency (F6-08) and the brake frequency (L4-00). When the brake release is detected, this terminal output is valid. |
| 43 | Motor Zero Speed | When using a PG card, if the feedback speed from the PG card is below the motor zero speed judgment threshold (F6-09) set speed, this terminal output is valid. |
| 44 | Low Current Output | When the inverter has low current output (the undercurrent action method is selected by parameter F9-24, which may not report a low current output fault or low current output warning), this terminal output is valid. |
| 45 | Three-phase output contactor Action | It is necessary to configure the digital input terminal as 49 (inverter enable) for use together. When the inverter status enable terminal is active, the output terminal is effective. |
| 46 | dEB action | During the inverter deceleration process, when dEB action occurs, the output terminal is effective. |
| 47 | Stop brake closure | When the inverter is in a shutdown state, if the discharge resistor switch feedback status is closed, the output terminal is effective. |
| 49 | Positioning complete | When returning to the origin is completed, the output terminal is effective. |
| 50 | CANopen control output | CANopen control output, controlling the output terminal based on the status of the corresponding CANopen data bits. The CANopen and digital output port mapping table is as follows: Terminal; Parameter setting; Attribute; Index. RLY1; F6-00=50; RW; Index:2026 SubIndex:0x41 Data bit0. RLY2; F6-01=50; RW; Index:2026 SubIndex:0x41 Data bit1. DO1; F6-02=50; RW; Index:2026 SubIndex:0x41 Data bit3. DO2; F6-03=50; RW; Index:2026 SubIndex:0x41 Data bit4. |
| 51 | RS485 control output | RS-485 (Modbus) control output, controlling output terminals based on the status of RS-485 corresponding data bits. The RS-485 and digital output port mapping table is as follows: Terminal; Parameter setting; Attribute; Index. RLY1; F6-00=51; RW; 2640 data bit0. RLY2; F6-01=51; RW; 2640 data bit1. DO1; F6-02=51; RW; 2640 data bit3. DO2; F6-03=51; RW; 2640 data bit4. |
| 52 | Communication card control output | Communication card control output, controls output terminals based on the status of the corresponding data bits of the communication card. The mapping table for the communication card and digital output ports is as follows: Terminal; Parameter setting; Attribute; Index. RLY1; F6-00=52; RW; 2640 data bit0. RLY2; F6-01=52; RW; 2640 data bit1. DO1; F6-02=52; RW; 2640 data bit3. DO2; F6-03=52; RW; 2640 data bit4. |
| 66 | STO fault | When any type of STO fault occurs in the inverter, the terminal output is active. |
| 67 | Analog input threshold reached output | The inverter's multi-function output terminal operates when the analog input is between the high threshold and low threshold. Parameter F6-05 selects the analog input channel AI1, AI2, or AI3 to be compared. Parameter F6-06 sets the high threshold for analog input comparison (default 50%). Parameter F6-07 sets the low threshold for analog input comparison (default 10%). When the analog input > Parameter F6-06, the multi-function output terminal operates; When the analog input < Parameter F6-07, the multi-function output terminal stops output. |
| 68 | STO normal | When any type of STO fault occurs in the inverter, this terminal output is invalid. |
| 69 | Maximum roll diameter reached | When using tension control mode, this terminal output is valid when the roll diameter reaches parameter FB-26. |
| 70 | Reach empty roll diameter | When using tension control mode, this terminal output is valid when the roll diameter reaches parameter FB-27. |
| 71 | Belt break indication | When using tension control mode, if the enable belt break detection is active, the line speed exceeds parameter FB-45, the roll diameter error exceeds parameter FB-46, and the time exceeds parameter FB-47, a belt break occurs, and this terminal output is valid. |
| 72 | Tension PID feedback deviation fault | When using tension control mode, if the difference between the tension PID setpoint and the tension PID feedback value exceeds the PID feedback error level (parameter FB-48), and the error time exceeds the PID feedback error detection time (parameter FB-49), a PID feedback error anomaly (refer to PID feedback error handling (parameter FB-50)) occurs, and this terminal output is valid. |
| 73 | Over-torque 3 | When the inverter detects an over-torque event (over-torque detection threshold H3-06, over-torque judgment delay H3-07), this terminal output is valid. |
| 74 | Over-torque 4 | When the inverter detects an over-torque event (over-torque detection threshold H3-11, over-torque judgment delay H3-12), this terminal output is valid. |

| F6-04 | DO Terminal Valid Logic | Range: 0000H~FFFFH | Factory value: 0 |
| --- | --- | --- | --- |

This function code can be used to set different DO terminal effective logic levels. The table below lists the effective logic control bits for each DO terminal. If the corresponding control bit is 0, the effective logic level remains unchanged; if the corresponding control bit is 1, the effective level will be inverted.As shown in Table 2-17.

Table 2-17 Effective Logic Control Bits for DO Terminals

| Bit 4 | Bit 4 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- |
| DO2 | DO1 | Reserved | RLY2 | RLY1 |

| F6-05 | Source of DO Output AI | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |

F6-05 Source of AI for DO Output

0: AI1

1: AI2

2: AI3

3: Expansion Card Terminal AI10

4: Expansion Card Terminal AI11

| F6-06 | DO Output AI Upper Limit Value | Range: -100.00%～100.00% | Factory Value: 50.00% |
| --- | --- | --- | --- |
| F6-07 | DO Output AI Lower Limit Value | Range: -100.00%～100.00% | Factory Value: 10.00% |

When the multi-function digital output terminal（F6-00～F6-03）is configured as 67 (analog input threshold reached output), the corresponding multi-function digital output status is valid when the analog input signal exceeds F6-06; if the analog input signal is less than F6-07, the corresponding multi-function digital output status is invalid. Note that the setting value of F6-06 should be greater than the setting value of F6-07.

| F6-08 | DO Action Frequency | Range: 0.00Hz～599.00Hz | Factory Value: 0.00Hz |
| --- | --- | --- | --- |

F6-08 DO Action Frequency

When the actual output frequency of the inverter is greater than the DO action frequency (F6-08), if the output terminal function is set to 29, this terminal output is valid. When the actual output frequency of the inverter is less than the DO action frequency (F6-08), if the output terminal function is set to 30, this terminal output is valid.

| F6-09 | Motor Zero Speed Judgment Threshold | Range: 0rpm～65535rpm | Factory Value: 0rpm |
| --- | --- | --- | --- |

This parameter defines the motor zero-speed judgment threshold. When the motor speed is below this parameter setting, the multi-function output terminal set to 43 will output effectively, motor zero-speed judgment function as shown in Figure 2-28.

Figure 2-28 Motor Zero-Speed JudgmentFunction

When parameter F0-04 is set to open-loop control mode, estimated speed calculation is used; When parameter F0-04 is set to closed-loop control mode, actual feedback speed calculation is used, and an encoder must be installed on the motor to provide actual speed feedback, along with a PG card or MI pulse input terminal. With the installation of a PG card and an encoder on the motor for feedback, zero-speed control accuracy can be improved.

| F6-13 | AO1 Signal Type Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |
| F6-14 | AO1 output function selection | Range: 0～23 | Factory value: 0 |

F6-13 AO1 signal type selection

0: 0-10V Output Selection

1: 0-20mA Output Selection

F6-14 AO1 output function selection

Output function selection is shown in Table2-18.

Table 2-18 AO output function selection

| Set Value | Function | Description |
| --- | --- | --- |
| 0 | Output frequency | The maximumfrequency parameterF4-02 is 100%. |
| 1 | Frequency command | The maximumfrequency parameterF4-02 is 100%. |
| 2 | Motor operating frequency (Hz) | The maximumfrequency parameterF4-02 is 100%. |
| 3 | Output current (RMS) | Rated at 2.5 times the inverter's rated current as 100%. |
| 4 | Output voltage | Rated at 2 times the motor's rated voltage as 100%. |
| 5 | DC Bus Voltage | Rated at 900V as 100%. |
| 6 | power factor | Rated at ±1.000 as 100%. |
| 7 | Power | Rated at 2 times the inverter's rated power as 100%. |
| 8 | Output torque | Rated at 2 times the rated torque as 100% |
| 9 | AI1 percentage | (0 ～ 10V)/(0 ～ 20mA) corresponds to 0 ～ 100%. |
| 10 | AI2 percentage | (0 ～ 10V)/(0 ～ 20mA) corresponds to 0 ～ 100%. |
| 11 | AI3 percentage | (0 ～ 10V)/(0 ～ 20mA) corresponds to 0 ～ 100%. |
| 12 | Iq current command | Rated at 2.5 times the inverter's rated current as 100%. |
| 13 | Iq current feedback | Rated at 2.5 times the inverter's rated current as 100%. |
| 14 | Id current command | Rated at 2.5 times the inverter's rated current as 100%. |
| 15 | Id current feedback | Rated at 2.5 times the inverter's rated current as 100%. |
| 16 | Vq voltage command | With 600V as 100% . |
| 17 | Vd voltage command | With 600V as 100% . |
| 18 | Torque command | With rated torque as 100% . |
| 19 | PG2 frequency command | The maximumfrequency parameterF4-02 is 100%. |
| 20 | CANopen analog output | Provided for CANopen communication analog output, terminalAO1 corresponds to index 0x2026, sub-index 0xA1, terminalAO2 corresponds to index 0x2026, sub-index 0xA2, extended terminalAO10 corresponds to address 2026-AB, extended terminalAO11 corresponds to address 2026-AC. |
| 21 | RS485 analog output | Provided for RS-485 communication analog output, terminalAO1 corresponds to address 26A0H, terminalAO2 corresponds to address 26A1H, extended terminalAO10 corresponds to address 26AAH, extended terminalAO11 corresponds to address 26ABH. |
| 22 | Communication card analog output | Provided for communication card analog output, terminalAO1 corresponds to address 26A0H, terminalAO2 corresponds to address 26A1H, extended terminalAO10 corresponds to address 26AAH, extended terminalAO11 corresponds to address 26ABH. |
| 23 | Fixed voltage output | The output voltage ratio can be set by parameters F6-18 (corresponding to AO1) and F6-25 (corresponding to AO2), parameter settings 0.00 ～ 100.00% correspond to AO output 0 ～ 10V. |

| F6-15 | AO1 output gain | Range: 0.0～500.0% | Factory value: 100.0% |
| --- | --- | --- | --- |

This parameter is used to adjust the amplitude of the output signal from the multifunctional analog output terminalproportionally.

| F6-16 | AO1 reverse enable | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

0: Output Absolute Value

1: Reverse Output 0V/0mA, Forward Output (0～10V)/(0～20mA)

2: Reverse Output (5～0V)/(10～0mA), Forward Output (5～10V)/(10～20mA)

Figure 2-29 AO Reverse Enable

| F6-17 | AO1 Output Bias | Range: -100.00%～100.00% | Factory Value: 0.00 |
| --- | --- | --- | --- |

For example, using output frequency:

0 ～ 10V: ((Output Frequency/F4-02) * F6-15 + F6-17) * 10V

0 ～ 20mA: ((Output Frequency/F4-02) * F6-15 + F6-17) * 20mA

This parameter sets the voltage or current value corresponding to the 0 point of the analog output.

| F6-18 | AO1 Output Fixed Value | Range: 0.00～100.00% | Factory Value: 0.00 |
| --- | --- | --- | --- |

This parameter is used to set the percentage of fixed analog output, requiring setting the multi-function analog output to fixed voltage output (F6-14 = 23), the actual analog output terminal voltage or current value = F6-18 * 10V or 20mA.

| F6-19 | AO1 Filter Time | Range: 0.00～20.00 seconds | Factory value: 0.01 second |
| --- | --- | --- | --- |

The inverter software uses a digital low-pass filter to remove noise signals contained in the analog output signal from the control terminal. This parameter is used to set the time constant of the digital low-pass filter.

| F6-20 | AO2 Signal Type Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |
| F6-21 | AO2 Output Function Selection | Range: 0～23 | Factory value: 0 |

F6-20 AO2 Signal Type Selection

0: 0-10V Output Selection

1: 0-20mA Output Selection

F6-21 AO2 Output Function Selection

Refer to F6-14.

| F6-22 | AO2 Output Gain | Range: 0.0～500.0% | Factory value: 100.0% |
| --- | --- | --- | --- |

This parameter is used to adjust the amplitude of the output signal from the multifunctional analog output terminalproportionally.

| F6-23 | AO2 Reverse Enable | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

0: Output absolute value;

1: Reverse output 0V/0mA, forward output (0～10V)/(0～20mA);

2: Reverse output (5～0V)/(10～0mA), forward output (5～10V)/(10～20mA).

| F6-24 | AO2 Output Bias | Range: -100.00%～100.00% | Factory Value: 0.00 |
| --- | --- | --- | --- |

For example, using output frequency:

0 ～ 10V : ((Output frequency/F4-02) * F6-22) + F6-24) * 10V

0 ～ 20mA : ((output frequency / F4-02) * F6-22 + F6-24) * 20mA

This parameter sets the voltage or current value corresponding to the 0 point of the analog output.

| F6-25 | AO2 Output Fixed Value | Range: 0.00～100.00% | Factory Value: 0.00 |
| --- | --- | --- | --- |

This parameterisused to set the percentage of fixed analog output, requiring the multi-function analog output to be set as a fixed voltage output (F6-21 = 23), the actual analog output terminalvoltage or current value= F6-25 * 10V or 20mA.

| F6-26 | AO2 Filter Time | Range: 0.00～20.00 seconds | Factory value: 0.01 second |
| --- | --- | --- | --- |

The inverter software filters out noise signals contained in the analog signals output from the control terminals through a digital low-pass filter; this parameter is used to set the time constant of the digital low-pass filter.

| F6-27 | Frequency Reach 1 Detection Value | Range: 0.00Hz～599.00Hz | Factory Value: 50.00Hz |
| --- | --- | --- | --- |
| F6-28 | Frequency Reach 1 Amplitude | Range: 0.00Hz～599.00Hz | Factory setting: 2.00Hz |
| F6-29 | Frequency reaches 2 detection value | Range: 0.00Hz～599.00Hz | Factory Value: 50.00Hz |
| F6-30 | Frequency reaches 2 amplitude | Range: 0.00Hz～599.00Hz | Factory setting: 2.00Hz |

Two DO output frequency thresholds can be set, including frequency reach 1 detection value (F6-27) and frequency reach 2 detection value (F6-29). Two DO output frequency judgment width thresholds can be set, including frequency arrival 1 amplitude (F6-28) and frequency arrival 2 amplitude (F6-30). When the inverter output speed (frequency) reaches within the set DO output frequency threshold range, the corresponding multi-function output terminal will output effectively if it is set to 3 or 4 (parameters F6-00, F6-01, F6-02, F6-03).

2.8 F7 Group Auxiliary Functions and Keyboard Display

| F7-00 | JOG frequency setting | Range: 0.00～599.00Hz | Factory Value: 6.00Hz |
| --- | --- | --- | --- |
| F7-01 | JOG acceleration time | Range: 0.00～600.00 seconds | Factory Value: 10.00 seconds |
| F7-02 | JOG deceleration time | Range: 0.00～600.00 seconds | Factory Value: 10.00 seconds |

The above parameters are used to set the jogging operation frequency, the time to accelerate from 0.00Hz to the jogging operation frequency, and the time to decelerate from the jogging operation frequency to0.00Hz. During jogging operation, when a jogging command is received, the inverter controls the motor to accelerate to the jog frequency according to the jogging acceleration time. When the jogging command is removed, depending on the different stop methods, the motor decelerates and stops according to the jogging deceleration time or adopts other stopping methods.

| F7-03 | acceleration time 2 | Range: 0.00～600.00 seconds or 0.0～6000.0 seconds | Factory value: Model determined |
| --- | --- | --- | --- |
| F7-04 | deceleration time 2 | Range: 0.00～600.00 seconds or 0.0～6000.0 seconds | Factory value: Model determined |
| F7-05 | acceleration time 3 | Range: 0.00～600.00 seconds or 0.0～6000.0 seconds | Factory value: Model determined |
| F7-06 | deceleration time 3 | Range: 0.00～600.00 seconds or 0.0～6000.0 seconds | Factory value: Model determined |
| F7-07 | acceleration time 4 | Range: 0.00～600.00 seconds or 0.0～6000.0 seconds | Factory value: Model determined |
| F7-08 | Deceleration Time 4 | Range: 0.00～600.00 seconds or 0.0～6000.0 seconds | Factory value: Model determined |

For details on function codes F7-03 to F7-08, please refer to F0-13 and F0-14.

| F7-09 | Jump Frequency 1 Upper Limit | Range: 0.00～599.00Hz | Factory Value: 0.00 |
| --- | --- | --- | --- |
| F7-10 | Jump Frequency 1 Lower Limit | Range: 0.00～599.00Hz | Factory Value: 0.00 |
| F7-11 | Jump Frequency 2 Upper Limit | Range: 0.00～599.00Hz | Factory Value: 0.00 |
| F7-12 | Jump Frequency 2 Lower Limit | Range: 0.00～599.00Hz | Factory Value: 0.00 |
| F7-13 | Jump Frequency 3 Upper Limit | Range: 0.00～599.00Hz | Factory Value: 0.00 |
| F7-14 | Jump frequency 3 lower limit | Range: 0.00～599.00Hz | Factory Value: 0.00 |
| F7-15 | Jump frequency 4 upper limit | Range: 0.00～599.00Hz | Factory Value: 0.00 |
| F7-16 | Jump frequency 4 lower limit | Range: 0.00～599.00Hz | Factory Value: 0.00 |

Whenthemotorworks near the mechanical resonance point of the load, it may cause mechanical resonance of the load, thereby reducing control performance, and in severe cases, it may damage the load. To avoid resonance, the skip frequency function can be used to make the motor bypass mechanical resonance points. When the set frequency is within the skip frequency range, the actual operating frequency will runinthe vicinity of the nearest skip frequency boundary. By setting the skip frequency, the inverter avoids the mechanical resonance points of the load. Skip frequency functionas shown in Figure 2-30,this inverter can set4 skip frequency points,if two adjacent skip frequencies are set to the same value, this function does not work at that frequency.

Figure 2-30 Skip Frequency Points

| F7-17 | Fan control method | Range: 0～4 | Factory value: 4 |
| --- | --- | --- | --- |

0: After power-on of the inverter, the fan runs continuously.

1: The fan starts when the inverter is running and stops 1 minute after the inverter stops.

2: The fan operates according to the inverter's operating status; it starts when the inverter is running and stops when the inverter stops.

3: When the module temperature > 50°C, the fan starts; When module temperature < 40°C and the inverter stopsoperation, the fan shuts off.

4: The variable frequency operation fan starts, after the inverter stops, if the module temperature is below 50°C, the fan shuts off.

| F7-19 | Keyboard STOP key enable | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: The STOP key on the keyboard can only control shutdown when the operation command source selection parameter F0-05 = 0; in other cases, the STOP key on the keyboard is invalid.

1: Regardless of whether the operation command source selection is 0, the STOP key on the keyboard can always control the shutdown.

| F7-20 | Startup screen selection | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

0: Set frequency display

1: Output frequency display

2: User-defined display

3: Output current display.

| F7-21 | Page display selection | Range: 0～61 | Factory Value: 3 |
| --- | --- | --- | --- |

0: Output current

1: PG card feedback frequency

2: Motor actual operating frequency

3: DC Bus Voltage

4: Output voltage

5: Power factorangle

6: Output power

7: Actual motor operating speed

8: Output torque%

9: PG feedback value

10: PID feedback value%

11: AI1 %

12: AI2 %

13: AI3 %

14: IGBT temperature

15: Ambient temperature

16: Digital input terminal status

17: Digital output terminal status

18: Multi-speed status

21: Actual motor position

22: Pulse input frequency value

23: Pulse input position

24: Position tracking error

25: Overload count value

26: Ground short-circuit current threshold

27: Bus voltage fluctuation value

30: User physical quantity

31: H page value multiplied by coefficient K

32: Encoder Z phase count

33: Motor pulse count

34: Reserved

35: Speed/torque mode

36: Current carrier frequency

37: Reserved

38: Inverter status

39: Output torque Nt-m

40: Torque command

41: kWh

42: PID target value

43: PID compensation

44: PID output frequency

45: Reserved

46: Auxiliary frequency

47: Main frequency

48: Set frequency display

49: Reserved

50: Reserved

51: PMVVC torque compensation amount

54: Reserved

55: Current roll diameter

56: Current line speed

57: Tension reference value

59: U phase current AD value

60: V phase current AD value

61: W phase current AD value

| F7-23 | FM terminal function selection | Range: 0～10 | Factory value: 0 |
| --- | --- | --- | --- |

0: Operating frequency

1: Set frequency

2: Output current

3: Output voltage

4: Output power

5: AI1 value

6: AI2 value

7: AI3 value

8: Output torque absolute value

9: Motor speed

10: Bus voltage

| F7-24 | FMP Output Maximum Frequency | Range: 1.00%～100.00% | Factory Value: 50.00% |
| --- | --- | --- | --- |
| F7-26 | Cumulative Start-up Times | Range: 0～65535 | Factory value: 0 |
| F7-27 | Cumulative Start-up Days | Range: 0～65535 | Factory value: 0 |
| F7-28 | Cumulative Operating Minutes | Range: 0～1439 | Factory value: 0 |
| F7-29 | Cumulative Operating Days | Range: 0～65535 | Factory value: 0 |
| F7-30 | Cumulative Running Minutes | Range: 0～65535 | Factory value: 0 |

The above parameters are used to display the accumulated start-up and operation information of the inverter.

| F7-31 | Motor Running Time | Range: 0～1439 min | Factory setting: 0 min |
| --- | --- | --- | --- |
| F7-32 | Motor Running Days | Range: 0～65535 | Factory value: 0 |

The above parameters are used to display the accumulated running time of the motor.

| F7-33 | Password Input | Range: 0～65535 | Factory value: 0 |
| --- | --- | --- | --- |
| F7-34 | Password Setting | Range: 0～65535 | Factory value: 0 |

F7-34 supports user-defined password for keyboard lock. After setting, the keyboard is locked and other parameters cannot be modified. To unlock, enter the corresponding password in F7-33.

| F7-35 | Automatic Energy Saving Setting | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |
| F7-36 | Energy Saving Gain | Range: 10～1000 | Factory Value: 100 |

Using the inverter to drive the motor, one important goal is energy saving. Under the same torque output conditions, the voltage and current output by the inverter can vary, and different conditions correspond to different output powers of the inverter. If a combination of voltage and current can be chosen to reduce the output power of the inverter, the goal of energy saving is achieved.

The inverter, while running,real-time calculates the output power, adjusts the output voltage based on the output power, to make the output power reach the minimum value or maintain near the minimum value. The entire processisperformedwithout externalintervention, can be carried outautomatically, hence it is called the“automatic energy saving” function. To ensure that the acceleration and deceleration performance is not affected by the automatic energy saving function, during acceleration and deceleration, the automatic energy saving function is turned off, and only during steady-speed operation will it beactivated.

| F7-37 | Automatic Voltage Regulation | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |

When the bus voltage changes, the output voltage may also change, which could affect the motor control performance. Bus voltage compensation function (Automatic Voltage Regulation function, AVR) can compensate for output voltage variations caused by bus voltage changes, making motor control performance more stable.

0: Enable AVR Function

When the AVR function is enabled, the inverter calculates the output voltage based on the bus voltage, so the output voltage will not fluctuate with the bus voltage.

1: Disable AVR Function

When the AVR function is disabled, the inverter does not calculate the output voltage based on the bus voltage, causing the output voltage to fluctuate with the bus voltage, which in turn leads to fluctuations in motor current.

2: Disable AVR During Deceleration

The AVR function is disabled during deceleration but enabled in other states. Disabling the AVR function during deceleration can shorten the deceleration time.

| F7-42 | Output Phase Sequence Switching | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: Output according to the command direction.

1: Output frequency opposite to the command direction; forward rotation changes to reverse, and reverse changes to forward.

Note: The output phase sequence switching and forward/reverse prohibition selection can be used together. However, if the prohibited rotation direction set for operation is the same as the direction after phase sequence switching, the output phase sequence switching function will not be realized.

| F7-46 | CANopen Quick Stop Time | Range: 0.00～600.00 s | Factory setting: 1.00 s |
| --- | --- | --- | --- |

This parameter is used to set the CANopen quick stop time.

| F7-54 | W-hrs | Range: 0.0～6553.5 | Factory Value: 0.0 |
| --- | --- | --- | --- |

This parameter is used to display the accumulated energy output by the inverter.

| F7-57 | Software Version (H) | Range: 0.00～655.35 | Factory value: Model determined |
| --- | --- | --- | --- |
| F7-58 | Software Version (L) | Range: 0.00～655.35 | Factory value: Model determined |
| F7-59 | Software Release Date | Range: 0～65535 | Factory value: Model determined |

The above parameters are used to display the software version and release date information of the inverter.

| F7-61 | Enhanced Parameter Display Selection | Range: 0～1 | Factory value: 1 |
| --- | --- | --- | --- |

0: Display only 18 function parameter groups, i.e., F0-FF groups, U0-U1 groups.

1: Default display of 31 function parameter groups, i.e., F0～FF groups, U0～U1 groups, H0～H3 groups, L0～L8 groups.

| F7-62 | High Four Bits of Serial Code | Range: 0～65535 | Factory value:\ |
| --- | --- | --- | --- |
| F7-63 | Low Three Bits of Serial Code | Range: 0～65535 | Factory value:\ |

| F7-64 | Language Setting | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

In actual use, after modifying the language via the keyboard or host computer, it will be saved in this function code for easy recall of the last user-set language type after power loss.

0: Simplified Chinese

1: English

| F7-65 | Page Display Selection 2 | Range: 0～64 | Factory Value: 2 |
| --- | --- | --- | --- |

Page display selection 2 functions similar to F7-21, supporting only LED keyboards.

2.9 F8 Group Communication Parameters

| F8-00 | Baud Rate Setting | Range: 4.8kbps～115.2kbps | Factory value: 115.2kbps |
| --- | --- | --- | --- |

This parameter configures the baud rate for Modbus communication on the inverter's RS485 terminal. This parameter can only be set to one of the following values; otherwise, it will be forced to 9.6 kbps.

Setting range：

4.8：4800 bps

9.6：9600 bps

19.2：19200 bps

38.4：38400 bps

57.6：57600 bps

115.2：115200 bps

| F8-01 | Communication Data Format | Range：1～17 | Factory value：12 |
| --- | --- | --- | --- |

This parameter configures the data format for Modbus communication on the inverter's RS485 terminal.

0：7,N,1 for ASCII

1: 7,N,2 for ASCII

2: 7,E,1 for ASCII

3: 7,O,1 for ASCII

4: 7,E,2 for ASCII

5: 7,O,2 for ASCII

6: 8,N,1 for ASCII

7: 8,N,2 for ASCII

8: 8,E,1 for ASCII

9: 8,O,1 for ASCII

10: 8,E,2 for ASCII

11:8,O,2 for ASCII

12:8,N,1 for RTU

13:8,N,2 for RTU

14:8,E,1 for RTU

15:8,O,1 for RTU

16:8,E,2 for RTU

17:8,O,2 for RTU

| F8-02 | Communication Address | Range：1～254 | Factory value: 1 |
| --- | --- | --- | --- |

This parameter sets the address of the inverter as a slave during Modbus communication on the RS485 terminal.

| F8-03 | Response Delay | Range：0.0ms～200.0ms | Factory value：2.0ms |
| --- | --- | --- | --- |

This parameter sets the response delay time for Modbus communication on the inverter's RS485 terminal, generally no change is required.

| F8-04 | Communication timeout time | Range: 0.0s～100.0s | Factory Value: 0.0s |
| --- | --- | --- | --- |

This parameter sets the Modbus communication timeout time for the inverter's RS485 terminal; generally, it does not need to be changed.

| F8-05 | Communication error handling | Range: 0～3 | Factory Value: 3 |
| --- | --- | --- | --- |

This parameter sets the handling method after the Modbus communication timeout for the inverter's RS485 terminal.

0: Warning and continue running

1: Warning and decelerate to stop

2: Warning and free stop

3: No warning

| F8-06 | Communication given frequency | Range: 0.00Hz～599.00Hz | Factory Value: 0.00Hz |
| --- | --- | --- | --- |

This parameter is read-only and displays the set frequency under Modbus communication for the inverter's RS485 terminal.

| F8-07 | Communication decoding method | Range: 0～1 | Factory value: 1 |
| --- | --- | --- | --- |

Read/Write Permission: User Setting

Valid Range: 0: Use Decoding Method 1 (20xx)

1: Use Decoding Method 2 (60xx)

Table 2-19

| Communication Decoding Methods |  | Decoding Method 1 (20xx) | Decoding Method 2 (60xx) |
| --- | --- | --- | --- |
| Control Source | Digital Operator | No effect, control source: Digital Operator button control |  |
|  | External terminals | No effect, control: controlled by external terminals |  |
|  | RS-485 | Reference address area is 2000h~20FFh | Reference address area is 6000h~60FFh |
|  | CANopen | Reference address area is 2020-01h~2020-FFh | Reference address area is 2060-01h~2060-FFh |
|  | Communication card | Reference address area is 2000h~20FFh | Reference address area is 6000h~60FFh |

| F8-09 | Communication Card Type | Range: 0～12 | Factory value: 0 |
| --- | --- | --- | --- |

Read/write permission: User read-only

Valid range:

2: PROFIBUS-DP

3: CANopen

5: EtherNet/IP

6: EtherCAT

12: PROFINET

| F8-10 | Communication Card Version | Range: 0.0～6553.5 | Factory value: 0.0 |
| --- | --- | --- | --- |

Read/write permission: User read-only

Valid range: After module card connection, firmware version number is automatically written

| F8-11 | Communication Card Address | Range: 0～65535 | Factory value: 0 |
| --- | --- | --- | --- |

Read/Write Permission: User Setting

Valid range: PROFIBUS-DP: 1~125

EtherCAT: 1~65535

| F8-14 | CANopen Node Address | Range: 0～127 | Factory value: 0 |
| --- | --- | --- | --- |

Read/Write Permission: User Setting

Valid range: 0: Off ;1~127

| F8-15 | CAN Bus Communication Speed | Range: 0～5 | Factory value: 0 |
| --- | --- | --- | --- |

Read/Write Permission: User Setting

Valid range:

0: 1 Mbps

1: 500 Kbps

2: 250 Kbps

3: 125 Kbps

4: 100 Kbps

5: 50 Kbps

| F8-18 | CANopen warning record | Range: 0～65535 | Factory value: 0 |
| --- | --- | --- | --- |

Read/write permission: User read-only

Valid range:

bit 0: CANopen software disconnection 1 (CANopen Guarding Time out)

bit 1: CANopen software disconnection 2 (CANopen Heartbeat Time out)

bit 3: CANopen SDO transmission timeout warning (CANopen SDO Time out)

bit 4: CANopen SDO receive buffer overflow warning (CANopen SDO buffer overflow)

bit 5: CANopen hardware disconnection warning (Can Bus Off)

bit 6: CANopen format error warning (Error protocol of CANopen)

| F8-19 | CiA402 protocol selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

Read/Write Permission: User Setting

Valid range:

0: Enable custom communication decoding method (Refer to parameter F8-07 setting)

1: Enable CANopen standard CiA402 specification

| F8-20 | CANopen Communication Status | Range: 0～5 | Factory value: 0 |
| --- | --- | --- | --- |

Read/write permission: User read-only

Valid range:

0: Node reset status

1: Communication reset status

2: Reset completion status

3: Pre-operational status

4: Operational status

5: Stop status

| F8-21 | CiA402 operational status | Range: 0～14 | Factory value: 0 |
| --- | --- | --- | --- |

Read/write permission: User read-only

Valid range:

0: Power-on incomplete status

1: Prohibited operation status

2: Pre-excitation status

3: Excitation status

4: Operation allowed status

7: Quick stop action status

13: Error trigger action status

14: Error State

| F8-22 | CANopen Index Reset | Range: 0000H~FFFFH | Factory Value: 65535 |
| --- | --- | --- | --- |

Read/Write Permission: User Setting

Valid range:

bit 0: When CANopen is reset, reset the value at internal address 20XX to 0

bit 1: When CANopen is reset, reset the value at internal address 264X to 0

bit 2: When CANopen is reset, reset the value at internal address 26AX to 0

bit 3: When CANopen is reset, reset the value at internal address 60XX to 0

| F8-26 | Communication Card DHCP Enable | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

Read/Write Permission: User Setting

Valid range:

0: Disable DHCP function, use static IP

1: Enable DHCP function, use dynamic IP

| F8-27 | Communication Card IP 1 | Range: 0～255 | Factory value: 0 |
| --- | --- | --- | --- |
| F8-28 | Communication Card IP 2 | Range: 0～255 | Factory value: 0 |
| F8-29 | Communication Card IP 3 | Range: 0～255 | Factory value: 0 |
| F8-30 | Communication Card IP 4 | Range: 0～255 | Factory value: 0 |

Read/Write Permission: User Setting

Valid Range: 0~255

Use parameters specific to the EtherNet/IP communication card.

| F8-31 | Communication Card Subnet Mask 1 | Range: 0～255 | Factory value: 0 |
| --- | --- | --- | --- |
| F8-32 | Communication Card Mask 2 | Range: 0～255 | Factory value: 0 |
| F8-33 | Communication Card Mask 3 | Range: 0～255 | Factory value: 0 |
| F8-34 | Communication Card Mask 4 | Range: 0～255 | Factory value: 0 |

Read/Write Permission: User Setting

Valid Range: 0~255

Use parameters specific to the EtherNet/IP communication card.

| F8-35 | Communication Card Gateway 1 | Range: 0～255 | Factory value: 0 |
| --- | --- | --- | --- |
| F8-36 | Communication Card Gateway 2 | Range: 0～255 | Factory value: 0 |
| F8-37 | Communication Card Gateway 3 | Range: 0～255 | Factory value: 0 |
| F8-38 | Communication Card Gateway 4 | Range: 0～255 | Factory value: 0 |

Read/Write Permission: User Setting

Valid Range: 0~255

Use parameters specific to the EtherNet/IP communication card.

2.10 Group F9 Fault and Protection Parameters

| F9-00 | Protection Control Bit | Range: 000h～FFFFh | Factory value: 000h |
| --- | --- | --- | --- |

This function code can be used to mask certain fault protections.

When the setting value of this function code is 0200H, it can mask three fault protections: U phase current detection error (E033), V phase current detection error (E034), and W phase current detection error (E035).

When the setting value of this function code is 0008H, it can mask low frequency overload fault (E087), which is generally not recommended as it may damage the inverter.

When the setting value of this function code is 0001H, it can mask high ambient temperature fault (E017).

| F9-01 | Motor 1 Overload Selection | Range: 0～2 | Factory Value: 2 |
| --- | --- | --- | --- |
| F9-02 | Motor 1 Overload Time | Range: 30.0～600.0 seconds | Factory setting: 60.0 seconds |

Function code F9-01 is used to set the overload protection mode.

0: Constant Torque Output Motor

1: Variable Torque Output Motor

2: No Motor Overload Protection

Function code F9-02 is used to set the overload protection time when the motor current reaches 150% of the rated current. By setting an appropriate inverse-time curve, the operating time of Motor 1 under overload conditions can be made less than the set overload protection time for Motor 1, thereby achieving overload protection for Motor 1 and preventing damage due to overheating. When the overload time reaches the overload protection time for Motor 1, an overload fault for Motor 1 (E022) will be reported. This function is not enabled by default; if it needs to be activated, set F9-01 to 0 or 1.

When F9-01 is 0, the overload protection curve for the motor is shown in Figure 2-31. Here, T equals the set value of parameter F9-02, and “motor current percentage” refers to the ratio of the inverter output current to the motor rated current. When the motor fan is independently controlled, this inverse time curve can be selected. In this case, the fan speed is independent of the motor speed, so the cooling capacity does not decrease as the motor speed decreases, making the inverse time curve independent of the motor operating speed.

Figure 2-31Inverse Time Curve1

When F9-01 is 1, the overload protection curve for Motor 1 is shown in Figure 2-32. Among them,the “speed coefficient” is a function of motor speed, when the motor speed is greater than the rated speed, the speed coefficient equals 1, when the motor speed is less than the rated speed, the speed coefficient=1/(0.4+0.6*motor speed/motor rated speed).

Figure 2-32Inverse Time Curve2

When the motor uses coaxial cooling (fan connected to the motor shaft), the fan speed is the same as the motor speed, and a decrease in motor speed will reduce the fan's cooling capacity. In this case, it is recommended to set F9-01 to 1, the inverse time curve will be adjusted according to the motor speed, the motor overload capability will decrease with the reduction in speed, preventing overheating due to reduced fan cooling capacity. When the motor speed is zero, the fan stops rotating, at this time the speed factor is 2.5, the corresponding overload curve is shown in Fig.2-33. From Fig. 2-33it can be seen that when the current reaches 60% of the motor's rated current, the motor will report an overload fault after running for T time.

Fig. 2-33 Inverse Time Curve3

| F9-03 | Overvoltage Stall Mode | Range: 0～3 | Factory value: 1 |
| --- | --- | --- | --- |

This function code is used to set the overvoltage stall prevention mode. During motor deceleration, energy feedback may cause the bus voltage to rise; if the bus voltage becomes too high, it can lead to an overvoltage fault. By reducing the deceleration ramp, the increase in bus voltage can be suppressed, thus avoiding shutdown due to overvoltage.

0: Overvoltage Stall Mode 0

If the inverter detects that the bus voltage is higher than the set value of F9-04, the inverter will stop decelerating (the output frequency remains unchanged) until the bus voltage is lower than the set value of F9-58, at which point the inverter will continue to decelerate.

1: Overvoltage Stall Mode 1

During the deceleration process, dynamically adjust the speed curve to prevent overvoltage faults due to excessively high bus voltage.

2: Overvoltage Stall Mode2

During the deceleration process, dynamicallyaccurately and quicklyadjust the speed curve to prevent overvoltage faults due to excessively high bus voltage.

3: Overvoltage Stall Mode3

During deceleration, it is mainly used due to energy feedback, whichcauses overvoltage faults due to excessive bus voltage.

It should be noted that when the overvoltage stall prevention function is activated, the inverter's deceleration time will be longer than the set time. If automatic adjustment of the deceleration time is not allowed, this function needs to be disabled. To prevent overvoltage, the following measures can be taken:

1. Appropriately increase the deceleration time;

2. Install a braking resistor to dissipate the motor's feedback energy.

| F9-04 | overvoltage stall threshold | Range: 0.0～900.0V | Factory setting: 760.0V |
| --- | --- | --- | --- |

This function code is used to set the overvoltage stall threshold. When the set value is 0.0, the overvoltage stall prevention function is disabled. When the inverter is equipped with a braking unit and connected to a braking resistor, it is recommended to use this setting. When the set value is not 0.0, the overvoltage stall prevention function is effective. This parameter can be set according to the power supply and load conditions; if set too low, it may extend the deceleration time. If the set value exceeds the overvoltage protection point, the overvoltage stall prevention function is considered to be turned off.

| F9-07 | maximum current limit | Range: 0～250% | Factory value: 150% |
| --- | --- | --- | --- |

This function code is used to set the maximum current output of the inverter, which, together with the settings of function codes F3-23～F3-26, determines the output current limit of the inverter. The unit is %, based on the rated current of the inverter.

| F9-08 | Acceleration OC Stall Threshold | Range: 0%～200% | Factory value: 180% |
| --- | --- | --- | --- |
| F9-09 | Overcurrent Stall Limit Threshold | Range: 0%～100% | Factory setting: 100% |

Generally speaking, the faster the acceleration, the greater the torque current required. If the acceleration is too fast, it may lead to excessive motor current. To protect the motor and inverter, it is necessary to limit the current. The overcurrent prevention function during acceleration can prevent overcurrent situations caused by excessively fast acceleration. The principle of the overcurrent prevention function during acceleration is not complicated. When the current exceeds the set value, the inverter stops accelerating. Once the current drops below the set value, the inverter resumes acceleration, as shown in Figure2-34.

Figure 2-34 Overcurrent Stall Process

F9-08 Overcurrent Stall Threshold During Acceleration

This function code is used to set the overcurrent stall prevention threshold during acceleration, with the unit being %, based on the rated current of the inverter. When the inverter accelerates, if the output current exceeds the value set by F9-08, the inverter will stop accelerating. When the current falls below the value set by F9-08, the inverter resumes acceleration to the set frequency.

F9-09 Overcurrent Stall Limit Threshold

This function code is used to set the overcurrent stall prevention threshold during acceleration when weakening the magnetic field. When the motor operating frequency is greater than the rated frequency, the overcurrent stall prevention threshold during acceleration equals the value set by F9-08 multiplied by the value set by F9-09. For example: If F9-08 = 150%, and F9-09 = 80%, when the motor operating frequency is greater than the motor's rated frequency, the overcurrent stall prevention threshold during acceleration is: F9-08 set value × F9-09 set value = 150% × 80% = 120%.

Figure 2-35 Overcurrent Stall Prevention Threshold During Acceleration with Flux Weakening

It is important to note that when the overcurrent stall prevention function activates during acceleration, the inverter's acceleration time will be longer than the set time.

| F9-10 | Running OC Stall Threshold | Range: 0～200% | Factory value: 180% |
| --- | --- | --- | --- |
| F9-11 | Constant Speed OC Acceleration/Deceleration Selection | Range: 0～5 | Factory value: 0 |

The above two function codes are used to set the overcurrent stall prevention function during operation. Generally speaking, the larger the motor load, the greater the motor current. If the motor load is too large, it can lead to excessive motor current. If the motor load exceeds the motor's capacity, it may even result in loss of control. The overcurrent stall prevention function during operation can prevent the aforementioned issues of excessive current or loss of control. The principle of the overcurrent stall prevention function during operation is shown in Figure2-36. When the motor current exceeds the set value, the inverter begins to decelerate until the current drops below the allowable value, at which point the motor reaccelerates to the set frequency.

Figure 2-36 Overcurrent Stall Prevention Function During Operation

F9-10 Overcurrent Stall Threshold During Operation

This function code is used to set the overcurrent stall prevention threshold during operation, with the unit being %, based on the rated current of the inverter. When the inverter is running, if the output current exceeds the F9-10 setting, the inverter will decelerate according to the acceleration/deceleration time selected by F9-11 to prevent motor stall. When the output current falls below 95% of the F9-10 setting, the inverter will reaccelerate to the set frequency according to the acceleration/deceleration time selected by parameter F9-11.

F9-11 Constant Speed OC Acceleration/Deceleration Selection

This function code is used forconstant speedoperation overcurrentacceleration/deceleration selection

0: System Acceleration/Deceleration Time

1: First acceleration/deceleration time (F0-13 F0-14)

2: Second acceleration/deceleration time (F7-03 F7-04)

3: Third acceleration/deceleration time (F7-05 F7-06)

4: Fourth acceleration/deceleration time (F7-07 F7-08)

5: Automatic Acceleration/Deceleration Time

| F9-12 | Input Phase Loss Action Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |
| F9-13 | Input Phase Loss Filter Time | Range: 0.00～600.00 seconds | Factory Value: 0.20 seconds |
| F9-14 | Input Phase Loss Voltage Threshold | Range: 0.0～320.0V | Factory setting: 60.0V |

When the power supply is not correctly connected to the inverter, or when there is an abnormality in the power supply, an input phase loss fault may occur. When an input phase loss occurs, the inverter bus voltage may fluctuate significantly, causing fluctuations in motor torque or speed, and also affecting the lifespan of the bus capacitor. The input phase loss detection function can detect whether an input phase loss fault has occurred and take protective measures in time.

F9-12 Input Phase Loss Action Selection

This function code is used to set the inverter's action when a phase loss occurs on the input side.

0: Warning and decelerate to stop

1: Warning and free stop

F9-13 Input Phase Loss Filter Time

This function code is used to set the low-pass filter time required for phase loss detection, which generally does not need to be modified.

F9-14 Input Phase Loss Voltage Threshold

This function code is used to set the voltage threshold required for phase loss detection, which generally does not need to be modified.

| F9-15 | Output Phase Loss Action Selection | Range: 0～3 | Factory Value: 3 |
| --- | --- | --- | --- |
| F9-16 | Output Phase Loss Detection Time | Range: 0.000～65.535 seconds | Factory Value: According to model |
| F9-17 | Output Phase Loss Current Threshold | Range: 0.00～100.00% | Factory Value: According to model |

7.5kw and aboveOutput phase loss detection timeis 0.500; 5.5kW and belowOutput phase loss detection timeis 0.100.

7.5kW and aboveOutput phase loss current thresholdis 2.00; 5.5kW and belowOutput phase loss current thresholdis 6.00.

| F9-18 | Output Phase Loss Braking Time | Range: 0.000～65.535 seconds | Factory setting: 0.000 seconds |
| --- | --- | --- | --- |

When there is an abnormal connection between the motor and the inverter, output phase loss may occur. During output phase loss, the current in the disconnected phase of the motor is zero. To maintain motor operation, the current in the remaining phases will increase, and at the same time, speed and torque will experience significant fluctuations, which may cause damage to the motor and inverter. Output phase loss detection can identify motor phase loss conditions and take appropriate actions.

F9-15 Output Phase Loss Action Selection

This function code is used to set the inverter's action when output phase loss occurs.

0: Warning and continue running

1: Warning and decelerate to stop

2: Warning and free stop

3: No warning

F9-16 Output Phase Loss Detection Time

This function code is used to set the output phase loss detection time during operation, which generally does not need to be modified.

F9-17 Output Phase Loss Current Threshold

This function code is used to set the output phase loss current detection threshold, which generally does not need to be modified.

F9-18 Output Phase Loss Braking Time

This function code is used for output phase loss judgment at startup. If the set value is not zero, output phase loss judgment will be performed immediately upon startup.

The following will introduce four scenarios.

Example 1: F9-18 = 0, no pre-operation output phase loss detection. As shown in Figure 2-37,inverter operation, if anyphase output current is less than the threshold set by F9-17 and exceeds the time set by F9-16, the inverter will begin to execute the action set by F9-15.

Figure 2-37 F9-18=0

Example 2: The inverter is in a stopstate,F9-18 = 0 and F1-09 ≠ 0. As shown in Figure 2-38, when starting, DC braking is performed according to the settings of F1-08 and F1-09,during which phase loss detection is not performed. After DC braking is completed, the inverter begins to operate and performs phase loss detection according to method1.

Figure 2-38 F9-18 = 0 and F1-09 ≠ 0

Example 3: The inverter is in a stopstate,F9-18 ≠ 0 and F1-09 ≠ 0. During startup, DC braking is performed according to the time set in F9-18, followed by DC braking according to the time set in F1-09. Within the time set in F9-18, the DC braking currentsizeis20 times the value set in F9-57; Within the time set in F1-09, the DC braking currentsizeisthe value set in F1-08. Total DC braking time = the value set in F9-18 + the value set in F1-09.

Example 3-1: F9-18 ≠ 0 and F1-09 ≠ 0 (no output phase loss detected at startup), as shown in Figure 2-39.

Figure 2-39 F9-18 ≠ 0 and F1-09 ≠ 0 (No output phase loss detected at startup)

Example 3-2: F9-18 ≠ 0 and F1-09 ≠ 0, output phase loss detected at startup. As shown in Figure 2-40, if an output phase loss occurs within the time set by F9-18, after half of the time set by F9-18 has passed, the inverter begins to execute the action set by F9-15.

Figure 2-40 F9-18 ≠ 0 and F1-09 ≠ 0, (Output phase loss detected at startup)

Example 4: The inverter is in a stopped state, F9-18 ≠ 0 and F1-09 = 0, at startup, DC braking is performed according to the time set by parameter F9-18, with the DC braking current being 20 times the value set by F9-57.

Example 4-1: F9-18 ≠ 0 and F1-09 = 0 (no output phase loss detected at startup) , as shown in Figure 2-41.

Figure 2-41 F9-18 ≠ 0 and F1-09 = 0 (no output phase loss detected at startup)

Example 4-2: F9-18 ≠ 0 and F1-09 = 0, output phase loss detected at startup. As shown in Figure 2-42, if an output phase loss occurs within the time set by F9-18, after half of the time set by F9-18 has passed, the inverter begins to execute the action set by F9-15.

Figure 2-42 F9-18 ≠ 0 and F1-09 = 0 (output phase loss detected at startup)

| F9-20 | Ground Fault Current Threshold | Range: 0.0～6553.5% | Factory Value: 60.0 |
| --- | --- | --- | --- |
| F9-21 | Ground Fault Filter Time | Range: 0.00～655.35 seconds | Factory Value: 0.10 |

When a ground fault occurs, it may lead to excessive motor winding current, motor overheating, etc., which can severely damage the motor or inverter. Therefore, it is necessary to detect ground faults and address them promptly when they occur.

F9-20 Ground Fault Current Threshold

This function code is used to set the ground fault judgment current threshold, measured in %, based on the inverter's rated current.

F9-21 Ground Fault Filter Time

This function code is used to adjust the low-pass filter time for ground fault detection current, which generally does not need to be changed.

The above two function codes can be used to set the ground fault detection function. When the absolute value of the sum of the three-phase currents exceeds the set value of F9-20, a ground fault is detected.

| F9-22 | Low Current Set Threshold | Range: 0.0～100.0% | Factory Value: 0.0% |
| --- | --- | --- | --- |
| F9-23 | Low Current Detection Time | Range: 0.00～360.00 seconds | Factory Value: 0.00 seconds |
| F9-24 | Low Current Action Method | Range: 0～3 | Factory value: 0 |

Low current protection is designed to prevent the inverter or motor from operating for extended periods at low current levels. Current lower than normal values typically will not damage the inverter or motor, but may cause system abnormalities due to the motor's inability to deliver the desired torque.

F9-22 Low current set threshold

F9-23 Low current detection time

The above two function codes are used to set the low current detection conditions. F9-22 low current set threshold is in units of %, based on the rated current of the inverter. When the inverter output current is less than the F9-22 set value and persists for longer than the time set by F9-23, a low current fault is detected, and the inverter will determine subsequent actions according to the low current action method set by F9-24.

F9-24 Low current action method

0: No Function

Low current protection function is disabled, making the low current set threshold (F9-22) and low current detection time (F9-23) invalid.

1: Fault and Free Stop

When the low current protection condition is met, a low current fault is reported,Free shutdown.

2: Second deceleration stop due to fault

When the low current protection condition is met, a low current fault is reported, and the machine decelerates and stops according to the second deceleration time.

3: Alarm and continue running

When the low current protection condition is met, a low current warning is issued, but the machine does not shut down.

| F9-25 | Excessive slip detection value | Range: 0.0～100.0% | Factory Value: 0.0% |
| --- | --- | --- | --- |
| F9-26 | Excessive slip detection time | Range: 0.0～10.0 seconds | Factory setting: 1.0 second |
| F9-27 | Excessive slip action selection | Range: 0～3 | Factory value: 0 |

The slip is the difference between the synchronous speed of an asynchronous motor and the actual speed of the motor.Generally speaking, the greater the load torque, the greater the slip. If an excessive slip occurs, it may be due to an overload or other abnormal conditions, so timely action is required.

F9-25 Excessive Slip Detection Value

F9-26 Excessive Slip Detection Time

The above two function codes are used to set the excessive slip detection conditions. F9-25 Excessive Slip Detection Value is in units of %, based on the motor's rated slip. When the actual slip of the induction motor exceeds the F9-25 setting value and persists for longer than the time set by F9-26, an excessive slip fault is detected, and the inverter will take subsequent actions as determined by F9-27 Excessive Slip Action Selection.

F9-27 Excessive slip action selection

0: Warning and continue running

When the over-slip detection condition is met, an over-slip warning is issued, but the machine does not stop.

1: Fault and decelerate to stop

When the over-slip detection condition is met, an over-slip fault is reported, and the machine decelerates to stop.

2: Fault and free stop

When the over-slip detection condition is met, an over-slip fault is reported,Free stop.

3: No warning

Do not perform excessive slip fault detection.

| F9-36 | Over Torque Selection 1 | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |
| F9-37 | Over-torque threshold 1 | Range: 10～250% | Factory setting: 120% |
| F9-38 | Over-torque time 1 | Range: 0.1～60.0 seconds | Factory setting: 0.1 second |

The above function codes are used to set up the over-torque detection feature. The inverter issues a warning or stops operation when it detects excessive torque, serving to protect the motor and the inverter.

F9-36 Over-torque selection 1

0: Not Detected

1: Constant Speed Detection Continue Running

When running at constant speed, if Motor 1 experiences over-torque, a warning is issued but operation continues.

2: Constant Speed Detection Stop Running

When running at constant speed, if Motor 1 experiences over-torque, an over-torque fault is reported and operation stops.

3: Running Detection Continue Running

During operation, if Motor 1 experiences over-torque, a warning is issued but operation continues.

4: Running Detection Stop Running

During operation, if Motor 1 experiences over-torque, an over-torque fault is reported and operation stops.

F9-37 Over-torque threshold 1

F9-38 Over-torque time 1

When the inverter output current exceeds F9-37 (unit %, based on the inverter's rated current) and persists for longer than the set time of F9-38, the inverter will determine subsequent actions according to F9-36. As shown in Figure 2-43,when F9-36 is 1 or 3, if over-torque is detected, the inverter will display an over-torque warning, but the inverter will continue to run until the output current is less than 95% of the F9-37 set value, after which the warning will be cleared. As shown in Figure 2-44,when F9-36 is 2 or 4, if over-torque is detected, the inverter will report an over-torque fault and stop running until the fault is reset, after which it can resume operation.

Figure 2-43 Schematic Diagram of Over-torque Detection 1

Figure 2-44 Schematic Diagram of Over-torque Detection 2

| F9-46 | Abnormal Start Count | Range: 0～10 | Factory value: 0 |
| --- | --- | --- | --- |
| F9-47 | Abnormal Restart Reset Time | Range: 0.0～6000.0 seconds | Factory Value: 60.0 |

When an abnormality (overcurrent and overvoltage) occurs, the inverterwillgenerally stop running until the fault is reset and a run command is received, after which it will restart. The automatic restart function can automatically clear faults after they occur and control the inverter to restart, achieving continuous motor operation. After a fault occurs, if the number of automatic restart attempts is not zero, the inverter fault will be cleared, and a speed search will be performed before controlling the motor to run to the set frequency. If the number of retries after an abnormality is zero, the fault will not be cleared, and the inverter will remain in a shutdown state.

F9-46 Abnormal Restart Attempts

This function code is used to set the number of automatic restarts after an abnormality. If set to zero, the inverter will not automatically restart after an abnormality. When automatically restarting after an abnormality, the inverter will start according to the settings in F1-02. If the number of abnormalities exceeds the value set in F9-46, the fault will not automatically reset; it must be manually reset and a new run command received before operation can continue.

F9-47 Abnormal Restart Reset Time

After an abnormal restart occurs, if no further abnormalities occur within the time set by this function code, the F9-46 Abnormal Restart Attempts will be reset to the set value.

| F9-48 | PTC Action Selection | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |
| F9-49 | PTC Threshold | Range: 0.0～100.0% | Factory value: 50.0% |
| F9-50 | PT Detection Threshold 1 | Range: 0.000～10.000V | Factory value: 5.000V |
| F9-51 | PT Detection Threshold 2 | Range: 0.000～10.000V | Factory value: 7.000V |
| F9-52 | PT protection frequency | Range: 0.00～599.00Hz | Factory Value: 0.00Hz |
| F9-53 | PT Action Delay Time | Range: 0～6000 seconds | Factory value: 60 seconds |

Example of PT100 wiring and usage:

Connect PT100 between AO1 and GND, short-circuit AO1 and AI1

Step one, F6-13 “AO1 signal type”, select 1 “0-20mA output”

Step two, F6-14 “AO1 output function”, select 23 “fixed voltage output”

Step three, F6-18 “AO1 output fixed value”, can be set to 50.00% (corresponding to 10mA)

Step four, F5-20 “AI1 signal type”, select 0 “0-10V input selection”

Step five, F5-21 “AI1 function selection”, select 11 "Thermistor PT100 value"

Step six, based on the AO1 output current and thermistor resistance value, the voltage drop can be calculated, then adjust F9-50 “PT detection threshold 1” and F9-51 “PT detection threshold 2”

By obtaining the motor temperature through sensors, the motor can be protected according to the motor temperature. When the motor temperature exceeds a certain value, the motor operation will stop to prevent overheating damage. Thermistors are typically used to measure motor temperature, common thermistors include PTC resistors and PT100 resistors.

F9-48 PTC Action Selection

This function code is used to set the inverter action when PTC overheating occurs.

0: Warning and continue running

When motor overheating is detected, a motor overheat warning is issued, and the inverter continues to run.

1: Fault and decelerate to stop

When motor overheating is detected, a motor overheat fault is issued, and the inverter decelerates to stop.

2: Fault and free stop

When motor overheating is detected, a motor overheat fault is issued, and the inverter free stops.

3: No warning

Do not perform PTC detection.

F9-49 PTC Threshold

This function code is used to set the PTC overheating detection threshold, the unit is %, and the reference value is the maximum value of the analog input. When using the PTC overheating detection function, the corresponding analog input terminal must be set to voltage signal input, and the function of this analog input terminal is “Thermistor PTC Input”. When the feedback voltage reaches the set value of F9-49, the inverter will operate according to the method set in F9-48.

F9-50 PT Detection Threshold 1

F9-51 PT Detection Threshold 2

The above two function codes are used to set the PT100 over-temperature detection threshold.

F9-52 PT Protection Frequency

F9-53 PT Action Delay Time

The above two function codes are used to set the inverter's action when a PT100 over-temperature is detected. When using the PT100 over-temperature detection function, the corresponding analog input terminal must be set to voltage signal input. The function of this analog input terminal is “Thermistor PT100 Value”. When the feedback voltage is less than the set value of F9-50, the motor operates normally; When the feedback voltage is between the set values of F9-50 and F9-51, the inverter will run to the set frequency of F9-52 after the set time of F9-52. When the feedback voltage exceeds the set value of F9-51, the inverter will operate according to the method set in F9-48.

| F9-54 | STO Lock Function | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set whether an STO fault is allowed to reset automatically. If F9-54 is 0, after an STO fault is reported, even if the STO signal returns to normal, the STO fault cannot be cleared; it requires a power-off and power-on cycle for the STO fault to be cleared. If F9-54 is 1, after an STO fault is reported, if the STO signal returns to normal, the STO fault will be automatically cleared, and the inverter will return to its normal state.

| F9-57 | Output Phase Loss Threshold 2 | Range: 0.00%～100.00% | Factory Value: 2.00% |
| --- | --- | --- | --- |

This function code is used to set the current threshold for phase loss detection during startup. For details, refer to F9-15～F9-18.

| F9-58 | Overvoltage Stall Recovery Threshold | Range: 0.0V～900.0V | Factory Value: 630.0V |
| --- | --- | --- | --- |

This function code is used to set the recovery threshold for overvoltage stall prevention. For details, refer to F9-03.

2.11 FA Group PID Function

The PID principle block diagram is shown inFigure 2-45:

Figure 2-45 PID Principle Block Diagram

| FA-00 | PID Feedback Type Selection | Range: 0～8 | Factory value: 0 |
| --- | --- | --- | --- |

0: No Function

1: Negative Feedback Analog Input

2: Negative Feedback Pulse No Direction

3: Negative Feedback Pulse With Direction

4: Positive Feedback Analog Input

5: Positive Feedback Pulse No Direction

6: Positive Feedback Pulse With Direction

7: Negative Feedback Communication Input

8: Positive Feedback Communication Input

Positive Feedback: If the feedback value is less than the PID setpoint, the inverter output frequency decreases

Negative Feedback: If the feedback value is less than the PID setpoint, the inverter output frequency increases

| FA-01 | PID Setpoint Source Selection | Range: 0～6 | Factory value:1 |
| --- | --- | --- | --- |

0: Frequency Command

1: Parameter FA-02

2: RS485 Communication

3: Analog Input

4: CANopen

5: Retain

6: Communication Card

This parameter selects the channel for the PID target quantity.

| FA-02 | PID Setpoint | Range: -100.00%～100.00% | Factory Value: 50.00% |
| --- | --- | --- | --- |

When FA-01 (PID setpoint source) is set to 1, this parameter needs to be configured. This parameter is a relative value, where 100% corresponds to the maximum feedback value of the controlled system.

| FA-03 | PID Setpoint Change Time | Range: 0.00s～655.35s | Factory Value: 0.00 |
| --- | --- | --- | --- |

The time required for the PID setpoint (parameter FA-02) to change from 0.0% to 100.0%. When the PID setpoint changes, the actual setpoint does not respond immediately but changes linearly over the given time to prevent sudden changes in the setpoint.

| FA-04 | PID Feedback Filter Time | Range: 0.1s～300.0s | Factory value: 5.0 |
| --- | --- | --- | --- |

Filters the PID feedback value; this parameter helps reduce the impact of disturbances on the feedback but may degrade the response performance of the closed-loop control process.

| FA-05 | Proportional Coefficient 1 | Range: 0.00～100.00 | Factory Value: 88.00 |
| --- | --- | --- | --- |

The speed at which the deviation decreases depends on the proportional coefficient; the larger the proportional coefficient, the faster the deviation decreases. However, an excessively large proportional coefficient can lead to significant overshoot and oscillation, reducing stability, especially in systems with large lag. Reducing the proportional coefficient decreases the likelihood of system oscillation but slows down the response speed. When performing 2ms enhanced PID control (parameter FA-12 = 0), the number of decimal places for this parameter can be selected by parameter FA-53 bit 1, 0: 1 decimal place, 1: 2 decimal places.

| FA-06 | Integral Time 1 | Range: 0.00s～100.00s | Factory Value: 0.05 |
| --- | --- | --- | --- |

This parameter determines the strength of the integral action of the PID controller. The smaller the integral time, the stronger the integral action, which helps to reduce overshoot, decrease oscillation, and stabilize the system, but it will slow down the elimination of static error in the system.

| FA-07 | Differential Time 1 | Range: 0.00s～1.00s | Factory Value: 0.00 |
| --- | --- | --- | --- |

This parameter determines the strength of the rate-of-change adjustment of the PID controller. The longer the differential time, the greater the adjustment strength. When set appropriately, this parameter can reduce overshoot and shorten the adjustment time. Differential action amplifies noise interference, so excessive differential regulation is detrimental to the system's ability to resist interference. Additionally, when there is no change in the input, the differential action output is zero. Therefore, differential control is often combined with the other two control laws to form a PD controller or a PID controller.

| FA-08 | Proportional Coefficient 2 | Range: 0.00～100.00 | Factory value: 100.00 |
| --- | --- | --- | --- |

Refer to parameter FA-05; no further details are provided here.

| FA-09 | Integral Time 2 | Range: 0.00s～100.00s | Factory value: 0.08 |
| --- | --- | --- | --- |

Refer to parameter FA-06; no further details are provided here.

| FA-10 | Differential Time 2 | Range: 0.00s～1.00s | Factory Value: 0.00 |
| --- | --- | --- | --- |

Refer to parameter FA-07; no further details are provided here.

| FA-11 | PID Series/Parallel Selection | Range: 0～1 | Factory value: 1 |
| --- | --- | --- | --- |

0: Series, traditional PID control structure

1: Parallel, which separates proportional control, integral control, and differential control, allowing users to adjust P, I, and D controllers separately according to application needs. The parallel type PID is selected by default.

The block diagram of the PID series control is shown inFigure 2-46:

Figure 2-46 PID series control block diagram

The PID parallel control block diagram is shown inFigure 2-47:

Figure 2-47 PID Parallel Control Block Diagram

| FA-12 | PID Control Execution Cycle | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: Execute 2ms Enhanced PID Control

1: Execute 1ms Traditional PID Control

FA-12 = 0, the user selects to execute process PID control once every 2ms cycle, the PID output frequency reference base can be selected according to parameter FA-30, to choose 100.00% output corresponding to the maximum output frequency, or to choose 100.00% output corresponding to the auxiliary frequency. For example, if the user enables the main and auxiliary frequency function (assuming selecting main frequency + auxiliary frequency, parameter F0-07 = 1, F0-08 = 0), and the PID output frequency base selects the auxiliary frequency (FA-30 = 1), and the keyboard sets the auxiliary frequency to 40Hz, then the maximum PID output frequency is 40Hz.

FA-12 = 1, the user selects a 1ms cycle to execute the process PID control once, where a PID output of 100.00% corresponds to the maximum output frequency, and there is no auxiliary frequency option for the reference benchmark.

Additionally, the enhanced PID option (FA-12 = 0) supports soft start and automatic switching of two sets of PID parameters based on output frequency or deviation, while the traditional PID option (FA-12 = 1) does not have these features.

In some application scenarios, one set of PID parameters cannot meet the requirements of the entire operating process. In such cases, parameter FA-13 can be used to initiate the switching between two sets of PID parameters, with related parameters being FA-13 ~ FA-15.

| FA-13 | PID Parameter Switching Conditions | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

0: Use the first set of PID parameters FA-05 ~ FA-07

1: Automatically adjust according to the output frequency. Use the first set of PID parameters (FA-05 ~ FA-07) when operating at the minimum frequency (F2-04) or below, use the second set of PID parameters (FA-08 ~ FA-10) when operating at the maximum frequency (F4-02), and use a linear interpolation value of the two sets of PID parameters when the operating frequency is between F2-04 and F4-02.

2: Automatically switch based on the deviation between the setpoint and feedback. Use the first set of PID parameters (FA-05 ~ FA-07) when the absolute value of the deviation between the setpoint and feedback is less than the PID parameter switching deviation 1 (parameter FA-14). Use the second set of PID parameters (FA-08 ~ FA-10) when the absolute value of the deviation between the setpoint and feedback is greater than the PID parameter switching deviation 2 (parameter FA-15). When the absolute value of the deviation between the setpoint and feedback varies between FA-14 ~ FA-15, the PID parameters are the linear interpolation values of the two sets of PID parameters.

When FA-13 = 1,thePI parameter adjustment diagramis shown in Figure 2-48:

Figure 2-48 PI parameter adjustment diagram

When FA-13 = 2,thePI parameter adjustment diagramis shown in Figure 2-49:

Figure 2-49 PI parameter adjustment diagram

| FA-14 | PID Parameter Switching err1 | Range: 0.00%～100.00% | Factory setting: 10.00 |
| --- | --- | --- | --- |
| FA-15 | PID Parameter Switching Error 2 | Range: 0.00%～100.00% | Factory setting: 40.00 |
| FA-16 | Allow PID Reverse Delay | Range: 0.0s～6553.5s | Factory Value: 0.0 |

When parameter FA-16 ≠ 0, the reverse operation function is enabled after startup. For example, if FA-16 is set to 2.0, PID control is not allowed to change the direction of operation during the start-up period of 0 ~ 2 seconds (parameter FA-17 = 0), and after 2 seconds of start-up, PID control is automatically allowed to change the direction of operation (parameter FA-17 will be automatically updated to 1).

| FA-17 | PID Direction Change Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: Non-reversible running direction

1: Reversible running direction

| FA-18 | Feedback Suppression Deviation Rate | Range: 0%～100% | Factory value: 10% |
| --- | --- | --- | --- |

Parameters FA-18 and FA-19 are only valid when the air compressor application is selected (parameter L0-00 = 6) and 2ms enhanced PID control is performed (parameter FA-12 = 0).

| FA-19 | Feedback Suppression Gain | Range: 0～1000 | Factory value: 800 |
| --- | --- | --- | --- |

When the setpoint and feedback deviation are in opposite directions, parameters FA-18 and FA-19 provide feedback suppression anti-integral windup functionality to quickly exit the saturation state and respond to external inputs, avoiding the controller output lingering in the saturation zone for a long time, thus improving the controller's response capability. The controller performs anti-integral windup suppression based on the relationship between the feedback suppression deviation rate (parameter FA-18) and the 100ms deviation rate.

| FA-20 | PID Compensation Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: Parameter Setting

1: Analog Input

When FA-20 = 0, the PID compensation value (parameter FA-21) must be set.

| FA-21 | PID Compensation Value | Range: -100.0% to 100.0% | Factory Value: 0.0 |
| --- | --- | --- | --- |

The reference for this parameter is the maximum output frequency F4-02. Example: If the maximum output frequency parameter F4-02 = 50.00 Hz, and FA-21 is 10.0%, the PID compensation will increase the output frequency by 5.00 Hz.

| FA-22 | PID Deviation Deadband Limit | Range: 0.00%～100.00% | Factory Value: 0.06 |
| --- | --- | --- | --- |

When the PID control output exceeds FA-22, the PID regulation output becomes effective; otherwise, the PID regulator is inhibited. This parameter effectively prevents the actuator from oscillating when the PID output is small.

| FA-23 | PID Control Deviation Limit | Range: 0.00%～100.00% | Factory Value: 0.00 |
| --- | --- | --- | --- |

This parameter determines the level at which the deviation between the feedback and setpoint signals causes the PID regulation to stop, maintaining the previous output value. PID regulation output is only executed when the deviation between the feedback value and the setpoint exceeds the PID control deviation limit FA-23. Properly setting this parameter can adjust the accuracy and stability of the PID system,the function diagram is shown inFigure 2-50.

Figure 2-50 Diagram of PID Control Deviation Limit

| FA-24 | Integral Separation Level | Range: 0.00%～100.00% | Factory Value: 0.00 |
| --- | --- | --- | --- |

When the PID feedback overshoot is large at startup, integral separation can be enabled to reduce overshoot, with the parameter benchmark being the PID deviation.

When FA-24 ≠ 0, the integral separation function is activated, and it only operates once at startup. When the deviation between the setpoint and the feedback value exceeds the parameter FA-24, integral separation occurs to prevent excessive overshoot due to integral action; When the deviation is less than parameter FA-24, the integral action takes effect to eliminate steady-state error.

| FA-25 | Integral Upper Limit | Range: 0.00%～100.00% | Factory value: 100.0 |
| --- | --- | --- | --- |

This parameter is the upper limit of integration, with the reference being the maximum output frequency F4-02. When the integral value is too large, if the load suddenly changes, the response speed of the inverter slows down, which may cause motor slippage or mechanical damage. At this time, the parameter FA-25 can be appropriately reduced.

| FA-26 | Wake-up integral limit | Range: 0.0%～200.0% | Factory value: 50.0 |
| --- | --- | --- | --- |

This parameter is the upper limit of wake-up integration, used to reduce the reaction time from sleep to wake-up, with the reference being the maximum output frequency F4-02.

| FA-27 | Main Auxiliary Reverse Cut-off Frequency | Range: 0.00%～100.00% | Factory value: 10.0 |
| --- | --- | --- | --- |

In some cases, only when the PID output frequency is negative (i.e., the inverter runs in reverse), can the PID possibly control the setpoint and feedback to the same state. However, an excessive reverse frequency is not allowed in certain situations. Parameter FA-27 is used to determine the upper limit of the reverse frequency. The reference base for this parameter is the maximum output frequency F4-02.

| FA-28 | PID Output Positive Limit | Range: 0.00%～100.0% | Factory value: 100.0 |
| --- | --- | --- | --- |

This parameter is the upper limit of the PID control output command, with the reference base being the maximum output frequency F4-02.

| FA-29 | PID Output Negative Limit | Range: 0.00%～100.0% | Factory value: 100.0 |
| --- | --- | --- | --- |

When PID output reversal is allowed, the PID output is negative, and at this time, the output will be limited to the value set by parameter FA-29, which should be used in conjunction with parameter FA-17.

| FA-30 | PID output frequency reference | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: PID control output 100.00% corresponds to maximum output frequency F4-02

1: PID control output 100.00% corresponds to auxiliary frequency (if the auxiliary frequency command changes, the PID output frequency will also change)

This parameter is only valid when the main and auxiliary frequency function is enabled.

| FA-31 | PID output filter time | Range: 0.0s～2.5s | Factory Value: 0.0 |
| --- | --- | --- | --- |

This parameter is used to set the low-pass filter time for PID control output; the larger the parameter value, the greater the PID output filtering, and the slower the change in output frequency. Improper setting of parameter FA-31 may affect the response speed of the inverter, and even cause system oscillation.

PID soft startas shown in Figure 2-51. When the PID feedback overshoot is large at startup, soft start can be used to reduce the feedback overshoot; the soft start function only operates once at startup. When the soft start is enabled, it will first start according to the soft start frequency FA-33 and acceleration time FA-34. When the PID deviation is less than parameter FA-32, it switches back to normal PID control (when switching from soft start to PID control, the soft start frequency is used as the PID integral value to avoid discontinuity in frequency).

Figure 2-51PIDSoft StartSchematic Diagram

| FA-32 | soft start-PID switch value | Range: 0.00%～100.00% | Factory Value: 5.00 |
| --- | --- | --- | --- |

This parameter is based on the deviation between the PID setpoint and feedback value.

| FA-33 | soft start frequency | Range: 0.00Hz～599.00Hz | Factory Value: 0.00 |
| --- | --- | --- | --- |

When parameter FA-33 ≠ 0, the soft start function is enabled.

| FA-34 | soft start acceleration time | Range: 0.00s～600.00s | Factory value: 3.00 |
| --- | --- | --- | --- |

Time to accelerate from start to soft start frequency FA-33

| FA-35 | No-load current | Range: 0.00A～655.35A | Factory value: 0.00A |
| --- | --- | --- | --- |

When parameter FA-35 ≠ 0 and the inverter output current is greater than FA-35, it starts at the soft start frequency (parameter FA-33) and the soft start acceleration step (parameter FA-36), until the soft start acceleration time (parameter FA-34) is reached, then switches to normal PID control.

| FA-36 | Soft Start Acceleration Step | Range: 0.00s～600.00s | Factory Value: 0.10 |
| --- | --- | --- | --- |

Fuzzy PID control

FA-37 ~ FA-48 are parameters related to fuzzy PID control. In general, process control only requires ordinary PID adjustment; users do not need to enable fuzzy PID tuning functions unless necessary.

| FA-37 | Fuzzy PID Tuning Enable | Range: 0～1 | Factory value: 1 |
| --- | --- | --- | --- |

0: Fuzzy PID disabled, use only conventional PID

1: When PID control is not allowed to change the direction of operation (FA-17 = 0), enable fuzzy PID control, and perform real-time self-tuning of PID parameters during operation. Fuzzy PID control is enabled only when the air compressor application is selected (parameter L0-00 = 6) and 2ms enhanced PID control (parameter FA-12 = 0) is executed, or 1ms traditional PID control (parameter FA-12 = 1) is executed.

| FA-38 | Deviation Fuzzy Universe of Discourse - NB | Range: 0.00～100.00 | Factory Value: 5.00 |
| --- | --- | --- | --- |

Membership value of the linguistic term 'negative large' in the universe of discourse of the error fuzzy variable

| FA-39 | Deviation Fuzzy Domain-NS | Range: 0.00～100.00 | Factory setting: 2.00 |
| --- | --- | --- | --- |

Membership value of the linguistic term 'negative small' in the universe of discourse of the error fuzzy variable

| FA-40 | Deviation Fuzzy Domain-PS | Range: 0.00～100.00 | Factory setting: 2.00 |
| --- | --- | --- | --- |

The membership value of the linguistic term “positive small” in the domain of the deviation fuzzy variable fuzzy set

| FA-41 | Deviation Fuzzy Domain-PB | Range: 0.00～100.00 | Factory Value: 5.00 |
| --- | --- | --- | --- |

The membership value of the linguistic term “positive large” in the domain of the deviation fuzzy variable fuzzy set

| FA-42 | Deviation Rate Fuzzy Domain-NB | Range: 0.00～100.00 | Factory setting: 10.00 |
| --- | --- | --- | --- |

The membership value of the linguistic term “negative large” in the domain of the deviation rate fuzzy variable fuzzy set

| FA-43 | Deviation Rate Fuzzy Domain-NS | Range: 0.00～100.00 | Factory Value: 5.00 |
| --- | --- | --- | --- |

The membership value of the linguistic term “negative small” in the domain of the deviation rate fuzzy variable fuzzy set

| FA-44 | Deviation Rate Fuzzy Domain-PS | Range: 0.00～100.00 | Factory Value: 5.00 |
| --- | --- | --- | --- |

The membership value of the linguistic term “positive small” in the domain of the deviation rate fuzzy variable fuzzy set

| FA-45 | Fuzzy Set of Deviation Rate PB | Range: 0.00～100.00 | Factory setting: 10.00 |
| --- | --- | --- | --- |

The membership value of the linguistic term “positive large” in the domain of the deviation rate fuzzy variable fuzzy set

| FA-46 | Fuzzy PID Inference Rules | Range: 0～3 | Factory Value: 2 |
| --- | --- | --- | --- |

0: Fuzzy PID Inference Rule 0

1: Fuzzy PID Inference Rule 1

2: Fuzzy PID Inference Rule 2

3: Fuzzy PID Inference Rule 3

| FA-47 | Intermediate Value of Fuzzy Rule KP | Range: 0～100 | Factory Value: 50 |
| --- | --- | --- | --- |
| FA-48 | Intermediate Value of Fuzzy Rule KI | Range: 0～100 | Factory Value: 50 |

Parameters FA-47 and FA-48 are used to obtain the initial membership values for fuzzy variables in the control rule table where the fuzzy linguistic value is zero.

PID Abnormality Detection

Parameters related to PID abnormality detection processing are FA-49 ~ FB-53. When the AIx signal type parameters F5-20, F5-26, or F5-32 = 2 (i.e., selecting 4~20 mA analog input), parameters FA-49 and FA-50 are set effectively.

| FA-49 | Feedback Abnormality Detection Time | Range: 0.0s～3600.0s | Factory Value: 0.0 |
| --- | --- | --- | --- |

This parameter is used for detecting abnormal conditions or extremely slow response of the feedback analog signal; when FA-49 = 0, no detection is performed. When the sampled value of the analog signal is below the 4~20mA disconnection threshold (parameter F5-43) and persists for longer than FA-49, the feedback analog signal is considered abnormal, and the inverter performs the abnormal action as set by parameter FA-50, displaying an 'AFE' prompt on the operation panel.

| FA-50 | Feedback Disconnection Action Selection | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

0: Warning and continue running

1: Fault and deceleration stop

2: Fault and free stop

3: Warning and operation at the frequency before disconnection

| FA-51 | PID Feedback Abnormal Deviation | Range: 1.0%～50.0% | Factory value: 10.0 |
| --- | --- | --- | --- |

When the deviation between the setpoint and feedback signal exceeds the threshold (parameter FA-51) and persists for longer than the deviation abnormality detection time FA-52, a PID deviation fault occurs. If the function selection for output terminals F6-00 ~ F6-03 is 15, the output terminal will indicate a PID deviation warning.

| FA-52 | Abnormal Deviation Detection Time | Range: 0.1s～300.0s | Factory value: 5.0 |
| --- | --- | --- | --- |
| FA-53 | PID Control Flag | Range: 0000H～FFFFH | Factory Value: 2 |

PID control flag (bits 0 ~ 2 are valid)

bit 0: PID reverse action selection, 0: PID reverse based on PID calculation value, 1: reverse based on parameter F0-09;

bit 1: PID parameter Kp decimal point position selection, 0: 1 decimal place, 1: 2 decimal places;

bit 2: 0: no function, 1: when the main/auxiliary frequency function is enabled, the integral upper limit base value is the auxiliary frequency.

| FA-54 | PID Feedback Disconnection Threshold | Range: 0.00%～100.00% | Factory value: 0.00% |
| --- | --- | --- | --- |

When the PID feedback value falls below the PID feedback disconnection threshold and exceeds the time set by FA-52, the system will stop according to the setting in FA-50.

2.12 FB Group Tension Control Parameters

This chapter introduces several tension control schemes, mainly including tension closed-loop speed mode, line speed closed-loop mode, tension open/closed-loop torque mode, among four control methods (corresponding to FB-00 selection).

● Tension Closed-loop Speed Mode

Features: Through the use of a swing arm (floating roller) or tension sensor to feedback material tension, the inverter output frequency is adjusted in a closed loop to achieve constant control of the swing arm position or tension. Suitable for applications where there is a swing arm or tension sensor (where changes in speed have a slow impact on tension changes) and there is sufficient speed regulation margin.

The winder operates in closed-loop tension speed mode, with two AI signals receiving the swing arm position signal and the main traction line speed signal, respectively. Calculate the roll diameter through line speed, calculate the main frequency based on line speed, and adjust the output frequency together with the feedback loop of the rocker position. Compared with the main frequency + PID auxiliary frequency of general inverters, the addition of roll diameter calculation allows the main frequency setting to more accurately follow changes in line speed, thus making the control of the rocker position more stable.

● Constant Line Speed Control

Features: Suitable for applications without primary traction, where one machine in the winding/unwinding process operates in constant line speed mode, serving as the traction unit. The winder also serves as the main traction unit. To maintain the material running at a constant line speed, it is necessary to obtain the roll diameter value. The diagram above shows two feasible methods:

■ Using the counting signal input through DI terminals, calculate the roll diameter using the thickness accumulation method.

■ Install a speed measuring device on the material, transmit the signal to the inverter via pulse method, and calculate the roll diameter using line speed.

It is not necessary to use both of the above methods simultaneously as shown in the diagram; choose one of them or another feasible method. The winder operates in line speed closed-loop speed mode, with one AI channel receiving the line speed signal. By calculating the roll diameter based on line speed, the main frequency setting is determined according to the given operating line speed and the roll diameter, working together with the line speed feedback closed-loop regulation to decide the output frequency, achieving constant line speed operation without main traction. The unwinder can operate in one of the other three tension modes based on actual conditions.

● Closed-loop tension torque control

Features: Through feedback from tension sensors on material tension, closed-loop regulation of the inverter's output torque achieves constant tension control. This integrates mode 4 open-loop tension control torque method, using open-loop tension calculation to assist closed-loop tension regulation (in practice, mainly relying on closed-loop tension regulation, with the option to superimpose mode 4 open-loop tension control torque based on control effectiveness). The tension accuracy is high, suitable for materials with strong elasticity or situations where there is no speed adjustment margin.

The winder operates in closed-loop tension torque mode, with two AI signals receiving tension sensor signals and the line speed signal from the main traction unit, respectively. In implementation, an open-loop torque setting plus tension closed-loop regulation method is adopted. The calculation of roll diameter in open-loop tension is retained, while inertia and friction compensation can be omitted due to the addition of closed-loop regulation (or can be set to optimize response speed).

For elastic materials (with speed regulation margin) where tension sensors are used, it is also possible to consider using the tension closed-loop speed mode (Mode 1) to avoid the occurrence of elastic oscillation.

● Open-loop tension torque control

Features: No need for a swing arm (floating roller) or tension sensor; no tension closed-loop, slightly lower tension accuracy; suitable for applications with low tension accuracy requirements.

The winder operates in open-loop tension torque mode, using line speed to calculate the roll diameter, and output torque is calculated through material tension. Based on the actual system conditions, friction torque compensation or acceleration/deceleration inertia torque compensation can be selected.

The above typical application methods are only intended to illustrate the scenarios suitable for the four tension control modes and do not strictly limit them to these forms. Under basic conditions being met, flexible handling according to actual conditions is possible.

The previous section introduced four typical applications of tension control, whose implementation is mainly constrained by three essential conditions: roll diameter, line speed, and control quantity feedback,Table 2-20shows this.

Table2-20 Necessary Conditions for CM680 Tension Control Mode

| Function/Restriction | Roll Diameter[1] | Line Speed | Control Quantity Feedback |
| --- | --- | --- | --- |
| Tension Closed-loop Speed Mode | Necessary | Necessary[2] | Necessary |
| Line Speed Closed-loop Mode | Necessary | Not Necessary | Not Necessary |
| Tension Closed-loop Torque Mode | Necessary | Not Necessary | Necessary |
| Tension Open-loop Torque Mode | Necessary | Not Necessary | Not Necessary |

Note：

[1]：If the roll diameter source is selected as the line speed calculation channel (FB-25=0), then line speed is a necessary condition.

[2]：Inertia compensation, friction compensation are related to line speed, therefore, if considering improving the open-loop torque tension control accuracy, line speed is a necessary condition.

| FB-00 | Tension Control Mode Selection | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |

Set this parameter to select the tension control mode.

0: Tension control invalid, functions the same as a general inverter.

1: Tension closed-loop speed mode, requires tension/position detection and feedback, operates in speed mode, the inverter adjusts the output frequency by superimposing PID closed-loop calculations on the main frequency given based on line speed and roll diameter, to achieve stable set tension or position. Control methods can be selected from V/F control, sensorless vector control (SVC), and closed-loop speed sensor vector control (FVC).

2: Line speed closed-loop mode, the inverter adjusts the operating frequency according to changes in roll diameter, ensuring the system runs at a constant line speed. Control methods can be selected from V/F control, sensorless vector control (SVC), and closed-loop speed sensor vector control (FVC).

3: Tension closed-loop torque mode, requires tension detection and feedback, torque mode operation, the inverter adjusts by superimposing PID closed-loop calculation on the given torque, including acceleration/deceleration inertia torque compensation, friction torque compensation, etc., and can choose whether to superimpose mode 4 open-loop tension torque based on control effect. Select torque (TQC) control method to achieve ideal control results.

4: Open-loop tension torque mode, does not require tension/position detection and feedback, torque mode operation, the inverter controls the output torque to control the tension on the material. Select torque (TQC) control method to achieve ideal control results.

| FB-01 | Curling Mode | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: Winding mode

1: Unwinding mode

■ The tension taper function is invalid in unwinding mode.

■ In winding mode, the roller diameter will increase; in unwinding mode, the roller diameter will decrease.

| FB-02 | Load Side Mechanical Gear A | Range: 1～65535 | Factory Value: 100 |
| --- | --- | --- | --- |
| FB-03 | Mechanical gear B on the motor side | Range: 1～65535 | Factory Value: 100 |

Parameters FB-02, FB-03 are only applicable in tension control mode, as shown inFigure 2-52.

Figure 2-52 Tension control diagram

| FB-04 | PID Setpoint Source Selection | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

0: Parameter FB-05 serves as the PID target setpoint

1: RS-485 communication settings;

2: Analog input, must first set AI1/AI2/AI3 function to tension PID target value (Parameter F5-21 or F5-27 or F5-33 = 17).

| FB-05 | PID Target Setpoint | Range: 0.0%～100.0% | Factory value: 50.0% |
| --- | --- | --- | --- |

Effective when parameter FB-00 = 1 or 2. In line speed closed-loop speed mode (FB-00 = 2), AI1/AI2/AI3 functions must be set to line speed feedback (parameter F5-21 or F5-27 or F5-33 = 15), and FB-05 serves as the line speed PID setpoint. In other modes, when F5-21 or F5-27 or F5-33 = 15, FB-05 represents the actual line speed.

In tension closed-loop speed mode (FB-00 = 1), this parameter range 0.0 ~ 100.0% corresponds to tension feedback voltage 0~10 V; in line speed closed-loop speed mode (FB-00 = 2), it corresponds to 0 ~ maximum line speed (parameter FB-18).

| FB-06 | PID Feedback Source Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: Analog Input

1: Pulse Input

When FB-06 = 0, set the AI1/AI2/AI3 function to Tension PID Feedback Value (Parameter F5-21 or F5-27 or F5-33 = 14).

When FB-06 = 1, set the number of pulses per meter (FB-20).

| FB-07 | PID Parameter Adjustment Basis | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

0: No Switching

1: Automatic Adjustment Based on Roll Diameter

2: Switching Based on Operating Frequency

3: Automatic Switching Based on Setpoint and Feedback Deviation

When FB-07=0, use the first set of PID parameters FB-08~ FB-09

When FB-07=1, use the first set of PID parameters FB-08~ FB-09 for empty rolls, and the second set of PID parameters FB-10~ FB-11 for full rolls. When the current roll diameter varies between the maximum roll diameter (FB-26) and the minimum roll diameter (FB-27), the PID parameters are linearly interpolated values between the two sets of PID parameters.

FB-07=2, use the first set of PID parameters FB-08 ~ FB-09 when operating at the minimum frequency F2-04 and below, use the second set of PID parameters FB-10 ~ FB-11 when operating at the maximum frequency F4-02, and when the operating frequency varies between F2-04 ~ F4-02, the PID parameters are the linear interpolation values of the two sets of PID parameters.

FB-07=3, use the first set of PID parameters FB-08 ~ FB-09 when the feedback value is at the lower limit FB-16, use the second set of PID parameters FB-10 ~ FB-11 when the feedback value is at the upper limit FB-17, and when the feedback value varies between FB-16 ~ FB-17, the PID parameters are the linear interpolation values of the two sets of PID parameters.

When FB-07=1，the PI parameter adjustment diagramis shown in Figure 2-53:

Fig. 2-53 PI Parameter Adjustment Diagram

When FB-07=2, the PI parameter adjustment diagram is shown in Fig. 2-54:

Fig. 2-54 PI Parameter Adjustment Diagram

When FB-07=3, the PI parameter adjustment diagram is shown in Fig. 2-55:

Fig. 2-55 PI Parameter Adjustment Diagram

| FB-08 | PID Proportional Coefficient 1 | Range: 0.0～1000.0 | Factory value: 50.0 |
| --- | --- | --- | --- |
| FB-09 | PID Integral Time 1 | Range: 0.00s～500.00s | Factory Value: 1.00 |
| FB-10 | PID Proportional Coefficient 2 | Range: 0.0～1000.0 | Factory value: 50.0 |
| FB-11 | PID Integral Time 2 | Range: 0.00s～500.00s | Factory Value: 1.00 |
| FB-12 | PID Output Positive/Negative Selection | Range: 0～1 | Factory value: 0 |

Depending on different customer requirements, you canchoosethesuitablemethodbasedonTable2-21:

Table2-21 PID Output Polarity Selection

| Tension Feedback Value | Loose 0~100% Tight | Loose 0~100% Tight |
| --- | --- | --- |
| Winding | Positive Output | Negative Output |
| Unwinding | Negative Output | Positive Output |

| FB-13 | PID Output Positive Limit | Range: 0.00%～100.00% | Factory Value: 20.00 |
| --- | --- | --- | --- |
| FB-14 | PID Output Negative Limit | Range: 0.00%～100.00% | Factory Value: 20.00 |

Parameters FB-13 and FB-14 define the positive and negative limit percentages for PID control output, PID output limit range = FB-13 or FB-14 * maximum frequency parameter F4-02.

| FB-15 | PID Feedback Upper Limit Value | Range: 0.0%～100.0% | Factory value: 100.0 |
| --- | --- | --- | --- |
| FB-16 | PID Feedback Lower Limit | Range: 0.0%～100.0% | Factory Value: 0.0 |

Parameters FB-15 and FB-16 are only valid when FB-00 is set to 1 or 3.

| FB-17 | Linear Speed Input Source | Range: 0～5 | Factory value: 0 |
| --- | --- | --- | --- |

0: No Input

1: Analog Input

2: Communication Setting

3: PULSE Input (from PG Card)

4: Invalid

5: PULSE Input via DI6/DI7 Digital Terminals

When FB-00 = 2, this parameter setting is invalid.

When FB-17 = 1, AI1/AI2/AI3 functions must be set to line speed feedback (F5-21/ F5-27/ F5-33 = 15).

When FB-17 = 2, change the line speed set value through parameter FB-21; When FB-17 ≠ 2, the inverter temporarily stores the analog value or pulse command to parameter FB-21, at this time FB-21 is read-only.

When FB-17 = 3 or 5, it is necessary to set the number of pulses per meter for parameter FB-20.

Line Speed

In closed-loop speed control mode, it is necessary to calculate the winding synchronization frequency based on the line speed; when calculating the roll diameter using the line speed, the line speed is an essential parameter; At the same time, the line speed can be used to achieve pre-driving, inertia compensation, friction compensation, and other functions. Therefore, the line speed is an important part of tension control, and related parameters are FB-18 to FB-24.

| FB-18 | Maximum Linear Speed | Range: 0.00～650.00 | Factory Value: 100.00 |
| --- | --- | --- | --- |

This parameter sets the maximum linear speed. The actual linear speed value corresponding to inputs 100.0% on channels 1~5 of FB-17 (may not be the same as the maximum production line speed; avoid confusion when setting). When using linear speed to calculate reel diameter (FB-25 = 0), the larger this parameter, the larger the reel diameter value, and vice versa. Refer to this rule to adjust this parameter based on the deviation between the calculated reel diameter and the actual value.

| FB-19 | Minimum Linear Speed | Range: 0.00～650.00 | Factory Value: 0.00 |
| --- | --- | --- | --- |

When the linear speed setting is less than parameter FB-19, the inverter will stop calculating the reel diameter and maintain the current reel diameter.

| FB-20 | Pulses per Meter | Range: 0.0～6500.0 | Factory Value: 0.0 |
| --- | --- | --- | --- |

When the PID feedback source is selected as pulse input (FB-06 = 1), this parameter needs to be set;

When the linear speed input source is PULSE input or PULSE input via digital terminals (FB-17 = 3 or 5), this parameter needs to be set.

| FB-21 | current linear speed | Range: 0.00～650.00 | Factory Value: 0.00 |
| --- | --- | --- | --- |

When FB-17 = 2, change the line speed set value through parameter FB-21; When FB-17 ≠ 2, the inverter temporarily stores the analog value or pulse command in parameter FB-21, at which time FB-21 is read-only.

| FB-22 | linear speed filter time | Range: 0.00s～100.00s | Factory Value: 0.10 |
| --- | --- | --- | --- |

When the linear speed input source is PULSE input or PULSE input via DI6/DI7 digital terminals (FB-17 = 3 or 5), this parameter is valid and can suppress linear speed oscillation.

| FB-23 | linear speed acceleration time | Range: 0.00s～655.35s | Factory Value: 0.00 |
| --- | --- | --- | --- |
| FB-24 | linear speed deceleration time | Range: 0.00s～655.35s | Factory Value: 0.00 |

Parameters FB-23 and FB-24 are effective in the linear speed closed-loop speed mode (FB-00 = 2).

Roll Diameter

Roll diameter is a necessary parameter for four tension control modes. Choose a reasonable method for calculating roll diameter based on conditions, and set relevant parameters correctly to ensure the relative accuracy of the roll diameter value; otherwise, it can lead to abnormal tension control functions. Parameters related to roll diameter are FB-25 ~ FB-39. Illustration of Roll Diameter Parameter Settings and CalculationAs shown in Figure 2-56.

Figure 2-56 Illustration of Roll Diameter Parameter Settings and Calculation

| FB-25 | winding diameter calculation method selection | Range: 0～5 | Factory value: 0 |
| --- | --- | --- | --- |

0: Calculated through line speed

1: Calculated through analog input

2: Calculated through thickness integration, encoder at the load end input via PG card

3: Calculated through thickness integration, encoder at the motor end input via PG card

4: Calculated through thickness integration, encoder at the load end input via DI6/DI7

5: Through thickness integral calculation, the encoder at the motor end is input via DI6/DI7.

When FB-25 = 1, it is necessary to first set the AI1/AI2/AI3 functions to reel diameter (F5-21/ F5-27/ F5-33 = 16), at this time 10 V corresponds to the maximum reel diameter parameter FB-26.

When FB-25 = 2, a PG card must be connected, and the reel diameter is obtained through the encoder on the scroll bar. At this time, the signal pulses are connected to PG2 on the PG card, and the encoder type (F4-27) as well as the reference pulse input type (F4-30), the number of pulses per revolution (FB-32), the number of turns per layer (FB-33), and the material thickness (FB-34) are set to calculate the reel diameter.

When FB-25 = 3, a PG card must be connected, and the roll diameter is calculated by reverse inference through the motor encoder and gear ratio. At this time, the signal pulse should be connected to PG1 on the PG card, and parameters such as gear ratio (FB-02 and FB-03), encoder type (F4-27), encoder pulses (F4-28), number of turns per layer (FB-33), and material thickness (FB-34) should be set to calculate the roll diameter.

When FB-25 = 4 or 5, DI6 and DI7 functions are supported, and the reference pulse input type should be set to unidirectional input (F4-30 = 5). If the winding direction changes during operation, parameter FB-01 must also be modified.

The relationship between various roll diameter parameters FB-26, FB-27, FB-29/ FB-30/ FB-31, and FB-38 is shown inFigure 2-57.

Figure 2-57 Diagram of the relationship between roll diameter parameters

Dmax: Maximum roll diameter, corresponding parameter FB-26;

D0: Empty roll diameter, corresponding parameter FB-27;

Dinit: Initial roll diameter, corresponding parameter FB-29 or FB-30 or FB-31;

Dcur: Current roll diameter, corresponding parameter FB-38;

| FB-26 | Maximum Roll Diameter | Range: 1.0mm～6000.0mm | Factory value: 500.0 |
| --- | --- | --- | --- |
| FB-27 | Empty Roll Diameter | Range: 1.0mm～6000.0mm | Factory value: 100.0 |
| FB-28 | Initial Roll Diameter Selection | Range: 0～1 | Factory value: 0 |
| FB-29 | Initial Roll Diameter 0 | Range: 0.0mm～6000.0mm | Factory value: 100.0 |
| FB-30 | Initial Roll Diameter 1 | Range: 0.0mm～6000.0mm | Factory value: 100.0 |
| FB-31 | Initial Roll Diameter 2 | Range: 0.0mm～6000.0mm | Factory value: 100.0 |
| FB-32 | Pulses per Revolution | Range: 1ppr～60000ppr | Factory value: 1 |

This parameter indicates the number of pulses per revolution of the roll shaft.

| FB-33 | Number of Coils per Layer | Range: 1～10000 | Factory value: 1 |
| --- | --- | --- | --- |

This parameter indicates the number of revolutions required for one layer of material to be wound, generally set to 1 for wire and strip materials.

| FB-34 | Material Thickness | Range: 0.001～65.000 | Factory setting: 0.001 |
| --- | --- | --- | --- |

The number of decimal places for this parameter can be selected by parameter FB-80 bit 6, 0: 3 decimal places, 1: 2 decimal places.

| FB-35 | Roll Diameter Filter Time | Range: 0.00s～100.00s | Factory Value: 1.00 |
| --- | --- | --- | --- |

This parameter filters the calculated roll diameter results, suppressing fluctuations in the roll diameter, which can improve the instability of the roll diameter source (parameter FB-25). The larger the parameter setting, the smoother the calculated roll diameter value, but the greater the delay in roll diameter changes. Rule: When the roll diameter changes linearly, the time lag between the calculated roll diameter and the actual roll diameter is approximately equal to the value of this parameter.

| FB-36 | Roll Diameter Compensation Enable | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

This parameter is only effective when the tension closed-loop speed mode (FB-00 = 1) is set and the line speed input source FB-17 is not zero. When the mechanical gear ratio or linear speed cannot reach the precise condition, set this parameter to compensate for the roll diameter.

| FB-37 | Roll Diameter Calculation Delay Time | Range: 0.0s～6553.5s | Factory Value: 0.0 |
| --- | --- | --- | --- |

After the pre-drive signal is canceled, the roll diameter calculation restarts after this delay to avoid inaccurate roll diameter calculation results in a short time after the pre-drive ends, which could lead to system instability.

| FB-38 | Current Roll Diameter | Range: 1.0～6553.5 | Factory value: 100.0 |
| --- | --- | --- | --- |

When the inverter is not in a stopped state, this parameter is read-only.

| FB-39 | Minimum Frequency for Roll Diameter Calculation | Range: 0.00Hz～599.00Hz | Factory Value: 1.00 |
| --- | --- | --- | --- |

When the linear speed is lower than parameter FB-39, maintain the current roll diameter value; when the linear speed exceeds this parameter, perform the roll diameter calculation. For situations where the operating frequency is low or the roll diameter calculation during acceleration is inaccurate, this parameter can be appropriately set to resolve the issue.

To avoid excessive tension causing material breakage and to prevent the wire from being slack during startup, a soft start method is used at the moment of startup. The relevant parameters are FB-40 ~ FB-43, and it is only effective when parameter FB-00 = 1 or 2.

| FB-40 | Pre-drive Mode Selection | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

0: No Function

1: Pre-drive for Winding Mode

2: Pre-drive for Unwinding Mode

At the moment of winding startup, to prevent the wire from being slack, the tension convergence time can be extended by setting it to pre-drive for winding mode (FB-40 = 1). During unwinding, it can be set to pre-drive for unwinding mode (FB-40 = 2), allowing the motor to rotate in reverse to actively tighten the material. At this time, the output frequency limit is the main and auxiliary frequency reversal cutoff frequency (parameter FA-27). Parameter FB-40 is only valid when parameter FB-00 = 1.

| FB-41 | Pre-drive/PID Switching Point | Range: 0.0%～100.0% | Factory value: 15.0 |
| --- | --- | --- | --- |

Tension feedback value 0~100% corresponds to tension from loose to tight, parameter FB-41 reference base: FB-05, when setting FB-05 = 50%, FB-41 = 10%, the pre-drive range is 0~40%.

| FB-42 | soft start frequency | Range: 0.00Hz～599.00Hz | Factory setting: 2.00 |
| --- | --- | --- | --- |
| FB-43 | soft start acceleration time | Range: 0.00s～600.00s | Factory value: 3.00 |

When FB-40 = 1 or 2, at the instant of winding start, the sum of the synchronous frequency calculated based on line speed and the soft start frequency FB-42 is used as the set frequency, and acceleration starts according to time FB-43. After the soft start acceleration time, the system runs with the set frequency being the sum of the synchronous frequency calculated based on line speed and the auxiliary frequency adjusted by PI control.

Belt Breakage Detection

The parameters related to tension control detection and abnormal handling are FB-44 to FB-50. When the line speed input source FB-17 is not 0, and the roll diameter is calculated through the line speed (FB-25 = 0), the setting of parameter FB-44 is valid.

After enabling belt breakage detection, if the line speed exceeds parameter FB-45, the change in roll diameter exceeds parameter FB-46, and the time exceeds the time set by parameter FB-47, a belt breakage will occur. In the event of a belt breakage, the inverter will display bEb, stop in free-run mode, and can simultaneously use the DO1 or DO2 output terminal function (setting value 46) as a belt breakage indication.

| FB-44 | Broken Belt Detection Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |
| FB-45 | Minimum Line Speed for Broken Belt Detection | Range: 0.0～3000.0 | Factory Value: 0.0 |
| FB-46 | Belt Breakage Detection Roll Diameter Change | Range: 1.0mm～6000.0mm | Factory value: 100.0 |
| FB-47 | Belt Breakage Detection Time | Range: 0.00s～100.00s | Factory Value: 1.00 |
| FB-48 | Tension Feedback Error Threshold | Range: 0%～100% | Factory setting: 100% |
| FB-49 | Tension Error Detection Time | Range: 0.0s～10.0s | Factory value: 0.5 |
| FB-50 | Tension Error Abnormal Handling | Range: 0～2 | Factory value: 0 |

0: Warning and Continue Running

1:Faultand Free Stop

2:Faultand Decelerate to Stop

When the deviation between the Tension PID target setpoint and the Tension PID feedback value exceeds the parameter FB-48 Tension Feedback Error Threshold, and the error time exceeds the parameter FB-49 Tension Error Detection Time, a PID Feedback Deviation Exception is generated, and the inverter reports fault “A011”.

| FB-51 | PID Output Gain | Range: 0.0～200.0 | Factory value: 100.0 |
| --- | --- | --- | --- |

This parameter, together with the PID output positive/negative limit parameters FB-13 or FB-14, determines the final output frequency limit in the tension closed-loop or line speed closed-loop modes, with the reference being the current output frequency of the inverter.

| FB-52 | Tension Reference Source Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: RS485 Communication

1: Analog Input

When FB-52 = 0, the zero-speed tension parameter value FB-54 can be set via the operation panel through communication.

When FB-52 = 1, the AI1/AI2/AI3 functions must be set to tension setting (F5-21/ F5-27/ F5-33 = 18), at this time FB-54 is read-only.

| FB-53 | Maximum Tension Value | Range: 0～65535 | Factory value: 0 |
| --- | --- | --- | --- |
| FB-54 | Tension Setpoint | Range: 0～65535 | Factory value: 0 |

When parameter FB-52 = 1, parameter FB-54 is read-only, at this time the analog value 10V corresponds to the maximum tension value FB-53.

Inertia and friction compensation

This section applies only to torque mode. Properly setting these parameters can optimize tension control performance and improve tension stability or enhance system response speed. The parameters related to inertia and friction torque compensation are FB-58 to FB-62. It is recommended to set these parameters in open-loop torque control mode (FB-00 = 4). In closed-loop torque control mode (FB-00 = 3), they generally do not need to be set.

In open-loop torque mode, during system acceleration and deceleration, the output torque must provide material tension as well as additional torque to overcome the system's moment of inertia.

Consider inertia compensation when any of the following phenomena occur:

● Material tension is too low during winding acceleration

● Material tension is too high during winding deceleration

● Material tension is too high during unwinding acceleration

● Material tension is too low during unwinding deceleration

The inertia of the winding/unwinding system generally consists of system inertia and material inertia. When setting parameters, choices can be made based on the relationship between these two quantities. If the material is heavier and the shaft is lighter, then only the material inertia parameter needs to be set, and vice versa.

| FB-58 | Sliding Friction Compensation Tension | Range: 0.0%～100.0% | Factory Value: 0.0 |
| --- | --- | --- | --- |

During constant speed operation of the motor, the output torque is used not only to establish material tension but also to overcome rotational friction. When this part of the torque cannot be ignored, compensation for the friction torque is required.

| FB-59 | Material Inertia Compensation Coefficient | Range: 0～30000 | Factory value: 0 |
| --- | --- | --- | --- |

Material inertia compensation coefficient = material density × material width. Density unit: kg/m³, width unit: m. The material inertia on the reel changes with the reel diameter. Based on parameters FB-02, FB-03, FB-27, FB-38, FB-59, etc., the inverter automatically calculates the flywheel inertia of the material to obtain the inertia compensation torque.

| FB-60 | Acceleration Inertia Compensation Gain | Range: 0.0%～1000.0% | Factory Value: 0.0 |
| --- | --- | --- | --- |
| FB-61 | Inertia Compensation Filter Time | Range: 0.00～100.00 | Factory Value: 5.00 |
| FB-62 | Deceleration Inertia Compensation Gain | Range: 0.0%～1000.0% | Factory Value: 0.0 |

In some cases, fine-tuning parameters FB-60 or FB-62 can optimize control performance. For example, during acceleration of the winding process, if the material tension is too low, parameter FB-60 can be increased to enhance the compensation effect; otherwise, reduce this parameter. The same applies to deceleration.

Startup Compensation

In certain situations, the winding shaft may have significant startup friction, which can cause difficulty in starting. At this point, torque compensation can be applied at zero speed during startup, and the compensation should be withdrawn once normal operation resumes to ensure constant tension. Relevant parameters are FB-55 to FB-57.

Zero-speed tension calculation depends on the value of bit 1 of the tension control flag parameter FB-80. By default, bit 1 of FB-80 is 0, which means zero-speed tension calculation is not enabled. At this time, if the operating frequency is less than the zero-speed tension threshold FB-57, the tension setpoint is the zero-speed tension value FB-56; Conversely, the tension setpoint is the target setpoint FB-05, with a hysteresis zone for the tension setpoint in between, maintaining a constant value.

When the tension control flag parameter bit 1 of FB-80 is 1, zero-speed tension calculation is effective, at this time:

When the line speed is less than the zero-speed tension threshold FB-57,

零速张力=线速度FB−21×PID目标设定值FB−05−零速张力FB−56最大线速度FB−18∗零速张力阈值FB−57+零速张力FB−56

Conversely, the tension setpoint is the target setpoint FB-05. After startup, the final tension setpoint will vary with the line speed and the zero-speed tension threshold FB-57.

| FB-55 | Zero-speed Tension Reference Source | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

0: No Input

1: Communication settings

2: Analog Input

When FB-55 = 1, the zero-speed tension parameter value FB-56 can be set via the operation panel through communication;

When FB-55 = 2, it is necessary to first set the AI1/AI2/AI3 function to zero-speed tension setting (F5-21/ F5-27/ F5-33 = 19), at this time FB-56 is read-only.

| FB-56 | Zero-speed Tension Setpoint | Range: 0～65535 | Factory value: 0 |
| --- | --- | --- | --- |

When FB-55 is set to 2, parameter FB-56 is read-only, and the analog input of 10 V corresponds to the maximum tension parameter FB-53.

| FB-57 | Zero-speed tension threshold | Range: 0.00%～100.00% | Factory Value: 5.00 |
| --- | --- | --- | --- |

When bit 1 of the tension control flag parameter FB-80 is 0, if the operating frequency is less than FB-57, the tension value is the parameter FB-56, achieving static friction tension compensation.

Taper Tension

In certain situations, to ensure flat winding, it is required that the tension decreases as the roll diameter increases. By properly setting the taper-related parameters (FB-63 ~ FB-70), this can be achieved. Parameters FB-63 ~ FB-70 are only valid in the winding mode (FB-01=0). Parameter FB-63 can select different taper tension curves for tension calculation, including single-segment linear or curved lines, as well as multi-segment linear or curved lines. Figure 2-58 shows the settings of taper tension-related parameters and the schematic diagram of taper tension calculation.

Figure 2-58 Schematic Diagram of Taper Tension-Related Parameter Settings and Taper Tension Calculation

| FB-63 | Tension Taper Curve Selection | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |

0: Taper Calculation Invalid

1: Curve Taper

2: Straight Taper

3: Multi-segment Curve Taper

4: Multi-segment Straight Taper

When FB-63 = 1, generate a curve based on parameter FB-65, and fine-tune the curve using parameter FB-66.

When FB-63 = 2, generate a straight taper based on parameter FB-65.

When FB-63 = 3, parameters FB-65, FB-69, and FB-70 determine the multi-segment curve taper, and parameters FB-67 and FB-68 determine the curve inflection points.

When FB-63 = 4, parameters FB-65, FB-69, and FB-70 determine the multi-segment straight taper, and parameters FB-67 and FB-68 determine the straight inflection points.

Single-stage linear taper output tension and roll diameter relationshipas shown in Fig. 2-59.

Fig. 2-59 Schematic diagram of single-stage linear taper output tension and roll diameter relationship

Multi-stage linear taper output tension and roll diameter relationshipas shown in Fig. 2-60.

Fig. 2-60 Schematic diagram of multi-stage linear taper output tension and roll diameter relationship

| FB-64 | Tension Taper Setting Source | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: Communication settings

1: Input analog setting

| FB-65 | Taper Setting Value | Range: 0%～100% | Factory value: 0 |
| --- | --- | --- | --- |
| FB-66 | Taper Curve Compensation Value | Range: 0.0～6000.0 | Factory Value: 0.0 |

When the tension taper curve selects curve taper (FB-63 = 1), the curve is generated by the taper setting value FB-65 and fine-tuned by parameter FB-66.

| FB-67 | Taper Calculation Roll Diameter 1 | Range: 1.0～6000.0 | Factory value: 6000.0 |
| --- | --- | --- | --- |
| FB-68 | taper calculation roll diameter 2 | Range: 1.0～6000.0 | Factory value: 6000.0 |
| FB-69 | multi-segment taper value 1 | Range: 0～100 | Factory value: 0 |
| FB-70 | multi-segment taper value 2 | Range: 0～100 | Factory value: 0 |

Pre-drive

The pre-drive function is used for automatic roll changing applications. When replacing a full roll with a new one, the new roll needs to be pre-rotated to match the line speed of the material in operation, ensuring a smooth roll change without shock. At this time, the pre-drive function using DI terminals (F5-00 ~ F5-07 = 94) must be utilized. The parameters related to pre-drive are FB-71 ~ FB-73; this function requires accurate line speed and roll diameter measurements. When this function is active, roll diameter calculation and tension PI control are halted, as shown in Figure2-61.

Figure 2-61 Pre-drive Schematic Diagram

| FB-71 | pre-drive frequency gain | Range: -50.0%～50.0% | Factory Value: 0.0 |
| --- | --- | --- | --- |

When the pre-drive terminal is effective, the inverter automatically calculates the output frequency based on the line speed detection value and the current roll diameter to match the line speed.

| FB-72 | pre-drive acceleration time | Range: 0.00s～655.35s | Factory Value: 0.00 |
| --- | --- | --- | --- |
| FB-73 | pre-drive deceleration time | Range: 0.00s～655.35s | Factory Value: 0.00 |

Parameters FB-72 ~ FB-73 are the time for accelerating from the soft start frequency (FB-42) to the pre-drive frequency or decelerating from the pre-drive frequency to the soft start frequency during automatic roll change.

| FB-74 | speed limit gain | Range: 0.0s～6553.5s | Factory value: 100.0 |
| --- | --- | --- | --- |

In tension control mode, when the analog detection value of line speed is also used as a speed limit (parameter F3-17 = 3), this parameter can be adjusted to change the size of the speed limit.

Parameters FB-75 ~ FB-79 are reserved; their default values should be used, and users should not modify them, as improper settings may cause the winding system to become unbalanced.

| FB-75 | soft start/PI switch LSErr | Range: 0～100 | Factory Value: 5 |
| --- | --- | --- | --- |

Line Speed Closed-loop Mode (FB-00 = 2), Pre-drive/PID Regulation Switching Line Speed Error Level, Reference Benchmark: Maximum Line Speed FB-18.

| FB-76 | soft start/PI switch LS1 | Range: 0.0%～10.0% | Factory Value: 0.4 |
| --- | --- | --- | --- |

Tension Closed-loop Speed Mode (FB-00 = 1) Valid, Pre-drive and PI Regulation Switching Interval Line Speed Critical Value 1, Reference Benchmark: Maximum Line Speed FB-18.

| FB-77 | soft start/PI switch LS2 | Range: 0.0%～10.0% | Factory Value: 0.7 |
| --- | --- | --- | --- |

Tension Closed-loop Speed Mode (FB-00 = 1) Valid, Pre-drive and PI Regulation Switching Interval Line Speed Critical Value 2, Reference Benchmark: Maximum Line Speed FB-18.

| FB-78 | Soft Start/PI Switch 1 Setting | Range: 0.0～40.0% | Factory Value: 2.0% |
| --- | --- | --- | --- |

Tension closed-loop speed mode (FB-00 = 1) is valid, the critical value for switching between pre-drive and PI regulation based on line speed when the threshold is 1, reference standard: FB-05.

| FB-79 | Unwinding Soft Start PI Switch Point | Range: 0.0～40.0% | Factory Value: 5.0% |
| --- | --- | --- | --- |

Tension closed-loop speed mode (FB-00 = 1) is valid, the threshold for switching between pre-drive and PI regulation during unwinding, reference standard: FB-05.

| FB-80 | Tension Control Flag | Range: 0000H～FFFFH | Factory Value: 0000H |
| --- | --- | --- | --- |

Tension control flag (bits 0 ~ 6 are valid)

bit 0: Taper calculation invalid;

bit 1: Enable switching between zero-speed tension command and normal tension command;

bit 2: Inertia torque compensation and acceleration/deceleration compensation switching, 0: Inertia compensation effective, 1: Acceleration/deceleration compensation effective;

bit 3: Reel diameter filtering using moving average method;

bit 4: Inertia torque compensation calculation selection, 0: Based on linear speed, 1: Based on operating frequency;

bit 5: PI output reverse limit selection, 0: PI output reverse limit selection FB-13, 1: Select FB-14;

bit 6: Material thickness display decimal places selection, 0: 3 decimal places, 1: 2 decimal places.

Function Block Diagram

The given torque and frequency are the target control quantities for tension control. Users can refer to the data flow of the above two key variables in the overall block diagram, query some sub-function diagrams, and refer to relevant parameter descriptions to achieve the tension control function. The setting value of parameter FB-00 determines the method of tension control implementation. The product integrates four tension control modes,as shown in Figures 2-62, 2-63, 2-64, and2-65.

● FB-00 = 1, Tension Closed-loop Speed Mode

Figure 2-62 Tension Closed-loop Speed ModeSchematic Diagram

● FB-00 = 2, Line Speed Closed-loop Speed Mode

Figure 2-63 Line SpeedClosed-loop Speed ModeSchematic Diagram

● FB-00 = 3, Tension Closed-loop Torque Mode

Figure 2-64 Tension Closed-loop Torque Mode Schematic Diagram

● FB-00 = 4, Tension Open-loop Torque Mode

Figure 2-65 Schematic Diagram of Tension Open-loop Torque Mode

2.13 FC Group Position Control Parameters

| FC-00 | P2P Position Mode | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the P2P (point-to-point) position control mode, as shown in Figure 2-66.

0: Relative P2P Position Control

1: Absolute P2P Position Control

Figure 2-66  P2P PositionControlModeSchematic

| FC-01 | Return-to-Origin Mode Selection | Range: 0000H~FFFFH | Factory value: 0 |
| --- | --- | --- | --- |

bit0～bit3: Homing Method Selection

0: Home in Forward Direction, PL Forward Limit as Home Enable;

1: Home in Reverse Direction, NL Reverse Limit as Home Enable;

2: Home in Forward Direction, ORG: OFF→ON as Home Enable;

3: Home in Reverse Direction, ORG: OFF→ON as Home Enable;

4: Forward direct search for Z pulse as home position enable;

5: Reverse direct search for Z pulse as home position enable;

6: Forward direction to home position, ORG: ON→OFF as home position enable;

7: Reverse direction to home position, ORG: ON→OFF as home position enable;

8: Current position as home position.

bit4～bit7: Z signal setting (to be used with bit0～bit3 options 0, 1, 2, 3, 6, 7), decimal value meanings are as follows:

0: Return to find Z pulse;

1: Do not return to find Z pulse, continue forward to find Z pulse;

2: Never search for Z pulse.

Greater than 2: Parameter setting is invalid.

bit8～bit11: Selection of action upon reaching limit (to be used with bit0～bit3 options 2, 3, 4, 5, 6, 7), decimal value meanings are as follows:

0: Display fault when position limit is reached;

1: Run in reverse direction when position limit is reached.

Greater than 1: Parameter setting is invalid

bit12～bit15: Reserved

The return-to-origin method corresponding to the set value of FC-01 is explained through the following diagram, where the decimal value of bit0~3 is represented by A, and the decimal value of bit4~7 is represented by B.

| FC-02 | Position Control Command Source | Range: 0～5 | Factory value: 0 |
| --- | --- | --- | --- |

0: External Terminal Input

2: RS485 communication

3: CANopen

5: Communication Expansion Card

| FC-03 | First Segment Return-to-Origin Speed | Range: 0.00Hz～599.00Hz | Factory default: 8.00Hz |
| --- | --- | --- | --- |
| FC-04 | Second Segment Return-to-Origin Speed | Range: 0.00Hz～599.00Hz | Factory setting: 2.00Hz |

First segment return-to-origin speed, used for the initial stage of position control, to run from the starting position to the target position at a faster speed, thereby reducing the time to reach the target position; Second segment return-to-origin speed, used for subsequent stages of position control, to run towards the target position at a slower speed, preventing significant overshoot upon reaching the target position.

| FC-05 | Machine tool application enable | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

When enabling machine tool application control, this function must be activated.

0: Off

1: Enable

| FC-06 | Error amount for searching Z point | Range: 0～65535 | Factory value: 1 |
| --- | --- | --- | --- |

This parameter is only used in P2P mode; when the absolute value of the position error is less than this parameter value, it is determined that the search for point Z has been successful.

| FC-07 | Position control acceleration time | Range: 0.00s～655.35s | Factory setting: 1.00s |
| --- | --- | --- | --- |
| FC-08 | Position control deceleration time | Range: 0.00s～655.35s | Factory setting: 3.00s |

The acceleration and deceleration times for position control are shown in Figure 2-67.

Figure 2-67 PositionsSchematic Diagram of Control for Acceleration and Deceleration Time

| FC-12 | Positioning Stiffness Adjustment Coefficient | Range: 0.10～10.00 | Factory Value: 1.00 |
| --- | --- | --- | --- |

Used to adjust the stiffness during the shaft locking period; the higher the stiffness adjustment coefficient, the better the positioning stiffness, but excessive stiffness may lead to system instability.

| FC-13 | Positioning Jitter Adjustment Coefficient | Range: 0.00～1.20 | Factory value: 3.00 |
| --- | --- | --- | --- |

Used to adjust the position loop bandwidth after positioning completion; the smaller this coefficient, the less the effect of the position loop after positioning completion.

| FC-20 | Internal Positioning Command | Range: -32767～32767 | Factory value: 0 |
| --- | --- | --- | --- |

This parameter sets the target position for single-point positioning and must be used in conjunction with the external terminal selection for single-point positioning enable (35).

| FC-21 | Feedback Position Arrival Error | Range: 0～65535 | Factory value: 10 |
| --- | --- | --- | --- |

When the absolute value of the position error is less than this parameter value, it is determined that the feedback position has been reached. For example, setting the target position to 500, FC-21 set to 50, after positioning is completed, positions between 450~550 are considered successful.

| FC-22 | Pulse Input Filter | Range: 0.000～65.535 | Factory Value: 0.100 |
| --- | --- | --- | --- |

PG2 Pulse Input Digital Low-pass Filter Time Constant, the larger this value, the better the input signal interference resistance, but the greater the delay.

| FC-23 | Pulse Speed Mode | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

PG2 Pulse as Speed Command Mode Selection, 0 indicates PG2 as electrical speed, 1 indicates PG2 as mechanical speed.

| FC-24 | Position Loop Proportional Coefficient | Range: 0.00～40.00 | Factory Value: 8.00 |
| --- | --- | --- | --- |

Position Closed-loop Control Proportional Coefficient, the larger this parameter, the faster the position tracking, but too large may lead to system instability.

| FC-25 | Position Loop Feedforward Coefficient | Range: 0～100 | Factory value: 30 |
| --- | --- | --- | --- |

Only effective for internal positioning (MIx=35) and pulse tracking position control (MIx=37). The larger the setting value of this parameter, the smaller the pulse tracking error and the faster the position control response, but it can easily cause position overshoot.

| FC-26 | Position Curve Time | Range: 0.00s～655.35s | Factory setting: 3.00s |
| --- | --- | --- | --- |

Effective for single-point positioning (ON) when the multi-function input terminal is set to 35. The longer the setting value, the longer the positioning time.

| FC-27 | P2P Maximum Frequency | Range: 0.00Hz～599.00Hz | Factory value: 10.00Hz |
| --- | --- | --- | --- |

This parameter sets the maximum speed frequency during position control acceleration and deceleration periods. Used in conjunction with FC-07 and FC-08.

| FC-28 | Multi-segment position 1(revs) | Range: -30000～30000 | Factory value: 0 |
| --- | --- | --- | --- |
| FC-29 | Multi-segment position 1(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-30 | Multi-segment position 2(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-31 | Multi-segment Position 2(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-32 | Multi-segment Position 3(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-33 | Multi-segment Position 3(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-34 | Multi-segment Position 4(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-35 | Multi-segment Position 4(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-36 | Multi-segment Position 5(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-37 | Multi-segment Position 5(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-38 | Multi-segment Position 6(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-39 | Multi-segment Position 6(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-40 | Multi-segment Position 7(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-41 | Multi-segment Position 7(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-42 | Multi-segment Position 8(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-43 | Multi-segment Position 8(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-44 | Multi-segment Position 9(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-45 | Multi-segment Position 9(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-46 | Multi-segment Position 10(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-47 | Multi-segment Position 10(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-48 | Multi-segment Position 11(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-49 | Multi-segment Position 11(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-50 | Multi-segment Position 12(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-51 | Multi-segment Position 12(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-52 | Multi-segment Position 13(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-53 | Multi-segment Position 13(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-54 | Multi-segment Position 14(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-55 | Multi-segment Position 14(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-56 | Multi-segment Position 15(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-57 | Multi-segment Position 15(pulse) | Range: -32767～32767 | Factory value: 0 |
| FC-58 | Multi-segment Position 0(revs) | Range: -30000～30000 | Factory value: 0 |
| FC-59 | Multi-segment Position 0(pulse) | Range: -32767～32767 | Factory value: 0 |

The above parameters can be used to set 16 multi-segment positions, where Multi-segment Position n(revs) indicates the number of revolutions, and Multi-segment Position n(pulse) represents the pulse count. The final position setting is calculated as: Position Setting = Multi-segment Position n(revs) × Pulses per Revolution + Multi-segment Position n(pulse), n=1～15.

When the multifunctional digital input terminal function selection is set to multi-speed/multi-position switching (34) and the terminal status is active, the target position is selected through the multi-segment position terminal status and multi-segment position parameter settings. The correspondence table for multi-segment position selection target positions and terminal status is shown inTable 2-22, where DI terminal status 1 indicates that the terminal status is valid, and 0 indicates that the terminal status is invalid.

Table 2-22 Correspondence Table for Multi-Segment Position Selection Target Positions and Terminal Status

| DI4 | DI3 | DI2 | DI1 | Position Command Circle Selection | Position Command Pulse Selection |
| --- | --- | --- | --- | --- | --- |
| \ | \ | \ | \ | When configuring internal single-point positioning, FC-20 serves as the position command. |  |
| 0 | 0 | 0 | 1 | FC-28 | FC-29 |
| 0 | 0 | 1 | 0 | FC-30 | FC-31 |
| 0 | 0 | 1 | 1 | FC-32 | FC-33 |
| 0 | 1 | 0 | 0 | FC-34 | FC-35 |
| 0 | 1 | 0 | 1 | FC-36 | FC-37 |
| 0 | 1 | 1 | 0 | FC-38 | FC-39 |
| 0 | 1 | 1 | 1 | FC-40 | FC-41 |
| 1 | 0 | 0 | 0 | FC-42 | FC-43 |
| 1 | 0 | 0 | 1 | FC-44 | FC-45 |
| 1 | 0 | 1 | 0 | FC-46 | FC-47 |
| 1 | 0 | 1 | 1 | FC-48 | FC-49 |
| 1 | 1 | 0 | 0 | FC-50 | FC-51 |
| 1 | 1 | 0 | 1 | FC-52 | FC-53 |
| 1 | 1 | 1 | 0 | FC-54 | FC-55 |
| 1 | 1 | 1 | 1 | FC-56 | FC-57 |
| 0 | 0 | 0 | 0 | FC-58 | FC-59 |

2.14 FD Group Multi-Speed and Simple PLC Function

This inverter can set up to 16 speed segments, and specific configurations can be found in the descriptions for F5-00 to F5-07.

| FD-00 | Multi-Speed 0 | Range: -100% to 100% | Factory value: 0.00% |
| --- | --- | --- | --- |
| FD-01 | Multi-Speed 1 | Range: -100% to 100% | Factory value: 0.00% |
| FD-02 | Multi-Speed 2 | Range: -100% to 100% | Factory value: 0.00% |
| FD-03 | Multi-Speed 3 | Range: -100% to 100% | Factory value: 0.00% |
| FD-04 | Multi-Speed 4 | Range: -100% to 100% | Factory value: 0.00% |
| FD-05 | Multi-Speed 5 | Range: -100% to 100% | Factory value: 0.00% |
| FD-06 | Multi-speed 6 | Range: -100% to 100% | Factory value: 0.00% |
| FD-07 | Multi-speed 7 | Range: -100% to 100% | Factory value: 0.00% |
| FD-08 | Multi-speed 8 | Range: -100% to 100% | Factory value: 0.00% |
| FD-09 | Multi-speed 9 | Range: -100% to 100% | Factory value: 0.00% |
| FD-10 | Multi-speed 10 | Range: -100% to 100% | Factory value: 0.00% |
| FD-11 | Multi-speed 11 | Range: -100% to 100% | Factory value: 0.00% |
| FD-12 | Multi-Speed 12 | Range: -100% to 100% | Factory value: 0.00% |
| FD-13 | Multi-Speed 13 | Range: -100% to 100% | Factory value: 0.00% |
| FD-14 | Multi-Speed 14 | Range: -100% to 100% | Factory value: 0.00% |
| FD-15 | Multi-Speed 15 | Range: -100% to 100% | Factory value: 0.00% |

| FD-16 | PLC Operation Mode | 0: Single Run Stop at End 1: Hold after Single Operation Completion 2: Continuous Run | Factory Value:0 |
| --- | --- | --- | --- |

There are three operating modes for the PLC:

0: Single Run Stop at End

The inverter automatically stops after completing all steps, and to restart, a new operation command must be given.

1: Hold after Single Operation Completion

After completing all steps, the inverter automatically maintains operation at the last step frequency until a stop command is received.

2: Continuous Run

After completing all steps, the inverter automatically starts the next cycle until a stop command is received.

| FD-17 | PLC Power Failure Memory Selection | 0: No Memory on Power Failure/Shutdown 1: Memory on Power Failure 2: Memory on Shutdown 3: Memory on Power Failure/Shutdown | Factory value: 0 |
| --- | --- | --- | --- |

PLC power failure memory refers to remembering the operating step and running time of the PLC before a power failure, and continuing to run from the remembered step and running time when powered on again. Choosing not to remember means that each time it is powered on, the PLC process restarts from step zero.

PLC shutdown memory refers to recording the current operating step and running time of the PLC when it is shut down, and continuing to run from the remembered step and running time when started again. Choosing not to remember means that each start-up restarts the PLC process from step zero.

| FD-18 | PLC Operation Time Unit | 0: Second 1: Hour | Factory value: 0 |
| --- | --- | --- | --- |

The user can select an appropriate time unit based on the duration of operation; changing this setting will affect the runtime of each PLC segment.

| FD-19 | Multi-Speed 0 Command Setting | 0: Function Code FD-00 Setting 1: AI 2: VR 3: Function Code F0-19 Given | Factory value: 0 |
| --- | --- | --- | --- |

For multi-speed 0 command setting, in addition to choosing FD-00, there are multiple other options available for easy switching between multi-speed and other setting methods.

| FD-20 | PLC Segment 00 Running Time | 0.0~6553.5 | Factory Value: 0.0 |
| --- | --- | --- | --- |
| FD-21 | PLC Segment 00 Acceleration/Deceleration Settings | 0~3 | Factory value: 0 |
| FD-22 | PLC Segment 01 Running Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-23 | PLC Segment 01 Acceleration/Deceleration Settings | 0~3 | Factory value: 0 |
| FD-24 | PLC Segment 02 Running Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-25 | PLC Segment 02 Acceleration/Deceleration Settings | 0~3 | Factory value: 0 |
| FD-26 | PLC Segment 03 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-27 | PLC Segment 03 Acceleration/Deceleration Settings | 0~3 | Factory value: 0 |
| FD-28 | PLC Segment 04 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-29 | PLC Segment 04 Acceleration/Deceleration Settings | 0~3 | Factory value: 0 |
| FD-30 | PLC Segment 05 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-31 | PLC Segment 05 Acceleration/Deceleration Settings | 0~3 | Factory value: 0 |
| FD-32 | PLC Segment 06 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-33 | PLC Segment 06 Acceleration/Deceleration Settings | 0~3 | Factory value: 0 |
| FD-34 | PLC Segment 07 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-35 | PLC Segment 07 Acceleration/Deceleration Settings | 0~3 | Factory value: 0 |
| FD-36 | PLC Segment 08 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-37 | PLC Segment 08 Acceleration/Deceleration Setting | 0~3 | Factory value: 0 |
| FD-38 | PLC Segment 09 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-39 | PLC Segment 09 Acceleration/Deceleration Setting | 0~3 | Factory value: 0 |
| FD-40 | PLC Segment 10 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-41 | PLC Segment 10 Acceleration/Deceleration Setting | 0~3 | Factory value: 0 |
| FD-42 | PLC Segment 11 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-43 | PLC Segment 11 Acceleration/Deceleration Setting | 0~3 | Factory value: 0 |
| FD-44 | PLC Segment 12 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-45 | PLC Segment 12 Acceleration/Deceleration Setting | 0~3 | Factory value: 0 |
| FD-46 | PLC Segment 13 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-47 | PLC Segment 13 Acceleration/Deceleration Setting | 0~3 | Factory value: 0 |
| FD-48 | PLC Segment 14 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-49 | PLC Segment 14 Acceleration/Deceleration Setting | 0~3 | Factory value: 0 |
| FD-50 | PLC Segment 15 Operation Time | 0.0~6553.5 | Factory Value: 0.0 |
| FD-51 | PLC Segment 15 Acceleration/Deceleration Setting | 0~3 | Factory value: 0 |

When the Nth segment running time of the PLC is set to greater than 0, the step will be executed; otherwise, it will be skipped directly. For the Nth segment acceleration/deceleration settings of the PLC, you can choose the acceleration/deceleration time corresponding to the step.
The acceleration/deceleration time setting values correspond to the acceleration/deceleration as shown in Table2-23.

Table 2-23Acceleration/Deceleration Setting Table

| Set Value | Corresponding Acceleration/Deceleration | Corresponding Parameters |
| --- | --- | --- |
| 0 | Acceleration/Deceleration 1 | F0-13F0-14 |
| 1 | Acceleration and Deceleration 2 | F7-03F7-04 |
| 2 | Acceleration and Deceleration 3 | F7-05F7-06 |
| 3 | Acceleration and Deceleration 4 | F7-07F7-08 |

| FD-52 | PLC Current Work Step | 0~15 | Factory value: 0 |
| --- | --- | --- | --- |
| FD-53 | PLC Operation Time High Position | 0000H~FFFFH | Factory value: 0 |
| FD-54 | PLC runtime low position | 0000H~FFFFH | Factory value: 0 |

Parameter PLC current step records the sequence number of the current step being executed by the PLC, while PLC runtime records the time value that the PLC has been running.

Note: After the inverter completes all steps, it can be configured to output a signal indicating the completion of one cycle. When the PLC operation mode is set to stop after a single run or hold after a single run, the signal will only be output once. For cyclic operation, the signal will be output each time a cycle is completed.

2.15 FE Group Torque Control Parameters

| FE-00 | Torque mode selection | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

0: TQCPG Torque Control IM

For use in closed-loop torque mode for induction motors when PG card feedback is available.

1: TQCPG Torque Control PM

Used for closed-loop torque mode of permanent magnet synchronous motors with PG card feedback.

3: SVC Open-loop Torque Control

Used for open-loop torque mode of induction motors or permanent magnet synchronous motors without PG card feedback.

| FE-01 | Torque command source selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: Digital Operator

1: RS485 Communication

When this parameter is set to 0 or 1, the torque digital setpoint is determined by FE-02.

2: Analog signal input

The corresponding AI function must be selected as torque setting (set F5-21, F5-27, or F5-33 to 2).

3: CANopen

5: Communication Card

In this case, the torque command corresponds to communication address 0x6006; refer to the relevant communication card user manual for details.

| FE-02 | Torque digital setting | Range: -100.0% to 100.0% | Factory Value: 0.0% |
| --- | --- | --- | --- |

When FE-01 is set to 0 or 1, this parameter can be written with a torque digital set value; otherwise, this parameter is read-only, displaying only the torque digital set value.

| FE-03 | Forward Torque Frequency Limit | Range: 0%～120% | Factory value: 110% |
| --- | --- | --- | --- |
| FE-04 | Reverse Torque Frequency Limit | Range: 0%～120% | Factory value: 110% |

When F3-17 is set to 0 or 1, the above parameters are used to set the upper frequency limit for forward and reverse motor rotation, respectively.

| FE-05 | Torque Bias Selection | Range: 0～3 | Factory value: 0 |
| --- | --- | --- | --- |

0: No Function

By default, no torque bias superposition is performed.

1: Analog Input

The corresponding AI function must be selected as torque compensation setting (set F5-21, F5-27, or F5-33 to 3).

2: Parameter FE-06 input

| FE-06 | Torque Bias Value | Range: -100.0% to 100.0% | Factory Value: 0.0% |
| --- | --- | --- | --- |

When FE-05=2, the torque bias value can be set through this parameter.

| FE-10 | Maximum torque command | Range: 0～500 | Factory Value: 100 |
| --- | --- | --- | --- |

This parameter sets the upper limit of the torque command, corresponding to 100% of the motor's rated torque.

| FE-11 | Forward Motor Torque Upper Limit | Range: 0～500 | Factory Value: 500 |
| --- | --- | --- | --- |
| FE-12 | Forward Braking Torque Upper Limit | Range: 0～500 | Factory Value: 500 |
| FE-13 | Reverse Electric Torque Upper Limit | Range: 0～500 | Factory Value: 500 |
| FE-14 | Reverse Braking Torque Upper Limit | Range: 0～500 | Factory Value: 500 |

FE-11～FE-12, FE-13～FE-14are the analog and communication givenvaluesformotor torque and braking torquelimits. When the actual direction of the motor rotation is the same as the direction of the electromagnetic torque generated by the motor, it is in the motor state; otherwise, it is in the braking state.

Note: This parameter is read-only, used to display the torque limit values given by the analog and communication, and cannot be set.

| FE-15 | Torque Filter Time | Range: 0.000s～1.000s | Factory Value: 0.000s |
| --- | --- | --- | --- |

This parameter sets the low-pass filter time constant for the torque command. Increasing this parameter can make the torque command change more smoothly, making control more stable, but the response will become slower.

| FE-17 | Zero Torque Mode Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

This parameter is effective only when FE-00=0 or 1 (closed-loop torque mode) and the speed limit value is 0.

0: Torque Mode

When the speed limit is 0, there is still excitation current in the motor, and the torque limit is set by FE-02 (torque digital setting).

1: Speed Mode

When the speed limit is 0, the actual control method switches from closed-loop torque control to closed-loop speed control, so that the motor always maintains zero speed, and the torque setting output by the speed loop is limited only by parameter F9-07.

2.16 U0 Group Fault Record Parameters

The fault record function records the name, power-on time, frequency, torque, voltage, current, and power device temperature at the moment of fault for a certain number of inverter faults, providing reference for subsequent fault diagnosis.

The current inverter software supports recording the fault codes of the last 10 faults, the power-on times at the moment of the last 6 faults, and physical quantity information.

Only faults that cause the inverter to shut down after they occur are recorded; undervoltage faults during shutdown are not recorded.

Fault record information is stored in EEPROM, and each time a fault occurs, the fault information is automatically updated and written into the EEPROM.

| U0-00 | Fault Record 1 | Range: 0～65535 | Factory value: 0 |
| --- | --- | --- | --- |
| U0-01 | Fault Record 2 | Range: 0～65535 | Factory value: 0 |
| U0-02 | Fault Record 3 | Range: 0～65535 | Factory value: 0 |
| U0-03 | Fault Record 4 | Range: 0～65535 | Factory value: 0 |
| U0-04 | Fault Record 5 | Range: 0～65535 | Factory value: 0 |
| U0-05 | Fault Record 6 | Range: 0～65535 | Factory value: 0 |
| U0-06 | Fault Record 7 | Range: 0～65535 | Factory value: 0 |
| U0-07 | Fault Record 8 | Range: 0～65535 | Factory value: 0 |
| U0-08 | Fault Record 9 | Range: 0～65535 | Factory value: 0 |
| U0-09 | Fault Record 10 | Range: 0～65535 | Factory value: 0 |

The above parameters are used to record the fault codes of the last 10 faults of the inverter. The smaller the fault record number (the minimum value is 1), the closer the corresponding fault is to the present. For example, the most recent fault will always be displayed in U0-00, and each subsequent fault will increment the count of all previous faults by 1. If the number of faults exceeds 6 or 10, the earliest fault record information will be overwritten. For the fault codes corresponding to fault types, please refer to the fault table.

| U0-10 | Fault Output 1 | Range: 0～65535 | Factory value: 0 |
| --- | --- | --- | --- |
| U0-11 | Fault Output 2 | Range: 0～65535 | Factory value: 0 |
| U0-12 | Fault Output 3 | Range: 0～65535 | Factory value: 0 |
| U0-13 | Fault output 4 | Range: 0～65535 | Factory value: 0 |

When the inverter fails, and the set values of parameters U0-10 to U0-13 match the fault code, the corresponding RLY1/RLY2 and DO1/DO2 outputs can be enabled by setting F6-00 to F6-03 to 35 to 38. The four fault outputs can operate independently without affecting each other.

| U0-14 | Fault 1 - Motor speed | Range: -32767 to 32767 r/min | Factory value: 0 |
| --- | --- | --- | --- |
| U0-15 | Fault 1 - Torque command | Range: -3276.7 to 3276.7 | Factory value: 0 |
| U0-16 | Fault 1 - Input terminals | Range: 0000H~FFFFH | Factory value: 0 |
| U0-17 | Fault 1 - Output terminals | Range: 0000H~FFFFH | Factory value: 0 |
| U0-18 | Fault 1 - Inverter Status | Range: 0～65535 | Factory value: 0 |
| U0-19 | Fault 1 - Frequency Command | Range: 0.00 to 655.35 Hz | Factory Value: 0.00 |
| U0-20 | Fault 1 - Output Frequency | Range: 0.00 to 599.99 Hz | Factory Value: 0.00 |
| U0-21 | Fault 1 - Output Voltage | Range: 0.0 to 6553.5 V | Factory Value: 0.0 |
| U0-22 | Fault 1 - DC Voltage | Range: 0.0 to 6553.5 V | Factory Value: 0.0 |
| U0-23 | Fault 1-Output Current | Range: 0.0 to 655.35 A | Factory Value: 0.00 |
| U0-24 | Fault 1-IGBT Temperature | Range: -3276.7 ～ 3276.7°C | Factory Value: 0.0 |
| U0-25 | Fault 1-Capacitor Temperature | Range: -3276.7 ～ 3276.7°C | Factory Value: 0.0 |

The above parameters are used to record the relevant information of the inverter during the most recent fault.

| U0-26 | Fault 2-Output Frequency | Range: 0.00～599.00Hz | Factory Value: 0.00 |
| --- | --- | --- | --- |
| U0-27 | Fault 2-Direct Current Voltage | Range: 0.0～6553.5V | Factory Value: 0.0 |
| U0-28 | Fault 2-Output Current | Range: 0.00～655.35A | Factory Value: 0.00 |
| U0-29 | Fault 2 - IGBT Temperature | Range: -3276.7～3276.7°C | Factory Value: 0.0 |

The above parameters are used to record the relevant information of the inverter during the second-to-last fault.

| U0-30 | Fault 3 - Output Frequency | Range: 0.00 to 599.99 Hz | Factory Value: 0.00 |
| --- | --- | --- | --- |
| U0-31 | Fault 3 - DC Voltage | Range: 0.0 to 6553.5 V | Factory Value: 0.0 |
| U0-32 | Fault 3 - Output Current | Range: 0.0 to 655.35 A | Factory Value: 0.00 |
| U0-33 | Fault 3 - IGBT Temperature | Range: -3276.7 ～ 3276.7°C | Factory Value: 0.0 |

The above parameters are used to record the relevant information of the inverter during the third-to-last fault.

| U0-34 | Fault 4 - Output Frequency | Range: 0.00 to 599.99 Hz | Factory Value: 0.00 |
| --- | --- | --- | --- |
| U0-35 | Fault 4-Direct Current Voltage | Range: 0.0 to 6553.5 V | Factory Value: 0.0 |
| U0-36 | Fault 4-Output Current | Range: 0.0 to 655.35 A | Factory Value: 0.00 |
| U0-37 | Fault 4-IGBT Temperature | Range: -3276.7 ～ 3276.7°C | Factory Value: 0.0 |

The above parameters are used to record the relevant information of the inverter during the fourth-to-last fault.

| U0-38 | Fault 5-Output Frequency | Range: 0.00 to 599.99 Hz | Factory Value: 0.00 |
| --- | --- | --- | --- |
| U0-39 | Fault 5-Direct Current Voltage | Range: 0.0 to 6553.5 V | Factory Value: 0.0 |
| U0-40 | Fault 5-Output Current | Range;0.0 ～ 655.35A | Factory Value: 0.00 |
| U0-41 | Fault 5-IGBT Temperature | Range: -3276.7 ～ 3276.7°C | Factory Value: 0.0 |

The above parameters are used to record the relevant information of the inverter during the fifth-to-last fault.

| U0-42 | Fault 6-Output Frequency | Range: 0.00 to 599.99 Hz | Factory Value: 0.00 |
| --- | --- | --- | --- |
| U0-43 | Fault 6-Direct Current Voltage | Range: 0.0 to 6553.5 V | Factory Value: 0.0 |
| U0-44 | Fault 6-Output Current | Range: 0.0 to 655.35 A | Factory Value: 0.00 |
| U0-45 | Fault 6-IGBT Temperature | Range: -3276.7 ～ 3276.7°C | Factory Value: 0.0 |

The above parameters are used to record the relevant information of the inverter at the time of the sixth last fault.

| U0-46 | Number of Days Since Fault 1 Occurred | Range: 0 ～ 65536 days | Factory value: 0 |
| --- | --- | --- | --- |
| U0-47 | Fault 1 Occurrence Minutes | Range: 0 ～ 1439 minutes | Factory value: 0 |
| U0-48 | Fault 2 Occurrence Days | Range: 0 ～ 65536 days | Factory value: 0 |
| U0-49 | Fault 2 Occurrence Minutes | Range: 0 ～ 1439 minutes | Factory value: 0 |
| U0-50 | Fault 3 Occurrence Days | Range: 0 ～ 65536 days | Factory value: 0 |
| U0-51 | Fault 3 Occurrence Minutes | Range: 0 ～ 1439 minutes | Factory value: 0 |
| U0-52 | Fault 4 Occurrence Days | Range: 0 ～ 65536 days | Factory value: 0 |
| U0-53 | Minutes since Fault 4 occurred | Range: 0 ～ 1439 minutes | Factory value: 0 |
| U0-54 | Days since Fault 5 occurred | Range: 0 ～ 65536 days | Factory value: 0 |
| U0-55 | Minutes since Fault 5 occurred | Range: 0 ～ 1439 minutes | Factory value: 0 |
| U0-56 | Days since Fault 6 occurred | Range: 0 ～ 65536 days | Factory value: 0 |
| U0-57 | Minutes since Fault 6 occurred | Range: 0 ～ 1439 minutes | Factory value: 0 |

The above parameters are used to record the time elapsed since the inverter control board was powered on at each fault occurrence, regardless of whether the inverter is running.

2.17 U1 Group Status Monitoring Parameters

| U1-00 | DI terminal status | Range: 0000H~FFFFH | Factory value: 0 |
| --- | --- | --- | --- |

This parameter is read-only and is used to display the status of the inverter's DI terminals. The relationship between each bit and the DI terminals is shown inTable 2-24.

Table 2-24 DI Terminal Status

| Bit0 | Bit1 | Bit2 | Bit3 | Bit4 | Bit5 | Bit6 | Bit7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DI1 | DI2 | DI3 | DI4 | DI5 | DI6 | DI7 | HDI8 |
| Bit8 | Bit9 | Bit10 | Bit11 | Bit12 | Bit13 | Bit14 | Bit15 |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| U1-01 | DO terminal status | Range: 0000H~FFFFH | Factory value: 0 |
| --- | --- | --- | --- |

This parameter is read-only and used to display the status of the inverter's DO terminals. The relationship between each bit and the DO terminals is shown inTable 2-25.

Table 2-25 DO Terminal Status

| Bit0 | Bit1 | Bit2 | Bit3 | Bit4 | Bit5 | Bit6 | Bit7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RLY1 | RLY2 | D01 | D02 | Reserved | Reserved | Reserved | Reserved |
| Bit8 | Bit9 | Bit10 | Bit11 | Bit12 | Bit13 | Bit14 | Bit15 |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| U1-02 | Set frequency | Range: 0～599.00Hz | Factory Value: 50.00 |
| --- | --- | --- | --- |

This parameter is read-only and used to display the current set frequency.

| U1-03 | External frequency record | Range: 0～599.00Hz | Factory Value: 60.00 |
| --- | --- | --- | --- |

This parameter is read-only. When the frequency source is selected as external terminals, this parameter stores the frequency command value when the inverter fails.

| U1-04 | PID feedback value | Range: -200.00%～200.00% | Factory value: 0.00% |
| --- | --- | --- | --- |

This parameter is typically read-only and used to display the PID feedback value. However, when the PID feedback type is selected as communication (FA-00=7 or 8), this parameter can be written to set the PID feedback value.

| U1-05 | KP gain monitoring value | Range: 0.00～100.00% | Factory Value: 0.00 |
| --- | --- | --- | --- |
| U1-06 | KI Gain Monitoring Value | Range: 0.00～100.00s | Factory Value: 0.00 |
| U1-07 | KD Gain Monitoring Value | Range: 0.00～1.00 | Factory Value: 0.00 |

The above parameters are read-only, used for displaying the PID proportional, integral, and differential gain values.

2.18 H0 Group Other Motor Parameters

When users need to switch between two or more motors, they can achieve motor switching through H0-00, and the CM680 supports switching of up to four induction motors. Parameters such as nameplate data and motor parameter identification can be set separately for each of the four motors. H0-01～H0-12, H0-13～H0-21, H0-22～H0-30 correspond to the parameters of the second, third, and fourth induction motors, respectively. All parameters in the H0 group have the same content definitions and usage methods as the first motor-related parameters in the F4 group, and will not be repeated here; users can refer to the explanations of the first motor-related parameters in the F4 group.

| F0-00 | Model Number | Range: 0～65535 | Factory value: Model determined |
| --- | --- | --- | --- |

1: Induction Motor 1

2: Induction Motor 2

3: Induction Motor 3

4: Induction Motor 4

| H0-01 | M2 Maximum Frequency | Range: 0.00～599.00Hz | Factory Value: 50.00 |
| --- | --- | --- | --- |
| H0-02 | M2 Rated Frequency | Range: 0.00～599.00Hz | Factory Value: 50.00 |
| H0-03 | M2 Rated Voltage | Range: 0.0～510.0V | Factory Value: 380.0 |
| H0-04 | IM2 Rated Power | Range: 0.00～655.35kW | Factory Value: 0.00 |
| H0-05 | IM2 Number of Poles | Range: 2～20 | Factory value: 4 |
| H0-06 | IM2 Rated Current | Range: 0.00～655.35A | Factory Value: 13.00 |
| H0-07 | IM2 Rated Speed | Range: 0～65535r/min | Factory Value: 1410 |
| H0-08 | IM2 No-load Current | Range: 0.00～13.00A | Factory Value: 0.00 |
| H0-09 | IM2 Stator Resistance | Range: 0.000～65.535Ω | Factory Value: 0.000 |
| H0-10 | IM2 Rotor Resistance | Range: 0.000～65.535Ω | Factory Value: 0.000 |
| H0-11 | IM2 Mutual Inductance | Range: 0.0～6553.5mH | Factory Value: 0.0 |
| H0-12 | IM2 Leakage Inductance | Range: 0.00～655.35mH | Factory Value: 0.00 |

The above parameters correspond to the second induction motor parameters.

| H0-13 | M3 Maximum Frequency | Range: 0.00～599.00Hz | Factory Value: 50.00 |
| --- | --- | --- | --- |
| H0-14 | M3 Rated Frequency | Range: 0.00～599.00Hz | Factory Value: 50.00 |
| H0-15 | M3 Rated Voltage | Range: 0.0～510.0V | Factory Value: 380.0 |
| H0-16 | IM3 rated power | Range: 0.00～655.35kW | Factory Value: 0.00 |
| H0-17 | IM3 number of poles | Range: 2～20 | Factory value: 4 |
| H0-18 | IM3 rated current | Range: 0.00～655.35A | Factory Value: 13.00 |
| H0-19 | IM3 rated speed | Range: 0～65535r/min | Factory Value: 1410 |
| H0-20 | IM3 no-load current | Range: 0.00～13.00A | Factory Value: 0.00 |
| H0-21 | IM3 Stator Resistance | Range: 0.000～65.535Ω | Factory Value: 0.000 |

The above parameters correspond to the third induction motor parameters.

| H0-22 | M4 Maximum Frequency | Range: 0.00～599.00Hz | Factory Value: 50.00 |
| --- | --- | --- | --- |
| H0-23 | M4 Rated Frequency | Range: 0.00～599.00Hz | Factory Value: 50.00 |
| H0-24 | M4 Rated Voltage | Range: 0.0～510.0V | Factory Value: 380.0 |
| H0-25 | IM4 Rated Power | Range: 0.00～655.35kW | Factory Value: 0.00 |
| H0-26 | IM4 Number of Poles | Range: 2～20 | Factory value: 4 |
| H0-27 | IM4 Rated Current | Range: 0.00～655.35A | Factory Value: 13.00 |
| H0-28 | IM4 Rated Speed | Range: 0～65535r/min | Factory Value: 1410 |
| H0-29 | IM4 No-load Current | Range: 0.00～13.00A | Factory Value: 0.00 |
| H0-30 | IM4 Stator Resistance | Range: 0.000～65.535Ω | Factory Value: 0.000 |

The above parameters correspond to the fourth induction motor parameters.

2.19 H1 Group Other Motor VF Control

Used for setting the VF control of the second, third, and fourth induction motors.

| H1-00 | M2 Multi-point VF Frequency Point 1 | Range: 0.00～599.00Hz | Factory value: 0.5 |
| --- | --- | --- | --- |
| H1-01 | M2 Multi-point VF Voltage Point 1 | Range: 0.0～480.0V | Factory Value: 2 |
| H1-02 | M2 Multi-point VF Frequency Point 2 | Range: 0.00～599.00Hz | Factory value: 1.5 |
| H1-03 | M2 Multi-point VF Voltage Point 2 | Range: 0.0～480.0V | Factory value: 10 |
| H1-04 | M2 Multi-point VF Frequency Point 3 | Range: 0.00～599.00Hz | Factory Value: 3 |
| H1-05 | M2 Multi-point VF Voltage Point 3 | Range: 0.0～480.0V | Factory value: 22 |

The above function codes are used to set the multi-segment (custom) V/F curve for the second induction motor, as shown in Figure 2-68. Define the multi-segment V/F curve according to H1-00 to H1-05 and H0-02 and H0-03, as shown in the figure below. In the figure below, f1, f2, f3, and fb represent the first, second, third frequency points, and the rated frequency of M2 multi-point VF, respectively; V1, V2, V3, and Vb represent the first, second, third voltage points, and the rated voltage of M2 multi-point VF, respectively.

Note: Generally, V1<V2<V3<Vb, f1<f2<f3<fb. Setting the voltage too high at low frequencies may cause the motor to overheat or even burn out, and the inverter may experience overcurrent or overvoltage protection.

Figure 2-68 Multi-segment (Custom) V/F Curve for the Second Induction Motor

| H1-06 | M2 Torque Compensation Gain | Range: 0～10 | Factory value: 1 |
| --- | --- | --- | --- |
| H1-07 | M2 Slip Compensation Gain | Range: 0.00～10.00 | Factory value: 0 |

The above two function codes are only applicable when driving the second induction motor in VF and VVC control modes.

H1-06 Torque Compensation Gain, used to adjust the torque compensation level. By adjusting the output voltage, the motor's load-carrying capacity is adjusted. The greater the torque compensation gain, the better the load-carrying capacity, but the output current will increase. If set too high, it may lead to overcurrent faults; reducing the torque compensation gain will decrease the load-carrying capacity accordingly.

H1-07 Torque Filter Time, used to adjust the filter time for torque compensation. If the filter time is set too large, control stability improves, but the control response deteriorates. When the filter time is toosmall, the response is faster, but it may lead to instability. Adjustments can be made according to actual conditions.

| H1-08 | M3 Multi-point VF Frequency Point 1 | Range: 0.00～599.00Hz | Factory value: 0.5 |
| --- | --- | --- | --- |
| H1-09 | M3 Multi-point VF Voltage Point 1 | Range: 0.0～480.0V | Factory Value: 2 |
| H1-10 | M3 Multi-point VF Frequency Point 2 | Range: 0.00～599.00Hz | Factory value: 1.5 |
| H1-11 | M3 Multi-point VF Voltage Point 2 | Range: 0.0～480.0V | Factory value: 10 |
| H1-12 | M3 Multi-point VF Frequency Point 3 | Range: 0.00～599.00Hz | Factory Value: 3 |
| H1-13 | M3 Multi-point VF Voltage Point 3 | Range: 0.0～480.0V | Factory value: 22 |

The above function codes are used to set the multi-segment (custom) V/F curve for the third induction motor, as shown in Figure 2-69. Define the multi-segment V/F curve according to H1-08 to H1-13 and H0-14 and H0-15, as shown in the figure below. In the figure below, f1, f2, f3, and fb represent the M3 multi-point VF frequency points 1, 2, 3, and the M3 rated frequency, respectively, while V1, V2, V3, and Vb represent the M3 multi-point VF voltage points 1, 2, 3, and the M3 rated voltage, respectively.

Note: Generally, V1<V2<V3<Vb, f1<f2<f3<fb. Setting the voltage too high at low frequencies may cause the motor to overheat or even burn out, and the inverter may experience overcurrent or overvoltage protection.

Figure 2-69 Thrinduction Motor Multi-Segment (Custom) V/F Curve

| H1-14 | M3 Torque Compensation Gain | Range: 0～10 | Factory value: 1 |
| --- | --- | --- | --- |
| H1-15 | M3 Slip Compensation Gain | Range: 0.00～10.00 | Factory value: 0 |

The above two function codes are only applicable for driving third induction motors under VF and VVC control modes.

H1-14 Torque compensation gain, used to adjust the torque compensation level. By adjusting the output voltage, the motor's load-carrying capacity is adjusted. The greater the torque compensation gain, the better the load-carrying capacity, but the output current will increase. If set too high, it may lead to overcurrent faults; reducing the torque compensation gain will decrease the load-carrying capacity accordingly.

H1-15 Torque filter time, used to adjust the filter time of the torque compensation. If the filter time is set too large, control stability improves, but the control response deteriorates. When the filter time is toosmall, the response is faster, but it may lead to instability. Adjustments can be made according to actual conditions.

| H1-16 | M4 Multi-point VF Frequency Point 1 | Range: 0.00～599.00Hz | Factory value: 0.5 |
| --- | --- | --- | --- |
| H1-17 | M4 Multi-point VF Voltage Point 1 | Range: 0.0～480.0V | Factory Value: 2 |
| H1-18 | M4 Multi-point VF Frequency Point 2 | Range: 0.00～599.00Hz | Factory value: 1.5 |
| H1-19 | M4 Multi-point VF Voltage Point 2 | Range: 0.0～480.0V | Factory value: 10 |
| H1-20 | M4 Multi-point VF Frequency Point 3 | Range: 0.00～599.00Hz | Factory Value: 3 |
| H1-21 | M4 Multi-point VF Voltage Point 3 | Range: 0.0～480.0V | Factory value: 22 |

The above function codes are used to set the multi-segment (custom) V/F curve for the fourth induction motor, as shown in Figure 2-70. Define multi-segment V/F curves according to H1-16～H1-21 and H0-23 and H0-24, as shown in the figure below. In the figure below, f1, f2, f3, and fb represent M4 multi-point VF frequency point 1, M4 multi-point VF frequency point 2, M4 multi-point VF frequency point 3, and M4 rated frequency, respectively. V1, V2, V3, and Vb represent M4 multi-point VF voltage point 1, M4 multi-point VF voltage point 2, M4 multi-point VF voltage point 3, and M4 rated voltage, respectively.

Note: Generally, V1<V2<V3<Vb, f1<f2<f3<fb. Setting the voltage too high at low frequencies may cause the motor to overheat or even burn out, and the inverter may experience overcurrent or overvoltage protection.

Figure 2-70 Fourth Induction Motor Multi-Segment (Custom) V/F Curve

| H1-22 | M4 Torque Compensation Gain | Range: 0～10 | Factory value: 1 |
| --- | --- | --- | --- |
| H1-23 | M4 Slip Compensation Gain | Range: 0.00～10.00 | Factory value: 0 |

The above two function codes apply only to the situation where the fourth induction motor is driven in VF and VVC control modes.

H1-22 Torque compensation gain, used to adjust the size of torque compensation. By adjusting the output voltage, the motor's load-carrying capacity is adjusted. The greater the torque compensation gain, the better the load-carrying capacity, but the output current will increase. If set too high, it may lead to overcurrent faults; reducing the torque compensation gain will decrease the load-carrying capacity accordingly.

H1-23 Torque filter time, used to adjust the filter time for torque compensation. If the filter time setting is too large, control stability improves, but the control response deteriorates. When the filter time is tooshort,the response is faster, but it may lead to instability. Adjustments can be made according to actual conditions.

2.20 H3 Group Other motor fault parameters

| H3-00 | Over-torque selection 2 | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |
| H3-01 | Over-torque threshold 2 | Range: 10%～250% | Factory value: 120 |
| H3-02 | Over-torque time 2 | Range: 0.1～60.0s | Factory value: 0.1 |

The above function codes are used to set up the over-torque detection function for Motor 2. The inverter issues a warning or stops when it detects excessive torque on Motor 2, protecting both the motor and the inverter.

H3-00 Over-torque Selection 2

0: Not Detected

1: Constant Speed Detection Continue Running

During constant speed operation, if Motor 2 experiences over-torque, a warning is issued but operation continues.

2: Constant Speed Detection Stop Running

During constant speed operation, if Motor 2 experiences over-torque, an over-torque fault is reported and operation stops.

3: Running Detection Continue Running

During operation, if Motor 2 experiences over-torque, a warning is issued but operation continues.

4: Running Detection Stop Running

During operation, if Motor 2 experiences over-torque, an over-torque fault is reported and operation stops.

H3-01 Over-torque threshold 2

H3-02 Over-torque time 2

When the inverter output current exceeds H3-01 (unit %, based on the rated current of the inverter) and persists for longer than the time set by H3-02, the inverter will determine subsequent actions according to H3-00. When H3-00 is 1 or 3, as shown in Figure 2-71, if over-torque is detected, the inverter will display an over-torque warning but continue to operate until the output current falls below 95% of the H3-01 set value, at which point the warning will be cleared. When H3-00 is 2 or 4, as shown in Figure 2-72, if over-torque is detected, the inverter will report an over-torque fault and stop operation until the fault is reset, after which it can resume operation.

Figure 2-71Motor 2 Over-torque Detection (H3-00=1or 3)

Figure 2-72Motor 2 Over-torque Detection (H3-00=2or4)

| H3-03 | Motor 2 overload selection | Range: 0～2 | Factory Value: 2 |
| --- | --- | --- | --- |
| H3-04 | Motor 2 overload time | Range: 30.0～600.0s | Factory Value: 60 |

Function Code H3-03 is used to set the overload protection mode for Motor 2.

0: Constant Torque Output Motor

1: Variable Torque Output Motor

2: No Motor Overload Protection

Function Code H3-04 is used to set the overload protection time for Motor 2 when the current reaches 150% of the rated current. By setting an appropriate inverse-time curve, the operating time of Motor 2 under overload conditions can be made less than the set overload protection time for Motor 2, thereby achieving overload protection and preventing Motor 2 from being damaged due to overheating. When the overload time reaches the motor 2 overload protection time, an alarm for motor 2 overload fault (E023) will be reported. This function is not enabled by default; if it needs to be enabled, set H3-03 to 0 or 1.

When H3-03 is 0, the overload curve of the corresponding motor is determined by H3-04, denoted as time T, and the inverse-time curve determined by it is shown inFigure 2-73. Inthis,“Motor 2 Current Percentage” refers to the ratio of the inverter output current to the motor rated current. When the motor fan is independently controlled, this inverse time curve can be selected. In this case, the fan speed is independent of the motor speed, so the cooling capacity does not decrease as the motor speed decreases, making the inverse time curve independent of the motor operating speed.

Figure 2-73 Motor 2 Overload Protection Curve (H3-03=0)

When H3-03 is 1, the overload curve of motor 2 is determined by H3-04 and the motor speed, as shown in Figure2-74. The “speed coefficient” is a function of motor speed, when the motor speed exceeds the rated speed, the speed coefficient equals 1, when the motor speed is less than the rated speed, the speed coefficient = 1/(0.4 + 0.6 * motor speed / motor 2 rated speed).

Figure 2-74 Motor 2 Overload Protection Curve (H3-03=1)

When the motor uses coaxial cooling (fan connected to the motor shaft), the fan speed is the same as the motor speed, and a decrease in motor speed will reduce the fan's cooling capacity. In this case, it is recommended to set H3-03 to 1, the inverse time curve will adjust according to the motor speed, the motor overload capacity will decrease as the speed decreases, preventing overheating due to reduced fan cooling capacity. When the motor speed is zero, the fan stops rotating, at this point the speed coefficient is 2.5, the corresponding overload curve is shown inFigure 2-75, according to the figure, when the current reaches 60% of the motor's rated current, the motor will report an overload fault after running for T time.

Figure 2-75 Motor 2 Overload Protection Curve (Coaxial Cooling)

| H3-05 | Over-torque selection 3 | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |
| H3-06 | Over-torque threshold 3 | Range: 10%～250% | Factory value: 120 |
| H3-07 | Over-torque time 3 | Range: 0.1～60.0s | Factory value: 0.1 |

The above function codes are used to set up the over-torque detection function for Motor 3. The inverter issues a warning or stops operation when it detects excessive torque on Motor 3, thus protecting the motor and the inverter.

H3-05 Over-torque Selection 3

0: Not Detected

1: Constant Speed Detection Continue Running

During constant speed operation, if Motor 3 experiences over-torque, a warning is issued but operation continues.

2: Constant Speed Detection Stop Running

During constant speed operation, if Motor 3 experiences over-torque, an over-torque fault is reported and operation stops.

3: Running Detection Continue Running

During operation, if Motor 3 experiences over-torque, a warning is issued but operation continues.

4: Running Detection Stop Running

During operation, if motor 3 experiences over-torque, an over-torque fault is reported and the operation stops.

H3-06 Over-torque threshold 3

H3-07 Over-torque time 3

When the inverter output current exceeds H3-06 (unit %, based on the inverter's rated current) and persists for longer than the time set by H3-07, the inverter will determine subsequent actions according to H3-05. When H3-05 is 1 or 3,the over-torque detection is shown in Figure 2-76. If over-torque is detected, the inverter will display an over-torque warning, but the inverter will continue to operate until the output current is less than 95% of the H3-06 set value, after which the warning will be cleared. When H3-05 is 2 or 4, the over-torque detection is shown in Figure 2-77. If over-torque is detected, the inverter reports an over-torque fault and stopsoperation until the fault is reset, after which it can resume operation.

Figure 2-76 Over-torque detection for Motor 3 (H3-05=1or 3)

Figure 2-77 Over-torque detection for Motor 3 (H3-05=2or4)

| H3-08 | Motor 3 overload selection | Range: 0～2 | Factory Value: 2 |
| --- | --- | --- | --- |
| H3-09 | Motor 3 overload time | Range: 30.0～600.0s | Factory Value: 60 |

Function code H3-08 is used to set the overload protection mode for Motor 3.

0: Constant Torque Output Motor

1: Variable Torque Output Motor

2: No Motor Overload Protection

Function code H3-09 is used to set the overload protection time for Motor 3 when the current reaches 150% of the rated current.

By setting an appropriate inverse time curve, the operating time of Motor 3 under overload conditions is made less than the set overload protection time for Motor 3, thereby achieving overload protection for Motor 3 and preventing damage due to overheating. When the overload time reaches the overload protection time for Motor 3, an overload fault for Motor 3 (E134) will be reported. This function is not enabled by default; if it needs to be enabled, H3-08 must be set to 0 or 1.

When H3-08 is 0, the overload curve for the corresponding motor is determined by H3-09, denoted as T, and the inverse time curve determined by it is shown in Figure2-78. Inthiscase,the“Motor 3 current percentage” refers to the ratio of the inverter output current to the rated current of Motor 3. When the motor fan is independently controlled, this inverse time curve can be selected. In this case, the fan speed is independent of the motor speed, so the cooling capacity does not decrease as the motor speed decreases, making the inverse time curve independent of the motor operating speed.

Figure 2-78 Motor 3 Overload Protection Curve (H3-08=0)

When H3-08 is 1, the overload curve of Motor 3 is determined by H3-09 and the motor speed, as shown in Figure7-79. Among them, the “speed coefficient” is a function of the motor speed; when the motor speed exceeds the rated speed, the speed coefficient equals 1; when the motor speed is less than the rated speed, speed coefficient=1/(0.4+0.6*motor speed/Motor 3 rated speed)

Figure 2-79 Motor 3 Overload Protection Curve (H3-08=1)

When the motor uses coaxial cooling (fan connected to the motor shaft), the fan speed is the same as the motor speed, and a decrease in motor speed will reduce the fan's cooling capacity. In this case, it is recommended to set H3-08 to 1, so that the inverse-time curve adjusts according to the motor speed, reducing the motor's overload capacity as the speed decreases to prevent overheating due to reduced fan cooling efficiency. When the motor speed is zero, the fan stops rotating. At this time, the speed factor is 2.5, and the corresponding overload curve is shown in Figure2-80. From the figure, it can be seen that when the current reaches 60% of the motor's rated current, Motor 3 will report an overload fault after running for T time.

Figure 2-80 Overload Protection Curve for Motor 3 (Axial Cooling)

| H3-10 | Over-torque selection 4 | Range: 0～4 | Factory value: 0 |
| --- | --- | --- | --- |
| H3-11 | Over-torque threshold 4 | Range: 10%～250% | Factory value: 120 |
| H3-12 | Over Torque Time 4 | Range: 30.0～600.0s | Factory value: 0.1 |

The above function codes are used to set the over-torque detection function for Motor 4. The inverter issues a warning or shuts down when it detects excessive torque in Motor 4, protecting both the motor and the inverter.

H3-10 Over-torque Selection 4

0: Not Detected

1: Constant Speed Detection Continue Running

During constant-speed operation, if Motor 4 experiences over-torque, a warning is issued but the operation continues.

2: Constant Speed Detection Stop Running

When running at constant speed, if motor 4 experiences over-torque, an over-torque fault is reported and operation stops.

3: Running Detection Continue Running

During operation, if motor 4 experiences over-torque, a warning is issued but operation continues.

4: Running Detection Stop Running

During operation, if motor 4 experiences over-torque, an over-torque fault is reported and operation stops.

H3-11 Over-torque threshold 4

H3-12 Over-torque time 4

When the inverter output current exceeds H3-11 (unit %, based on the inverter's rated current) and persists for longer than the time set in H3-12, the inverter will determine subsequent actions according to H3-05. When H3-10 is 1 or 3, as shown in Figure 2-81, if over-torque is detected, the inverter will display an over-torque warning, but the inverter will continue to operate until the output current is less than 95% of the H3-11 set value, after which the warning will be cleared. When H3-10 is 2 or 4, as shown in Figure 2-82, if over-torque is detected, the inverter reports an over-torque fault and stopsoperation until the fault is reset, after which it can resume operation.

Figure 2-81 Motor 4 Over-Torque Detection (H3-10=1or 3)

Figure 2-82 Motor 4 Over-Torque Detection (H3-10=2or4)

| H3-13 | Motor 4 Overload Selection | Range: 0～2 | Factory Value: 2 |
| --- | --- | --- | --- |
| H3-14 | Motor 4 Overload Time | Range: 30.0～600.0s | Factory Value: 60 |

Function code H3-13 is used to set the overload protection mode for Motor 4.

0: Constant Torque Output Motor

1: Variable Torque Output Motor

2: No Motor Overload Protection

Function code H3-14 is used to set the overload protection time for Motor 4 when the current reaches 150% of the rated current. By setting appropriate inverse-time curves, the operating time of Motor 4 under overload conditions is made less than the set overload protection time for Motor 4, thereby achieving overload protection and preventing Motor 4 from being damaged due to overheating. When the overload time reaches the motor 4 overload protection time, an E135 motor 4 overload fault will be reported. This function is not enabled by default; if it needs to be enabled, set H3-13 to 0 or 1.

When H3-13 is 0, the overload curve for the corresponding motor is determined by H3-14, denoted as T, and the inverse-time curve is shown in Figure2-83. Inthiscase,the“Motor 4 Current Percentage” refers to the ratio of the inverter output current to the rated current of Motor 4. When the motor fan is independently controlled, this inverse time curve can be selected. In this case, the fan speed is independent of the motor speed, so the cooling capacity does not decrease as the motor speed decreases, making the inverse time curve independent of the motor operating speed.

Figure 2-83 Motor 4 Overload Protection Curve (H3-13=0)

When H3-13 is 1, the overload curve for Motor 4 is determined by H3-14 and the motor speed, as shown in Figure2-84. Among them,“speed coefficient” is a function of motor speed, when the motor speed is greater than the rated speed, the speed coefficient equals 1, when the motor speed is less than the rated speed,speed coefficient=1/(0.4+0.6*motor speed/motor 4 rated speed)

Figure 2-84 Motor 4 overload protection curve (H3-13=1)

When the motor uses coaxial cooling (fan connected to the motor shaft), the fan speed is the same as the motor speed, and a decrease in motor speed will reduce the fan's cooling capacity. In this case, it is recommended to set H3-13 to 1, the inverse time curve will be adjusted according to the motor speed, the motor overload capacity will decrease as the speed decreases, preventing motor overheating due to reduced fan cooling capacity. When the motor speed is zero, the fan stops rotating. At this time, the speed factor is 2.5, and the corresponding overload curve is shown in Figure2-85. From the figure, it can be seen that when the current reaches 60% of the motor's rated current, Motor 4 will report an overload fault after running for T time.

Figure 2-85 Overload Protection Curve for Motor 4 (Axial Cooling)

2.21 L0 Group System Control Parameters

| L0-00 | APP MACRO | Range: 0～65535 | Factory value: 0 |
| --- | --- | --- | --- |
| L0-01 | System Debugging Flag | Range: 0～65535 | Factory value: 0 |
| L0-02 | Debugging Flag | Range: 0～65535 | Factory value: 0 |
| L0-03 | Debug Flag 1 | Range: 0～65535 | Factory Value: 64 |
| L0-05 | System Control Parameters | Range: 0～65535 | Factory value: 0 |
| L0-06 | Special Control Flags | Range: 0～65535 | Factory value: 0 |
| L0-09 | KPD Automatic Screen Refresh Enable | Range: 0～1 | Factory value: 0 |
| L0-10 | KPD Screen Refresh Time Setting | 0.00～6.00s | Factory Value: 0.15 |
| L0-11 | Low Speed Debugging Position | 0～65535 | Factory value: 1 |

This group of function codes is used for professional personnel to debug; detailed introduction is not provided here.

2.22 L1 Group User Function Code Customization

| L1-00 | Frequency Source Selection (HAND) | Range: 0～9 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the frequency source when in manual mode.

0: Digital Operator

1: RS485 Communication

2: Analog Input

3: External Up/Down Input

4: Pulse Input Without Direction

5: Pulse Input With Direction

6: CANopen Input

7: Reserved

8: Communication Card Input

9: PID

| L1-01 | Operation Command Source (HAND) | Range: 0～5 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set the operation command source when in manual mode.

0: Digital Operator

1: External Terminal Input

2: RS485 Communication Input

3: CANopen Input

4: Reserved

5: Communication Card Input

2.23 L2 Group Optimal Control Parameters

| L2-06 | Dead Time Compensation Mode | Range: 0～5 | Factory value: 4 |
| --- | --- | --- | --- |

This parameter is used to set the dead time compensation method.

0: Disable dead time compensation

4: Use dead time compensation method 4

5: Use dead time compensation method 5

| L2-07 | Dead Time Compensation Value | Range: 0～65535 | Factory Value: 180 |
| --- | --- | --- | --- |
| L2-08 | Tube Voltage Drop Compensation Value | Range: 0～65535 | Factory Value: 5 |
| L2-09 | Dead Time Compensation 4 Slope | Range: 0～65535 | Factory Value: 200 |
| L2-10 | Dead Time Compensation 5 Slope | Range: 0～65535 | Factory Value: 5000 |

The above parameters are used to adjust the dead time compensation curve, generally no modification is required.

| L2-17 | BrakeUnit EnableVoltage | Range:330.0～900.0V | Factory Value: 740.0V |
| --- | --- | --- | --- |

This parameter sets the DC bus voltage threshold for activating the braking unit. Users can select an appropriate braking resistor to achieve optimal deceleration characteristics; this parameter can be modified during operation.

| L2-18 | Undervoltage Protection Value | Range: 250.0～440.0V | Factory Value: 360.0V |
| --- | --- | --- | --- |

This parameter sets the undervoltage threshold for the inverter's DC bus voltage. When the DC bus voltage falls below this value, an undervoltage fault is triggered, causing the inverter to stop output and the motor to free stop.

| L2-19 | Zero Speed Operation Selection | Range: 0～2 | Factory value: 0 |
| --- | --- | --- | --- |

0: Wait for Output

When the control mode is VF or VVC, output waits, and the inverter enters a waiting state (U, V, W 无voltage output);

In other control modes, zero-speed operation is executed.

1: Zero-speed operation

When the control method is set to induction motor VF, the inverter will enter the operating state, but the actual output will be zero;

When the control method is set to VVC, the inverterworks in the DC braking state, and the motor current is DC;

In other control modes, zero-speed operation is executed.

2: Output at Minimum Frequency

The inverter will operate according to the set values of multi-point VF frequency point 1 (F2-04) and multi-point VF voltage point 1 (F2-05).

| L2-22 | Random PWM | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: Off

1: Enable

| L2-24 | Back EMF Identification Frequency | Range:0～1000% | Factory value:50 |
| --- | --- | --- | --- |

For synchronous motors with higher rated frequencies (e.g., several hundred Hz), the frequency ratio during operation can be appropriately reduced during the rotational self-learning process.

2.24 L4 Group Brake Function Parameters

| L4-00 | Brake Frequency | Range: 0.00～599.00Hz | Factory Value: 0.00 |
| --- | --- | --- | --- |

This function code is used to set the brake frequency.

| L4-01 | Brake Control Enable Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used to set brakecontrol enable.

| L4-02 | Brake Release Frequency | Range: 0.00～20.00Hz | Factory Value: 2.5 |
| --- | --- | --- | --- |

This function code is used to setbrake release frequency.

| L4-03 | Brake Release Frequency Maintenance | Range: 0.00～20.0 seconds | Factory Value: 1.0 |
| --- | --- | --- | --- |

This function code is used to setbrake release frequency hold.

| L4-04 | Brake Current Limit Value | Range: 50～200% | Factory value: 120 |
| --- | --- | --- | --- |

This function code is used to setbrake current limit value.

| L4-05 | Brake Close Frequency | Range: 0.00～20.00Hz | Factory Value: 1.50 |
| --- | --- | --- | --- |

This function code is used to setbrake close frequency.

| L4-06 | Brake Close Delay Time | Range: 0.0～20.0 seconds | Factory Value: 0.0 |
| --- | --- | --- | --- |

This function code is used to setbrake release delay time.

| L4-07 | Brake Close Frequency Maintenance | Range: 0.0～20.0 seconds | Factory Value: 1.0 |
| --- | --- | --- | --- |

This function code is used to setbrake release frequency maintenance.

2.25 L5 Group Sleep and Wake Function Parameters

This group of parameters is mainly used to achieve sleep and wake functions in constant pressure water supply applications, where the inverter stops running during the sleep period. After the wake-up delay within the sleep zone, the inverter starts running again, ending the sleep mode.

The sleep and wake functions require setting parameters such as sleep threshold, sleep delay, wake threshold, and wake delay. In general, the wake frequency (L5-02) should be set greater than or equal to the sleep frequency (L5-01). When the sleep frequency is 0, the sleep and wake functions are invalid.

Sleep and wake-up are divided into three cases:

1. Frequency command (without using process PID, parameter FA-00 = 0, only valid in VF control, i.e., VF sleep and wake-up)

After the output frequency reaches the sleep frequency (parameter L5-01), the inverter maintains operation at the sleep frequency and begins the sleep delay (parameter L5-03). After the delay time has elapsed, it stops directly at 0 Hz. When the frequency command reaches the wake-up frequency (parameter L5-02), after the wake-up delay (parameter L5-04), the inverter starts to accelerate to the set frequency according to the specified acceleration time.The frequency command sleep and wake-up process is shown in Figure 2-86.

Figure 2-86 Frequency CommandSleep and Wake-up Schematic Diagram

2. PID Output Frequency Command (using process PID, parameter FA-00 ≠ 0 and parameter L5-00 = 0, i.e., PID sleep and wake-up)

When the PID output frequency command reaches the sleep frequency (parameter L5-01), the inverter begins to enter sleep mode. After the sleep delay (parameter L5-03) has elapsed, it stops directly at 0 Hz. If the sleep delay time has not been reached, the output frequency remains at the lower limit frequency (parameter F0-11, and F0-11 ≠ 0) or the minimum output frequency (parameter F2-04, if the lower limit frequency F0-11 = 0), waiting for the sleep time to reach before entering the sleep state. When the PID output frequency command reaches the wake-up frequency (parameter L5-02), the inverter starts the wake-up delay (parameter L5-04). After the delay time has elapsed, the inverter accelerates to the PID output frequency as set. The PID output frequency command sleep and wake-up process is shown in Figure 2-87.

Figure 2-87 PID output frequency commandsleep and wake-up diagram

3. PID feedback value (using process PID, parameter FA-00 ≠ 0 and parameter L5-00 = 1, also for PID sleep and wake-up)

When the PID feedback value reaches the sleep threshold (parameter L5-01), the inverter begins to sleep. After the sleep delay (parameter L5-03) has elapsed, it stops directly at 0 Hz. If the sleep delay time has not been reached, the output frequency remains at the lower limit frequency (parameter F0-11, and F0-11 ≠ 0) or the minimum output frequency (parameter F2-04, if the lower limit frequency F0-11 = 0), waiting for the sleep time to reach before entering the sleep state. When the PID feedback value reaches the wake-up threshold (parameter L5-02), the inverter starts the wake-up delay (parameter L5-04). After the delay time has elapsed, the inverter accelerates to the PID output frequency according to the set acceleration time. The PID feedback value sleep and wake-up process is shown in Figure 2-88.

Figure 2-88 PIDFeedback Value Sleep and Wake-Up Schematic

When the sleep function is controlled by the PID setpoint and feedback value (the above cases 2 and 3), the inverter frequency source must be selected as PID (parameter F0-06 = 9).

| L5-00 | Sleep Method Reference Selection | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

0: PID Command Arrival

1: PID Feedback Arrival

When parameter L5-00 = 0, the units for parameters L5-01 and L5-02 automatically change to frequency, and the setting range automatically changes to 0.00~599.00 Hz.

When parameter L5-00 = 1, parameters L5-01 and L5-02 units automatically change to percentage, and the reference base becomes the percentage of feedback, with the setting range automatically changing to 0.00~200.00%.

| L5-01 | Sleep Threshold | Range: 0.00Hz～599.00Hz | Factory Value: 0.00 |
| --- | --- | --- | --- |

During the operation of the inverter, when the set frequency is less than the sleep frequency (parameter L5-01, L5-00 = 0), or the PID negative feedback value is greater than the sleep threshold (parameter L5-01, L5-00 = 1), the inverter begins to enter sleep mode. After the sleep delay (parameter L5-03) has elapsed, it stops directly at 0 Hz.

| L5-02 | Wake Threshold | Range: 0.00Hz～599.00Hz | Factory Value: 0.00 |
| --- | --- | --- | --- |

If the inverter is in sleep mode and the current operating command is valid, when the set frequency is greater than the wake-up frequency (parameter L5-02, L5-00 = 0), or the PID negative feedback value is less than the wake-up threshold (parameter L5-02, L5-00 = 1), after the wake-up delay (L5-04), the inverter starts to accelerate to the PID output frequency according to the set acceleration time.

| L5-03 | Sleep Delay | Range: 0.0s～6000.0s | Factory Value: 0.0 |
| --- | --- | --- | --- |
| L5-04 | Wake-up Delay | Range: 0.00s～600.00s | Factory Value: 0.00 |

2.26 L6 Group Counting

| L6-00 | Final Count Value Setting | Range: 0～65500 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used for counting functions, and can set the final count value (maximum value).

When L6-00 is 0, the counting function is disabled. When L6-00 is not 0, the counting function is active. The counter's input point can be triggered by the multifunction terminal DI6 (digital input function set to 23 for input counting), as shown in Figure 2-89, when the count reaches the L6-00 set value, the inverter can issue a final count reached signal through the digital output terminal.

Figure 2-89 Final Count Value SettingDiagram

| L6-01 | Intermediate Count Value Setting | Range: 0～65500 | Factory value: 0 |
| --- | --- | --- | --- |

This function code is used for counting functions, and can set the intermediate (set) count value. This function code must be used in conjunction with the final count value setting (L6-00).

AsshowninFig.2-90,whenthecountvalueincreasestothesetvalueofL6-01,theinvertercanissueasetcountreachedsignalviadigitaloutputterminalsandcontinuecountinguntilthefinalcountvalue.

Fig.2-90IntermediateCountValueSettingDiagram

| L6-02 | Count Reached E.F Enable | Range: 0～1 | Factory value: 0 |
| --- | --- | --- | --- |

Thisfunctioncodeisusedtosettheinverter'sactionwhenthecountisreached.

0:Countreached,noEF;

Whenthecountisreached,theinvertercontinuestooperate.

1:Countreached,EF;

Whenthecountisreached,theinverterstopsoperationandindicatesanexternalfault.

2.27L7GroupAIMulti-pointCurveSetting

| L7-00 | AI Curve Selection | Range: 0～7 | Factory value: 0 |
| --- | --- | --- | --- |

ThisfunctioncodeisusedtosettheAIcurveadjustmentmethod.

0: General Curve

Thesetvalueis0,allanaloginputsignalsarecalculatedusingbiasandgainmethods.

1: AI1 Three-point Curve

Set value to 1, AI1 uses the frequency and voltage/current correspondence method (parameters L7-01 ~ L7-06) for calculation, other analog input signals use the bias and gain method for calculation.

2: AI2 Three-point Curve

Set value to 2, AI2 uses the frequency and voltage/current correspondence method (parameters L7-07 ~ L7-12) for calculation, other analog input signals use the bias and gain method for calculation.

3: Three-point curve for AI1 and AI2

Set value to 3, AI1 and AI2 use the frequency and voltage/current correspondence method (parameters L7-01 ~ L7-12) for calculation, other analog input signals use the bias and gain method for calculation.

4: Three-point Curve for AI3

Set value to 4, AI3 uses the frequency and voltage/current correspondence method (parameters L7-13 ~ L7-18) for calculation, other analog input signals use the bias and gain method for calculation.

5: Three-point Curve for AI1 and AI3

Set value to 5, AI1 and AI3 use the frequency and voltage/current correspondence method (parameters L7-01 ~ L7-06, parameters L7-13 ~ L7-18) for calculation, other analog input signals use the bias and gain method for calculation.

6: Three-point Curve for AI2 and AI3

Set value to 6, AI2 and AI3 use the frequency and voltage/current correspondence method (parameters L7-07 ~ L7-18) for calculation, other analog input signals use the bias and gain method for calculation.

7: Three-point curve for AI1, AI2, and AI3

The set value is 7, and the frequency and voltage/current correspondence method (parameters L7-01 to L7-18) is used for AI1, AI2, and AI3.

| L7-01 | AI1 Minimum Point Input Value | Range: 0.00～20.00 | Factory Value: 0.00 |
| --- | --- | --- | --- |
| L7-02 | AI1 Minimum Point Percentage | Range: 0.00%～100.00% | Factory Value: 0.00 |
| L7-03 | AI1 Midpoint Input Value | Range: 0.00～20.00 | Factory Value: 5.00 |
| L7-04 | AI1 Midpoint Percentage | Range: 0.00～100.00% | Factory Value: 50.00 |
| L7-05 | AI1 Maximum Point Input Value | Range: 0.00～20.00 | Factory setting: 10.00 |
| L7-06 | AI1 Maximum Point Percentage | Range: 0.00～100.00% | Factory value: 100.00 |

The above function codes are used to adjust the AI1 input signal using a three-point curve, which is effective when the AI1 curve is selected as a three-point curve.

L7-01, L7-03, and L7-05 are the actual input voltage or current values. If the input signal type is 0-10V, entering 1.00 represents 1V; If the input type is 0-20mA or 4-20mA, entering 5.00 represents 5mA. Note that when setting, it must be ensured that L7-01 < L7-03 < L7-05. L7-02, L7-04, and L7-05 are the numerical values corresponding to the respective input values entered into the inverter, with units in %. 100% represents the maximum value; for example, when the analog input function is set to input frequency, 100% corresponds to the maximum frequency (F4-02). When the AI1 input type is 0-10V and used as a frequency command, the curve relationships corresponding to different settings of L7-01 to L7-06 are shown in Figure2-91.

Figure 2-91 AI1 Multi-point Curve

| L7-07 | AI2 Minimum Point Input Value | Range: 0.00～20.00 | Factory Value: 0.00 |
| --- | --- | --- | --- |
| L7-08 | AI2 Minimum Point Percentage | Range: 0.00～100.00% | Factory Value: 0.00 |
| L7-09 | AI2 Midpoint Input Value | Range: 0.00～20.00 | Factory Value: 5.00 |
| L7-10 | AI2 Midpoint Percentage | Range: 0.00～100.00% | Factory Value: 50.00 |
| L7-11 | AI2 Maximum Point Input Value | Range: 0.00～20.00 | Factory setting: 10.00 |
| L7-12 | AI2 Maximum Point Percentage | Range: 0.00～100.00% | Factory value: 100.00 |

The above function codes are used for adjusting the AI2 input signal using a three-point curve, which is effective when the AI2 curve is selected as a three-point curve. Refer to the relevant descriptions for L7-01 to L7-06.

| L7-13 | AI3 Minimum Point Input Value | Range: 0.00～20.00 | Factory Value: 0.00 |
| --- | --- | --- | --- |
| L7-14 | AI3 Minimum Point Percentage | Range: 0.00～100.00% | Factory Value: 0.00 |
| L7-15 | AI3 Midpoint Input Value | Range: 0.00～20.00 | Factory Value: 5.00 |
| L7-16 | AI3 Intermediate Point Percentage | Range: 0.00～100.00% | Factory Value: 50.00 |
| L7-17 | AI3 Maximum Point Input Value | Range: 0.00～20.00 | Factory setting: 10.00 |
| L7-18 | AI3 Maximum Point Percentage | Range: 0.00～100.00% | Factory value: 100.00 |

The above function codes are used for adjusting the AI3 input signal using a three-point curve, which is effective when the AI3 curve is selected as a three-point curve. Refer to the relevant descriptions for L7-01 to L7-06.

2.28 L8 Group Application Macro Parameters

By selecting application macro parameters, the system automatically imports function parameters related to the selected industry application, thus simplifying the parameter settings for users when choosing different on-site applications.

Supports custom application macros, allowing users to edit 50 function code indexes themselves, and stores the custom function code indexes and function code values even after power loss. Additionally, there are ten industry application macros: air compressor, fan, water pump, conveyor belt, CNC machine, packaging, textile machine, electric drill high-speed machine, process PID, process PID main-secondary frequency. When different industry applications are selected, the system automatically updates the relevant function parameters, eliminating the need for user settings.

| L8-00 | Industry Application Macro Selection | Range: 0～12 | Factory value: 0 |
| --- | --- | --- | --- |

0: No Function

1: Custom Application

2: Air Compressor Application

3: Fan Application

4: Pump Application

5: Conveyor Belt Application

6: CNC Machine Tool Application

7: Packaging Application

8: Textile Machine Application

9: High-Speed Electric Drill

10: Not Applicable

11: Process PID Application

12: Process PID Master-Slave Frequency Application

Note: After selecting an application macro, some parameter default values will be automatically set according to the selected application macro type.

L8-00=1, User-Defined Parameter Setting:

| L8-01~ L8-50 | Application Macro Parameters | Range: 0.00～29.00 | Factory Value: 0.00 |
| --- | --- | --- | --- |

The custom option allows users to add up to 50 function code parameter settings (L8-01~L8-50). For example, if a certain on-site application requires setting an asynchronous motor to operate in torque mode with a carrier frequency of 6KHz and a maximum torque of 150%, then the parameters should be set as follows: L8-01=0.03, L8-02=14.00, L8-03=0.15, L8-04=14.10; F0-03=2 (torque mode), FE-00=2 (IM open-loop torque control), F0-15=6 (carrier frequency), FE-10=150% (maximum torque command). After power-off, when powered on again, the parameters added by the custom application macro (L8-00=1) will be automatically imported.

Precautions:

■ The setting values of parameters L8-01~L8-50 cannot be the index values of hidden attribute function codes (function codes reserved for display, such as L8-01 cannot be set to 25.00, i.e., L3-00).

■ The setting values of parameters L8-01~L8-50 cannot exceed the total number of members in the set function group, for example, the display range of the F0 group is F0-00~F0-18, L8-01 cannot be set to 0.19 (i.e., F0-19).

■ The setting values for parameters L8-01 to L8-50 cannot be L8 group parameters, such as L8-01 cannot be set to 29.XX (i.e., L8-XX).

■ When the setting values for parameters L8-01 to L8-50 are not 0.00, user-defined index items increase; otherwise, they decrease. For example, if L8-01=1.00, L8-02=1.01 (i.e., F1-00, F1-01), and L8-03=0.00, two custom function parameters are defined.

■ When adding application parameters, it is mandatory to set them consecutively starting from L8-01 without skipping over 0.00 to set the next parameter, such as L8-01=1.00 (i.e., F1-00), L8-02=0.00, and L8-03 cannot be set to 1.01.

■ When reducing application parameters, they must be set to 0.00 sequentially from the last non-0.00 parameter (L8-XX~L8-01).

If the operation does not follow the above rules, the keyboard will display an “**ERR**” error, indicating that the setting has failed.

L8-00=2, air compressor application parameter settingsas shown in Table 2-26.

Table 2-26 Air Compressor Application Parameters

| Parameter | Parameter Name | Set Value |
| --- | --- | --- |
| F0-01 | Light/Heavy Load Selection | 1 (Heavy Load) |
| F0-03 | Control Mode | 0 (Speed Mode) |
| F0-04 | Speed Mode Selection | 0 (V/F) |
| F0-05 | Run Command Source Selection | 1 (External Terminal Input) |
| F0-06 | Frequency Source Selection | 2 (Analog Input) |
| F0-09 | Forward/Reverse Prohibited Selection | 1 (Reverse Prohibited) |
| F0-11 | Lower Limit Frequency | 20.00 (Hz) |
| F0-13 | Acceleration time 1 | 20.00 (s) |
| F0-14 | Deceleration time 1 | 20.00 (s) |
| F0-15 | Carrier frequency | 2 (kHz) |
| F1-12 | Stopping Method | 0 (Coast to Stop) |
| F5-21 | AI1 function selection | 0 (No Function) |
| F5-27 | AI2 Function Selection | 1 (Frequency Setting) |

L8-00=3, Fan application parameter settingsas shown in Table 2-27.

Table 2-27 Fan Application Parameters

| Parameter | Parameter Name | Set Value |
| --- | --- | --- |
| F0-01 | Light/Heavy Load Selection | 1 (Heavy Load) |
| F0-03 | Control Mode | 0 (Speed Mode) |
| F0-04 | Speed Mode Selection | 0 (V/F) |
| F0-05 | Run Command Source Selection | 1 (External Terminal Input) |
| F0-06 | Frequency Source Selection | 2 (Analog Input) |
| F0-09 | Forward/Reverse Prohibited Selection | 1 (Reverse Prohibited) |
| F0-10 | Upper Limit Frequency | 50.00 (Hz) |
| F0-11 | Lower Limit Frequency | 35.00 (Hz) |
| F0-13 | Acceleration time 1 | 15.00 (s) |
| F0-14 | Deceleration time 1 | 15.00 (s) |
| F0-15 | Carrier frequency | 2 (kHz) |
| F1-12 | Stopping Method | 1 (Free Stop) |
| F1-29 | Instant Power Failure Restart Method | 2 (Minimum Frequency Tracking) |
| F2-00 | V/F Voltage Selection | 2 (Square V/F Curve) |
| F5-04 | DI5 Terminal Function Selection | 16 (AI2 Input Frequency Command) |
| F5-20 | AI1 signal type selection | 0 (0-10V Input Selection) |
| F5-21 | AI1 function selection | 1 (Frequency Setting) |
| F5-26 | AI2 Signal Type Selection | 1 (0-20mA Input Selection) |
| F5-27 | AI2 Function Selection | 1 (Frequency Setting) |
| F6-02 | DO1 Terminal Function Selection | 11 (Fault Indication) |
| F6-03 | DO2 Terminal Function Selection | 1 (Inverter Running) |
| F6-20 | AO2 Signal Type Selection | 0 (0-10V Output Selection) |
| F9-46 | Abnormal Start Count | 5 |
| F9-47 | Abnormal Restart Reset Time | 60.0s |
| L1-01 | Operation Command Source (HAND) | 0 (Digital Operator) |
| L1-00 | Frequency Source Selection (HAND) | 0 (Digital Operator) |
| L7-00 | AI Curve Selection | 1 (AI1 three-point curve) |

L8-00=4, Pump application parameter settingsas shown in Table 2-28.

Table 2-28 Pump application parameters

| Parameter | Parameter Name | Set Value |
| --- | --- | --- |
| F0-01 | Light/Heavy Load Selection | 1 (Heavy Load) |
| F0-03 | Control Mode | 0 (Speed Mode) |
| F0-04 | Speed Mode Selection | 0 (V/F) |
| F0-05 | Run Command Source Selection | 1 (External Terminal Input) |
| F0-06 | Frequency Source Selection | 2 (Analog Input) |
| F0-09 | Forward/Reverse Prohibited Selection | 1 (Reverse Prohibited) |
| F0-10 | Upper Limit Frequency | 50.00 (Hz) |
| F0-11 | Lower Limit Frequency | 35.00 (Hz) |
| F0-13 | Acceleration time 1 | 15.00 (s) |
| F0-14 | Deceleration time 1 | 15.00 (s) |
| F1-29 | Instant Power Failure Restart Method | 2 (Minimum Frequency Tracking) |
| F2-00 | V/F Voltage Selection | 2 (Square V/F Curve) |
| F9-46 | Abnormal Start Count | 5 |
| F9-47 | Abnormal Restart Reset Time | 60.0s |

L8-00=5, Conveyor belt application parameter settingsas shown in Table 2-29.

Table 2-29 Conveyor belt application parameters

| Parameter | Parameter Name | Set Value |
| --- | --- | --- |
| F0-01 | Light/Heavy Load Selection | 1 (Heavy Load) |
| F0-03 | Control Mode | 0 (Speed Mode) |
| F0-04 | Speed Mode Selection | 0 (V/F) |
| F0-05 | Run Command Source Selection | 1 (External Terminal Input) |
| F0-06 | Frequency Source Selection | 2 (Analog Input) |
| F0-09 | Forward/Reverse Prohibited Selection | 1 (Reverse Prohibited) |
| F0-10 | Upper Limit Frequency | 50.00 (Hz) |
| F0-11 | Lower Limit Frequency | 35.00 (Hz) |
| F0-13 | Acceleration time 1 | 10.00 (s) |
| F0-14 | Deceleration time 1 | 10.00 (s) |

L8-00=6, Machine tool application parameter settingsas shown in Table 2-30.

Table 2-30 Machine tool application parameters

| Parameter | Parameter Name | Set Value |
| --- | --- | --- |
| F0-01 | Light/Heavy Load Selection | 1 (Heavy Load) |
| F0-03 | Control Mode | 0 (Speed Mode) |
| F0-04 | Speed Mode Selection | 0 (V/F) |
| F0-05 | Run Command Source Selection | 1 (External Terminal Input) |
| F0-06 | Frequency Source Selection | 2 (Analog Input) |
| F0-13 | Acceleration time 1 | 5.00 (s) |
| F0-14 | Deceleration time 1 | 5.00 (s) |
| F0-15 | Carrier frequency | 2 (kHz) |
| F1-08 | Braking Current Level | 20% |
| F1-24 | Stopping brake time | 0.3s |
| F1-25 | Braking Start Frequency | 0.00Hz |
| F5-02 | DI3 Terminal Function Selection | 1 (Multi-speed/multi-position 1) |
| F5-03 | DI4 Terminal Function Selection | 2 (Multi-speed/multi-position 2) |
| F5-21 | AI1 function selection | 1 (Frequency Setting) |
| F5-27 | AI2 Function Selection | 1 (Frequency Setting) |
| F6-00 | RLY1 terminal function selection | 11 (Fault Indication) |
| F6-02 | DO1 Terminal Function Selection | 1 (Inverter Running) |
| F6-03 | DO2 Terminal Function Selection | 2 (Reach set frequency) |
| F7-37 | Automatic Voltage Regulation | 1 (Cancel AVR function) |

L8-00=7, Packaging application parameter settingsas shown in Table 2-31.

Table 2-31 Packaging Application Parameters

| Parameter | Parameter Name | Set Value |
| --- | --- | --- |
| F0-01 | Light/Heavy Load Selection | 1 (Heavy Load) |
| F0-03 | Control Mode | 0 (Speed Mode) |
| F0-04 | Speed Mode Selection | 0 (V/F) |
| F0-05 | Run Command Source Selection | 2 (RS485 Communication Input) |
| F0-06 | Frequency Source Selection | 0 (Digital Operator) |
| F0-13 | Acceleration time 1 | 10.00 (s) |
| F0-14 | Deceleration time 1 | 10.00 (s) |
| F1-16 | S Acceleration Time 1 | 0.20s |
| F1-17 | S Acceleration Time 2 | 0.20s |
| F1-18 | S Deceleration Time 1 | 0.20s |
| F1-19 | S Deceleration Time 2 | 0.20s |
| F5-08 | Terminal Command Mode | 1 (2-Wire Mode 1) |
| F5-20 | AI1 signal type selection | 0 (0-10V Input Selection) |
| F5-21 | AI1 function selection | 1 (Frequency Setting) |

L8-00=8, Textile Application Parameter SettingsAs Shown in Table 2-32.

Table 2-32 Textile Application Parameters

| Parameter | Parameter Name | Set Value |
| --- | --- | --- |
| F0-01 | Light/Heavy Load Selection | 1 (Heavy Load) |
| F0-03 | Control Mode | 0 (Speed Mode) |
| F0-04 | Speed Mode Selection | 0 (V/F) |
| F0-05 | Run Command Source Selection | 1 (External Terminal Input) |
| F0-06 | Frequency Source Selection | 1 (RS485 Communication) |
| F0-13 | Acceleration time 1 | 10.00 (s) |
| F0-14 | Deceleration time 1 | 10.00 (s) |
| F1-16 | S Acceleration Time 1 | 0.20s |
| F1-17 | S Acceleration Time 2 | 0.20s |
| F1-18 | S Deceleration Time 1 | 0.20s |
| F1-19 | S Deceleration Time 2 | 0.20s |
| F9-08 | Acceleration OC Stall Threshold | 180% |
| F9-10 | Operation OC Stall Threshold | 180% |
| F9-37 | Over-torque threshold 1 | 200% |
| F7-17 | Fan control method | 2 (Stop with Inverter Shutdown) |

L8-00=9, High-Speed Drill Application Parameter SettingsAs Shown in Table 2-33.

Table 2-33 High-Speed Drill Application Parameters

| Parameter | Parameter Name | Set Value |
| --- | --- | --- |
| F0-01 | Light/Heavy Load Selection | 1 (Heavy Load) |
| F0-03 | Control Mode | 0 (Speed Mode) |
| F0-04 | Speed Mode Selection | 0 (V/F) |
| F0-05 | Run Command Source Selection | 1 (External Terminal Input) |
| F0-06 | Frequency Source Selection | 2 (Analog Input) |
| F0-09 | Forward/Reverse Prohibited Selection | 1 (Reverse Prohibited) |
| F0-13 | Acceleration time 1 | 4.00 (s) |
| F0-14 | Deceleration time 1 | 3.00 (s) |
| F0-15 | Carrier frequency | 2 (kHz) |
| F1-08 | Braking Current Level | 20% |
| F1-24 | Stopping brake time | 0.3s |
| F1-25 | Braking Start Frequency | 0.00Hz |
| F5-21 | AI1 function selection | 1 (Frequency Setting) |
| F6-00 | RLY1 terminal function selection | 11 (Fault Indication) |
| F6-02 | DO1 Terminal Function Selection | 1 (Inverter Running) |
| F6-03 | DO2 Terminal Function Selection | 2 (Reach set frequency) |
| F7-37 | Automatic Voltage Regulation | 1 (Cancel AVR function) |
| F9-15 | Output Phase Loss Action Selection | 1 (Warning and Decelerate to Stop) |
| F9-36 | Over Torque Selection 1 | 2 (Constant Speed Detection Stop Operation) |
| F9-37 | Over-torque threshold 1 | 120% |
| F9-38 | Over-torque time 1 | 0.1s |

L8-00=11, Process PID Application Parameter SettingsAs Shown in Table 2-34.

Table 2-34 Process PID Application Parameters

| Parameter | Parameter Name | Set Value |
| --- | --- | --- |
| F0-03 | Control Mode | 0 (Speed Mode) |
| F0-04 | Speed Mode Selection | 0 (V/F) |
| F0-05 | Run Command Source Selection | 1 (External Terminal Input) |
| F0-06 | Frequency Source Selection | 9 (PID) |
| F0-13 | Acceleration time 1 | 3.00 (s) |
| F0-14 | Deceleration time 1 | 3.00 (s) |
| F5-21 | AI1 function selection | 5 (PID Feedback Value) |
| FA-00 | PID Feedback Type Selection | 1 (Negative Feedback Analog Input) |
| FA-01 | PID Setpoint Source Selection | 1 (Parameter FA-02) |
| FA-02 | PID Setpoint | 50.00% |
| FA-05 | Proportional Coefficient 1 | 10.00 |
| FA-06 | Integral Time 1 | 1.00 |
| FA-11 | PID Series/Parallel Selection | 1 (Parallel) |
| FA-17 | PID Direction Change Selection | 0 (Prohibited) |
| L7-00 | AI Curve Selection | 1 (AI1 Three-point Curve) |
| L7-13 | AI3 Minimum Point Input Value | 0.00 |
| L7-15 | AI3 Midpoint Input Value | 9.99 |
| L7-16 | AI3 Intermediate Point Percentage | 100.00% |

L8-00=12, Process PID + Main Auxiliary Frequency Application Related Parameter SettingsAs Shown in Table 2-35.

Table 2-35 PID + Main Frequency Application Parameters

| Parameter | Parameter Name | Set Value |
| --- | --- | --- |
| F0-03 | Control Mode | 0 (Speed Mode) |
| F0-04 | Speed Mode Selection | 0 (V/F) |
| F0-05 | Run Command Source Selection | 1 (External Terminal Input) |
| F0-06 | Frequency Source Selection | 9 (PID) |
| F0-07 | Auxiliary Frequency Source | 3 (Analog Input) |
| F0-13 | Acceleration time 1 | 3.00 (s) |
| F0-14 | Deceleration time 1 | 3.00 (s) |
| F5-21 | AI1 function selection | 5 (PID Feedback Value) |
| F5-26 | AI2 Signal Type Selection | 1 (0-20mA Input Selection) |
| F5-27 | AI2 Function Selection | 12 (Auxiliary Frequency Setting) |
| F5-38 | Analog Bias Negative Frequency | 0 (Forward/Reverse by Operator) |
| F5-30 | AI2 Gain | 100.0% |
| FA-00 | PID Feedback Type Selection | 1 (Negative Feedback Analog Input) |
| FA-01 | PID Setpoint Source Selection | 1 (Parameter FA-02) |
| FA-02 | PID Setpoint | 50.00% |
| FA-05 | Proportional Coefficient 1 | 10.00 |
| FA-06 | Integral Time 1 | 1.00 |
| FA-11 | PID Series/Parallel Selection | 1 (Parallel) |
| FA-17 | PID Direction Change Selection | 1 (Enabled) |
| FA-27 | Main Auxiliary Reverse Cut-off Frequency | 10.0% |
| L7-00 | AI Curve Selection | 1 (AI1 Three-point Curve) |
| L7-13 | AI3 Minimum Point Input Value | 0.00 |
| L7-15 | AI3 Midpoint Input Value | 9.99 |
| L7-16 | AI3 Intermediate Point Percentage | 100.00% |

3 Basic Configuration and Function Application

3.1 VFD Basic Application

3.1.1 Operation Command Setting

The operation command is used to control the inverter's start, stop, forward/reverse rotation, jog operation, etc. The operation commands are divided into three types: keyboard, terminal, and communication. Parameter F0-05 is used to select the operation command method.

Table 3-1 Operation Command Setting Related Parameters

| Function Code | Function Code Definition | Value Range | Factory Value | Description |
| --- | --- | --- | --- | --- |
| F0-05 | Run Command Source Selection | 0: Digital Operator 1: External Terminal Input 2: RS485 Communication Input 3: CANopen Input 4: Reserved 5: Communication Card Input | 0 | Selects the input channel for the inverter control command. Inverter control commands include: start, stop, forward rotation, reverse rotation, jog, etc. 0: Digital Operator Selecting this command channel allows control commands to be input via the RUN, STOP, JOG buttons on the keyboard, suitable for initial debugging. 1: External terminal input Select this command channel to control commands via the inverter's DI terminals. The DI terminal control commands can be set according to different scenarios, such as start/stop, forward/reverse, jogging, two/three-wire mode, multi-speed, etc., suitable for most applications. 2, 3, 5: Communication command channels Communication methods include RS485, CANopen, communication cards. Select this command channel to input control commands via remote communication. The inverter needs to have a communication card installed to achieve communication with the higher-level machine. Suitable for long-distance control or centralized control of multiple equipment systems. |

Set operation instructions via the 'keyboard'

When F0-05 is set to 0, use the keyboard buttons “RUN”, “STOP”, “JOG”, etc., to give the inverter operation commands.

Pressing the “RUN” button starts the inverter, and you can see the indicator light above the “RUN” button light up.

Pressing the “STOP” button while the inverter is running will stop the inverter. Before stopping, the “RUN” button indicator light will flash until it stops, at which point the indicator light above the “STOP” button will light up.

Setting operation commands through external terminals

When F0-05 is set to 1, operation commands are given to the inverter through DI terminals to control its start and stop.

Parameter F5-08 sets the terminal command method, with a total of 6 methods, as shown inTable 3-2below:

Table 3-2 Terminal Command Mode Setting Parameters

| Function Code | Function Code Definition | Value Range | Factory Value | Explanation |
| --- | --- | --- | --- | --- |
| F5-08 | Run Command Source Selection | 0: No Function 1: 2-Wire Mode 1 2: 2-wire Mode 2 3: 3-wire 4: 2-wire Mode 1/Fast Start 5: 2-wire Mode 2/Fast Start 6: 3-wire Fast Start | 1 | Select different terminal combinations to set the inverter operation mode. |

2-Wire Mode 1

F5-08 = 1, set to commonly used 2-wire mode 1.

The DI1 terminal is used for forward operation, and the DI2 terminal is used for reverse operation. Connect the forward operation switch to the DI1 terminal and the reverse operation switch to the DI2 terminal.

Table 3-3 2-Wire Mode 1 Related Parameters

| Function Code | Function Code Definition | Set Value | Explanation |
| --- | --- | --- | --- |
| F5-08 | Terminal Command Mode | 1 | 2-Wire Mode 1 |
| F5-00 | DI1 Terminal Function Selection | 0 | When F5-08 is changed to 1, the DI1 terminal defaults to forward operation. |
| F5-01 | DI2 Terminal Function Selection | 0 | When F5-08 is changed to 1, the DI2 terminal defaults to reverse operation. |
| F0-05 | Run Command Source Selection | 1 | External Terminal Input |

Table 3-4 2-Wire Mode 1

| SW1 | SW2 | Run Command |
| --- | --- | --- |
| 1 | 0 | Forward Operation |
| 0 | 1 | Reverse Operation |
| 1 | 1 | Stop |
| 0 | 0 | Stop |

Figure 3-1 Two-Wire Operation Mode 1

Two-Wire Mode 2

F5-08 = 2, set to Two-Wire Mode 2.

The DI1 terminal is used to control the run command, and the DI2 terminal is used to control the forward/reverse direction.

Table 3-5 Parameters Related to Two-Wire Mode 2

| Function Code | Function Code Definition | Set Value | Description |
| --- | --- | --- | --- |
| F5-08 | Terminal Command Mode | 2 | Two-Wire Mode 2 |
| F5-00 | DI1 Terminal Function Selection | 0 | Default for controlling the run command |
| F5-01 | DI2 Terminal Function Selection | 0 | Default for controlling the forward/reverse operation direction |
| F0-05 | Run Command Source Selection | 1 | External Terminal Input |

Table 3-6 Two-Wire Mode 2

| SW1 | SW2 | Run Command |
| --- | --- | --- |
| 1 | 0 | Forward Operation |
| 1 | 1 | Reverse Operation |
| 0 | 1 | Stop |
| 0 | 0 | Stop |

Figure 3-2 Two-Wire Operation Mode 2

Three-Wire

F5-08 = 3, set to Three-Wire mode.

The DI1 terminal is used to control the run command, the DI2 terminal is used to control the forward/reverse direction, and the DI3 terminal is used to control the run enable.

Table 3-7 Three-wire Related Parameters

| Function Code | Function Code Definition | Set Value | Description |
| --- | --- | --- | --- |
| F5-08 | Terminal Command Mode | 3 | Three-Wire |
| F5-00 | DI1 Terminal Function Selection | 0 | Default for controlling the run command |
| F5-01 | DI2 Terminal Function Selection | 0 | Default for controlling the forward/reverse operation direction |
| F5-02 | DI3 Terminal Function Selection | 0 | Default is the Run Enable Terminal |
| F0-05 | Run Command Source Selection | 1 | External Terminal Input |

Table 3-8 Three-wire

| SW1 | SW2 | SW3 | Run Command |
| --- | --- | --- | --- |
| 1 | 0 | 1 | Forward Operation |
| 1 | 1 | 1 | Reverse Operation |
| x | x | 0 | Stop |
| 0 | x | x | Stop |

Figure 3-3 Three-wire Operation Mode

Set the operation command via communication

When F0-05 is set to 2, 3, or 5, the inverter's start and stop commands are given through communication.

This series supports multiple communication methods: 485, CANopen, PROFIBUS-DP, EtherCAT, Profinet, and Ethernet. Except for 485, other communications require the installation of a communication card. According to the communication protocol, relevant parameters need to be set in the F8 group to match the corresponding serial port protocol.

The following uses 485 communication as an example to explain the process of setting the operation command via communication.

Figure 3-4 Schematic Diagram of Communication Given Operation Command

Application Example, Communication Control of Inverter Forward Operation. Send write command data “01 06 20 00 00 12 02 07”. The data is in hexadecimal, and the meaning of each data bit is shown in the table below. For more communication addresses and commands, see Appendix B.

Table 3-9 Meaning of Instruction Data Bits

| Data | Meaning |
| --- | --- |
| 01 | Inverter Communication Address |
| 06 | Write Command |
| 20 00 | Control Command Communication Address |
| 00 12 | Forward Operation Command |
| 02 07 | CRC Check Low and High Bits |

Complete Data Transmission Process:

Host Sends: 01 06 20 00 00 12 02 07

Slave Responds: 01 06 20 00 00 12 02 07

3.1.2 Frequency Command Setting

The inverter is equipped with 2 frequency setting channels, named as the main frequency source and auxiliary frequency source, which can operate independently or switch at any time, and even be combined through set calculation methods to meet different control requirements on site.

Main Frequency Source Selection

The inverter has 10 main frequency sources, including digital operator, RS485 communication, analog input, external Up/Down, CANopen, PID, multi-speed, etc., one of which can be selected through F0-06.

Figure 3-5 Schematic Diagram of Main Frequency Source Selection

From the different frequency sources shown in the figure, it can be seen that the operating frequency of the inverter can be determined by function codes, adjusted manually in real-time, set by an analog signal, given by multi-speed terminal commands, regulated in a closed loop by the built-in PID controller using external feedback signals, or controlled via communication from a higher-level machine.

The figure above provides the function codes related to each frequency source setting; refer to the detailed description of the corresponding function code when setting.

Usage method with auxiliary frequency setting

There are 7 sources for the auxiliary frequency, which are selected through F0-07 setting.

Figure 3-6 Schematic diagram of auxiliary frequency setting sources

In actual use, the relationship between the target frequency and the main and auxiliary frequency sources is set through F0-10.

3.1.3 Start/Stop Methods

Start Method

The inverter has three start modes: direct start, speed tracking start, and asynchronous motor pre-magnetization start, which are selected through function parameter F1-00.

Direct Start

F1-00=0, direct start mode, suitable for most small inertia loads, the frequency curve during the start process is shown in the following figure. The 'DC Braking' function before startup is suitable for driving elevators, hoisting loads, etc. The 'Startup Frequency' is suitable for equipment that requires torque impact at startup, such as concrete mixers.

Figure 3-7 Direct Start with DC Braking Schematic

Figure 3-8 Direct Start without DC Braking Schematic

Speed Tracking Start

F1-00≠0, for speed tracking start, speed tracking is applicable to punch presses, fans, and other high-inertia load applications.

First, look at the startup curve process when F1-00=0:

Figure 3-9 Startup Curve When Speed Search Function is Disabled

After giving the stop command, the inverter decelerates to a stop. If the frequency has not reduced to zero and a run command is given again, the frequency accelerates from zero to the set frequency.

When F1-00=1, it is for tracking start from the maximum frequency, and the frequency curve during the start-up process is as shown in the following figure:

Figure 3-10 Tracking Start from Maximum Frequency

After giving the stop command, the inverter decelerates to a stop. If the frequency has not reduced to zero and a run command is given again, the frequency directly decreases from the rated frequency to the set frequency.

When F1-00=2, it is for tracking start from the current frequency, and the frequency curve during the start-up process is as shown in the following figure:

Figure 3-11 Start-up with Tracking from Current Frequency

After a stop command is given, the inverter decelerates to stop. If the frequency has not reduced to zero and a run command is given again, the frequency will directly reach the set frequency. When F1-00=3, it indicates start-up with tracking from the minimum frequency, and the frequency curve during the start-up process is as shown in the following figure:

Figure 3-12 Start-up with Tracking from Minimum Frequency

This start-up process is consistent with when F1-00=0.

Stopping Methods

The inverter has two stopping modes: deceleration stop and free stop, which are selected by function code F1-12.

Deceleration Stop

F1-12=0 indicates deceleration stop. After a stop command becomes effective, the inverter reduces the output frequency according to the deceleration time, and stops after the frequency drops to 0.

The frequency curve is as follows:

Figure 3-13 Deceleration stop output frequency curve

Free stop

After the stop command becomes effective, the inverter immediately terminates the output, at which point the motor stops freely according to mechanical inertia.

The frequency curve is as follows:

Figure 3-14 Free stop output frequency curve

Acceleration and deceleration time settings

Acceleration time refers to the time required for the inverter to accelerate from zero frequency to F4-03 (rated frequency) base frequency. Deceleration time refers to the time required for the inverter to decelerate from F4-03 (rated frequency) base frequency to zero frequency.

Figure 3-15  Acceleration and deceleration time diagram

The CM680 provides four sets of acceleration and deceleration times, and users can switch and select using digital input terminals DI (terminal input functions 8, 9). The four sets of acceleration and deceleration times are set through the following function codes:

First Group: F0-13 F0-14  Second Group: F7-03 F7-04

Third Group: F7-05 F7-06  Fourth Group: F7-07 F7-08

Application Example:

Taking DI3 and DI4 as input switching terminals as an example, the following are the setup steps.

1. Set parameters F5-02 and F5-03 to select DI3 and DI4 terminals as input switching terminals.

Table 3-10 Input Terminal Control for Acceleration/Deceleration Switching

| Function Code | Function Code Definition | Set Value | Description |
| --- | --- | --- | --- |
| F5-02 | DI3 Terminal Function Selection | 8 | 1-2 Segment Acceleration/Deceleration Switching |
| F5-03 | DI4 Terminal Function Selection | 9 | 3-4 Segment Acceleration/Deceleration Switching |

2. By setting four groups of acceleration/deceleration parameters, set the corresponding acceleration/deceleration times.

Table 3-11 Input Terminal Settings for Acceleration/Deceleration Times

| DI4 Terminal Status | DI3 Terminal Status | Acceleration and deceleration time settings |
| --- | --- | --- |
| OFF | OFF | First Group Acceleration/Deceleration Time F0-13, F0-14 |
| OFF | ON | Second Group Acceleration/Deceleration Time F7-03, F7-04 |
| ON | OFF | Third Group Acceleration/Deceleration Time F7-05, F7-06 |
| ON | ON | Fourth Group Acceleration/Deceleration Time F7-07, F7-08 |

3. When setting F0-12 (Speed Curve Time Unit), the decimal point position of the four groups of acceleration/deceleration times will change, and the corresponding acceleration/deceleration times will also change. This should be noted during actual use.

4. Set F1-20 (Automatic Acceleration/Deceleration Selection) to choose the inverter's acceleration/deceleration method.

0: Linear Acceleration and Deceleration

1: Automatic Acceleration, Linear Deceleration

2: Linear Acceleration, Automatic Deceleration

3: Automatic Acceleration and Deceleration

4: Automatic Acceleration, Deceleration Suppression

3.2 MotorDebugging Methods

3.2.1 Asynchronous Motor VF and VVC Control Debugging Methods

Both asynchronous motor VF control and VVC control can be used to drive asynchronous motors, and their debugging methods are almost identical; this section will introduce them together. The list of parameters related to asynchronous motor VF and VVC debugging is shown in Table3-12.

Table 3-12 List of Parameters Related to Asynchronous Motor VF and VVC Debugging

| Parameter Number | Parameter Name |
| --- | --- |
| F4-03 | M1 Rated Frequency |
| F4-04 | M1 Rated Voltage |
| F2-04 | M1 Multi-point VF Frequency Point 1 |
| F2-05 | Multi-point VF Voltage Point 1 |
| F2-06 | Multi-point VF Frequency Point 2 |
| F2-07 | Multi-point VF Voltage Point 2 |
| F2-08 | Multi-point VF Frequency Point 3 |
| F2-09 | Multi-point VF Voltage Point 3 |
| F2-10 | Slip Filter Time |
| F2-11 | Slip Compensation Gain |
| F2-14 | Vibration Compensation Gain |
| F3-29 | Torque Filter Time |
| F7-37 | Automatic Voltage Regulation |

3.2.1.1 Basic Settings

The basic settings for asynchronous motor VF and VVC debugging mainly include the setting of motor nameplate parameters, which generally do not require motor parameter identification. The setup process is shown in Figure3-16.

Fig.3-16 Basic Settings for Asynchronous Motor VF and VVC Tuning

The basic settings for VF and VVC are as follows:

(1) Parameter reset; Set parameter F0-18 to 9 to restore the inverter parameters to factory settings.

(2) Set motor nameplate parameters; set the parameters as shown in Table3-13:

Table3-13 Motor Nameplate Parameters

| Parameter Number | Parameter Name |
| --- | --- |
| F4-00 | IM/PM Motor Selection |
| F4-03 | IM1 Rated Frequency |
| F4-04 | IM1 Rated Voltage |
| F4-05 | IM1 Rated Power |
| F4-06 | IM1 Number of Poles |
| F4-07 | IM1 Rated Current |
| F4-08 | IM1 Rated Speed |

(3) Perform motor parameter identification (optional, generally not required); Set parameter F4-01 to 6 (IM rotational identification) or 2 (IM static identification), press the run button to start the identification, and after calibration, the motor parameters shown in Table3-14 will be automatically updated.

Table3-14 Identified Motor Parameters

| Parameter Number | Parameter Name |
| --- | --- |
| F4-10 | IM1 No-load Current |
| F4-11 | IM1 Stator Resistance |
| F4-12 | IM1 Rotor Resistance |
| F4-13 | IM1 Mutual Inductance |
| F4-14 | IM1 Leakage Inductance |

(4) Set the control mode to VF or VVC; Set F0-03 to 0 for speed mode, and set F0-04 to 0 to configure the control mode as VF control; if F0-04 is set to 2, it will be asynchronous motor VVC.

3.2.1.2 No-load Testing

After basic settings are completed, no-load testing can be performed. Operate the motor at different frequencies; if the motor vibrates, increase F2-14 'Oscillation Suppression Gain'.

3.2.1.3 Full-load Testing

After completing no-load testing, load testing can be conducted. If the motor's load capacity is insufficient, try increasing F2-01 'Torque Compensation Gain'. In the case of full load operation, if the actual motor speed is lower than the set speed, you can try increasing F2-11 “Slip Compensation Gain”. Conversely, if the actual motor speed is higher than the set speed, you can try decreasing F2-11 “Slip Compensation Gain”.

3.2.2 Permanent Magnet Motor VVC Commissioning Method

PMVVC is a sensorless vector control method for permanent magnet synchronous motors, and the parameters required for commissioning are shown in Table3-15:

Table3-15 List of Parameters Related to Permanent Magnet Motor VVC Commissioning

| Parameter Number | Parameter Name |
| --- | --- |
| F1-01 | Initial Position Identification Method |
| F1-36 | PM Voltage Pulse Width |
| F1-37 | PM High-Frequency Injection Frequency |
| F1-38 | PM High-Frequency Injection Amplitude |
| F2-01 | Torque Compensation Gain |
| F3-23 | I/F Mode Current Command |
| F3-24 | IF Switching Frequency 1 |
| F3-25 | IF Switching Frequency 2 |
| F3-29 | Torque Filter Time |
| F3-41 | VVC High-Pass Frequency Compensation |
| F3-42 | VVC Frequency Compensation Gain |

3.2.2.1 Basic Settings

PMVVC basic setup includes motor nameplate parameter settings and motor parameter identification; the setup process is shown in Figure3-17.

The basic setup steps for PMVVC are as follows:

(1) Parameter reset; Set parameter F0-18 to 9 to restore the inverter parameters to factory settings.

(2) Set the motor nameplate parameters; set the parameters as shown in Table3-16:

Table3-16 Motor Nameplate Parameters

| Parameter Number | Parameter Name |
| --- | --- |
| F4-00 | Motor Type Selection |
| F4-03 | M1 Rated Frequency |
| F4-04 | M1 Rated Voltage |
| F4-15 | PM rated power |
| F4-16 | PM pole number |
| F4-17 | PM rated current |
| F4-18 | PM rated speed |

(3)Perform motor parameter identification; Set parameter F4-01 to 5 (PM rotational identification) or 13 (PM static identification), press the run button to start the identification, and after the adjustment ends, the motor parameters shown in Table3-17 will be automatically updated.

Fig.3-17 Basic Settings for VVC Tuning of Permanent Magnet Motor

Table3-17 Identified Motor Parameters

| Parameter Number | Parameter Name |
| --- | --- |
| F4-20 | PM Stator Resistance |
| F4-21 | PM D-axis Inductance |
| F4-22 | PM Q-axis Inductance |
| F4-23 | PM Ke Parameter |

In Table3-17, the Ke parameter refers to the effective value of the back EMF at 1000 RPM. The actual Ke value can be obtained during rotational identification, while static identification will estimate Ke based on motor parameters.

If an error occurs during identification or the identification results are not within a reasonable range, re-identification is required. The following table lists possible identification errors. When an error occurs, refer to Table3-18 for troubleshooting.

Table3-18 Identification-related Faults

| Fault Code | Fault Description |
| --- | --- |
| E040 Motor Parameter Identification Error | Motor Parameter Identification Error |
| E142 No Feedback Current During Motor Parameter Identification | No Feedback Current During Identification |
| E143 Motor Phase Missing During Parameter Identification | Phase Missing During Identification |

（4）Set the Control Method to PMVVC；Set F0-03 to 0 for Speed Mode, and set F0-04 to 2 to configure the control method as PMVVC.

（5）Initial Angle Identification Method Selection；Select the initial angle identification method via parameter F1-01 as shown in Table3-19.

Table3-19 Initial Position Identification Methods

| F1-01 | Description |
| --- | --- |
| 0 | Do Not Perform Initial Angle Identification |
| 1 | Positive Suction Method |
| 3 | Pulse Injection Method 1 |
| 4 | Pulse Injection Method 2 |

If reverse start is not allowed, options 3 or 4 should be prioritized; if reverse start is allowed or if methods 3 and 4 are ineffective, the Positive Suction Method can be attempted.

3.2.2.2 No-load Testing

After basic settings are completed, no-load testing can be conducted. The no-load testing procedure for a permanent magnet motor VVC is shown in Figure3-18.

The no-load testing steps for a permanent magnet motor VVC are as follows:

(1) Start the motor and run it to 1/2 rated speed. If overcurrent occurs during startup, handle it according to the F1-01 setting: if F1-01=1, the current loop bandwidth can be reduced; If F1-01=3 or 4, you can reduce F1-36 “PM voltage pulse width”. When reverse operation is not allowed, if the motor reverses, you can increase F1-36 “PM voltage pulse width”. If the no-load current is too high (exceeding 60% of the rated current), you can increase “F2-01 torque compensation gain”.

（2）Control the motor to run at the rated speed

If the motor exhibits periodic low-frequency oscillation, you can try increasing F3-42 “VVC frequency compensation gain”, or increasing F3-41 “VVC frequency compensation high-pass”. If the motor experiences severe vibration, try reducing F3-42 “VVC frequency compensation gain”, or reducing F3-41 “VVC frequency compensation high-pass”.

(3) Control the motor to run at the maximum speed

If loss of control occurs during acceleration, try increasing F3-42 “VVC frequency compensation gain”, or increase the carrier frequency (parameter F0-15, the carrier frequency generally needs to be more than 10 times the highest output frequency).

Fig.3-18 Permanent Magnet Motor VVC No-load Debugging Flowchart

3.2.2.3 Full Load Debugging

After the no-load debugging is completed, load testing can be conducted. The permanent magnet motor VVC load debugging process is shown in Fig.3-19.

Fig.3-19 Permanent Magnet Motor VVC Load Debugging Flowchart

By default, I/F control is used at 1/10 rated speed. If I/F control is not required, set F3-25 to zero.

The full-load commissioning steps are as follows:

(1) Low-speed load capacity adjustment; Run the motor to 1/10 of the rated speed. If the motor's load-carrying capacity is insufficient, increase F3-23 “I/F mode current command”. If the low-frequency current is too high, reduce F3-23 “I/F mode current command”.

(2) Load testing during acceleration; Accelerate from 1/10 of the rated speed to the rated speed. If instability occurs, try increasing F3-30 “VVC voltage feedforward gain”.

(3) Rated speed load testing; If the motor exhibits periodic low-frequency oscillation, you can try increasing F3-41 “VVC frequency compensation high-pass” or F3-42 “VVC frequency compensation gain”; If the motor experiences severe vibration, try reducing F3-41 “VVC frequency compensation high-pass” or F3-42 “VVC frequency compensation gain”.

3.2.3 Asynchronous Motor and Permanent Magnet Motor SVC Commissioning Methods

3.2.3.1 SVC Introduction

SVC is a sensorless vector control method that can perform speed or torque control on asynchronous motors, synchronous motors, and synchronous reluctance motors. The SVC commissioning parameters are shown in Table3-20.

Table3-20 SVC Commissioning Parameters Table

| Parameter Number | Parameter Name |
| --- | --- |
| F3-00 | Speed loop PI parameter auto-calculation function enable bit (bit 0 of F3-00, set to 1 to enable) |
| F3-01 | ASR1/2 Switching Frequency |
| F3-02 | Zero Speed Bandwidth |
| F3-03 | ASR1 Low-Speed Bandwidth |
| F3-04 | ASR2 High-Speed Bandwidth |
| F3-05 | ASR Zero Speed Gain |
| F3-06 | ASR Zero Speed Integral |
| F3-07 | ASR1 Low-Speed Gain |
| F3-08 | ASR1 Low-Speed Integral Time |
| F3-09 | ASR2 High-Speed Gain |
| F3-10 | ASR2 High-Speed Integral Time |
| F3-14 | ASR Output Filter Time |
| F3-23 | I/F Mode Current Command |
| F3-26 | Weak Magnetism Proportion Coefficient |
| F3-27 | Weak Magnetism Integral Coefficient |
| F4-09 | Inertia Per Unit Value |
| L2-09 | Dead Time Compensation 4 Slope |

3.2.3.2 Basic Settings

The basic settings for induction motor SVC include: setting the motor nameplate parameters, motor parameter identification, etc., as shown in Figure3-20.

Figure3-20 Induction Motor SVC Basic Settings Flowchart

(1) Parameter Reset

Set parameter F0-18 to 9 to restore the inverter parameters to factory settings.

(2) Select Motor Type

Set parameter F4-00 = 0 for an induction motor.

Set the motor parameters according to the actual motor nameplate parameters as shown in Table3-21:

Table3-21 Motor Nameplate Parameters

| Parameter Number | Parameter Name |
| --- | --- |
| F4-03 | IM1 Rated Frequency |
| F4-04 | IM1 Rated Voltage |
| F4-05 | IM1 Rated Power |
| F4-06 | IM1 Number of Poles |
| F4-07 | IM1 Rated Current |
| F4-08 | IM1 Rated Speed |

(4) Perform Motor Parameter Identification

Confirm that the motor is disconnected from the load. In the disconnected state, it is recommended to set F4-01 = 6 for rotating parameter identification; If disconnection is not possible, set F4-01 = 2 for static parameter identification. Press the “RUN” button to start the identification process. After the motor parameter identification is completed, the motor parameters shown in table3-22will be automatically updated.

Table3-22 Motor Parameters After Identification

| Parameter Number | Parameter Name |
| --- | --- |
| F4-10 | IM1 No-load Current |
| F4-11 | IM1 Stator Resistance |
| F4-12 | IM1 Rotor Resistance |
| F4-13 | IM1 Mutual Inductance |
| F4-14 | IM1 Leakage Inductance |

The above parameters refer to the values per phase. If an error occurs during identification or the identification results are not within a reasonable range, re-identification is required.

Basic settings for permanent magnet motor SVC include motor nameplate parameter settings and motor parameter identification, as shown in Figure3-21.

Figure3-21 Basic Setting Flowchart for Permanent Magnet Motor SVC

(1) Parameter Reset

Set parameter F0-18 to 9 to restore the inverter parameters to factory settings.

(2) Select Motor Type

When setting parameter F4-00 = 1, it is for surface-mounted synchronous motors, corresponding to Ld = Lq; When F4-00 = 2, it is for interior permanent magnet synchronous motors, corresponding to Ld ≠ Lq.

(3) Set the motor nameplate parameters according to Table3-23:

Table3-23 Motor Nameplate Parameters

| Parameter Number | Parameter Name |
| --- | --- |
| F4-03 | M1 Rated Frequency |
| F4-04 | M1 Rated Voltage |
| F4-15 | PM rated power |
| F4-16 | PM pole number |
| F4-17 | PM rated current |
| F4-18 | PM rated speed |

(4) Perform Motor Parameter Identification

Confirm that the motor is disconnected from the load. In the case of disconnection, it is recommended to set F4-01 = 5 for rotational parameter identification. Press the “RUN” button to start the identification process. After the motor parameter identification is completed, the motor parameters shown in Table3-24 will be automatically updated.

Table3-24 Identified Motor Parameters

| Parameter Number | Parameter Name |
| --- | --- |
| F4-20 | PM Stator Resistance |
| F4-21 | PM D-axis Inductance |
| F4-22 | PM Q-axis Inductance |
| F4-23 | PM Ke Parameter (V/krpm) |
| F4-26 | PM Pole Angle (Degrees) |

3.2.3.3 No-Load Testing

After basic settings are completed, no-load testing can be conducted. The SVC no-load testing procedure is shown in Figure3-22.

Figure3-22SVC No-Load Testing Flowchart

3.2.3.4 Load Testing

After no-load testing is completed, load testing can be conducted. The SVC load testing procedure is shown in Figure3-23.

Figure3-23SVC Load Testing Flowchart

3.3 Input and Output Terminal Descriptions

3.3.1 Digital Input Terminal Functions (DI)

The CM680 series inverter is equipped with 8 multifunctional digital input terminals, among which HDI8 can be used as a high-speed pulse input terminal. Each DI terminal can select any one of the DI terminal functions.

Table 3-25 DI Terminal Function Parameters List

| F5-00 | DI1 Terminal Function Selection | Range: 0～94 | Factory value: 0 |
| --- | --- | --- | --- |
| F5-01 | DI2 Terminal Function Selection | Range: 0～94 | Factory value: 0 |
| F5-02 | DI3 Terminal Function Selection | Range: 0～94 | Factory value: 0 |
| F5-03 | DI4 Terminal Function Selection | Range: 0～94 | Factory value: 0 |
| F5-04 | DI5 Terminal Function Selection | Range: 0～94 | Factory value: 0 |
| F5-05 | DI6 Terminal Function Selection | Range: 0～94 | Factory value: 0 |
| F5-06 | DI7 Terminal Function Selection | Range: 0～94 | Factory value: 0 |
| F5-07 | HDI8 Terminal Function Selection | Range: 0～94 | Factory value: 0 |

Table 3-26 Detailed Explanation of DI Terminal Function Selection

| Set Value | Function | Description |
| --- | --- | --- |
| 0 | No function | Even if there is a signal input to the inverter, it will not operate; unused terminals can be set to no function to prevent accidental operation. |
| 1 | Multi-speed/Multi-point Position 1 | A total of 16 segments can be set through the combination of states of four digital input terminals, detailed combinations are shown below. |
| 2 | Multi-speed/Multi-point Position 2 | A total of 16 segments can be set through the combination of states of four digital input terminals, detailed combinations are shown below. |
| 3 | Multi-speed/Multi-point Position 3 | A total of 16 segments can be set through the combination of states of four digital input terminals, detailed combinations are shown below. |
| 4 | Multi-speed/Multi-point Position 4 | A total of 16 segments can be set through the combination of states of four digital input terminals, detailed combinations are shown below. |
| 5 | Fault Reset | External fault reset function. It has the same function as the STOP key on the digital operator. |
| 6 | Jog | Jog operation, which has the same function as the JOG key on the keyboard. The jog operation set frequency, acceleration and deceleration times refer to F7-00, F7-01, F7-02. |
| 7 | Speed Hold | During the inverter's acceleration or deceleration process, if the external terminal status meets the speed hold conditions, the inverter maintains its current speed. When the speed hold condition is canceled, the inverter resumes accelerating or decelerating to the set value. |
| 8 | 1-2 segment acceleration/deceleration switching | First and second acceleration/deceleration time switching, by default using the first and fourth acceleration/deceleration times, which are switched through the first and fourth acceleration/deceleration switching frequency (F1-15). Selecting multi-function input terminals, the first and second acceleration/deceleration times can be switched by changing the terminal status. |
| 9 | 3-4 segment acceleration/deceleration switching | Third and Fourth Acceleration/Deceleration Time Switching, Select Multi-function Input Terminal, you can switch between the third and fourth acceleration/deceleration times by changing the terminal status. |
| 10 | External Fault (F1-20) | External Fault Input (External Fault), when a multi-function digital IO input is detected as an external fault, the inverter will decelerate and stop according to the emergency or forced shutdown deceleration method (F1-23), displaying EF on the digital operator until the external terminal status returns to normal, after which the inverter can resume operation through fault reset (RESET). |
| 11 | Base Block | Base Block (Base Block) Input, when a multi-function digital IO input is detected as base block, the inverter immediately stops output, and the motor freely decelerates to a stop, displaying B.B on the digital operator. |
| 12 | Stop Output | Output Stop, when the set multifunctional digital input terminalisvalid, the inverter will immediately stop output, and the motor will decelerate to a stop freely. The inverter enters output wait mode until the input IO signal returns to normal, at which point the inverter restartstothe current set frequency. |
| 13 | Automatic Acceleration/Deceleration Prohibited | To cancel automatic acceleration/deceleration settings, set the automatic acceleration/deceleration (F1-20) to non-linear acceleration/deceleration. By switching the state of the multifunctional input IO port, you can switch between automatic acceleration/deceleration and linear acceleration/deceleration modes. |
| 14 | No function |  |
| 15 | AI1 Input Frequency Command | The speed command comes from AI1. When the set multifunctional input terminal status is valid, the inverter's frequency command source is forced to AI1. At the same time, if other AI command sources are set, AI1 has the highest priority. |
| 16 | AI2 Input Frequency Command | Speed command comes from AI2, when the set multifunctional input terminalstatus is valid, the inverter's frequency command source is forced toAI2. |
| 17 | AI3 Input Frequency Command | Speed Command comesfromAI3, when the set multi-function input terminalstatus is valid, the inverter frequency command source is forcibly set toAI3. |
| 18 | Deceleration stop (F1-20) | Forced shutdown, when the set multifunctional input terminalstatus is valid, the inverter will decelerate and stop according to the emergency or forced shutdown deceleration method(F1-23). |
| 19 | Frequency Increase External Command | Frequency increment command, the frequency command source (F0-06 = 3) needs to be set to external UP/DOWN terminals, when the set multifunctional input terminal status is valid, the inverter's frequency setting will increaseby one unit, if the input terminalstatus remains continuously valid, the frequency command will incrementally increase to the maximum frequency command value based on the settings for the external terminalUP/DOWN key mode (F5-09) and the external terminalUP/DOWN key acceleration/deceleration rate (F5-10). |
| 20 | External frequency decrease command | Frequency decrement command, the frequency command source (F0-06 = 3) must be set to external UP/DOWN terminals, when the designated multifunctional input terminal status is valid, the inverter's frequency setting will decrease by one unit. If the input terminal remains valid, the frequency command will decrement to the minimum frequency command value according to the settings for the external terminal UP/DOWN key mode (F5-09) and the external terminal UP/DOWN key acceleration/deceleration rate (F5-10). |
| 21 | PID function disabled | PID function cancellation, when the set multifunctional input terminalstatus is valid, thePID function becomes invalid. |
| 22 | Clear counter | Counter reset, when the designated multifunctional input terminal status is valid, the inverter will reset the current display count to zero. When the input terminal status switches to invalid, the inverter will resume counting. |
| 23 | Input Count (MI6) | When the status of the DI6 input terminal is validonce (the current software limits it to only the DI6 input terminal), the count value displayed on the digital operator will automatically increase by 1. The reset judgment value for the count value is determined by the final count value reaching the set value (L6-00). |
| 24 | External Forward Jog | The operation command source needs to be set to external terminals (F0-05 = 1). When the set multifunctional input terminal status is valid, the inverter will perform forward jogging operation. When executing the JOG command in torque mode, the inverter forcibly switches to speed mode. After the JOG command disappears, it automatically returns to speed mode. |
| 25 | External Reverse Jog | The operation command source needs to be set to external terminals (F0-05 = 1). When the set multifunctional input terminal status is valid, the inverter will perform reverse jogging operation. When executing the JOG command in torque mode, the inverter forcibly switches to speed mode, and after the JOG command disappears, it automatically returns to speed mode. |
| 26 | Torque/Speed Mode | Torque and Speed Mode Switching, when the set multifunctional input terminal status is valid, the inverter will operate in torque mode; otherwise, it will operate in speed mode. |
| 27 | Speed Loop 1/2 Switching | ASR1/ASR2 Switching, switching of speed loop control parameters for low-speed and high-speed zones, when the set multifunctional input terminalstatus is valid, the inverter will perform speed control using the high-speed zone control parameters (ASR2), otherwise, it will use the low-speed zone control parameters (ASR1). |
| 28 | External Fault Free Stop | Emergency Stop, when the set multifunctional input terminalstatus is valid, the inverter immediately stops output and displays EF1 on theDigital Operator, the motorfreely decelerates to a stop. When the external input terminalstatus returns to normal, aRESET must be performed to clear the fault before operation can resume. |
| 29 | Motor Y-connection confirmation signal | Motor coil star connection confirmation, when operating in V/F mode, if the state of the set multifunctional input terminalis effective, the inverter operates according to the firstset of V/F curves. |
| 30 | Motor delta connection confirmation signal | Motor coil delta connection confirmation, when operating in V/F mode, if the state of the set multifunctional input terminalis effective, the inverter operates according to the secondset of V/F curves. |
| 31 | High torque compensation amount | Torque command bias, torque command bias source (FE-05) set to 3 (torque command bias source is external terminal), multifunctional input terminalset to 31, when the multifunctional terminalinput status is effective, the high torque command bias compensation value is high torque command compensation(FE-07), multifunctional input terminalset to32, when the multifunctional terminalinput status is effective, the medium torque command bias compensation value is medium torque command compensation(FE-08), multifunctional input terminalset to33, when the multifunctional terminalinput status is effective, the low torque command bias compensation value is low torque command compensation(FE-09). |
| 32 | Intermediate Torque Compensation | Please see the setting value is 31. |
| 33 | Low Torque Compensation | Please see the setting value is 31. |
| 34 | Multi-speed/Position Switching | Terminal selection for multi-speed and multi-position function switching, when the set multifunctional input terminal status is valid, the terminal set for multi-speed/multi-point position function is used to select multi-position commands; otherwise, it is used to select multi-speed commands. |
| 35 | Internal Positioning Enable | Enable single-point positioning, when the set multifunctional input terminalstatus is valid, the inverter performs internal single-point positioning according to the encoder's internal positioning position (FC-20), supporting only the FOCPG control mode. |
| 36 | Multi-point Position Input | When the set multifunctional input terminal status is valid, the inverter writes the current position information into the corresponding multi-segment position command parameter, which is used for manually calibrating the multi-segment position command parameter settings. |
| 37 | Pulse Input Position Control | Enable full-range position control pulse command input, when the inverter frequency command source is selected as pulse input (F0-06 = 4, 5), if the set multifunctional input terminalstatus is valid, the pulse input on the PG card serves as the position command, it is recommended to set the APR feedforward gain (FC-25) to 0. |
| 38 | Prohibit Writing to EEPROM | Prohibit writing to EEPROM, when the set multifunctional input terminalstatus is valid, parameter modifications will not be saved toEEPROM, and will not be retained after power loss. |
| 39 | Torque Command Direction Switching | Torque command direction, applicableto torque control mode, when the torque command source isAI1/AI2/AI3, if the set multifunctional input terminalstatus is valid, the torque commandautomatically converts to a negative value. |
| 40 | Motor Free Stop | Forced free stop, during the operation of the inverter, when the set multifunctional input terminal status is valid, the inverter immediately stops output, and the motor coasts to a stop. |
| 41 | Manual Mode Enable | HAND switch, when the set multifunctional input terminal status is valid, the inverter switches to HAND mode, and the frequency command source and operation command source switch to being determined by parameters L1-00 and L1-01. When the multi-function input terminal status isinvalid, the inverter will stop (it will not be able to start in standby mode). |
| 42 | Automatic mode enable | When AUTO switching is enabled, if the set multi-function input terminal status is valid, the inverter switches to AUTO mode, and the frequency command source and operation command source switch to those determined by parameters F0-06 and F0-05. When the multi-function input terminal status is invalid, the inverter will stop (it will not be able to start in standby mode). |
| 43 | Enable resolution switching | Enable resolution switching, when the set multi-function input terminalstatus is valid, the frequency base value (maximum inverter frequency) for analog input frequency calculation switches from F4-02 to F5-40. |
| 48 | Mechanical gear ratio switching | Mechanical gear ratio switching, when the set multifunctional input terminalstatus is valid, the mechanical gear ratio switches to the second group (switching from F4-36, F4-37 to F4-38, F4-39). |
| 49 | Inverter Enable | Inverter enable, by default, the inverter enable is valid, allowing the inverter to control start and stop. When the inverter is not enabled, the run command is invalid. If the inverter enable fails during operation, the motor will freely decelerate to a stop. Switching the set multifunctional input terminalstatus can switch the inverter enable status. Once the multifunctional input terminalis configured as the inverter enable, the inverter can only be started by setting the multifunctional input terminalstatus to valid. |
| 50 | Slave dEB (instant stop without stopping) action input | Slave station dEb action input, when the master station performs a dEb action, this information is input to notify the slave station to also perform a dEb action, ensuring that the slave station can stop simultaneously. |
| 53 | CANopen Quick Stop Trigger | CANopen rapid stop trigger, when controlled via CANopen, if the set multifunctional input terminalis effective, the inverter's operating state is forcibly switched to rapid stop. |
| 56 | Local/Remote Switching | LOCAL/REMOTE switching selection, when LOCAL/REMOTE actions are selected and set to LOCAL/REMOTE switching (L1-02 is not 0), the higher-level machine will display the LOC/REM status; if the set multifunctional input terminalis effective, the inverter operates in LOCAL mode, otherwise, REMOTE mode. |
| 70 | Auxiliary Frequency Forced to 0 | Auxiliary frequency forced to 0, when the auxiliary frequency function is enabled (auxiliary frequency source F0-07 is not 0), if the set multifunctional input terminalis effective, the auxiliary frequency is forced to 0; if PID is the main frequency, PID will continue to operate. |
| 71 | PID function disabled, PID output forced to 0 | PID function disabled, PID output forced to 0, auxiliary frequency function enabled (auxiliary frequency source F0-07 is not 0), and when using the PID function, if the configured multi-function input terminaliseffective,thePID function stops,the integral value is reset to zero,and the PID output is cleared. |
| 72 | PID function disabled, PID maintains current output level | PID function disabled, PID maintains previous output value, auxiliary frequency function enabled (auxiliary frequency source F0-07 is not 0), and when using the PID function, if the configured multi-function input terminaliseffective,thePID function stops,and the PID maintains the previous output. |
| 73 | Force PID integral gain to 0 | Force PID integral gain to 0, no integration action, if the configured multi-function input terminaliseffective,thePID continues to operate, but the integrator stops working, and the integrator output is cleared. |
| 74 | Invert PID feedback | Invert PID feedback, when the set multifunctional input terminaliseffective,the sign of the PID feedback value is reversed; if the PID feedback is positive, it is converted to negative, and if the PID feedback is negative, it is converted to positive. |
| 83 | Select bit0 for multiple induction motors | Multiple (inductive) motor selection, when the set multifunctional input terminalis effective, switch different motor parameters according to the terminal status combination, the operating frequency and V/F curve parameters corresponding to different motor parameters need to refer to the basic parameter group. |
| 84 | Multiple Induction Motor Selection Bit 1 | Same as Above. |
| 86 | Diameter Reset | Diameter reset, under tension control mode (tension control mode FB-00 is not 0), when the set multifunctional input terminalis effective, the current diameter (FB-38) is reset to the initial diameter or empty roll diameter. |
| 87 | Initial Diameter Selection 1 | Initial diameter selection, under tension control mode (tension control mode FB-00 is not 0), when the set multifunctional input terminal is effective, the current diameter (FB-38) can be reset to the initial diameter (FB-30, FB-31). |
| 88 | Initial Diameter Selection 2 | Same as Above. |
| 89 | Tension PID Control Integral Reset | Tension PID control integral reset, under tension control mode (tension control modeFB-00 is not 0), when the designated multifunctional input terminalis effective, the integrator output is cleared. |
| 90 | Diameter Calculation Hold | Winding diameter calculation hold, when the designated multifunctional input terminalis effective, stopupdating the winding diameter calculation. |
| 91 | Diameter Mode Selection (0: Winding In 1: Winding Out) | Drum Mode Selection, when the designated multi-function input terminalis effective, the drum mode switches to winding out; otherwise, it is winding in. |
| 92 | Disable Tension PID | Disable tension PID, when the designated multifunctional input terminalis effective, disabletensionPID function. |
| 93 | Pause Tension PID | Pause tension PID, when the designated multifunctional input terminalis effective, pause tensionPID function. |
| 94 | Automatic Diameter Switching | Automatic winding diameter switching, when the designated multifunctional input terminalis effective,automatic switching of winding diameter. |

3.3.2 Digital Output Terminal Functions (DO)

The CM680 series inverter is equipped with 2 multi-function digital output terminals and two multi-function relay output terminals.

Table 3-27 List of Parameters Related to D0 Terminal Function

| F6-00 | RLY1 terminal function selection | Range: 0～76 | Factory value: 11 |
| --- | --- | --- | --- |
| F6-01 | RLY2 terminal function selection | Range: 0～76 | Factory value: 1 |
| F6-02 | DO1 Terminal Function Selection | Range: 0～76 | Factory value: 0 |
| F6-03 | DO2 Terminal Function Selection | Range: 0～76 | Factory value: 0 |
| F6-04 | DO Terminal Valid Logic | Range: 0000H~FFFFH | Factory value: 0 |
| F6-05 | Source of DO Output AI | 0: AI1 1: AI2 2: AI3 | Factory value: 0 |
| F6-06 | DO Output AI Upper Limit Value | -100.00%～100.00% | 50.00 |
| F6-07 | DO Output AI Lower Limit Value | -100.00%～100.00% | 10.00 |
| F6-08 | DO Action Frequency | 0.00～599.00Hz | 0.00 |

Table 3-28 Detailed Description of DO Terminal Function

| Set Value | Function | Description |
| --- | --- | --- |
| 0 | No function | Disable multi-function digital output function |
| 1 | Inverter running | Inverter running, this terminal output is valid. |
| 2 | Reach set frequency | Reach set frequency, when the inverter output frequency deviation from the set frequency is within a certain range (±2Hz), this terminal output is valid. |
| 3 | Reach frequency 1 detection value | Reach frequency 1 detection value, when the inverter output frequency deviation from the frequency 1 detection value (F6-27) is less than the frequency 1 amplitude (F6-28), this terminal output is valid. |
| 4 | Reach frequency 2 detection value | When the inverter output frequency deviates from the frequency arrival 2 detection value (F6-29) by less than the frequency arrival 2 amplitude (F6-30), this terminal is valid. |
| 5 | Zero Speed Command Running | This terminal is valid when the frequency command is 0 during inverter operation. |
| 6 | Zero Speed Command Operation (Including STOP) | This terminal is valid when the inverter frequency command is 0 (including when the inverter is stopped). |
| 7 | Over Torque 1 | When the inverter detects an over torque condition (over torque action method, detection threshold, and detection time refer to parameters F9-36 ~ F9-38), this terminal is valid. |
| 8 | Over Torque 2 | When the inverter detects an over torque (for over torque action method, detection threshold, and detection time, refer to parameters H3-00 ~ H3-02), this terminal is active. |
| 9 | Inverter Ready | When the inverter has no fault (including during stop and operation), this terminal is active. |
| 10 | Low Voltage Warning | When the inverter experiences DC bus under-voltage (including during stop and operation, and when the DC bus voltage is below the low voltage threshold L2-18), this terminal is active. |
| 11 | Fault Indication | This terminal is active when the inverter has a fault (excluding undervoltage during shutdown, B.B. fault, dEb fault). |
| 12 | Brake release | This terminal is effective when the inverter operating frequency is not 0 or greater than the motor zero-speed threshold (F6-09). |
| 13 | Overtemperature warning | This terminal is effective when the inverter detects an overtemperature warning for the IGBT or bus capacitor. |
| 14 | Software Brake Action Indication | This terminal is effective when the inverter's discharge resistor switch actuates. |
| 15 | PID feedback abnormality | This terminal is effective when the inverter detects that the deviation between the PID setpoint and feedback exceeds the set threshold (FA-51) and persists for the duration (FA-52). |
| 16 | Excessive slip | This terminal is effective when the inverter detects an excessive slip fault. |
| 17 | Count Value Reached Not Reset | When the inverter executes an external counter, if the count value equals the set value of parameter L6-01, this terminal is active. If the setting value of parameter L6-01 > the setting value of parameter L6-00, this terminal output is invalid. |
| 18 | Count Value Resets to Zero | When the inverter executes an external counter, if the count value equals the setting value of parameter L6-00, this terminal output is valid. |
| 19 | Base Block State | When the inverter stops output due to base block (B.B.), this terminal outputs effectively. |
| 20 | Warning Indication | When the inverter detects a warning, this terminal outputs effectively. |
| 21 | Overvoltage Warning | When the inverter detects that the bus voltage exceeds the set overvoltage warning threshold (0.9 times the overvoltage threshold, no warning state output), this terminal outputs effectively. |
| 22 | Overcurrent Stall Warning | When the inverter detects an overcurrent slip prevention warning, this terminal output is valid. |
| 23 | Overvoltage Stall Warning | When the inverter detects an overvoltage slip prevention warning, this terminal output is valid. |
| 24 | Inverter Operation Source | When the inverter operation command source is not the keyboard (F0-05 is not 0), this terminal output is valid. |
| 25 | Forward Command | When the inverter's direction command is forward, this terminal output is valid. |
| 26 | Reverse Command | When the inverter's direction command is reverse, this terminal output is valid. |
| 29 | Over Frequency | When the actual output frequency of the inverter exceeds the DO action frequency (F6-08), this terminal output is valid. |
| 30 | Under Frequency | When the actual output frequency of the inverter is less than the DO action frequency (F6-08), this terminal outputs a valid signal. |
| 31 | Motor Star Connection Switch Command | Motor Star Connection Switch Command, set the multi-function digital input terminal for star connection confirmation signal (DIx = 29) and delta connection confirmation signal (DIx = 30), set the motor star-delta switching enable (F7-48 = 1). When the inverter output frequency is below the motor star-delta switching frequency (F7-47) setting minus 2Hz and the duration is greater than the motor star-delta switching delay (F7-49), this terminal outputs a valid signal. |
| 32 | Motor Delta Connection Switching Command | To switch the motor connection from star to delta, set the multifunction digital input terminal for star connection confirmation signal (DIx = 29) and delta connection confirmation signal (DIx = 30), enable the motor star-delta switching (F7-48 = 1). When the inverter output frequency is higher than the motor star-delta switching frequency (F7-47) setting plus 2Hz and the duration exceeds the motor star-delta switching delay (F7-49), this terminal becomes active. |
| 33 | Zero Frequency Operation | When the inverter is running and the output frequency is zero, this terminal becomes active. |
| 34 | Output zero frequency (including STOP) | This terminal is valid when the inverter output frequency is zero or stopped. |
| 35 | Fault output selection 1 | This terminal is valid when the inverter encounters a fault set by U0-10. |
| 36 | Fault output selection 2 | This terminal is valid when the inverter encounters a fault set by U0-11. |
| 37 | Fault output selection 3 | This terminal is valid when the inverter encounters a fault set by U0-12. |
| 38 | Fault output selection 4 | This terminal is valid when the inverter encounters a fault set by U0-13. |
| 39 | Position reached | This terminal is valid when the position is reached in the inverter's positioning mode. |
| 40 | Target frequency reached | This terminal is valid when the inverter output frequency reaches the set frequency (including stop and frequency command of 0). |
| 41 | Multi-point Positioning Arrival | When the position error of multi-point positioning of the inverter is less than the allowable position arrival error range of the encoder (FC-21), this terminal output is valid. |
| 42 | Mechanical Brake Release | It is necessary to set the DO action frequency (F6-08) and the brake frequency (L4-00). When the brake release is detected, this terminal output is valid. |
| 43 | Motor Zero Speed | When using a PG card, if the feedback speed from the PG card is below the motor zero speed judgment threshold (F6-09) set speed, this terminal output is valid. |
| 44 | Low Current Output | When the inverter experiences a low current fault (the low current action method is selected by parameter F9-24, which may not report a low current fault or low current warning), this terminal output is valid. |
| 45 | Three-phase Output Contactor Action | It is necessary to configure the digital input terminal as 49 (inverter enable) for use together. When the inverter status enable terminal is active, the output terminal is effective. |
| 46 | dEB action | During the inverter deceleration process, when dEB action occurs, the output terminal is effective. |
| 47 | Stop brake closure | When the inverter is in a shutdown state, if the discharge resistor switch feedback status is closed, the output terminal is effective. |
| 49 | Positioning complete | When returning to the origin is completed, the output terminal is effective. |
| 50 | CANopen control output | CANopen Control Output, controlling the output terminals based on the status of the corresponding CANopen data bits. The mapping table for CANopen and digital outputs is as follows: Terminal; Parameter setting; Attribute; Index. RLY1; F6-00=50; RW; Index:2026 SubIndex: 0x41 Data bit0. RLY2; F6-01=50; RW; Index:2026 SubIndex: 0x41 Data bit1. DO1; F6-02=50; RW; Index:2026 SubIndex: 0x41 Data bit3. DO2; F6-03=50; RW; Index:2026 SubIndex: 0x41 Data bit4. |
| 51 | RS485 control output | RS-485 (Modbus) control output, controlling the output terminals based on the status of the corresponding RS-485 data bits. The RS-485 and digital output port mapping table is as follows: Terminal; Parameter setting; Attribute; Index. RLY1; F6-00=51; RW; 2640 data bit0. RLY2; F6-01=51; RW; 2640 data bit1. DO1; F6-02=51; RW; 2640 data bit3. DO2; F6-03=51; RW; 2640 data bit4. |
| 52 | Communication card Control Output | Communication card control output, controlling the output terminals based on the status of the corresponding communication card data bits. The communication card and digital output port mapping table is as follows: Terminal; Parameter setting; Attribute; Index. RLY1; F6-00=52; RW; 2640 data bit0. RLY2; F6-01=52; RW; 2640 data bit1. DO1; F6-02=52; RW; 2640 data bit3. DO2; F6-03=52; RW; 2640 data bit4. |
| 66 | STO fault | When any type of STO fault occurs in the inverter, the terminal output is active. |
| 67 | Analog input threshold reached output | The inverter's multi-function output terminal operates when the analog input is between the high threshold and low threshold. Parameter F6-05 selects the analog input channel AI1, AI2, or AI3 to be compared. Parameter F6-06 sets the high threshold for analog input comparison (default 50%). Parameter F6-07 sets the low threshold for analog input comparison (default 10%). When the analog input > Parameter F6-06, the multi-function output terminal operates; When the analog input < Parameter F6-07, the multi-function output terminal stops output. |
| 68 | STO normal | When any type of STO fault occurs in the inverter, this terminal output is invalid. |
| 69 | Maximum roll diameter reached | When using tension control mode, this terminal output is valid when the roll diameter reaches parameter FB-26. |
| 70 | Reach empty roll diameter | When using tension control mode, this terminal output is valid when the roll diameter reaches parameter FB-27. |
| 71 | Belt break indication | When using tension control mode, if the enable belt break detection is active, the line speed exceeds parameter FB-45, the roll diameter error exceeds parameter FB-46, and the time exceeds parameter FB-47, a belt break occurs, and this terminal output is valid. |
| 72 | Tension PID feedback deviation fault | When using tension control mode, if the difference between the tension PID setpoint and the tension PID feedback value exceeds the PID feedback error level (parameter FB-48), and the error time exceeds the PID feedback error detection time (parameter FB-49), a PID feedback error anomaly (refer to PID feedback error handling (parameter FB-50)) occurs, and this terminal output is valid. |
| 73 | Over-torque 3 | When the inverter detects an over-torque event (over-torque detection threshold H3-06, over-torque judgment delay H3-07), this terminal output is valid. |
| 74 | Over-torque 4 | When the inverter detects an over-torque event (over-torque detection threshold H3-11, over-torque judgment delay H3-12), this terminal output is valid. |

3.3.3 Analog Input Terminal Functions (AI)

The CM680 series inverter is equipped with 3 AI terminals.

Table 3-29 AI Terminal Function Selection Parameter List

| F5-21 | AI1 function selection | Range: 0～20 | Factory value: 1 |
| --- | --- | --- | --- |
| F5-27 | AI2 Function Selection | Range: 0～20 | Factory value: 0 |
| F5-33 | AI3 Function Selection | Range: 0～20 | Factory value: 0 |

Table 3-30 Detailed Description of AI Terminal Functions

| Set Value | Function | Description |
| --- | --- | --- |
| 0 | No function | AI terminal has no function |
| 1 | Frequency setting | When used as frequency setting, the frequency source (F0-06) must be set to 2 |
| 2 | Torque setting | When used as torque setting, the inverter control mode (F0-03) must be set to 2 (torque mode), the torque command source selection (FE-01) must be set to 2, and parameters F5-21, F5-27, or F5-33 must be set to 2. |
| 3 | Torque compensation setting | When used as torque compensation, the inverter must set the torque command bias source (FE-05) to 1 (from external analog input). |
| 4 | PID target value | When used as a PID target value, the PID feedback terminal (FA-00 not equal to 0) must be set, and there are two other cases: 1. The PID target value source (FA-01) is set to 0 (from frequency setting), parameters F5-21 or F5-27 or F5-33 are set to 1 or 4; if both 1 and 4 are set in the analog input simultaneously, AI1 is prioritized as the PID target value. 2. The PID target value source (FA-01) is set to 3 (from external analog input). The PID target value will be displayed in percentage form via parameter FA-02 (PID target value setting) (precision 0.01%). |
| 5 | PID feedback value | When used as PID feedback, PID feedback selection (FA-00) must be set to 1 or 4, and the feedback value is displayed in percentage form via U1-04 (Communication setting PID feedback value)（precision0.01%）. |
| 6 | Thermistor PTC Input | When used as PTC input, it is necessary to switch between PTC input and PT100 input by checking if parameter F9-52 (PT100 Class 1 Protection Frequency) is set to 0. |
| 7 | Forward Torque Limit | When the analog input is used as torque limit for four-quadrant operation, parameters F5-21, F5-27, or F5-33 should be set to 7, 8, 9, or 10. The torque limit values are displayed through parameters FE-11 to FE-14 (shared variables with torque limits received via communication), and the diagram below illustrates the torque limit for four-quadrant operation: |
| 8 | reverse torque limit |  |
| 9 | regenerative torque limit |  |
| 10 | Forward/Reverse Torque Limit |  |
| 11 | Thermistor PT100 value | An external PT100 thermistor is required to detect whether the motor is overheated. |
| 12 | Auxiliary frequency setting | When used as an auxiliary frequency input, the auxiliary frequency source (F0-07) must be set to 3 (external analog input). |
| 13 | PID offset | When used as a PID compensation amount, it is necessary to set the PID feedback selection (FA-00 not equal to 0) to be valid, set the PID compensation amount selection (FA-20) to 1 (from external analog input), the intelligent start frequency command (FA-33) to 0, and the application macro parameter (L0-00) to 6 (for air compressor industry applications). The change in the analog input compensation amount can be displayed in parameter FA-21 (PID compensation) as a percentage (precision 0.1%). |
| 14 | Tension PID Feedback Value | When used as tension PID feedback, the tension control mode selection (FB-00) must be set to 1 (speed closed-loop mode). |
| 15 | Line Speed Feedback | As tension control line speed feedback. |
| 16 | roll diameter feedback | When used as roll diameter feedback (including initial roll diameter and real-time roll diameter calculation feedback), the tension control selection (FB-00 not zero) must be enabled, the roll diameter source (FB-25) must be set to 1 for analog input as roll diameter feedback (FB-38), and the initial roll diameter source (FB-28) must be set to 1 for analog input as the initial roll diameter (FB-29). |
| 17 | Tension PID Setpoint | When used as tension PID setpoint, the tension control mode selection (FB-00) must be set to 1 (speed closed-loop mode), and the PID target source setting (FB-04) must be set to 2 (analog input). When the inverter is in operation, the PID target value setting (FB-05) isdisplayed. |
| 18 | Tension Setpoint | When setting the tension command, the tension control mode selection (FB-00) must be set to 3/4 (torque mode), the tension command source selection (FB-52) to 1 (analog input), and the maximum tension value (FC-53) should be set according to the site conditions (default value is 0). Parameters F5-21, F5-27, or F5-33 should be set to 18. When the inverter is in operation, the tension setting is displayed through parameter (FB-54). |
| 19 | Zero-speed tension setting | When setting the zero-speed tension command, the tension control mode selection (FB-00) must be set to 3/4 (torque mode), the zero-speed tension command source selection (FB-55) to 2 (analog input), and the maximum tension value (FB-53) should be set according to the site conditions (default value is 0). When the inverter is in operation, the zero-speed tension setting is displayed through parameter (FB-56). |
| 20 | Tension Taper Setting | When setting the tension taper, the tension control mode selection (FB-00) must be set to 3/4 (torque mode), the tension taper setting source selection (FC-64) must be set to 1 (analog input), and parameters F5-21, F5-27, or F5-33 must be set to 20. During the inverter operation, the tension taper setting is displayed through parameter (FC-65). |

3.3.4 Analog Output Terminal Functions (AO)

The CM680 series inverter is equipped with 2 AO terminals.

Table 3-31 List of Terminal Function Parameters

| F6-13 | AO1 Signal Type Selection | 0: 0-10V Output Selection 1: 0-20mA Output Selection | Factory value: 0 |
| --- | --- | --- | --- |
| F6-14 | AO1 output function selection | Range: 0～23 | Factory value: 0 |
| F6-15 | AO1 output gain | Range: 0.0～500.0% | Factory value: 100.0% |
| F6-16 | AO1 reverse enable | 0: Absolute value output 1: Negative value outputs as 0V 2: 5V as center point | Factory value: 0 |
| F6-17 | AO1 Output Bias | Range: -100.00%～100.00% | Factory value: 0.00% |
| F6-18 | AO1 Output Fixed Value | Range: 0.00～100.00% | Factory value: 0.00% |
| F6-19 | AO1 Filter Time | Range: 0.00～20.00 seconds | Factory value: 0.01 second |
| F6-20 | AO2 Signal Type Selection | 0: 0-10V Output Selection 1: 0-20mA Output Selection | Factory value: 0 |
| F6-21 | AO2 Signal Type Selection | Range: 0～23 | Factory value: 0 |
| F6-22 | AO2 Output Gain | Range: 0.0～500.0% | Factory value: 100.0% |
| F6-23 | AO2 Reverse Enable | 0: Absolute value output 1: Negative value outputs as 0V 2: 5V as center point | Factory value: 0 |
| F6-24 | AO2 Output Bias | Range: -100.00%～100.00% | Factory value: 0.00% |
| F6-25 | AO2 Output Fixed Value | Range: 0.00～100.00% | Factory value: 0.00% |
| F6-26 | AO2 Filter Time | Range: 0.00～20.00 seconds | Factory value: 0.01 second |

Table 3-32 Detailed Explanation of Terminal Output Functions

| Set Value | Function | Description |
| --- | --- | --- |
| 0 | Output frequency | The maximumfrequency parameterF4-02 is 100%. |
| 1 | Frequency command | The maximumfrequency parameterF4-02 is 100%. |
| 2 | Motor operating frequency (Hz) | The maximumfrequency parameterF4-02 is 100%. |
| 3 | Output current (RMS) | Rated at 2.5 times the inverter's rated current as 100%. |
| 4 | Output voltage | Rated at 2 times the motor's rated voltage as 100%. |
| 5 | DC Bus Voltage | Rated at 450V (230V model) / 900V (460V model) for 100%. |
| 6 | power factor | Rated at ±1.000 as 100%. |
| 7 | Power | Rated at 2 times the inverter's rated power as 100%. |
| 8 | Output torque | Rated at 2 times the rated torque as 100% |
| 9 | AI1 percentage | 0 ～ 10V/0 ～ 20mA corresponds to 0 ～ 100%. |
| 10 | AI2 percentage | 0 ～ 10V/0 ～ 20mA corresponds to 0 ～ 100%. |
| 11 | AI3 percentage | 0 ～ 10V/0 ～ 20mA corresponds to 0 ～ 100%. |
| 12 | Iq Current Command | Rated at 2.5 times the inverter's rated current as 100%. |
| 13 | Iq Current Feedback | Rated at 2.5 times the inverter's rated current as 100%. |
| 14 | Id Current Command | Rated at 2.5 times the inverter's rated current as 100%. |
| 15 | Id Current Feedback | Rated at 2.5 times the inverter's rated current as 100%. |
| 16 | Vq Voltage Command | Rated at 300V (230V model) / 600V (460V model) for 100%. |
| 17 | Vd Voltage Command | Rated at 300V (230V model) / 600V (460V model) for 100%. |
| 18 | Torque command | With rated torque as 100% . |
| 19 | PG2 Frequency Command | The maximumfrequency parameterF4-02 is 100%. |
| 20 | CANopen Analog Output | Provided for CANopen communication analog output, terminalAO1 corresponds to address 2026-A1, terminalAO2 corresponds to address 2026-A2, extended terminalAO10 corresponds to address 2026-AB, extended terminalAO11 corresponds to address 2026-AC. |
| 21 | RS485 analog output | Provided for RS-485 communication analog output, terminalAO1 corresponds to address 26A0H, terminalAO2 corresponds to address 26A1H, extended terminalAO10 corresponds to address 26AAH, extended terminalAO11 corresponds to address 26ABH. |
| 22 | Communication card analog output | Provided for communication card analog output, terminalAO1 corresponds to address 26A0H, terminalAO2 corresponds to address 26A1H, extended terminalAO10 corresponds to address 26AAH, extended terminalAO11 corresponds to address 26ABH. |
| 23 | Fixed voltage output | The output voltage ratio can be set by parameters F6-18 (corresponding to AO1) and F6-25 (corresponding to AO2), parameter settings 0.00 ～ 100.00% correspond to AO output 0 ～ 10V. |

3.4 Control performance

3.4.1 Setting of VF curves

This inverter is equipped with multipleV/Fcurves and can customizeV/Fcurves to meet different load conditions. When F2-00=0, it is a multi-segmentV/Fcurve, relevant parameters are shown in Table 3-33. Among them,V1<V2<V3,F1<F2<F3. When F2-00=1, the inverter output voltage is proportional to the 1.5 power of frequency. When F2-00=2, the inverter output voltage is proportional to the square of frequency.

Table 3-33  Parameters related to V/F curve setting

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F2-00 | V/F Voltage Selection | 0 | 0: General V/F Curve 1: 1.5 Power V/F Curve 2: 2 Power V/F Curve | Multi-segment (custom) V/F curve, defined according to F2-04 to F2-09 and F4-03 and F4-04, as shown in the figure below. In the figure, f1, f2, f3, and fb represent M1 multi-point VF frequency point 1, M1 multi-point VF frequency point 2, M1 multi-point VF frequency point 3, and M1 rated frequency, respectively. V1, V2, V3, and Vb represent M1 multi-point VF voltage point 1, M1 multi-point VF voltage point 2, M1 multi-point VF voltage point 3, and M1 rated voltage, respectively. Note: Generally, V1<V2<V3<Vb, f1<f2<f3<fb. Setting the voltage too high at low frequencies may cause the motor to overheat or even burn out, and the inverter may experience overcurrent or overvoltage protection. |
| F2-04 | M1 Multi-point VF Frequency Point 1 | 0.50Hz | 0.00~599.00Hz |  |
| F2-05 | M1 Multi-point VF Voltage Point 1 | 2.0V | 0.0~480.0V |  |
| F2-06 | M1 Multi-point VF Frequency point 2 | 1.50Hz | 0.00~599.00Hz |  |
| F2-07 | M1 Multi-point VF Voltage point 2 | 10.0V | 0.0~480.0V |  |
| F2-08 | M1 Multi-point VF Frequency point 3 | 3.00Hz | 0.00~599.00Hz |  |
| F2-09 | M1 Multi-point VF Voltage point 3 | 22.0V | 0.0~480.0V |  |
| F4-03 | M1 Rated Frequency | 50.00Hz | 0.00~599.00Hz |  |
| F4-04 | M1 Rated Voltage | 380.0V | 0.0~510.0V |  |

3.4.2 Torque compensation

Torque compensation is a commonly used stage in motor control, which can be used to improve load-carrying capacity and enhance control performance. The torque compensation function in this section can be used for theVFcontrol of asynchronous motors and theSVCcontrol of permanent magnet synchronous motors. Based on the operating state of the motor (such as motor speed), the torque compensation amount is calculated. This compensation amount is then multiplied by a certain gain, passed through a low-pass filter, and added as a voltage compensation value to the output voltage to achieve the purpose of torque compensation. Relevant parameters are shown in Table 3-34.

Table 3-34 Torque Compensation Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F2-01 | Torque Compensation Gain | 1 | Induction Motor: 0~10 Permanent Magnet Synchronous Motor: 0~5000 | This parameter is used to adjust the torque compensation level. By adjusting the output voltage, the motor's load-carrying capacity can be adjusted. The greater the torque compensation gain, the better the load-carrying capacity, but the output current will increase. If set too high, it may lead to overcurrent faults; reducing the torque compensation gain will decrease the load-carrying capacity accordingly. |
| F2-02 | Torque Filter Time | 0.500s | 0.001~10.000s | This parameter is used to adjust the filter time for torque compensation. If the filter time is set too high, control stability improves, but response becomes slower. When the filter time is set too small, the response is faster, but it may lead to instability. Adjust according to actual conditions. |
| H1-06 | M2 Torque Compensation Gain | 1 | Induction Motor: 0~10 Permanent Magnet Synchronous Motor: 0~5000 | This parameter is used to adjust the torque compensation level. By adjusting the output voltage, the motor's load-carrying capacity can be adjusted. The greater the torque compensation gain, the better the load-carrying capacity, but the output current will increase. If set too high, it may lead to overcurrent faults; reducing the torque compensation gain will decrease the load-carrying capacity accordingly. |
| H1-14 | M3 Torque Compensation Gain | 1 |  |  |
| H1-22 | M4 Torque Compensation Gain | 1 |  |  |

3.4.3 Slip Compensation

Slip compensation is a commonly used stage in the control of asynchronous motors. Due to the existence of slip, the speed of an asynchronous motor often does not equal the synchronous speed. If slip is not compensated for, it may result in insufficient precision in the speed control of the asynchronous motor. Proper slip compensation can improve the speed accuracy of the asynchronous motor. Based on the mathematical model of the motor, the slip is calculated using information such as motor current and voltage. The slip is then added to the synchronous speed to achieve slip compensation. Since the calculated slip may have errors, the slip compensation gain can be increased to adjust it. Additionally, the calculated slip usually contains noise, which can be eliminated or reduced using a low-pass filter. Relevant parameters are shown in Table 3-35.

Table 3-35 Slip Compensation Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F2-10 | Slip Compensation Time | 0.100s | 0.001~10.000s | This parameter is used to set the filter time for slip compensation. The larger the value of this function code, the slower the compensation response; the smaller the value, the faster the response, but it may lead to system instability. Adjust according to actual conditions. |
| F2-11 | Slip Compensation Gain | 0.00 (1.00 in VVC mode) | 0.00~10.00 | This parameter is used to set the gain for slip compensation. By adjusting the slip compensation gain, the motor speed accuracy can be improved. When using VF control: Under electric load conditions, if the motor speed is lower than the actual speed, the slip compensation gain can be appropriately increased; otherwise, it should be decreased. In power generation load conditions, if the motor speed is higher than the actual speed, the slip compensation gain can be appropriately increased; conversely, it should be decreased. When using VVC control for induction motors: Under electric load conditions, if the motor speed is lower than the actual speed, the slip compensation gain can be appropriately increased; otherwise, it should be decreased. |
| H1-07 | M2 Slip Compensation Gain |  |  |  |
| H1-15 | M3 Slip Compensation Gain |  |  |  |
| H1-23 | M4 Slip Compensation Gain |  |  |  |
| F2-12 | Generation Slip Compensation Gain | 1.00 | 0.00~1.00 | This parameter is used to adjust the slip compensation gain in generation mode, applicable only to IMVVC control mode. Whenusinginduction motor VVC control, under generation load conditions, if the motor speed is higher than the actual speed, the generation slip compensation gain can be appropriately increased; conversely, the generation slip compensation gain should be reduced. |

3.4.4 Current Limiting

The current limiting function restricts torque current by limiting the torque current setpoint, as shown in Table 3-36.

Table 3-36 Current Limit Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-07 | maximum current limit | 150% | 0~250%(100% corresponding to the inverter's rated current) | This parameter is used to set the maximum current output of the inverter, which, together with the settings of parameters F3-23 to F3-25, determines the current limit of the inverter's output. |
| F3-23 | I/F Mode Current Command | 40% | 0%～150% | When F1-00 is 1, this parameter sets the percentage of the DC drive current relative to the motor rated current during the synchronous motor flux zero position alignment period; When F3-24 is zero, this parameter sets the percentage of the low-speed magnetization current relative to the motor rated current when the SVC observer mode is running; When F3-24 is non-zero, this parameter sets the percentage of the current command relative to the motor rated current when the motor I/F mode is running. |
| F3-24 | IF Switching Frequency 1 | 20.00Hz | 0.00～599.00 Hz | When this parameter is non-zero, the synchronous motor will be forced to operate in I/F mode below this frequency. |
| F3-25 | IF Switching Frequency 2 | 20.00Hz | 0.00～599.00 Hz | This parameter defaults to F3-25 and generally does not need to be changed. |

3.4.5 Parameter Management Settings

Parameter management settings are achieved through a function code parameter setting, enabling operations such as write protection for related parameters, value reset, parameter reinitialization, modification of read-only parameters, and program reset. Currently, parameter management settings are configured through function codeF0-18, as shown in Table 3-37.

Table 3-37 Parameter Management Settings

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F0-18 | Parameter Management | 0 | 0: No Function 1: Parameter Write Protection 5: Reset kWH Display to Zero 6: Reset PLC 7: Reset CAN Slave 9: Reset to 50Hz Factory Values 11: Reset to 50Hz Factory Value | 0: No Function; 1: After entering 1, most function codes will be locked and cannot be modified; only parameter management (F0-18) and password input (F7-33) can be modified. This value is written to EEPROM and remains effective after power loss. Only by setting parameter management (F0-18) to 0 can other parameter values be modified; 5: Can reset the inverter's energy statistics to zero; 7: Reset CAN Slave; 8: No Function; 9: Restore function codes to factory settings, which resets them to default values. If the inverter has been set with a password (via F7-34), the password must be removed (via F7-33) before restoring factory settings; 11: Restore function codes to factory settings, but retain all user-defined parameter values of the application macro. |

Precautions:

1. When the parameter management setting is6or7, the control board needs to be powered off and then powered on again after setting.

2. If the frequency command source is theDigital Operatorknob, and the parameter management setting is7, to achieve reverse operation, it is necessary to simultaneously set the analog frequency negative value reversal (F5-38) to1(allowing negative frequency input, positive frequency forward rotation, negative frequency reverse rotation; theDigital Operatorand external terminals cannot control forward/reverse rotation).

3.4.6 Command Source Setting (AUTO)

In automatic operation mode(AUTO/REMOTE), users can select the frequency source (F0-06) and the run command source (F0-05) through function code settings to ensure that the inverter operates normally under different input command methods, as shown in Table 3-38.

Table 3-38 Parameters related to command source setting

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F0-06 | Frequency command source selection | 0 | 0: Digital Operator 1: RS485 Communication 2: Analog Input 3: External Up/Down Input 4: Pulse Input Without Direction 5: Pulse Input With Direction 6: CANopen Input 8: Communication Card Input 9: PID 10: Digital Terminal Multi-Speed | 0: Set the frequency value via the keypad; set the frequency value at the “F xx.xx Hz” display on the main interface. Use the up and down buttons to move the arrow icon to the 'F xx.xx Hz' position, then press the 'OK' button, the frequency value will flash. Modify the frequency value using the left and right arrows and the up and down arrows, and press the 'ESC' key to exit after modification. 1: Set the frequency value via RS485 communication; the master device must be connected to the inverter's 485+ and 485-, then the frequency can be set through the communication method. For details, refer to Appendix A MODBUS Communication Protocol. 2: Set the frequency value via an analog signal. The frequency value can be set through three analog input channels on the inverter; it is necessary to set the analog input function of the corresponding channel to “frequency setting” (set through F5-21, F5-27, or F5-33). 3: Set the frequency value via DI terminals; set the F5 group DI terminal functions to 19 and 20 (external command for frequency increase, external command for frequency decrease), then use the corresponding DI terminals to adjust the frequency value. 4: Reference pulse input type selection (F4-30). 5: Reference pulse input type selection (F4-30). 6: Set the frequency value through CANopen communication; the master device uses the CANopen protocol to communicate with the inverter and set its frequency value. 7: Reserved 8: Set the frequency value through a communication card; the inverter must be equipped with a communication card to communicate with the master device, which can use the corresponding bus communication protocol to set the inverter's frequency. 9: The frequency setting value comes from process PID control. The PID function can be configured using the FA group function codes; the output of the PID controller serves as the inverter's frequency setting value. For details, refer to the introduction of the 'PID function' in the FA group. 10: Set the frequency value through DI terminals. The F5 group 'Input Terminals' and FD group 'Multi-Speed and Simple PLC Function' function codes can be used to set multi-speeds and the correspondence between DI input signals and multi-speeds. For details, refer to the introductions of these two groups of function codes. |
| F0-05 | Run Command Source Selection | 0 | 0: Digital Operator 1: External Terminal Input 2: RS485 Communication Input 3: CANopen Input 5: Communication Card Input | 0: Select this command channel to control the inverter's operation and shutdown via the RUN, STOP, JOG buttons on the keyboard. 1: Select this command channel to control the inverter through digital input terminals. 2: Select this command channel to send commands to the inverter via RS485 communication, controlling the inverter to start, stop, etc. 3: Select this command channel; the inverter requires a CANopen communication card, allowing the master station to send control commands using the CANopen protocol. 4: Reserved 5: Select this command channel; the inverter requires other communication cards (such as Profinet, Profibus-DP, EtherCAT communication cards), allowing the master station to send control commands using the corresponding bus communication protocol. |

Precautions:

1. The command source can only be modified when the inverter is stopped;

2. The command source needs to be set through the multifunctional input terminals by selecting theAUTOmode;

3. Some command source methods require additional accessories, such as communication cards;

4. The factory default frequency and operation command source is set toAUTOmode, and it will revert toAUTOmode upon power-up. If the multifunctional input terminal P is configured for switching betweenHANDandAUTO, the mode will be determined based on the status of the multifunctional input terminal. That is, if the external terminal is in theOFFstate, the inverter will not accept any operation signals and cannot executeJOGcommands;

5. At the same time, configure digital inputs (F5-00 ~ F5-06) as multi-speed commands for frequency sources, and configure analog inputs (F5-21, F5-27, F5-33) as frequency command sources. The priority order is: multi-speed> analog input frequency source > frequency command source selection input (F0-06);

6. When digital inputs (F5-00 ~ F5-06) are used as operation command sources, the priority order is: digital inputs > operation command source selection (F0-05).

3.4.7 Stopping Methods

Users can select different normal stopping methods, such as deceleration stop and free-run stop, based on actual application scenarios through function codes. The stopping method can be modified during operation. After the inverter receives a shutdown command, it reduces the output frequency according to the selected stopping method until the output stops. Two stopping methods are shown in Figure 3-24:

Figure 3-24 Stopping Method Diagram

Relevant parameters are shown in Table 3-39:

Table 3-39 Relevant Parameters Table

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-12 | Stopping Method | 0 | 0: Deceleration Stop 1: Free Stop | 0: Deceleration Stop, the inverter decelerates according to the set deceleration time until it reaches 0 or the first multi-point VF frequency point M1 (F2-04) and then stops output drive; 1: Free Stop, after receiving the shutdown command, the inverter immediately stops output, and the motor freely runs down to a stop according to the load inertia. |

Precautions:

1. When mechanical stopping is required, to avoid personal injury or material waste, it is recommended to set it to deceleration stop, and the length of the deceleration time should be determined based on the characteristics of the site debugging.

2. When mechanical stopping is required, if the load inertia is very small and there is no requirement for the motor stop time, or if the motor idling has no impact, and the load inertia is large, it is recommended to set it to free stop, such as in applications like fans, punch presses, etc.

3.4.8 Forward/Reverse Rotation Prohibition Selection

In some application scenarios, the motor can only operate in one direction; if it operates in the opposite direction to the predetermined direction, it could cause equipment damage or other unknown losses. The motor's operating direction can be limited through software. When the motor phase sequence is connected incorrectly, the desired motor rotation can be switched by adjusting the inverter output drive phase sequence without changing the actual wiring order. The diagram for forward/reverse rotation prohibition selection is shown in Figure 3-25:

Figure 3-25 Diagram of Forward/Reverse Rotation Prohibition Selection

The phase sequence switching diagram is shown in Figure 3-26:

Figure 3-26 Phase Sequence Switching Diagram

Related parameters as shown in Table 3-40:

Table 3-40 Forward/Reverse Rotation Prohibition Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F0-09 | Forward/Reverse Prohibited Selection | 0 | 0: Forward/Reverse Enabled 1: Reverse Prohibited 2: Forward Prohibited | Setting value to 0: Normal output according to command direction; Setting value to 1: Only forward rotation regardless of set direction; Setting value to 2: Only reverse rotation regardless of set direction; |
| F7-42 | Output Phase Sequence Switching | 0 | 0~1 | 0: Output according to the command direction. 1: Output frequency opposite to the command direction; forward rotation changes to reverse, and reverse changes to forward. Note: The output phase sequence switching and forward/reverse prohibition selection can be used together. However, if the prohibited rotation direction set for operation is the same as the direction after phase sequence switching, the output phase sequence switching function will not be realized. |

Precautions:

1. Do not modify the forward/reverse prohibition selection during VFD operation;

2. Do not modify the output phase sequence switching during VFD operation;

Output phase sequence switching and forward/reverse prohibition selection can be used together; however, if the prohibited rotation direction setting matches the direction after phase sequence switching, the output phase sequence switching function will not be realized.

3.4.9 Auxiliary Frequency Function (AUTO)

In some application scenarios, without changing the main frequency command, the auxiliary frequency command can be used to correct the main frequency command. Relevant parameters are shown in Table 3-41.

Table 3-41 Parameters for Auxiliary Frequency Function

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F0-07 | Auxiliary Frequency Source | 0 | 0: Disabled 1: Digital Operator 2: RS485 Communication 3: Analog Input 4: External Up/Down Input 5: Pulse Input 6: CANopen Input 8: Communication Card Input | 0: Disable; turn off the auxiliary frequency function. 1: Digital Operator; Set the auxiliary frequency through the keyboard, set the frequency value at the “Fxx.xxHz” display on the main interface. Use the up and down buttons to move the arrow icon to the 'F xx.xx Hz' position, then press the 'OK' button, the frequency value will flash. Modify the frequency value using the left and right arrows and the up and down arrows, and press the 'ESC' key to exit after modification. 2: RS485 Communication; Set the frequency value through RS485 communication; the host computer needs to connect to the inverter's 485+, 485- terminals, and then the frequency can be set via communication. For details, refer to Appendix A MODBUS Communication Protocol. 3: Analog Input; set the frequency value through an analog signal. The frequency value can be set through three analog input channels on the inverter. The corresponding channel's analog input function must be set to 'Auxiliary Frequency Setting' (set through F5-21, F5-27, or F5-33). 4: External Up/Down Input; Set the frequency value through DI terminals by setting the DI terminal function in group F5 to 19, 20 (external command for frequency increase, external command for frequency decrease), then adjust the frequency value using the corresponding DI terminals. 5: Pulse Input; refer to pulse input type selection (F4-30). 6: CANopen input; Selecting this command channel requires the installation of a CANopen communication card in the inverter, allowing the master device to send control commands to the inverter using the CANopen protocol. 7: Reserved 8: Communication card input; Set the frequency value through a communication card; the inverter must have a communication card installed to communicate with the master device, which can use the corresponding bus communication protocol to set the inverter's frequency. |
| F0-08 | Frequency Source Superposition Selection | 0 | 0: Main frequency + Auxiliary frequency 1: Main frequency - Auxiliary frequency 2: Auxiliary frequency - Main frequency 3: Auxiliary Frequency | 0: Main frequency + Auxiliary frequency; The actual set frequency of the inverter is the sum of the main frequency and the auxiliary frequency. 1: Main frequency - Auxiliary frequency; The actual set frequency of the inverter is the main frequency minus the auxiliary frequency. 2: Auxiliary frequency - Main frequency; The actual set frequency of the inverter is the auxiliary frequency minus the main frequency. 3: Auxiliary frequency; The actual set frequency of the inverter is the auxiliary frequency. |

Precautions:

1.The auxiliary frequency source can only be modified when the inverter is stopped;

2.The main and auxiliary frequency function selection can only be modified when the inverter is stopped;

3. Some command source methods require additional accessories, such as communication cards;

4. The main frequency and auxiliary frequency cannot be set to the same source;

5. When the digital input is configured as auxiliary frequency prohibition (F5-00 ~ F5-06), the auxiliary frequency output function will be disabled;

6. If the result of the subtraction between the main and auxiliary frequencies is negative, negative frequency input must be allowed (F5-38 set to 1) to reverse; otherwise, the output frequency will be 0. Additionally, pay attention to the setting of the main and auxiliary reversal cutoff frequency (FA-27) to avoid the desired reverse output being limited.

3.4.10 Rebound Energy Suppression

When the motor decelerates, the inverter may return energy, causing the bus voltage to rise. If the bus voltage is too high, it can lead to overvoltage faults. Rebound energy suppression can increase motor losses, reduce energy feedback, thereby lowering the probability of overvoltage faults, while also allowing the motor to decelerate more quickly.

During deceleration, increasing the motor's magnetic field/flux/excitation current can increase motor losses and reduce the energy fed back to the inverter. This function has two implementation forms,“Deceleration Method 1”and“Deceleration Method 2”. “Overvoltage Energy Suppression”will decelerate according to the set deceleration time value; the actual fastest deceleration time will not be less than the set deceleration time. “Traction energy control, automatically adjusts output frequency and output voltage to accelerate consumption of regenerative energy based on the inverter's capability; deceleration time is the result of automatic adjustment by the inverter. Relevant parameters are shown in Table 3-42.

Table 3-42 Parameters related to regenerative energy suppression

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-13 | Deceleration Method | 0 | 0~2 | 0:Nofunction; normal deceleration or stop, without regenerative energy suppression. 1: Deceleration method 1; During deceleration, the inverter will automatically adjust the deceleration curve based on the overvoltage stall threshold F9-04 and the bus voltage. 2: Deceleration method 2; During deceleration, when the bus voltage is too high, deceleration method 2 will be activated, and the inverter will automatically adjust the output frequency and output voltage to achieve the purpose of accelerating the consumption of regenerative energy. |

3.4.11 Zero Speed Operation Selection

This function determines the operating mode of the inverter when the given frequency is less than the minimum output frequency of the motor. At this time, there are three operating modes for the inverter:This function determines the operating mode of the inverter when the given frequency is less than the minimum output frequency of the motor. In this case, there are three operating modes for the inverter:

1. Output Wait, which means not operating, UVWthree phases have no output;

2. Zero Speed Operation, which means running at a given frequency of zero;

3. Run at the minimum frequency, as the name suggests, means running at the given frequency as the minimum frequency.

The parameters related to zero speed operation selection are shown in Table 3-43.

Table 3-43 Parameters Related to Zero Speed Operation Selection

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| L2-19 | Zero Speed Operation Selection | 0 | 0: Wait for Output 1: Zero Speed Position Control Output 2: Output at Minimum Frequency | 0: When the control mode is VF or VVC, the output waits, and the inverter will enter a waiting state (UVWNovoltage output). Under other control modes, zero-speed operation is performed. 1: When the control mode is induction motor VF, the inverter will enter an operating state, but the actual output is zero; When the control mode is VVC, the inverterworksin DC braking state, and the motor current is DC; under other control modes, zero-speed operation is performed. 2: The inverter will operate according to the set values of multi-point VF frequency point 1 (F2-04) and multi-point VF voltage point 1 (F2-05). |

3.4.12 Frequency Setting Processing

Process the frequency setpoint sent from the application layer to the drive layer to ensure it is within a reasonable range. When the frequency setpoint exceeds the maximum allowable frequency, the frequency setpoint is set to the maximum frequency. When the frequency setpoint is less than the minimum allowable frequency, it is set to the minimum frequency or zero according to the operating mode.Frequency setpoint processingRelevant parameters are shown in Table 3-44.

Table 3-44 Parameters related to frequency setpoint processing

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| L2-19 | Zero Speed Operation Selection | 0 | 0: Wait for Output 1: Zero Speed Position Control Output 2: Output at Minimum Frequency | When the given frequency is less than the 'Minimum Motor Output Frequency Setting,' the output mode is determined by this parameter. |
| F2-04 | M1 Multi-point VF Frequency Point 1 | 0.50Hz | 0.00~599.00 Hz | This parameter, together with 'Zero Speed Operation Selection,' determines the motor operation mode when the given frequency is below this parameter. At the same time, this parameter, together with the “Lower Limit Frequency,” determines the lower limit value of the given frequency. |
| H1-00 | M2 Multi-point VF Frequency Point 1 |  |  |  |
| H1-08 | M3 Multi-point VF Frequency Point 1 |  |  |  |
| H1-16 | M4 Multi-point VF Frequency Point 1 |  |  |  |
| F0-10 | Upper Limit Frequency | 599.00Hz | 0.0~599.00Hz | The upper limit of the given frequency; when setting this parameter, it must be greater than the “Lower Limit Frequency.” |
| F0-11 | Lower Limit Frequency | 0.00Hz | 0.0~599.00Hz | The lower limit of the given frequency; together with the “Minimum Motor Output Frequency Setting,” it determines the lower limit value of the given frequency. When setting this parameter, it must be less than the “Upper Limit Frequency.” |

3.4.13 External Frequency Command Limitation

If the inverter receives a frequency command that exceeds the upper limit frequency set by the parameter “Maximum Motor Operating Frequency,” the frequency command will be modified to the upper limit frequency set by the parameter “Maximum Motor Operating Frequency.” Relevant parameters are shown in Table 3-45.

Table 3-45 Parameters Related to External Frequency Command Limitation

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F4-02 | M1 Maximum Frequency | 50.00 or 60.0 Hz | 0.00~599.00Hz | This parameter is used to set the highest operating frequency of the inverter. If the input frequency command exceeds the value set by this parameter, the frequency command will be modified to this value. This parameter cannot be modified during operation. It can be selected which group of parameters to use through H0-00, or it can be chosen via digital input functions 83/84. |
| H0-01 | M2 Maximum Frequency |  |  |  |
| H0-13 | M3 Maximum Frequency |  |  |  |
| H0-22 | M4 Maximum Frequency |  |  |  |

3.4.14 Speed Curve

There are two modes for the speed curve: normal acceleration/deceleration mode and S-curve acceleration/deceleration mode. When all four function codes are zero, the speed curve operates in normal acceleration/deceleration mode; otherwise, it operates in S-curve acceleration/deceleration mode.

In general acceleration and deceleration modes, the slope of acceleration and deceleration will undergo sudden changes at the start or end of the process, causing significant variations in motor torque within a short period, which can impact both the motor and the load. To reduce this impact, S-curve acceleration and deceleration mode can be used. Under this mode, the slope of the speed curve is continuous, thereby reducing torque shock. Figure 3-27 shows the frequency and acceleration waveforms during normal acceleration and deceleration. It can be seen that the acceleration is discontinuous, with sudden changes occurring. In contrast, the acceleration during S-curve acceleration and deceleration, as shown in Figure 3-28, is continuously variable, thus using an S-curve for acceleration and deceleration can reduce the impact caused by sudden torque changes.

Fig.3-27 General Acceleration and Deceleration Mode

Fig.3-28  S Acceleration and Deceleration Mode

In S acceleration and deceleration mode, the total acceleration and deceleration time will belonger. When acceleratingfromzerotothemaximumoperatingfrequency, the total acceleration time= set acceleration time + (S acceleration time 1)/2 + (S acceleration time 2)/2, the total deceleration time = set deceleration time + (S deceleration time 3)/2 + (S deceleration time 4)/2. As shown in Fig. 3-29, S1 represents S acceleration time 1, S2 represents S acceleration time 2, S3 represents S deceleration time 3, and S4 represents S deceleration time 4.

Fig. 3-29  S Acceleration and Deceleration Time

The relevant parameters are shown in Table 3-46:

Table 3-46 Parameters Related to Speed Curves

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-16 | S Acceleration Time 1 | 0.2s | 0.00~25.00s (when F0-12=0) or 0.0~250.0s (when F0-12=1) | The above parameters are used to set the S acceleration and deceleration times. |
| F1-17 | S Acceleration Time 2 |  |  |  |
| F1-18 | S Deceleration Time 1 |  |  |  |
| F1-19 | S Deceleration Time 2 |  |  |  |
| F0-12 | Speed curve time unit | 0 | 0: Unit 0.01 seconds 1: Unit 0.1 seconds | This parameter is used to set the unit for speed curve time. |

3.4.15 Frequency hopping function

Whenoperating near the mechanical resonance point of the load, it may cause mechanical resonance of the load, thereby degrading control performance, and in severe cases, it may damage the load. To avoid resonance, the skip frequency function can be used to make the motor bypass mechanical resonance points. When the set frequency is within the frequency hopping range, the actual operating frequency will run at the frequency hopping boundary closest to the set frequency. By setting the frequency hopping points, the inverter avoids the mechanical resonance point of the load. This inverter can set 4 frequency hopping points. If two adjacent frequency hopping points are set to the same value, this function will not work at that frequency. The skip frequency function is shown in Figure 3-30, and related parameters are listed in Table 3-47.

Figure 3-30 Skip Frequency Function

Table 3-47 Parameters for Skip Frequency Function

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F7-09 | Jump Frequency 1 Upper Limit | 0.00Hz | 0.00~599.00Hz | These eight parameters are used to set prohibited frequency ranges; the inverter's frequency setting will skip these ranges, but the frequency output remains continuous. During acceleration and deceleration, the inverter's output frequency will still pass through the prohibited operating frequency ranges. |
| F7-10 | Jump Frequency 1 Lower Limit |  |  |  |
| F7-11 | Jump Frequency 2 Upper Limit |  |  |  |
| F7-12 | Jump Frequency 2 Lower Limit |  |  |  |
| F7-13 | Jump Frequency 3 Upper Limit |  |  |  |
| F7-14 | Jump frequency 3 lower limit |  |  |  |
| F7-15 | Jump frequency 4 upper limit |  |  |  |
| F7-16 | Jump frequency 4 lower limit |  |  |  |

3.4.16 Automatic Acceleration/Deceleration

In practical applications, the setting of acceleration and deceleration times is influenced by factors such as load conditions and motor inertia, and may require multiple adjustments to determine. If the acceleration time is too short, it may cause excessive acceleration current, leading to overcurrent; If the deceleration time is too short, it may result in excessively high bus voltage, causing overvoltage. The automatic acceleration/deceleration function can automatically adjust the acceleration and deceleration times based on actual conditions, simplifying the commissioning process.

The automatic acceleration/deceleration function consists of two parts: automatic acceleration and automatic deceleration. The automatic acceleration function allows the motor to accelerate to the set frequency at the fastest acceleration time while ensuring a stable starting current and preventing stalling. The automatic deceleration function allows the motor to decelerate to the set frequency or stop operation at the fastest deceleration time while ensuring that overvoltage does not occur in the absence of a braking resistor. The automatic acceleration function and the automatic deceleration function can be individually enabled or disabled to meet different requirements, as shown in Table 3-48.

Table 3-48 Parameters Related to Automatic Acceleration/Deceleration

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-20 | Automatic Acceleration and Deceleration Selection | 0 | 0: Linear Acceleration and Deceleration 1: Automatic acceleration with linear deceleration; 2: Linear acceleration with automatic deceleration; 3: Automatic acceleration and deceleration; 4: Automatic Acceleration/Deceleration Suppression. | 0: Linear Acceleration/Deceleration; both automatic acceleration and deceleration functions are disabled. 1: Automatic Acceleration with Linear Deceleration; automatic acceleration function is enabled, automatic deceleration function is disabled. 2: Linear Acceleration with Automatic Deceleration; automatic acceleration function is disabled, automatic deceleration function is enabled. 3: Automatic Acceleration/Deceleration; both automatic acceleration and deceleration functions are enabled. 4: Automatic Acceleration/Deceleration Suppression; when current is too high, the automatic acceleration function is enabled; when bus voltage is too high, the automatic deceleration function is enabled; in other cases, both automatic acceleration and deceleration functions are disabled. |
| F1-21 | Automatic Acceleration and Deceleration Kp | 2 | 0~65535 | When the automatic acceleration/deceleration function is enabled, this setting is used to configure the PI regulator parameters to adjust the acceleration/deceleration slope. |
| F1-22 | Automatic Acceleration and Deceleration Ki | 400 | 0~65535 |  |

3.4.17 Overcurrent Stall

3.4.17.1 Overcurrent Stall During Acceleration

Generally speaking, the faster the acceleration, the greater the torque current required. If the acceleration is too fast, it may lead to excessive motor current. To protect the motor and inverter, it is necessary to limit the current. The overcurrent stall function during acceleration can prevent overcurrent situations caused by too rapid acceleration.

The principle of the overcurrent stall function during acceleration is shown in Figure 3-31. When the detected current exceeds the set value, the inverter stops accelerating until the current drops below the set value, at which point the inverter resumes acceleration.

Figure 3-31 Overcurrent Stall During Acceleration

For the weak sector overcurrent stall prevention level, please refer to parameterF9-09explanation. The protection curve is shown in Figure 3-32.

Figure 3-32 Weak Sector Current Stall

When the overcurrent stall prevention function is activated, the inverter's acceleration time will be longer than the set time. If the system enters a stall state due to an undersized motor or operating under factory settings, reduce the parameterF9-08set value.

If the acceleration time cannot be extended due to this function, it needs to be disabled. To prevent overcurrent, the following measures can be taken:

1.appropriately increase the acceleration time;

2.Set parameterF1-20automatic acceleration/deceleration selection to1,3or4for automatic acceleration.

Relevant parameters are shown in Table 3-49as follows:

Table 3-49 Parameters Related to Overcurrent Stall During Acceleration

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-08 | Acceleration OC Stall Threshold | 180% | 0~200% | This parameter is used to set the stall prevention threshold during acceleration, the unit is %, based on the rated current of the inverter. When the inverter accelerates, if the output current exceeds the value set by F9-08, the inverter will stop accelerating. When the current falls below the value set by F9-08, the inverter resumes acceleration to the set frequency. |
| F9-09 | Overcurrent Stall Limit Threshold | 100% | 0~100% | This parameter is used to set the overcurrent stall prevention threshold during acceleration when demagnetization occurs. When the motor operating frequency is greater than the rated frequency, the overcurrent stall prevention threshold during acceleration equals the value set by F9-08 multiplied by the value set by F9-09. |

3.4.17.2 Overcurrent Stall During Operation

Generally speaking, if the motor load is greater, the motor current will also be greater. If the motor load is too high, it can lead to excessive motor current. If the motor load exceeds the motor's capacity, it may even result in loss of control. The overcurrent stall function during operation can prevent the aforementioned situations of excessive current or loss of control.

The principle of overcurrent flux weakening function during operation is shown in Figure 3-33, and the relevant parameters are shown in Table 3-50. When the motor current exceeds the set value, the inverter begins to decelerate until the current drops below the allowable value, at which point the motor reaccelerates to the set frequency.

Figure 3-33 Overcurrent Flux Weakening During Operation

Table 3-50 Parameters for Overcurrent Flux Weakening During Operation

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-10 | Running OC Stall Threshold | 180% | 0~200% | This parameter is used to set the threshold for preventing overcurrent flux weakening during operation, with the unit being %, based on the rated current of the inverter. When the inverter is running, if the output current exceeds the F9-10 setting, the inverter will decelerate according to the acceleration/deceleration time selected by F9-11 to prevent motor stall. When the output current is below 95% of the value set in F9-10, the inverter will reaccelerate to the set frequency according to the acceleration/deceleration time selected in parameter F9-11. |
| F9-11 | Constant Speed OC Acceleration/Deceleration Selection | 0 | 0: System Acceleration/Deceleration Time; 1: First Acceleration/Deceleration Time; 2: Second Acceleration/Deceleration Time; 3: Third Acceleration/Deceleration Time; 4: Fourth Acceleration/Deceleration Time; 5: Automatic Acceleration/Deceleration Time. | This parameter is used to select the acceleration/deceleration time for overcurrent stall action. |

3.4.18 Overvoltage Stall

When the motor decelerates, it may feed energy back to the inverter, causing the inverter's bus voltage to rise. If the bus voltage is too high, it may lead to an overvoltage fault. The overvoltage stall function adjusts the slope of the speed curve to avoid overvoltage as much as possible.

When an excessively high bus voltage is detected, the motor deceleration slope is reduced. When the bus voltage is not excessively high, the normal deceleration slope is maintained or the maximum deceleration slope is used for deceleration. In this way, the bus voltage can be maintained within a reasonable range during motor deceleration, avoiding overvoltage conditions.

Overvoltage stall has several implementation methods; Figure 3-34 shows one method. When the bus voltage reaches the overvoltage detection level, the inverter stops decelerating, at which point the bus voltage begins to drop. Once it falls to the overvoltage recovery level, the inverter resumes deceleration.

Figure 3-34 Overvoltage Stall Prevention Method 1

When overvoltage stall prevention is activated, the inverter's deceleration time will be longer than the set time. If automatic adjustment of the deceleration time is not allowed, this function must be disabled. To prevent overvoltage, the following measures can be taken:

1.Reasonably increase deceleration time;

2.Install braking resistor to dissipate motor feedback energy.

Figure 3-35shows another implementation method, where during motor deceleration, the inverter's deceleration slope is controlled to ensure that the bus voltage does not exceed the set value. Relevant parameters are shown in Table 3-51.

Figure 3-35 Overvoltage Stall Prevention Method 2

Table 3-51 Overvoltage Stall Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-03 | Overvoltage Stall Mode | 0 | 0: Overvoltage Stall Mode 0 1: Overvoltage Stall Mode 1 | 0: Overvoltage stall mode 0; If the inverter detects that the bus voltage is higher than the set value of F9-04, the inverter will stop decelerating (the output frequency remains unchanged) until the bus voltage is lower than the set value of F9-58, at which point the inverter will continue to decelerate. 1: Overvoltage stall mode 1; During the deceleration process, dynamically adjust the speed curve to prevent overvoltage faults due to excessively high bus voltage. |
| F9-04 | overvoltage stall threshold | 760.0V | 0.0~900.0V | When the set value is 0.0, the overvoltage stall prevention function is disabled. When the inverter is equipped with a braking unit and connected to a braking resistor, it is recommended to use this setting. When the set value is not 0.0, the overvoltage stall prevention function is effective. This parameter can be set according to the power supply and load conditions; if set too low, it may extend the deceleration time. When the set value exceeds the overvoltage protection point, the overvoltage stall prevention function is considered to be disabled. |
| F9-05 | overvoltage stall deceleration time sec | 600.00 | 0.00~655.35 | overvoltage stall deceleration time |

3.4.19 Incremental PG Card

Related parameters are shown in Table 3-52:

Table 3-52 Incremental PG Card Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F4-27 | Encoder Type Selection | 0 | 0: No PG Card 1: ABZ Encoder 2: Reserved 3: Resolver | Used to select the encoder type. |
| F4-28 | Encoder Pulse Count | 1024 | 6~20000 | This parameter is used to set the number of lines for the selected incremental encoder, indicating the number of A/B signal pulses output per revolution of the encoder. The higher this value, the higher the resolution of the encoder, and the higher the speed and position measurement accuracy. If this parameter is set incorrectly, it can cause overcurrent in the inverter output and motor overspeed during closed-loop speed control. When using a permanent magnet motor, it can also result in incorrect initial rotor position identification. When changing the encoder results in a change in the number of encoder lines, it is necessary to perform the rotor position dynamic identification again (parameter F4-01=4). |
| F4-29 | Pulse input type | 0 | 0: No Function 1: A phase leading B phase indicates forward rotation; 2: B phase leading A phase indicates forward rotation; 3: A phase pulse B phase direction HL | Used to set the pulse input type. |

3.4.20 PID function

PID principle block diagram as shown in Figure 3-36:

Figure 3-36 PID principle block diagram

PID series control block diagram as shown in Figure 3-37:

Figure 3-37 PID series control block diagram

PID parallel control block diagram as shown in Figure 3-38:

Figure 3-38 PIDParallel Control Block Diagram

Relevant parameters are shown in Table 3-53:

Table 3-53PID Function Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| FA-00 | PID Feedback Type Selection | 0 | 0: No Function 1: Negative Feedback Analog Input 2: Negative Feedback Pulse No Direction 3: Negative Feedback Pulse With Direction 4: Positive Feedback Pulse Input 5: Positive Feedback Pulse No Direction 6: Positive Feedback Pulse With Direction 7: Negative Feedback Communication Input 8: Positive Feedback Communication Input | Positive Feedback: If the feedback value is less than the PID setpoint, the inverter output frequency increases; Negative Feedback: If the feedback value is less than the PID setpoint, the inverter output frequency decreases. |
| FA-01 | PID Setpoint Source Selection | 0 | 0: Frequency Command 1: Parameter FA-02 2: RS485 Communication 3: Analog Input 4: CANopen 5: Retain 6: Communication Card | This parameter selects the channel for the PID target value. |
| FA-02 | PID Setpoint | 50.00% | -100.00%～100.00% | When FA01 (PID setpoint source) is set to 1, this parameter needs to be configured. This parameter is a relative value, where 100% corresponds to the maximum feedback value of the controlled system.- |
| FA-03 | PID Setpoint Change Time | 0.00s | 0.00s～655.35s | The time required for the PID setpoint (parameter FA-02) to change from 0.0% to 100.0%. When the PID setpoint changes, the actual setpoint does not respond immediately but changes linearly over the given time to prevent sudden changes in the setpoint. |
| FA-04 | PID Feedback Filter Time | 5.0s | 0.1s～300.0s | Filters the PID feedback value; this parameter helps reduce the impact of disturbances on the feedback but may degrade the response performance of the closed-loop control process. |
| FA-05 | Proportional Coefficient 1 | 88.00% | 0.00%～100.00% | The speed at which the deviation decreases depends on the proportional coefficient; the larger the proportional coefficient, the faster the deviation decreases. However, an excessively large proportional coefficient can lead to significant overshoot and oscillation, reducing stability, especially in systems with large lag. Reducing the proportional coefficient decreases the likelihood of system oscillation but slows down the response speed. When performing 2ms enhanced PID control (parameter FA-12 = 0), the number of decimal places for this parameter can be selected by parameter FA-53 bit 1, 0: 1 decimal place, 1: 2 decimal places. |
| FA-06 | Integral Time 1 | 0.05s | 0.00s～100.00s | This parameter determines the strength of the integral action of the PID controller. The smaller the integral time, the stronger the integral action, which helps to reduce overshoot, decrease oscillation, and stabilize the system, but it will slow down the elimination of static error in the system. |
| FA-07 | Differential Time 1 | 0.00s | 0.00s～1.00s | This parameter determines the strength of the rate-of-change adjustment of the PID controller. The longer the differential time, the greater the adjustment strength. When set appropriately, this parameter can reduce overshoot and shorten the adjustment time. Differential action amplifies noise interference, so excessive differential regulation is detrimental to the system's ability to resist interference. Additionally, when there is no change in the input, the differential action output is zero. Therefore, differential control is often combined with the other two control laws to form a PD controller or a PID controller. |
| FA-08 | Proportional Coefficient 2 | 100.00 | 0.00～100.00 | Refer to parameter FA-05; no further details are provided here. |
| FA-09 | Integral Time 2 | 0.08s | 0.00s～100.00s | Refer to parameter FA-06; no further details are provided here. |
| FA-10 | Differential Time 2 | 0.00s | 0.00s～1.00s | Refer to parameter FA-07; no further details are provided here. |
| FA-11 | PID Series/Parallel Selection | 1 | 0: Kp, Kp*Ki, Kp*Kd 1: Kp, Ki, Kd | 0: Series, traditional PID control structure 1: Parallel, which separates proportional control, integral control, and differential control, allowing users to adjust P, I, and D controllers separately according to application needs. The parallel type PID is selected by default. |
| FA-12 | PID Control Execution Cycle | 0 | 0: Execute 2ms Enhanced PID Control 1: Execute 1ms Traditional PID Control | FA-12 = 0, the user selects to execute process PID control once every 2ms cycle, the PID output frequency reference base can be selected according to parameter FA-30, to choose 100.00% output corresponding to the maximum output frequency, or to choose 100.00% output corresponding to the auxiliary frequency. For example, if the user enables the main and auxiliary frequency function (assuming selecting main frequency + auxiliary frequency, parameter F0-07 = 1, F0-08 = 0), and the PID output frequency base selects the auxiliary frequency (FA-30 = 1), and the keyboard sets the auxiliary frequency to 40Hz, then the maximum PID output frequency is 40Hz. FA-12 = 1, the user selects a 1ms cycle to execute the process PID control once, where a PID output of 100.00% corresponds to the maximum output frequency, and there is no auxiliary frequency option for the reference benchmark. Additionally, the enhanced PID option (FA-12 = 0) supports soft start and automatic switching of two sets of PID parameters based on output frequency or deviation, while the traditional PID option (FA-12 = 1) does not have these features. |
| FA-13 | PID Parameter Switching Conditions | 0 | 0: No Function 1: Switch Based on Output Frequency 2: Switch Based on PID Deviation | 0: Use the first set of PID parameters FA-05 ~ FA-07 1: Automatically adjust according to the output frequency. Use the first set of PID parameters (FA-05 ~ FA-07) when operating at the minimum frequency (F2-04) or below, use the second set of PID parameters (FA-08 ~ FA-10) when operating at the maximum frequency (F4-02), and use a linear interpolation value of the two sets of PID parameters when the operating frequency is between F2-04 and F4-02. 2: Automatically switch based on the deviation between the setpoint and feedback. Use the first set of PID parameters (FA-05 ~ FA-07) when the absolute value of the deviation between the setpoint and feedback is less than the PID parameter switching deviation 1 (parameter FA-14). Use the second set of PID parameters (FA-08 ~ FA-10) when the absolute value of the deviation between the setpoint and feedback is greater than the PID parameter switching deviation 2 (parameter FA-15). When the absolute value of the deviation between the setpoint and feedback varies between FA-14 ~ FA-15, the PID parameters are the linear interpolation values of the two sets of PID parameters. |
| FA-14 | PID Parameter Switching err1 | 10.00% | 0.00%～100.00% |  |
| FA-15 | PID Parameter Switching Error 2 | 40.00% | 0.00%～100.00% |  |
| FA-16 | Allow PID Reverse Delay | 0.0s | 0.0s～6553.5s | When parameter FA-16 ≠ 0, the reverse operation function is enabled after startup. For example, if FA-16 is set to 2.0, PID control is not allowed to change the direction of operation during the start-up period of 0 ~ 2 seconds (parameter FA-17 = 0), and after 2 seconds of start-up, PID control is automatically allowed to change the direction of operation (parameter FA-17 will be automatically updated to 1). |
| FA-17 | PID Direction Change Selection | 0 | 0: Disabled 1: Enable | 0: Non-reversible running direction 1: Reversible running direction |
| FA-18 | Feedback Suppression Deviation Rate | 10% | 0%～65535% | When the setpoint and feedback deviation are in opposite directions, parameters FA-18 and FA-19 provide feedback suppression anti-integral windup functionality to quickly exit the saturation state and respond to external inputs, avoiding the controller output lingering in the saturation zone for a long time, thus improving the controller's response capability. The controller performs anti-integral windup suppression based on the relationship between the feedback suppression deviation rate (parameter FA-18) and the 100ms deviation rate. |
| FA-19 | Feedback Suppression Gain | 800 | 0～1000 |  |
| FA-20 | PID Compensation Selection | 0 | 0: Parameter Setting 1: Analog Input | When FA-20 = 0, the PID compensation value (parameter FA-21) must be set. |
| FA-21 | PID Compensation Value | 0.0 | -100.0～100.0 | The reference for this parameter is the maximum output frequency F4-02. Example: If the maximum output frequency parameter F4-02 = 50.00 Hz, and FA-21 is 10.0%, the PID compensation will increase the output frequency by 5.00 Hz. |
| FA-22 | PID Deviation Deadband Limit | 0.06% | 0.00%～100.00% | When the PID control output exceeds FA-22, the PID regulation output becomes effective; otherwise, the PID regulator is inhibited. This parameter effectively prevents the actuator from oscillating when the PID output is small. |
| FA-23 | PID Control Deviation Limit | 0.00% | 0.00%～100.00% | This parameter determines the level at which the deviation between the feedback and setpoint signals causes the PID regulation to stop, maintaining the previous output value. PID regulation output is only executed when the deviation between the feedback value and the setpoint exceeds the PID control deviation limit FA-23. Properly setting this parameter can adjust the accuracy and stability of the PID system. The functional diagram is as follows: |
| FA-24 | Integral Separation Level | 0.00% | 0.00%～100.00% | When the PID feedback overshoot is large at startup, integral separation can be enabled to reduce overshoot, with the parameter benchmark being the PID deviation. When FA-24 ≠ 0, the integral separation function is activated, and it only operates once at startup. When the deviation between the setpoint and the feedback value exceeds the parameter FA-24, integral separation occurs to prevent excessive overshoot due to integral action; When the deviation is less than parameter FA-24, the integral action takes effect to eliminate steady-state error. |
| FA-25 | Integral Upper Limit | 100.0% | 0.00%～100.00% | This parameter is the upper limit of integration, with the reference being the maximum output frequency F4-02. When the integral value is too large, if the load suddenly changes, the response speed of the inverter slows down, which may cause motor slippage or mechanical damage. At this time, the parameter FA-25 can be appropriately reduced. |
| FA-26 | Wake-up Integral Limit | 50.0% | 0.0%～200.0% | This parameter is the upper limit of wake-up integration, used to reduce the reaction time from sleep to wake-up, with the reference being the maximum output frequency F4-02. |
| FA-27 | Main Auxiliary Reverse Cut-off Frequency | 10.0% | 0.00%～100.00% | In some cases, only when the PID output frequency is negative (i.e., the inverter runs in reverse), can the PID possibly control the setpoint and feedback to the same state. However, an excessive reverse frequency is not allowed in certain situations. Parameter FA-27 is used to determine the upper limit of the reverse frequency. The reference base for this parameter is the maximum output frequency F4-02. |
| FA-28 | PID Output Positive Limit | 100.0% | 0.00%～100.00% | This parameter is the upper limit of the PID control output command, with the reference base being the maximum output frequency F4-02. |
| FA-29 | PID Output Negative Limit | 100.0% | 0.00%～100.00% | When PID output reversal is allowed, the PID output is negative, and at this time, the output will be limited to the value set by parameter FA-29, which should be used in conjunction with parameter FA-17. |
| FA-30 | PID output frequency reference | 0 | 0～1 | 0: PID control output 100.00% corresponds to the maximum output frequency F4-02; 1: PID control output 100.00% corresponds to the auxiliary frequency (if the auxiliary frequency command changes, the PID output frequency will also change). |
| FA-31 | PID output filter time | 0.0s | 0.0s～2.5s | This parameter is used to set the low-pass filter time for PID control output; the larger the parameter value, the greater the PID output filtering, and the slower the change in output frequency. Improper setting of parameter FA-31 may affect the response speed of the inverter, and even cause system oscillation. |
| FA-32 | Soft Start - PID Switching Value | 5.00% | 0.00%～100.00% | The reference for this parameter is the deviation between the PID setpoint and feedback value. |
| FA-33 | Soft Start Frequency | 0.00Hz | 0.00Hz～599.00Hz | When parameter FA-35 ≠ 0 and the inverter output current is greater than FA-35, it starts at the soft start frequency (parameter FA-33) and the soft start acceleration step (parameter FA-36), until the soft start acceleration time (parameter FA-34) is reached, then switches to normal PID control. |
| FA-34 | soft start acceleration time | 3.00s | 0.00s～600.00s |  |
| FA-35 | No-load Current | 0.00A | 0.00A～655.35A |  |
| FA-36 | Soft Start Acceleration Step | 0.10s | 0.00s～600.00s |  |
| FA-37 | Fuzzy PID Tuning Enable | 1 | 0: Fuzzy PID Tuning Disabled 1: Fuzzy PID Tuning Enabled | 0: Fuzzy PID disabled, use only conventional PID 1: When PID control is not allowed to change the direction of operation (FA-17 = 0), enable fuzzy PID control, and perform real-time self-tuning of PID parameters during operation. Fuzzy PID control is enabled only when the air compressor application is selected (parameter L0-00 = 6) and 2ms enhanced PID control (parameter FA-12 = 0) is executed, or 1ms traditional PID control (parameter FA-12 = 1) is executed. |
| FA-38 | Deviation Fuzzy Universe of Discourse - NB | 5.00 | 0.00～100.00 | Membership value of the linguistic term 'negative large' in the fuzzy set domain of the error variable. |
| FA-39 | Deviation Fuzzy Domain-NS | 2.00 | 0.00～100.00 | Membership value of the linguistic term 'negative small' in the fuzzy set domain of the error variable. |
| FA-40 | Deviation Fuzzy Domain-PS | 2.00 | 0.00～100.00 | Membership value of the linguistic term 'positive small' in the fuzzy set domain of the error variable. |
| FA-41 | Deviation Fuzzy Domain-PB | 5.00 | 0.00～100.00 | The membership value of the linguistic term “Positive Large” in the domain of the deviation fuzzy variable fuzzy set. |
| FA-42 | Deviation Rate Fuzzy Domain-NB | 10.00 | 0.00～100.00 | The membership value of the linguistic term “Negative Large” in the domain of the deviation rate fuzzy variable fuzzy set. |
| FA-43 | Deviation Rate Fuzzy Domain-NS | 5.00 | 0.00～100.00 | The membership value of the linguistic term “Negative Small” in the domain of the deviation rate fuzzy variable fuzzy set. |
| FA-44 | Deviation Rate Fuzzy Domain-PS | 5.00 | 0.00～100.00 | The membership value of the linguistic term “Positive Small” in the domain of the deviation rate fuzzy variable fuzzy set. |
| FA-45 | Fuzzy Set of Deviation Rate PB | 10.00 | 0.00～100.00 | The membership value of the linguistic term “Positive Large” in the domain of the deviation rate fuzzy variable fuzzy set. |
| FA-46 | Fuzzy PID Inference Rules | 2 | 0～3 | 0: Fuzzy PID Inference Rule 0 1: Fuzzy PID Inference Rule 1 2: Fuzzy PID Inference Rule 2 3: Fuzzy PID Inference Rule 3 |
| FA-47 | Intermediate Value of Fuzzy Rule KP | 50 | 0～100 | Parameters FA-47 and FA-48 are used to obtain the initial membership values for the linguistic terms that are zero in the fuzzy control rule table for fuzzy variables such as Kp, Ki, and Kd. |
| FA-48 | Intermediate Value of Fuzzy Rule KI | 50 | 0～100 |  |
| FA-49 | Feedback Abnormality Detection Time | 0.0s | 0.0s～3600.0s | This parameter is used for detecting abnormal conditions or extremely slow response of the feedback analog signal; when FA-49 = 0, no detection is performed. When the sampled value of the analog signal is below the 4~20mA open-circuit threshold (parameter F5-43) for a duration exceeding FA-49, the inverter will feedback an abnormal analog signal and handle the fault according to the setting of parameter FA-50. |
| FA-50 | Feedback Disconnection Action Selection | 0 | 0: Warning and continue running; 1: Warning and decelerate to stop; 2: Warning and free stop; 3: Run at the frequency before the open circuit. | Used to set the handling method for feedback open-circuit faults |
| FA-51 | PID Feedback Abnormal Deviation | 10.0% | 1.0%～50.0% | When the deviation between the setpoint and feedback signal exceeds the threshold (parameter FA-51) and persists for longer than the deviation abnormality detection time FA-52, a PID deviation fault occurs. If the function selection for output terminals F6-00 ~ F6-03 is 15, the output terminal will indicate a PID deviation warning. |
| FA-52 | Abnormal Deviation Detection Time | 5.0s | 0.1s～300.0s |  |
| FA-53 | PID Control Flag | 2 | 0～65535 | PID control flag (bits 0 ~ 2 are valid) bit 0: PID reverse action selection, 0: PID reverse based on PID calculation value, 1: Reverse based on parameter F0-09. bit 1: Selection of decimal places for PID parameter Kp, 0: 1 decimal place, 1: 2 decimal places. bit 2: 0: No function, 1: When the main and auxiliary frequency function is enabled, the integral upper limit base value is the auxiliary frequency. |

3.5 Application Control

3.5.1 Jog Operation

Jog operation is a common function of inverters, often used during equipment debugging or when precise adjustments are needed. During jog operation, upon receiving a jog command, the inverter will control the motor to accelerate to the jogging frequency according to the jog acceleration time. After the jog command is removed, depending on the stopping method, the motor will decelerate and stop according to the jog deceleration time or use other stopping methods, as shown in Table 3-54.

Table 3-54 Jog Operation Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F7-00 | JOG frequency setting | 6.00Hz | 0.00~599.00Hz | The above parameters are used to set the jogging operation frequency, the time to accelerate from 0.00Hz to the jogging operation frequency, and the time to decelerate from the jogging operation frequency to0.00Hz. During jog operation, when the jog command is received, the inverter controls the motor to accelerate to the electric frequency according to the jog acceleration time. When the jog command is removed, depending on the different stop modes, the motor decelerates and stops according to the jog deceleration time or adopts other stop methods。 |
| F7-01 | JOG acceleration time | 10.00s | 0.00~600.00s or 0.0~6000.0s |  |
| F7-02 | JOG deceleration time | 10.00s | 0.00~600.00s or 0.0~6000.0s |  |
| F0-12 | Speed curve time unit | 0 | 0: Acceleration/Deceleration Unit 0.01 seconds 1: Acceleration/Deceleration Unit 0.1 seconds | This parameter is used to set the unit for speed curve time. |

3.5.2 Fan Control

Inverters are generally equipped with fans. When the fans operate, they can increase air circulation, thereby reducing the internal temperature of the inverter. The relevant parameters are shown in Table 3-55. The fan operation status can be determined by the inverter's operating status, module temperature, etc. Currently, several different fan operation modes are provided, which can be selected according to needs.

Table 3-55 Fan Control Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F7-17 | Fan control method | 3 | 0: Fan runs continuously; 1: STOP one minute after shutdown; 2: Stops running with the inverter; 3: Starts when the temperature reaches 50°C; 4: Stops when the temperature is below 50°C during shutdown. | 0: After power-on of the inverter, the fan runs continuously. 1: The fan starts when the inverter is running and stops 1 minute after the inverter stops. 2: The fan operates according to the inverter's operating status; it starts when the inverter is running and stops when the inverter stops. 3: When the module temperature > 50°C, the fan starts; When the module temperature < 40°C and the inverter stopsoperating, the fan turns off. 4: The fan starts when the inverter is running at variable frequency, and after the inverter stops, the fan turns off when the module temperature is below 50°C. |

3.5.3 Load Torque Compensation

In inverters, aPIregulator is commonly used for speed control. If a higher system speed response is required, the bandwidth of thePIregulator needs to be increased. However, in practical applications, the bandwidth of thePIregulator is subject to many constraints and may not be adjustable to the desired value. At this point, if it is still necessary to further improve the system speed response, load torque compensation can be added on top of thePIregulator.

A disturbance observer (DOB) is used to calculate the load torque, which is then added to the output of the speed loopPIregulator to achieve load torque compensation. The parameters related to load torque compensation are shown in Table 3-56.

Table 3-56 Parameters Related to Load Torque Compensation

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F3-13 | DOB Compensation Gain | 0 | 0~5000 | This parameter sets the magnitude of the load torque compensation gain. |

3.5.4 Base Block

When the external controller detects an abnormal condition, it may be necessary for the inverter to stop immediately; at this time, the base block function of the inverter needs to be used. When a base block signal is received, the inverter stopsPWMoutput, and after the base block signal is removed, the inverter resumes operation after a certain delay (which can be set through parameters). Additionally, if the inverter's stopping method is free stop, after stopping, it must wait for the time set by this function code before it can run again, as shown in Table 3-57.

Table 3-57 Base Block Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-31 | Base Blockade Interruption Time | 0.5s | 0.0~5.0s | Used to set the waiting time from when the inverter stops output to when it restarts. |

3.5.5 Bus Voltage Compensation

When the bus voltage changes, the output voltage may also change, which could affect the motor control performance. The bus voltage compensation function (Automatic Voltage Regulation function,AVR) can compensate for changes in output voltage caused by variations in bus voltage, making motor control performance more stable.

When the bus voltage increases, the output duty cycle is appropriately reduced, and when the bus voltage decreases, the output duty cycle is appropriately increased, thus maintaining a constant output voltage. In actual use, the duty cycle calculated by the control algorithm can be multiplied by the reciprocal of the bus voltage reference value and the actual value to achieve this. The bus voltage compensation related parameters are shown in Table 3-58.

Table 3-58 Bus Voltage Compensation Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F7-37 | Automatic Voltage Regulation | 0 | 0: Enable AVR function; 1: Disable AVR function; 2: Disable AVR during deceleration. | 0: AVR function enabled, the inverter calculates the output voltage based on the bus voltage, ensuring the output voltage does not fluctuate with changes in bus voltage. 1: AVR function disabled, the inverter does not calculate the output voltage based on the bus voltage, leading to fluctuations in output voltage and motor current due to bus voltage variations. 2: AVR function disabled during deceleration, but enabled in other states. Disabling AVR during deceleration can reduce the deceleration time. |

3.5.6 Frequency Holding

In applications such as elevators and cranes, to improve reliability or positioning accuracy, it is often necessary to run at a certain frequency for a period of time during startup before accelerating to the set frequency, and during shutdown, to run at a certain frequency for a period of time before decelerating to zero speed. The above can be achieved through the frequency holding function.

The frequency holding function includes start-up holding and stop holding, which process the speed curve during start-up and stop, respectively. As shown in Figure 3-39, the parameters for start-up holding frequency, start-up holding time, stop holding frequency, and stop holding time can be adjusted to meet different requirements.

Fig. 3-39 Frequency Holding

Parameters related to frequency holding are shown in Table 3-59:

Table 3-59 Parameters Related to Frequency Holding

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-06 | Startup Hold Time | 0.00s | 0.00~600.00s | F1-06 Start Holding Time and F6-07 Start Holding Frequency can be used to set the start frequency holding function. |
| F1-07 | Startup Hold Frequency | 0.00s | 0.00~599.00Hz |  |
| F1-26 | Stop Holding Time | 0.00s | 0.00~600.00s | F1-26 Stop Holding Time and F1-27 Stop Holding Frequency can be used to set the stop frequency holding function. |
| F1-27 | Parking Hold Frequency | 0.00s | 0.00~599.00Hz |  |

3.5.7 Start/Stop DC Braking

When the inverter starts, if the motor is still rotating, it may cause significant impact. To avoid this situation, the motor can be braked first, and then started at zero speed after it has stopped rotating. The DC braking function is activated by injecting DC current into the motor windings, which generates a braking torque to stop the motor, as shown in Figure 3-40.

When the inverter stops, there may be instances where the motor has not completely stopped. To prevent this, the stop DC braking function can be used to brake the motor after it has stopped, ensuring that the motor does not rotate after stopping. The stop DC braking function injects DC current into the motor windings during the stop process, generating a braking torque to achieve the purpose of braking. Start DC braking is typically applied in situations such as fans, where the motor may still be rotating when the inverter starts. In this case, start DC braking can be used to stop the motor before starting. Stop DC braking ensures that the motor stops rotating when parking.

Figure 3-40 Start/Stop DC Braking

Parameters related to start/stop DC braking are shown in Table 3-60:

Table 3-60 Start/Stop DC Braking

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-08 | Braking Current Level | 0 | 0~100% | Used to set the magnitude of the start and stop DC braking current, the unit is %, based on the motor's rated current. |
| F1-09 | Startup Braking Time | 0.0 | 0.0~60.0s | Used to set the duration of the start DC braking. When the start braking time is set to zero, the start DC braking is invalid. |
| F1-10 | DC Braking Ratio Coefficient | 2000 | 0~65535 | When using VF or induction motor VVC control, F1-10 DC braking ratio coefficient and F1-11 DC braking integral coefficient can be used to adjust the current PI regulator parameters. |
| F1-11 | DC Braking Integral Coefficient | 100 | 0~65535 |  |
| F1-24 | Braking Time | 0.0s | 0.0~60.0s | This parameter sets the duration of the stop DC braking. If DC braking is required during stopping, parameter F1-12 motor stop method must be set to deceleration stop for this function to be effective. When the stop brake time is set to 0.0, DC braking during stop is invalid. |
| F1-25 | Braking Start Frequency | 0.0Hz | 0.00~599.00Hz | This parameter is used to set the starting frequency for stop DC braking. When the inverter decelerates to the frequency value set by this function code, DC braking begins. |

3.5.8 Instantaneous Power Failure Restart

Generally, when power is lost, the inverter will shut down due to undervoltage, and even if the power returns to normal within a short period, the inverter will not restart automatically. The instantaneous power failure restart function can automatically start the inverter when power is restored after a brief outage, achieving the goal of continuous motor operation.

After an undervoltage fault occurs, if the bus voltage returns to normal within a certain time (which can be set through parameters), a speed search is performed, followed by controlling the motor to run at the set frequency. If the bus voltage does not return to normal within the specified time, the system will shut down. Parameters related to instantaneous power failure and restart are shown in Table 3-61.

Table 3-61 Instantaneous Power Failure and Restart

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-29 | Instant Power Failure Restart Method | 0 | 0: Stop operation; 1: Start speed tracking from the frequency before the power failure; 2: Start speed tracking from the minimum output frequency. | 0: Stop operation; in case of an instantaneous power failure, stop operation and cannot automatically restart. 1: Track current speed; The inverter starts from the frequency before power-off and performs speed tracking downwards. Once the motor frequency is detected, it accelerates from the detected motor frequency to the set frequency. It is recommended to select this option when the motor inertia is large. 2: minimum frequency tracking; The inverter starts from the lowest frequency and tracks upwards. Once the motor frequency is detected, it accelerates from the detected motor frequency to the set frequency. It is recommended to select this option when the motor inertia is small. |
| F1-30 | Allowed Power Failure Time | 2.0s | 0.0~20.0s | When the power outage duration is less than the value set for this function code, automatic restart according to the settings of F1-29 is allowed. If the power outage duration exceeds the time set for this function code, automatic restart will not be performed. |

3.5.9 Instant Stop Without Stopping

When the grid voltage drops, the inverter may shut down due to undervoltage. Uncontrolled abnormal shutdowns can cause significant impacts, leading to system damage and serious consequences. The instant stop without stopping function can, in such cases, allow the motor to decelerate in a controlled manner, reducing the impact on the system.

When a grid voltage sag (bus voltage below a certain value) is detected, the instantaneous stop prevention function starts working, adjusting the deceleration slope to put the motor into a power generation state, feeding energy back to the inverter, preventing the bus voltage from dropping rapidly, thus avoiding the inverter shutting down due to undervoltage.Instantaneous Stop PreventionRelevant parameters are shown in Table 3-62.

Table 3-62 Parameters for Instantaneous Stop Prevention

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-32 | dEb recovery threshold | 40.0V | 0.0~200.0V | This parameter is used to set the dEb recovery voltage value. By default, dEb recovery voltage = dEb action voltage + (F1-32 setting value). |
| F1-33 | dEb action bias threshold | 40.0V | 0.0~200.0V | This parameter is used to define the voltage value at which dEb begins to act. By default, dEb action voltage = undervoltage protection value + 60V + (F1-33 setting value). |
| F1-34 | dEb deceleration selection | 0 | 0: No Action 1: Enable, no recovery; 2: Enable, recovery; | 0: When the grid voltage drops, the inverter does not decelerate, and the inverter may stop due to undervoltage. 1: When the grid voltage drops, reaching the dEb action voltage, the inverter controls the motor to decelerate, and when the grid voltage returns to normal, it still controls the motor to decelerate to zero before stopping. 2: When the grid voltage drops, reaching the dEb action voltage, the inverter controls the motor to decelerate, and when the grid voltage returns to normal, it maintains the current frequency for a period (F1-35) before accelerating to the target frequency. |
| F1-35 | dEb recovery time | 3.0s | 0.0~25.0s | This parameter is used to set the time that the inverter maintains operation at the current frequency when power is restored. After this period ends, the inverter accelerates back to the given frequency. |
| L2-18 | Undervoltage Protection Value | 360.0V | 250.0~440.0V | A voltage below this level will trigger under-voltage protection. |

3.5.10 Speed Tracking

Speed tracking is applicable for punch presses, fans, and otherlargeinertia load applications. In control modes with an encoder, F1-00 must be set to a non-zero value for the inverter to start speed tracking based on the encoder feedback. When using free stop (that is, inertial stop) for parking, or when the inverter suddenly stops due to a fault, the motor will decelerate under the effect of friction. If the motor inertia is large, the time required for the motor to come to a complete stop will be longer. If the inverter starts directly while the motor has not completely stoppedrotating, it may cause significant impact. The speed tracking function can obtain the motor speed and then control the motor to run to the set frequency based on this speed. This not only shortens the restart time but also avoids excessive shock.

The principle of speed tracking is to inject a certain voltage or current signal into the motor, and determine the actual speed of the motor based on the current or required voltage combined with the motor model. After obtaining the actual speed, it is used as the speed setpoint, and then the motor runs along the normal speed curve to the set frequency. The parameters related to speed tracking are shown in Table 3-63:

Table 3-63 Parameters Related to Speed Tracking

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-00 | Startup Speed Tracking | 0 | 0: No action; 1: Track from maximum frequency; 2: Track from start-up frequency; 3: Track from minimum frequency. | 0: Speed tracking function disabled. 1: Track using the maximum output frequency; Upon restart, track the speed downward from the maximum output frequency until the motor speed is detected, then control the motor to run at the set frequency. 2: Upon restart, track the speed from the given frequency at startup, and ultimately control the motor to run at the set frequency. 3: Upon restart, track the speed from the minimum frequency, and after tracking the motor speed, control the motor to run at the set frequency. |
| F1-03 | Speed Tracking Maximum Current | 100% | 20~200% | The inverter starts speed tracking only when the output current exceeds this set value. The larger this parameter is set, the faster the tracking speed, but setting it too large may cause overcurrent or overload faults. |

3.5.11 Abnormal Restart

After an abnormality occurs, the inverter usually stops running and will only restart after the fault has been reset and a run command is received. The automatic restart function after an abnormality can automatically clear the fault and control the inverter to restart when a fault occurs, achieving the purpose of continuous motor operation, as shown in Table 3-64.

After a fault occurs, if the number of automatic restart attempts after an abnormality (F9-46) is not zero, the inverter fault will be cleared, and speed search will be performed, followed by controlling the motor to run at the set frequency. If the number of retries after an abnormality is zero, the fault will not be cleared, and the inverter will remain in a shutdown state.

Table 3-64 Parameters for Automatic Restart After Abnormality

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F1-02 | Abnormal Restart Method | 0 | 0: Stop operation; 1: Track current speed; 2: Minimum frequency tracking. | 0: Shut down after an abnormality, and the inverter does not automatically restart after the fault is reset. 1: When restarting after an abnormality, perform speed tracking from the current speed, and then control the motor to run at the set frequency after the speed is tracked. 2: When an abnormality occurs and the system restarts, it performs speed tracking from the minimum frequency. Once the speed is tracked, the motor is controlled to run to the set frequency. |
| F9-46 | Abnormal restart count | 0 | 0~10 | This parameter is used to set the number of automatic restarts after an abnormality. If set to zero, the inverter will not automatically restart after an abnormality. When automatically restarting after an abnormality, the inverter will start according to the settings in F1-02. If the number of abnormalities exceeds the value set in F9-46, the fault will not automatically reset. It must be manually reset and a new operation command received before it can continue running. |
| F9-47 | Abnormal Restart Reset Time | 60.0s | 0.0~6000.0s | After an abnormal restart, if no further abnormalities occur within the time set by this function code, the F9-46 abnormal restart count will be reset to the set value. |

3.5.12 Vibration Suppression

When usingVFcontrol to drive an asynchronous motor, there may be vibration issues. If the vibration is severe, it could lead to a shutdown, preventing normal operation. Therefore, when designingVFcontrol algorithms, vibration suppression functions are generally essential. Some self-tuning algorithms also require vibration suppression functions.

By appropriately adjusting the given frequency based on the motor's operating status, the purpose of vibration suppression can be achieved. Relevant parameters are shown in Table 3-65. The vibration suppression effect can be adjusted through the parameter“Oscillation Suppression Gain”. The higher the“Oscillation Suppression Gain”, the better the vibration suppression effect, but the dynamic performance of the motor will decrease. Conversely, the worse the vibration suppression effect, the better the dynamic performance of the motor. In actual use, this parameter can be adjusted according to specific needs.

Table 3-65 Parameters Related to Vibration Suppression

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F2-14 | Oscillation Suppression Gain | 1000 | 0~10000 | If oscillation occurs when the motor is under VF control, this parameter value can be adjusted to improve the oscillation condition. The greater the oscillation suppression gain, the better the oscillation suppression effect, but the dynamic performance of the motor will decrease; conversely, the poorer the vibration suppression effect, the better the dynamic performance of the motor. |

3.5.13 Braking Unit Control

When the motor is in generator mode and feeds energy back to the inverter, the bus voltage of the inverter will rise. If not controlled, it may trigger an overvoltage fault, preventing the inverter from continuing to operate. The braking unit can dissipate the energy fed back by the motor, thus avoiding excessive bus voltage. When using this function, an external braking resistor is required for the inverter. When the bus voltage is detected to be higher than the set voltage, the braking unit activates. At this point, the braking resistor is equivalent to being connected in parallel with the bus capacitor, thereby releasing the energy stored in the capacitor and reducing the bus voltage to achieve the purpose of limiting the bus voltage. The parameters related to the control of the braking unit are shown in Table 3-66:

Table 3-66 Parameters Related to Braking Unit Control

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| L2-17 | Braking Resistor Activation Voltage | 740.0V | 700.0~900.0 V | This parameter is used to set the activation voltage of the braking unit. Users can select an appropriate braking resistor to achieve optimal deceleration characteristics. |

3.5.14 Excessive slip

Slip is the difference between the synchronous speed of an asynchronous motor and the motor's actual speed; generally, the greater the load torque, the larger the slip. If excessive slip occurs, it may be due to an overload or other abnormal conditions, so timely action is necessary. Relevant parameters are shown in Table 3-67.

Detecting excessive slip is relatively straightforward; continuously monitor the slip value of the motor. When the absolute value of the slip exceeds a certain threshold (which can be set via parameters), take appropriate actions such as continuing operation, issuing a warning, or initiating a fault shutdown.

Table 3-67 Parameters related to excessive slip

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-25 | Excessive slip detection value | 0 | 0.0~100.0% | The unit for excessive slip detection is %, based on the motor's rated slip. When the actual slip of an asynchronous motor exceeds the set value F9-25 and persists for longer than the time set in F9-26, an over-slip fault is detected, and the inverter will take subsequent actions as determined by the setting in F9-27. |
| F9-26 | Excessive slip detection time | 1.0s | 0.0~10.0s |  |
| F9-27 | Excessive slip action selection | 0 | 0: Warning and continue running; 1: Warning and decelerate to stop; 2: Warning and free stop; 3: No warning. | 0: When the over-slip detection conditions are met, an over-slip warning is issued, but the machine does not stop. 1: When the over-slip detection conditions are met, an over-slip fault is reported, and the machine decelerates to stop. 2: When the conditions for excessive slip detection are met, an excessive slip fault is reported,automatic shutdown. 3: No excessive slip fault detection is performed. |

3.5.15 Automatic energy saving

Using a frequency converter to drive a motor is primarily aimed at energy conservation. Under the same torque output conditions, the voltage and current output by the frequency converter can vary; different scenarios correspond to different output powers of the frequency converter. If a combination of voltage and current can be selected to reduce the output power of the frequency converter, the goal of energy conservation is achieved.

While running, the frequency converter continuously calculates the output power and adjusts the output voltage based on the output power to achieve the minimum output power or maintain it near the minimum value. The entire process can be carried out automatically without external intervention, hence it is referred to as“automatic energy-saving”function. To ensure that the acceleration and deceleration performance is not affected by the automatic energy-saving function, this feature is disabled during acceleration and deceleration, and only activated during steady-speed operation. The parameters related to automatic energy-saving are shown in Table 3-68as follows:

Table 3-68 Parameters Related to Automatic Energy-Saving

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F7-35 | Automatic Energy Saving Setting | 0 | 0: Disabled 1: Enable | After the automatic energy-saving function is enabled, the voltage will be output normally during acceleration and deceleration. During constant-speed operation, the output voltage will be automatically adjusted based on the load condition. When the load is small, the output voltage will automatically decrease, thereby achieving the purpose of energy saving. |
| F7-36 | Energy Saving Gain | 100% | 10~1000% | When F7-35 is set to 1, this parameter can be used to adjust the energy-saving gain. The factory setting value is 100%. If the energy-saving effect is poor, F7-36 can be reduced; if the motor oscillates, F7-36 should be increased. |

3.6 Tension Control

This chapter introduces several tension control schemes, mainly including tension closed-loop speed mode, line speed closed-loop mode, tension open/closed-loop torque mode, etc., for a total of four control methods (corresponding toFB-00selection).

● Tension Closed-loop Speed Mode

Features: Through the use of a swing arm (floating roller) or tension sensor to feedback material tension, the inverter output frequency is adjusted in a closed loop to achieve constant control of the swing arm position or tension. Suitable for applications where there is a swing arm or tension sensor (where changes in speed have a slow impact on tension changes) and there is sufficient speed regulation margin.

The winder operates in closed-loop tension speed mode, with twoAIsignals receiving the position signal of the lever and the line speed signal of the main traction machine, respectively. Calculate the roll diameter through line speed, calculate the main frequency based on line speed, and adjust the output frequency together with the feedback loop of the rocker position. Compared to the main frequency+PIDauxiliary frequency of general-purpose inverters, due to the addition of roll diameter calculation, the main frequency setting can more accurately follow the changes in line speed, thus making the control of the lever position more stable.

● Constant linear speed control

Features: Suitable for occasions without master traction, one machine in the winding/unwinding process operates in constant linear speed mode, serving as the traction role. The winder also serves as the main traction unit. To maintain the material running at a constant line speed, it is necessary to obtain the roll diameter value. The diagram above shows two feasible methods:

■ Using theDIterminal input pulse signal, the roll diameter is calculated using the thickness accumulation method.

■ A speed measurement device is installed on the material, transmitting pulses to the inverter, which calculates the roll diameter based on the linear speed.

It is not necessary to use both of the above methods simultaneously as shown in the diagram; choose one of them or another feasible method. The winder operates in closed-loop linear speed mode, with oneAIreceiving the linear speed signal. By calculating the roll diameter based on line speed, the main frequency setting is determined according to the given operating line speed and the roll diameter, working together with the line speed feedback closed-loop regulation to decide the output frequency, achieving constant line speed operation without main traction. The unwinder can operate in one of the other three tension modes based on actual conditions.

● Closed-loop tension torque control

Features: Through feedback from a tension sensor, the inverter's output torque is adjusted in a closed loop to achieve constant tension control. This system integrates mode4open-loop tension control torque, using open-loop tension calculation to assist closed-loop tension regulation (in practice, closed-loop tension regulation is the primary method, but mode4open-loop tension control torque can be optionally added based on control performance). The tension accuracy is high, suitable for materials with strong elasticity or situations where there is no speed adjustment margin.

The winder operates in tension closed-loop torque mode, with twoAIsignals receiving tension sensor signals and the main drive line speed signal, respectively. In implementation, an open-loop torque setting plus tension closed-loop regulation method is adopted. The calculation of roll diameter in open-loop tension is retained, while inertia and friction compensation can be omitted due to the addition of closed-loop regulation (or can be set to optimize response speed).

For elastic materials (with speed regulation margin), it is also possible to consider using tension closed-loop speed mode (mode1) to avoid the occurrence of elastic oscillation.

● Open-loop tension torque control

Features: No need for a swing arm (floating roller) or tension sensor; no tension closed-loop, slightly lower tension accuracy; suitable for applications with low tension accuracy requirements.

The winder operates in open-loop tension torque mode, using line speed to calculate the roll diameter, and output torque is calculated through material tension. Based on the actual system conditions, friction torque compensation or acceleration/deceleration inertia torque compensation can be selected.

The above typical application methods are only intended to illustrate the scenarios suitable for the four tension control modes and do not strictly limit them to these forms. Under basic conditions being met, flexible handling according to actual conditions is possible.

The previous section introduced four typical applications of tension control, which are mainly limited by the roll diameter, line speed, and feedback of control variables, as shown in Table 3-69.

Table 3-69 CM680Tension Control Mode Essential Conditions

| Function/Limitation | Roll Diameter[1] | Line Speed | Control Quantity Feedback |
| --- | --- | --- | --- |
| Tension Closed-loop Speed Mode | Necessary | Necessary[2] | Necessary |
| Line Speed Closed-loop Mode | Necessary | Not Necessary | Not Necessary |
| Tension Closed-loop Torque Mode | Necessary | Not Necessary | Necessary |
| Tension Open-loop Torque Mode | Necessary | Not Necessary | Not Necessary |

Note:

[1]: If the roll diameter source is selected as the line speed calculation channel (FB-25=0), then line speed is a necessary condition.

[2]: Inertia compensation, friction compensation, and line speed are related; therefore, if you want to improve the accuracy of open-loop torque tension control, line speed is a necessary condition.

The given torque and frequency are the target control quantities for tension control. Users can refer to the data flow of the above two key variables in the overall block diagram, query some sub-function diagrams, and refer to relevant parameter descriptions to achieve the tension control function. ParameterFB-00set value determines the method of tension control implementation. The product integrates four tension control modes, as shown in Figures 3-41 to Figure3-44.

● FB-00 = 1, Tension Closed-loop Speed Mode

Figure 3-41 Tension Closed-loop Speed Mode

● FB-00 = 2，Line Speed Closed-loop Speed Mode

Figure 3-42 Line Speed Closed-loop Speed Mode

● FB-00 = 3，Tension Closed-loop Torque Mode

Figure 3-43 Tension Closed-loop Torque Mode

● FB-00 = 4，Tension Open-loop Torque Mode

Figure 3-44 Tension Open-loop Torque Mode

Tension control related parameters are shown in Table 3-70:

Table 3-70 Tension Control Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| FB-00 | Tension Control Mode Selection | 0 | 0～4 | 0: Tension control invalid, functions the same as a general inverter. 1: Tension closed-loop speed mode, requires tension/position detection and feedback, operates in speed mode, the inverter adjusts the output frequency by superimposing PID closed-loop calculations on the main frequency given based on line speed and roll diameter, to achieve stable set tension or position. Control methods can be selected from V/F control, sensorless vector control (SVC), and closed-loop speed sensor vector control (FVC). 2: Line speed closed-loop mode, the inverter adjusts the operating frequency according to changes in roll diameter, ensuring the system runs at a constant line speed. Control methods can be selected from V/F control, sensorless vector control (SVC), and closed-loop speed sensor vector control (FVC). 3: Tension closed-loop torque mode, requires tension detection and feedback, torque mode operation, the inverter adjusts by superimposing PID closed-loop calculation on the given torque, including acceleration/deceleration inertia torque compensation, friction torque compensation, etc., and can choose whether to superimpose mode 4 open-loop tension torque based on control effect. Select torque (TQC) control method to achieve ideal control results. 4: Open-loop tension torque mode, does not require tension/position detection and feedback, torque mode operation, the inverter controls the output torque to control the tension on the material. Select torque (TQC) control method to achieve ideal control results. |
| FB-01 | Curling Mode | 0 | 0: Winding mode 1: Unwinding mode | The tension taper function is invalid in unwinding mode. In winding mode, the roller diameter will increase; in unwinding mode, the roller diameter will decrease. |
| FB-02 | Load Side Mechanical Gear A | 100 | 1～65535 | Parameters FB-02, FB-03 are only applicable in tension control mode. |
| FB-03 | Load Side Mechanical Gear B | 100 | 1～65535 |  |
| FB-04 | PID Setpoint Source Selection | 0 | 0～2 | 0: Parameter FB-05 as PID target setpoint; 1: RS-485 communication settings; 2: Analog input, must first set AI1/AI2/AI3 function to tension PID target value (Parameter F5-21 or F5-27 or F5-33 = 17). |
| FB-05 | PID Target Setpoint | 50.0% | 0.0～100.0% | Effective when parameter FB-00 = 1 or 2. In line speed closed-loop speed mode (FB-00 = 2), AI1/AI2/AI3 functions must be set to line speed feedback (parameter F5-21 or F5-27 or F5-33 = 15), and FB-05 serves as the line speed PID setpoint. In other modes, when F5-21 or F5-27 or F5-33 = 15, FB-05 represents the actual line speed. In tension closed-loop speed mode (FB-00 = 1), this parameter range 0.0 ~ 100.0% corresponds to tension feedback voltage 0~10 V; in line speed closed-loop speed mode (FB-00 = 2), it corresponds to 0 ~ maximum line speed (parameter FB-18). |
| FB-06 | PID Feedback Source Selection | 0 | 0: Analog Input 1: Pulse Input | When FB-06 = 0, set the AI1/AI2/AI3 function to Tension PID Feedback Value (Parameter F5-21 or F5-27 or F5-33 = 14). When FB-06 = 1, set the number of pulses per meter (FB-20). |
| FB-07 | PID Parameter Adjustment Basis | 0 | 0: No Switching 1: Automatic Adjustment Based on Roll Diameter 2: Switching Based on Operating Frequency 3: Automatic Switching Based on Setpoint and Feedback Deviation | When FB-07=0, use the first set of PID parameters FB-08~ FB-09 When FB-07=1, use the first set of PID parameters FB-08~ FB-09 for empty rolls, and the second set of PID parameters FB-10~ FB-11 for full rolls. When the current roll diameter varies between the maximum roll diameter (FB-26) and the minimum roll diameter (FB-27), the PID parameters are linearly interpolated values between the two sets of PID parameters. FB-07=2, use the first set of PID parameters FB-08 ~ FB-09 when operating at the minimum frequency F2-04 and below, use the second set of PID parameters FB-10 ~ FB-11 when operating at the maximum frequency F4-02, and when the operating frequency varies between F2-04 ~ F4-02, the PID parameters are the linear interpolation values of the two sets of PID parameters. FB-07=3, use the first set of PID parameters FB-08 ~ FB-09 when the feedback value is at the lower limit FB-16, use the second set of PID parameters FB-10 ~ FB-11 when the feedback value is at the upper limit FB-17, and when the feedback value varies between FB-16 ~ FB-17, the PID parameters are the linear interpolation values of the two sets of PID parameters. |
| FB-08 | PID Proportional Coefficient 1 | 50.0 | 0.0～1000.0 | Depending on different customer requirements, the appropriate method from the table below can be selected: Tension Feedback Value; Loose 0~100% Tight; Loose 0~100% Tight. Winding; Positive Output; Negative Output. Unwinding; Negative Output; Positive Output. |
| FB-09 | PID Integral Time 1 | 1.00s | 0.00～500.00s |  |
| FB-10 | PID Proportional Coefficient 2 | 50.0 | 0.0～1000.0 |  |
| FB-11 | PID Integral Time 2 | 1.00s | 0.00～500.00s |  |
| FB-12 | PID Output Positive/Negative Selection | 0 | 0～1 |  |
| FB-13 | PID Output Positive Limit | 20.00% | 0.00～100.00% | Parameters FB-13 and FB-14 define the positive and negative limit percentages for PID control output, PID output limit range = FB-13 or FB-14 * maximum frequency parameter F4-02. |
| FB-14 | PID Output Negative Limit | 20.00% | 0.00～100.00% |  |
| FB-15 | PID Feedback Upper Limit Value | 100.0% | 0.0%～100.0% | Parameters FB-15 and FB-16 are only valid when FB-00 is set to 1 or 3. |
| FB-16 | PID Feedback Lower Limit | 0.0% | 0.0%～100.0% |  |
| FB-17 | Linear Speed Input Source | 0 | 0: No Input 1: Analog Input 2: Communication Setting 3: PULSE Input (from PG Card) 4: Invalid 5: PULSE Input via DI6/DI7 Digital Terminals | When FB-00 = 2, this parameter setting is invalid; When FB-17 = 1, must first set AI1/AI2/AI3 function to line speed feedback (F5-21/ F5-27/ F5-33 = 15); When FB-17 = 2, change the line speed set value through parameter FB-21; When FB-17 ≠ 2, the inverter temporarily stores the analog value or pulse command to parameter FB-21, at this time FB-21 is read-only. When FB-17 = 3 or 5, the parameter FB-20 pulses per meter must be set. |
| FB-18 | Maximum Linear Speed | 100.00 | 0.00～650.00 | This parameter sets the maximum linear speed. The actual linear speed value corresponding to inputs 100.0% on channels 1~5 of FB-17 (may not be the same as the maximum production line speed; avoid confusion when setting). When using linear speed to calculate reel diameter (FB-25 = 0), the larger this parameter, the larger the reel diameter value, and vice versa. Refer to this rule to adjust this parameter based on the deviation between the calculated reel diameter and the actual value. |
| FB-19 | Minimum Linear Speed | 0.00 | 0.0～650.00 | When the linear speed setting is less than parameter FB-19, the inverter will stop calculating the reel diameter and maintain the current reel diameter. |
| FB-20 | Pulses per Meter | 0.0 | 0.0～6500.0 | When the PID feedback source is selected as pulse input (FB-06 = 1), this parameter needs to be set; When the linear speed input source is PULSE input or PULSE input via digital terminals (FB-17 = 3 or 5), this parameter needs to be set. |
| FB-21 | current linear speed | 0.00 | 0.00～650.00 | When FB-17 = 2, change the line speed set value through parameter FB-21; When FB-17 ≠ 2, the inverter temporarily stores the analog value or pulse command in parameter FB-21, at which time FB-21 is read-only. |
| FB-22 | linear speed filter time | 0.10s | 0.00s～100.00s | This parameter is valid when the linear speed input source is PULSE input or PULSE input via digital terminals (FB-17 = 3 or 5), which can suppress linear speed oscillation. |
| FB-23 | linear speed acceleration time | 0.00s | 0.00s～655.35s | Parameters FB-23 and FB-24 are effective in the linear speed closed-loop speed mode (FB-00 = 2). |
| FB-24 | linear speed deceleration time | 0.00s | 0.00s～655.35s |  |
| FB-25 | winding diameter calculation method selection | 0 | 0～5 | When FB-25 = 1, it is necessary to first set the AI1/AI2/AI3 functions to reel diameter (F5-21/ F5-27/ F5-33 = 16), at this time 10 V corresponds to the maximum reel diameter parameter FB-26. When FB-25 = 2, a PG card must be connected, and the reel diameter is obtained through the encoder on the scroll bar. At this time, the signal pulses are connected to PG2 on the PG card, and the encoder type (F4-27) as well as the reference pulse input type (F4-30), the number of pulses per revolution (FB-32), the number of turns per layer (FB-33), and the material thickness (FB-34) are set to calculate the reel diameter. When FB-25 = 3, a PG card must be connected, and the roll diameter is calculated by reverse inference through the motor encoder and gear ratio. At this time, the signal pulse should be connected to PG1 on the PG card, and parameters such as gear ratio (FB-02 and FB-03), encoder type (F4-27), encoder pulses (F4-28), number of turns per layer (FB-33), and material thickness (FB-34) should be set to calculate the roll diameter. When FB-25 = 4 or 5, DI6 and DI7 functions are supported, and the reference pulse input type should be set to unidirectional input (F4-30 = 5). If the winding direction changes during operation, parameter FB-01 must also be modified. |
| FB-26 | Maximum Roll Diameter | 500.0mm | 1.0～6000.0mm | The relationship diagram for various roll diameter parameters FB-26, FB-27, FB-29/ FB-30/ FB-31, and FB-38 is as follows: Dmax: Maximum roll diameter, corresponding to parameter FB-26; D0: Empty roll diameter, corresponding to parameter FB-27; Dinit: Initial roll diameter, corresponding to parameter FB-29 or FB-30 or FB-31; Dcur: Current roll diameter, corresponding parameter FB-38. |
| FB-27 | Empty Roll Diameter | 100.0mm | 1.0～6000.0mm |  |
| FB-28 | Initial Roll Diameter Selection | 0 | 0～1 |  |
| FB-29 | Initial Roll Diameter 0 | 100.0mm | 0.0～6000.0mm |  |
| FB-30 | Initial Roll Diameter 1 | 100.0mm | 0.0～6000.0mm |  |
| FB-31 | Initial Roll Diameter 2 | 100.0mm | 0.0～6000.0mm |  |
| FB-32 | Pulses per Revolution | 1ppr | 1～60000ppr | This parameter indicates the number of pulses per revolution of the roll shaft. |
| FB-33 | Number of Coils per Layer | 1 | 1～10000 | This parameter indicates the number of revolutions required for one layer of material to be wound, generally set to 1 for wire and strip materials. |
| FB-34 | Material Thickness | 0.001 | 0.001～65.000 | The number of decimal places for this parameter can be selected by parameter FB-80 bit 6, 0: 3 decimal places, 1: 2 decimal places. |
| FB-35 | Roll Diameter Filter Time | 1.00s | 0.00s～100.00s | This parameter filters the calculated roll diameter results, suppressing fluctuations in the roll diameter, which can improve the instability of the roll diameter source (parameter FB-25). The larger the parameter setting, the smoother the calculated roll diameter value, but the greater the delay in roll diameter changes. Rule: When the roll diameter changes linearly, the time lag between the calculated roll diameter and the actual roll diameter is approximately equal to the value of this parameter. |
| FB-36 | Roll Diameter Compensation Enable | 0 | 0～1 | This parameter is only effective when the tension closed-loop speed mode (FB-00 = 1) is set and the line speed input source FB-17 is not zero. When the mechanical gear ratio or linear speed cannot reach the precise condition, set this parameter to compensate for the roll diameter. |
| FB-37 | Roll Diameter Calculation Delay Time | 0.0s | 0.0s～6553.5s | After the pre-drive signal is canceled, the roll diameter calculation restarts after this delay to avoid inaccurate roll diameter calculation results in a short time after the pre-drive ends, which could lead to system instability. |
| FB-38 | Current Roll Diameter | 100.0 | 1.0～6553.5 | When the inverter is not in a stopped state, this parameter is read-only. |
| FB-39 | Minimum Frequency for Roll Diameter Calculation | 1.00Hz | 0.00～599.00Hz | When the linear speed is lower than parameter FB-39, maintain the current roll diameter value; when the linear speed exceeds this parameter, perform the roll diameter calculation. For situations where the operating frequency is low or the roll diameter calculation during acceleration is inaccurate, this parameter can be appropriately set to resolve the issue. |
| FB-40 | Pre-drive Mode Selection | 0 | 0: No Function 1: Pre-drive for Winding Mode 2: Pre-drive for Unwinding Mode | At the moment of winding startup, to prevent the wire from being slack, the tension convergence time can be extended by setting it to pre-drive for winding mode (FB-40 = 1). During unwinding, it can be set to pre-drive for unwinding mode (FB-40 = 2), allowing the motor to rotate in reverse to actively tighten the material. At this time, the output frequency limit is the main and auxiliary frequency reversal cutoff frequency (parameter FA-27). Parameter FB-40 is only valid when parameter FB-00 = 1. |
| FB-41 | Pre-drive/PID Switching Point | 15.0% | 0.0%～100.0% | Tension feedback value 0~100% corresponds to tension from loose to tight, parameter FB-41 reference base: FB-05, when setting FB-05 = 50%, FB-41 = 10%, the pre-drive range is 0~40%. |
| FB-42 | soft start frequency | 2.00Hz | 0.00～599.00Hz | When FB-40 = 1 or 2, at the instant of winding start, the sum of the synchronous frequency calculated based on line speed and the soft start frequency FB-42 is used as the set frequency, and acceleration starts according to time FB-43. After the soft start acceleration time, the system runs with the set frequency being the sum of the synchronous frequency calculated based on line speed and the auxiliary frequency adjusted by PI control. |
| FB-43 | soft start acceleration time | 3.00s | 0.00s～600.00s |  |
| FB-44 | Broken Belt Detection Selection | 0 | 0～1 | When the line speed input source FB-17 is not 0, and the roll diameter is calculated through the line speed (FB-25 = 0), the setting of parameter FB-44 is valid. After enabling belt breakage detection, if the line speed exceeds parameter FB-45, the change in roll diameter exceeds parameter FB-46, and the time exceeds the time set by parameter FB-47, a belt breakage will occur. When the belt breaks, the inverter will display E144 and stop in free run mode. At the same time, it can be used with DO1 or DO2 output terminal functions (setting value 46) as a broken belt indicator. |
| FB-45 | Minimum Line Speed for Broken Belt Detection | 0.0 | 0.0～3000.0 |  |
| FB-46 | Belt Breakage Detection Roll Diameter Change | 100.0 mm | 1.0～6000.0mm |  |
| FB-47 | Belt Breakage Detection Time | 1.00s | 0.00s～100.00s |  |
| FB-48 | Tension Feedback Error Level | 100% | 0%～100% | If the deviation between the tension PID target setpoint and the tension PID feedback value exceeds the tension feedback error threshold (parameter FB-48), and the error duration exceeds the tension error detection time (parameter FB-49), a PID feedback deviation fault is generated, and the inverter reports fault “E147” according to the handling method specified in parameter FB-50. |
| FB-49 | Tension Error Detection Time | 0.5s | 0.0s～10.0s |  |
| FB-50 | Tension Error Abnormal Handling | 0 | 0: Warning and Continue Running 1: Warning and free stop 2: Warning and decelerate to stop |  |
| FB-51 | PID Output Gain | 100.0 | 0.0～200.0 | Set PID output gain |
| FB-52 | Tension Reference Source Selection | 0 | 0: RS485 Communication 1: Analog Input | When FB-52 = 0, the zero-speed tension parameter value FB-54 can be set via the operation panel through communication. When FB-52 = 1, the AI1/AI2/AI3 functions must be set to tension setting (F5-21/ F5-27/ F5-33 = 18), at this time FB-54 is read-only. |
| FB-53 | Maximum Tension value | 0 | 0～65535 | When parameter FB-52 = 1, parameter FB-54 is read-only, at this time the analog value 10V corresponds to the maximum tension value FB-53. |
| FB-54 | Tension Set Value | 0 | 0～65535 |  |
| FB-55 | Zero-speed Tension Reference Source | 0 | 0: No Input 1: Communication settings 2: Analog Input | When FB-55 = 1, the zero-speed tension parameter value FB-56 can be set via the operation panel through communication; When FB-55 = 2, AI1/AI2/AI3 functions must first be set to zero-speed tension setting (F5-21/ F5-27/ F5-33 = 19), at this time FB-56 is read-only. |
| FB-56 | Zero-speed Tension Setpoint | 0 | 0～65535 | When FB-55 is set to 2, parameter FB-56 is read-only, and the analog input of 10 V corresponds to the maximum tension parameter FB-53. |
| FB-57 | Zero Speed Tension Level | 5.00% | 0.00～100.00% | When bit 1 of the tension control flag parameter FB-80 is 0, if the operating frequency is less than FB-57, the tension value is the parameter FB-56, achieving static friction tension compensation. |
| FB-58 | Sliding Friction Compensation Tension | 0.0% | 0.0%～100.0% | During constant speed operation of the motor, the output torque is used not only to establish material tension but also to overcome rotational friction. When this part of the torque cannot be ignored, compensation for the friction torque is required. |
| FB-59 | Material Inertia Compensation Coefficient | 0 | 0～30000 | Material inertia compensation coefficient = material density × material width. Density unit: kg/m³, width unit: m. The material inertia on the reel changes with the reel diameter. Based on parameters FB-02, FB-03, FB-27, FB-38, FB-59, etc., the inverter automatically calculates the flywheel inertia of the material to obtain the inertia compensation torque. |
| FB-60 | Acceleration Inertia Compensation Gain | 0.0% | 0.0～1000.0% | In some cases, fine-tuning parameters FB-60 or FB-62 can optimize control performance. For example, during acceleration of the winding process, if the material tension is too low, parameter FB-60 can be increased to enhance the compensation effect; otherwise, reduce this parameter. The same applies to deceleration. |
| FB-61 | Inertia Compensation Filter Time | 5.00 | 0.00～100.00 |  |
| FB-62 | Deceleration Inertia Compensation Gain | 0.0% | 0.0～1000.0% |  |
| FB-63 | Tension Taper Curve Selection | 0 | 0: Taper Calculation Invalid 1: Curve Taper 2: Straight Taper 3: Multi-segment Curve Taper 4: Multi-segment Straight Taper | When FB-63 = 1, generate a curve based on parameter FB-65, and fine-tune the curve using parameter FB-66. When FB-63 = 2, generate a straight taper based on parameter FB-65. When FB-63 = 3, parameters FB-65, FB-69, and FB-70 determine the multi-segment curve taper, and parameters FB-67 and FB-68 determine the curve inflection points. When FB-63 = 4, parameters FB-65, FB-69, and FB-70 determine the multi-segment straight taper, and parameters FB-67 and FB-68 determine the straight inflection points. |
| FB-64 | Tension Taper Setting Source | 0 | 0～1 | 0: Communication settings 1: Input analog setting |
| FB-65 | Taper Set Value | 0% | 0%～100% | When the tension taper curve selects curve taper (FB-63 = 1), the curve is generated by the taper setting value FB-65 and fine-tuned by parameter FB-66. |
| FB-66 | Taper Curve Compensation Value | 0.0 | 0.0～6000.0 |  |
| FB-67 | Taper Calculation Roll Diameter 1 | 6000.0 | 1.0～6000.0 | Diagram showing the relationship between multi-segment linear taper output tension and roll diameter: |
| FB-68 | taper calculation roll diameter 2 | 6000.0 | 1.0～6000.0 |  |
| FB-69 | multi-segment taper value 1 | 0 | 0～100 |  |
| FB-70 | multi-segment taper value 2 | 0 | 0～100 |  |
| FB-71 | pre-drive frequency gain | 0.0% | -50.0%～50.0% | When the pre-drive terminal is effective, the inverter automatically calculates the output frequency based on the line speed detection value and the current roll diameter to match the line speed. |
| FB-72 | pre-drive acceleration time | 0.00s | 0.00s～655.35s | Parameters FB-72 ~ FB-73 are the time for accelerating from the soft start frequency (FB-42) to the pre-drive frequency or decelerating from the pre-drive frequency to the soft start frequency during automatic roll change. |
| FB-73 | pre-drive deceleration time | 0.00s | 0.00s～655.35s |  |
| FB-74 | speed limit gain | 100.0s | 0.0s～6553.5s | In tension control mode, when the analog detection value of line speed is also used as a speed limit (parameter F3-17 = 3), this parameter can be adjusted to change the size of the speed limit. |
| FB-75 | soft start/PI switch LSErr | 5 | 0～100 | Parameters FB-75 ~ FB-79 are reserved; their default values should be used, and users should not modify them, as improper settings may cause the winding system to become unbalanced. |
| FB-76 | soft start/PI switch LS1 | 0.4% | 0.0%～10.0% |  |
| FB-77 | soft start/PI switch LS2 | 0.7% | 0.0%～10.0% |  |
| FB-78 | Soft start/PI switch 1 Set | 2.0 | 0.0～40.0 |  |
| FB-79 | Unwinding Soft Start PI Switch Point | 5.0 | 0.0～40.0 |  |
| FB-80 | Tension Control Flag | 0 | 0～65535 |  |

3.7 Faults and Protection

3.7.1 IGBT Overtemperature Protection

When theIGBTtemperature exceeds the overtemperature protection point, anIGBTovertemperature fault is reported, and the motor stops running; When the temperature is detected to be below the over-temperature fault temperature but above the over-temperature warning threshold, an over-temperature fault alarm is triggered, and the motor can continue to operate. IGBT over-temperature protection parameters are shown in Table 3-71:

Table 3-71 IGBT Over-Temperature Protection Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-41 | Over-Temperature Warning Threshold | Depends on the model power | 0.0~110.0°C | In the inverter specification module, the defined module over-temperature protection point is 95°C, and the module over-temperature warning threshold is 90°C. When the module temperature exceeds the lower value between the module over-temperature warning threshold and the F9-41 set temperature, an IGBT overheating warning (A009) is issued; When the module temperature exceeds the over-temperature protection point, an IGBT over-temperature fault (E016) is reported. When an IGBT over-temperature warning occurs, it does not affect the operation of the inverter; at this time, cooling measures can be taken in advance to prevent the inverter from experiencing an over-temperature fault. |

3.7.2 Low Current Protection

Low current protection is designed to prevent the inverter from operating for extended periods under conditions where the current is below normal levels. Operating below normal current may not cause damage to the inverter, but such abnormal operation could result in the system failing to achieve the expected load-carrying capacity, as shown in Table 3-72.

The user can determine the undercurrent threshold based on the normal operating current and the low current operation time that the system can withstand, and decide the actions the inverter should take after a low current occurs, such as free stop, deceleration stop, or only issuing a warning message while the inverter continues to operate normally. The low current warning can be automatically cleared when the current exceeds the set low current threshold1%. Undercurrent protection does not activate when the inverter is in sleep or standby mode.

Table 3-72 Low Current Protection Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-22 | Low Current Set Threshold | 0.0% | 0.0~100.0% | F9-22 low current set threshold is in units of %, based on the rated current of the inverter. When the inverter output current is less than the F9-22 set value and persists for longer than the time set by F9-23, a low current fault is detected, and the inverter will determine subsequent actions according to the low current action method set by F9-24. |
| F9-23 | Low Current Detection Time | 0.00 | 0.00~360.00s |  |
| F9-24 | Low Current Action Method | 0 | 0: No Function 1: Alarm and free stop; 2: Alarm and second deceleration stop; 3: Alarm and continue running. | 0: Low current protection function is off, low current setting threshold (F9-22) and low current detection time (F9-23) are both invalid. 1: When the low current protection condition is met, report a low current fault,and automatically stop. 2: When the low current protection condition is met, report a low current fault, and decelerate to stop according to the seconddeceleration time. 3: When the low current protection condition is met, issue a low current warning, but do not stop. |

3.7.3 Motor overheat protection

By obtaining the motor temperature through sensors, the motor can be protected based on its temperature. When the motor temperature exceeds a certain value, the motor operation is stopped to prevent overheating damage. Thermistors are commonly used to measure motor temperature, and common thermistors includePTCresistors andPT100resistors. Parameters related to motor overheat protection are shown in Table 3-73.

Table 3-73 Parameters Related to Motor Overheat Protection

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-48 | PTC Action Selection | 0 | 0: Warning and continue running 1: Warning and decelerate to stop 2: Warning and free stop 3: No warning | This parameter is used to set the inverter action when PTC overheats. |
| F9-49 | PTC Threshold | 50.0% | 0.0~100.0% | This parameter is used to set the PTC overtemperature detection threshold, with the unit being %, and the reference value being the maximum analog input value. When using the PTC overheating detection function, the corresponding analog input terminal must be set to voltage signal input, and the function of this analog input terminal is “Thermistor PTC Input”. When the feedback voltage reaches the set value of F9-49, the inverter will operate according to the method set in F9-48. |
| F9-50 | PT Detection Threshold 1 | 5.000 | 0.000~10.000V | Used to set the PT100 over-temperature detection threshold. |
| F9-51 | PT Detection Threshold 2 | 7.000 | 0.000~10.000V |  |
| F9-52 | PT Voltage 1 Protection Frequency | 0.00Hz | 0.00~599.00Hz | When using the PT100 over-temperature detection function, the corresponding analog input terminal must be set to voltage signal input. The function of this analog input terminal is “Thermistor PT100 Value”. When the feedback voltage is less than the set value of F9-50, the motor operates normally; When the feedback voltage is between the set values of F9-50 and F9-51, the inverter will run to the set frequency of F9-52 after the set time of F9-52. When the feedback voltage exceeds the set value of F9-51, the inverter will operate according to the method set in F9-48. |
| F9-53 | PT Action Delay Time | 60s | 0~6000s |  |

3.7.4 Motor Overload Protection

By setting an appropriate inverse-time curve, the operating time of the motor under overload conditions is made less than the set overload protection time, thereby achieving overload protection for the motor and preventing damage due to overheating. When the overload time reaches the overload protection time, an overload fault will be reported. This function is not enabled by default. If you need to enable it, set the parameter“MotornOverload Protection Selection”to0or1,n=1,2,3or4.

Taking Motor 1 as an example, when F9-01 is set to 0, the overload protection curve for Motor 1 is shown in Figure 3-45. Here, T equals the set value of parameter F9-02,“Motor Current Percentage” refers to the ratio of the inverter output current to the motor rated current. When the motor fan is independently controlled, this inverse time curve can be selected. In this case, the fan speed is independent of the motor speed, so the cooling capacity does not decrease as the motor speed decreases, making the inverse time curve independent of the motor operating speed.

Figure3-45 Inverse Time Curve1

When F9-01 is set to 1, the overloadprotectioncurvefor Motor 1 is shown in Figure3-46. Among them,the “speed coefficient” is a function of motor speed, when the motor speed is greater than the rated speed, the speed coefficient equals 1, when the motor speed is less than the rated speed, the speed coefficient=1/(0.4+0.6*motor speed/motor rated speed).

Figure3-46 Inverse Time Curve2

When the motor uses coaxial cooling (fan connected to the motor shaft), the fan speed is the same as the motor speed, and a decrease in motor speed will reduce the fan's cooling capacity. In this case, it is recommended to set F9-01 to 1, the inverse time curve will be adjusted according to the motor speed, the motor overload capability will decrease with the reduction in speed, preventing overheating due to reduced fan cooling capacity. When the motor speed is zero, the fan stops rotating, at which point the speed factor is 2.5, and the corresponding overload curve is shown in Figure 3-47. From Figure3-47,it can be seen that when the current reaches 60% of the motor rated current, the motor will report an overload fault after running for T time.

Fig.3-47 Inverse Time Curve3

Parameters related to motor overload protection are shown in Table 3-74:

Table3-74 Parameters Related to Motor Overload Protection

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-01 | Motor 1 Overload Selection | 2 | 0: Constant Torque Output Motor 1: Variable Torque Output Motor 2: No Motor Overload Protection | This parameter is used to set the overload protection mode. |
| F9-02 | Motor 1 Overload Time | 60.0s | 30.0~600.0s | This parameter is used to set the overload protection time when the motor current is 150% of the rated current. |
| H3-03 | Motor 2 overload selection | 2 | 0: Constant Torque Output Motor 1: Variable Torque Output Motor 2: No Motor Overload Protection | Refer to Motor 1 Overload Protection |
| H3-04 | Motor 2 overload time | 60.0s | 30.0~600.0s |  |
| H3-08 | Motor 3 overload selection | 2 | 0: Constant Torque Output Motor 1: Variable Torque Output Motor 2: No Motor Overload Protection |  |
| H3-09 | Motor 3 overload time | 60.0s | 30.0~600.0s |  |
| H3-13 | Motor 4 Overload Selection | 2 | 0: Constant Torque Output Motor 1: Variable Torque Output Motor 2: No Motor Overload Protection |  |
| H3-14 | Motor 4 Overload Time | 60.0s | 30.0~600.0s |  |

3.7.5 Fault Recording Function

The fault record function records the name, power-on time, frequency, torque, voltage, current, and power device temperature at the moment of fault for a certain number of inverter faults, providing reference for subsequent fault diagnosis.

The current inverter software supports recording the last10fault codes, the power-on times of the last6fault occurrences, and physical quantity information.

Only faults that cause the inverter to shut down after they occur are recorded; undervoltage faults during shutdown are not recorded.

Fault record information is stored inEEPROM, and it is automatically updated and written toEEPROMeach time a fault occurs.

Parameters related to the fault recording function are shown in Table 3-75:

Table 3-75 Parameters Related to Fault Recording

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| U0-00 | Fault Record 1 | 0 | 0～65535 | This set of parameters is used to record the fault codes of the inverter's last 10 faults. The smaller the fault record number (the minimum value is 1), the closer the corresponding fault is to the present. For example, the most recent fault will always be displayed in U0-00, and each subsequent fault will increment the count of all previous faults by 1. If the number of faults exceeds 6 or 10, the earliest fault record information will be overwritten. For the fault codes corresponding to fault types, please refer to the fault table. |
| U0-01 | Fault Record 2 | 0 | 0～65535 |  |
| U0-02 | Fault Record 3 | 0 | 0～65535 |  |
| U0-03 | Fault Record 4 | 0 | 0～65535 |  |
| U0-04 | Fault Record 5 | 0 | 0～65535 |  |
| U0-05 | Fault Record 6 | 0 | 0～65535 |  |
| U0-06 | Fault Record 7 | 0 | 0～65535 |  |
| U0-07 | Fault Record 8 | 0 | 0～65535 |  |
| U0-08 | Fault Record 9 | 0 | 0～65535 |  |
| U0-09 | Fault Record 10 | 0 | 0～65535 |  |
| U0-10 | Fault Output 1 | 0 | 0～65535 | When the inverter fails, and the set values of parameters U0-10 to U0-13 match the fault code, the corresponding RLY1/RLY2 and DO1/DO2 outputs can be enabled by setting F6-00 to F6-03 to 35 to 38. The four fault outputs can operate independently without affecting each other. |
| U0-11 | Fault Output 2 | 0 | 0～65535 |  |
| U0-12 | Fault Output 3 | 0 | 0～65535 |  |
| U0-13 | Fault output 4 | 0 | 0～65535 |  |
| U0-14 | Fault 1 - Motor speed | 0 | -32767～32767 | Used to record information of the last 6 faults |
| U0-15 | Fault 1 - Torque command | 0 | -3276.7～3276.7 |  |
| U0-16 | Fault 1 - Input terminals | 0 | 0000H~FFFFH |  |
| U0-17 | Fault 1 - Output terminals | 0 | 0000H~FFFFH |  |
| U0-18 | Fault 1 - Inverter Status | 0 | 0～65535 |  |
| U0-19 | Fault 1 - Frequency Command | 0.00 | 0.00～655.35Hz |  |
| U0-20 | Fault 1 - Output Frequency | 0.00 | 0.00～599.00Hz |  |
| U0-21 | Fault 1 - Output Voltage | 0.0 | 0.0～6553.5V |  |
| U0-22 | Fault 1 - DC Voltage | 0.0 | 0.0～6553.5V |  |
| U0-23 | Fault 1-Output Current | 0.00 | 0.00～655.35A |  |
| U0-24 | Fault 1-IGBT Temperature | 0.0 | -3276.7～3276.7℃ |  |
| U0-25 | Fault 1-Capacitor Temperature | 0.0 | -3276.7～3276.7℃ |  |
| U0-26 | Fault 2-Output Frequency | 0.00 | 0.00～599.00Hz |  |
| U0-27 | Fault 2-Direct Current Voltage | 0.0 | 0.0～6553.5V |  |
| U0-28 | Fault 2-Output Current | 0.00 | 0.00～655.35A |  |
| U0-29 | Fault 2 - IGBT Temperature | 0.0 | -3276.7～3276.7℃ |  |
| U0-30 | Fault 3 - Output Frequency | 0.00 | 0.00～599.00Hz |  |
| U0-31 | Fault 3 - DC Voltage | 0.0 | 0.0～6553.5V |  |
| U0-32 | Fault 3 - Output Current | 0.00 | 0.00～655.35A |  |
| U0-33 | Fault 3 - IGBT Temperature | 0.0 | -3276.7～3276.7℃ |  |
| U0-34 | Fault 4 - Output Frequency | 0.00 | 0.00～599.00Hz |  |
| U0-35 | Fault 4-Direct Current Voltage | 0.0 | 0.0～6553.5V |  |
| U0-36 | Fault 4-Output Current | 0.00 | 0.00～655.35A |  |
| U0-37 | Fault 4-IGBT Temperature | 0.0 | -3276.7～3276.7℃ |  |
| U0-38 | Fault 5-Output Frequency | 0.00 | 0.00～599.00Hz |  |
| U0-39 | Fault 5-Direct Current Voltage | 0.0 | 0.0～6553.5V |  |
| U0-40 | Fault 5-Output Current | 0.00 | 0.00～655.35A |  |
| U0-41 | Fault 5-IGBT Temperature | 0.0 | -3276.7～3276.7℃ |  |
| U0-42 | Fault 6-Output Frequency | 0.00 | 0.00～599.00Hz |  |
| U0-43 | Fault 6-Direct Current Voltage | 0.0 | 0.0～6553.5V |  |
| U0-44 | Fault 6-Output Current | 0.00 | 0.00～655.35A |  |
| U0-45 | Fault 6-IGBT Temperature | 0.0 | -3276.7～3276.7℃ |  |
| U0-46 | Number of Days Since Fault 1 Occurred | 0 | 0～65535 |  |
| U0-47 | Fault 1 Occurrence Minutes | 0 | 0～1439 |  |
| U0-48 | Fault 2 Occurrence Days | 0 | 0～65535 |  |
| U0-49 | Fault 2 Occurrence Minutes | 0 | 0～1439 |  |
| U0-50 | Fault 3 Occurrence Days | 0 | 0～65535 |  |
| U0-51 | Fault 3 Occurrence Minutes | 0 | 0～1439 |  |
| U0-52 | Fault 4 Occurrence Days | 0 | 0～65535 |  |
| U0-53 | Minutes since Fault 4 occurred | 0 | 0～1439 |  |
| U0-54 | Days since Fault 5 occurred | 0 | 0～65535 |  |
| U0-55 | Minutes since Fault 5 occurred | 0 | 0～1439 |  |
| U0-56 | Days since Fault 6 occurred | 0 | 0～65535 |  |
| U0-57 | Minutes since Fault 6 occurred | 0 | 0～1439 |  |

3.7.6 Over-torque Detection

The over-torque detection function protects the motor and inverter by limiting the motor torque. When the current exceeds the over-torque detection threshold and persists for the set time, the over-torque signal is set; when the current is less than 95% of the over-torque detection threshold*95%, the over-torque signal is cleared. Based on the over-torque signal, the inverter will report an over-torque fault and stop operation, or it will report an over-torque warning and continue running. The over-torque detection parameters are shown in Table 3-76:

Table 3-76 Over-torque Detection Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-36 | Over Torque Selection 1 | 0 | 0: Not Detected 1: Constant speed detection continues running; 2: Constant speed detection stops running; 3: Running detection continues running; 4: Running detection stops running. | 0: Not Detected 1: During constant speed operation, if Motor 1 experiences over-torque, a warning is issued but operation continues. 2: During constant speed operation, if Motor 1 experiences over-torque, an over-torque fault is reported and operation stops. 3: During operation, if Motor 1 experiences over-torque, an alarm is issued but operation continues. 4: During operation, if Motor 1 experiences over-torque, an over-torque fault is reported and operation stops. |
| F9-37 | Over-torque threshold 1 | 120 | 10%~250% | When the inverter output current exceeds F9-37 (unit %, based on the inverter's rated current) and persists for longer than the set time of F9-38, the inverter will determine subsequent actions according to F9-36. When F9-36 is set to 1 or 3, if over-torque is detected, the inverter will display an over-torque warning, but the inverter will continue to run until the output current is less than 95% of the F9-37 setting, after which the warning will be cleared. When F9-36 is set to 2 or 4, if over-torque is detected, the inverter reports an over-torque fault and stops running until the fault is reset, after which operation can resume. |
| F9-38 | Over-torque time 1 | 0.1s | 0.0~60.0s |  |
| H3-00 | Over-torque selection 2 | 0 | 0: Not Detected 1: Constant speed detection continues running; 2: Constant speed detection stops running; 3: Running detection continues running; 4: Running detection stops running. | Motor 1 over-torque detection reference |
| H3-01 | Over-torque threshold 2 | 120 | 10%~250% |  |
| H3-02 | Over-torque time 2 | 0.1s | 0.0~60.0s |  |
| H3-05 | Over-torque selection 3 | 0 | 0: Not Detected 1: Constant speed detection continues running; 2: Constant speed detection stops running; 3: Running detection continues running; 4: Running detection stops running. |  |
| H3-06 | Over-torque threshold 3 | 120 | 10%~250% |  |
| H3-07 | Over-torque time 3 | 0.1s | 0.0~60.0s |  |
| H3-10 | Over-torque selection 4 | 0 | 0: Not Detected 1: Constant speed detection continues running; 2: Constant speed detection stops running; 3: Running detection continues running; 4: Running detection stops running. |  |
| H3-11 | Over-torque threshold 4 | 120 | 10%~250% |  |
| H3-12 | Over Torque Time 4 | 0.1s | 0.0~60.0s |  |

Taking Motor 1 as an example, when the inverter output current exceeds F9-37 (unit %, based on the inverter's rated current) and persists for more than the time set by F9-38, the inverter will determine subsequent actions according to F9-36. As shown in Figure 3-48, when F9-36 is 1 or 3, if over-torque is detected, the inverter will display an over-torque warning, but the inverter will continue to operate until the output current is less than 95% of the F9-37 set value, at which point the warning will be cleared. As shown in Figure3-49, when F9-36 is 2 or 4, if over-torque is detected, the inverter will report an over-torque fault and stop operation until the fault is reset, after which it can resume operation.

Fig.3-48 Schematic Diagram of Over-torque Detection

Fig.3-49 Schematic Diagram of Over-torque Detection

Motor2~4Over-torque handling can refer to Motor1, and will not be repeated.

3.7.8 Ground Protection

When a ground fault occurs, it may lead to excessive motor winding current, motor overheating, etc., which can severely damage the motor or inverter. Therefore, it is necessary to detect ground faults and address them promptly when they occur.

According to Kirchhoff's Current Law, the sum of the three-phase currents is zero, but when a ground fault occurs, the sum of the three-phase currents is no longer zero. This phenomenon can be used to determine whether a ground fault has occurred. To prevent misjudgment, it is usually considered that a ground fault has occurred only when the sum of the three-phase currents exceeds a certain value. The parameters related to ground protection are shown in Table 3-77:

Table 3-77 Ground Protection Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-20 | Ground Fault Current Threshold | 60.0% | 0.0~6553.5% | When the inverter detects that the three-phase imbalance of the output current exceeds the set value of parameter F9-20, the ground protection activates, and the inverter immediately stops output. |
| F9-21 | Ground Fault Filter Time | 0.10s | 0.00~655.35s | This parameter is used to adjust the low-pass filter time for ground fault detection; it generally does not need to be changed. |

3.7.9 Undervoltage Protection

When the input voltage is too low, the inverter may not operate correctly. When undervoltage is detected, the inverter should stop running to prevent abnormal conditions. This is known as undervoltage protection, and related parameters are shown in Table 3-78.

Generally, the inverter input voltage detection circuit can indirectly determine whether the input voltage is too low through the bus voltage. When the bus voltage is below the undervoltage protection value, an undervoltage fault is reported; when the bus voltage is above the undervoltage recovery value (undervoltage protection value+60V), the undervoltage fault is automatically cleared, or it can be cleared after receiving a reset command. Depending on the different operating states of the inverter, undervoltage faults can be categorized as undervoltage during stop (E014), undervoltage during acceleration (E011), undervoltage during deceleration (E012), and undervoltage during constant speed (E013).

Table 3-78 Undervoltage Protection Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| L2-18 | Undervoltage Protection Value | 360.0V | 300~440.0V | This parameter is used to set the undervoltage protection value. When the inverter bus voltage falls below the undervoltage protection value, an undervoltage fault will be triggered, stopping the output and initiating a free stop. If an undervoltage fault occurs while the inverter is running, the inverter will stop output and initiate a free stop. The fault type depends on the acceleration or deceleration state at the time of the fault, which can be undervoltage during acceleration (E011), undervoltage during deceleration (E012), or undervoltage during constant speed (E013). The undervoltage fault must be cleared by pressing the reset button (STOP). However, if instantaneous stop and restart is set, it will automatically recover; please refer to the relevant descriptions for parameter F1-29 Instantaneous Power Failure Restart Method (F1-29) and Allowed Power Failure Time (F1-30). If an under-voltage fault occurs while the inverter is stopped, it will display Under-voltage During Stop (E014). This fault will not be recorded and can automatically recover when the input voltage exceeds the under-voltage protection value or 60V. |
| F9-06 | undervoltage fault automatic reset | 0 | 0: Disabled 1: Enable | When this parameter is set to 1, Under-voltage During Acceleration (E011), Under-voltage During Deceleration (E012), and Under-voltage During Constant Speed (E013) can automatically recover. |

3.7.10 Output Phase Loss

When there is an abnormal connection between the motor and the inverter, output phase loss may occur. When output phase loss occurs, the current in the disconnected phase of the motor is zero. To maintain motor operation, the current in the remaining phases generally increases, and there will be significant fluctuations in speed and torque, which may cause damage to the motor and inverter. Output phase loss detection can identify motor phase loss conditions and take appropriate actions.

When an output phase failure occurs, the current in the disconnected phase of the motor is zero, which can be used to detect the output phase failure. Output phase failure detection can be divided into startup output phase failure detection and operation output phase failure detection. As their names suggest, these methods detect whether an output phase failure has occurred during startup and operation, respectively. After detecting an output phase failure, appropriate actions can be taken as needed to protect the motor and inverter. The parameters related to output phase failure are shown in Table 3-79:

Table 3-79 Parameters Related to Output Phase Failure

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-15 | Output Phase Loss Action Selection | 3 | 0: Warning and continue running; 1: Warning and decelerate to stop; 2: Warning and free stop; 3: No warning. | This parameter is used to set the inverter's action when an output phase failure occurs. A setting value not equal to 3 will enable the output phase failure protection. |
| F9-16 | output phase loss deceleration time | 0.500s | 0.000~65.535s | This parameter is used to set the output phase loss detection time during operation, generally no modification is required. |
| F9-17 | Output Phase Loss Current Threshold | 1.00% | 0.00~100.00% | This parameter is used to set the output phase loss current detection threshold, generally no modification is required. |
| F9-18 | Output Phase Loss Braking Time | 0.000 | 0.000~65.535s | This parameter is used for output phase loss judgment at startup. If the set value is not zero, the output phase loss judgment will be performed at startup. |

The following will introduce four scenarios.

Example 1: F9-18 = 0, no pre-operation output phase loss detection. As shown in Figure3-50,duringoperation,if any phase output current is less than the threshold set by F9-17 and exceeds the time set by F9-16, the inverter will begin to execute the action set by F9-15.

Figure3-50 F9-18=0

Example 2: The inverter is in a stopstate,F9-18 = 0 and F1-09 ≠ 0. As shown in Fig.3-51,when starting, DC braking is performed according to the settings of F1-08 and F1-09,during which output phase loss detection is not performed. After DC braking is completed, the inverter begins to operate and performs phase loss detection according to method1.

Fig.3-51  F9-18 = 0 and F1-09 ≠ 0

Example 3: The inverter is in a stopstate,F9-18 ≠ 0 and F1-09 ≠ 0. During startup, DC braking is performed according to the time set in F9-18, followed by DC braking according to the time set in F1-09. Within the time set in F9-18, the DC braking currentsizeis20 times the value set in F9-57; Within the time set in F1-09, the DC braking currentsizeisthe value set in F1-08. Total DC braking time = the value set in F9-18 + the value set in F1-09.

Example 3-1: F9-18 ≠ 0 and F1-09 ≠ 0 (no output phase loss detected at startup), as shown in Fig. 3-52.

Fig.3-52  F9-18 ≠ 0 and F1-09 ≠ 0 (no output phase loss detected at startup)

Example 3-2: F9-18 ≠ 0 and F1-09 ≠ 0, output phase loss detected at startup. As shown in Fig. 3-53, if an output phase loss occurs within the time set by F9-18, after half of the time set by F9-18 has passed, the inverter begins to execute the action set by F9-15.

Fig.3-53  F9-18 ≠ 0 and F1-09 ≠ 0 (output phase loss detected at startup)

Example 4: The inverter is in a stopped state, F9-18 ≠ 0 and F1-09 = 0, at startup, DC braking is performed according to the time set by parameter F9-18, with the DC braking current being 20 times the value set by F9-57.

Example 4-1: F9-18 ≠ 0 and F1-09 = 0 (no output phase loss detected at startup), as shown in Fig.3-54.

Fig.3-54  F9-18 ≠ 0 and F1-09 = 0 (no output phase loss detected at startup)

Example 4-2: F9-18 ≠ 0 and F1-09 = 0, output phase loss detected at startup. As shown in Fig. 3-55, if an output phase loss occurs within the time set by F9-18, the inverter begins to execute the action set by F9-15 after half of the time set by F9-18 has elapsed.

Fig.3-55  F9-18 ≠ 0 and F1-09 = 0 (output phase loss detected at startup)

3.7.11 Input Phase Loss

When the power supply is not correctly connected to the inverter, or when there is an abnormality in the power supply, an input phase loss fault may occur. When an input phase loss occurs, the inverter bus voltage may fluctuate significantly, causing fluctuations in motor torque or speed, and also affecting the lifespan of the bus capacitor. The input phase loss detection function can detect whether an input phase loss fault has occurred and take protective measures in time.

When an input phase loss occurs, if the motor is carrying a heavy load, the bus voltage will fluctuate significantly. By monitoring the fluctuations in the bus voltage, it can be determined whether an input phase loss fault has occurred. When an input phase loss is detected, the machine should be shut down promptly to prevent damage to the motor or inverter. The parameters related to input phase loss are shown in Table 3-80:

Table 3-80 Parameters Related to Input Phase Loss

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F9-12 | Input Phase Loss Action Selection | 0 | 0: Warning and decelerate to stop; 1: Warning and free stop. | The inverter will perform protective actions for input phase loss based on the settings of parameter F9-12. |
| F9-13 | Input Phase Loss Filter Time | 0.20s | 0.00~600.00s | This parameter is used to set the low-pass filter time required for phase loss detection, which generally does not need to be modified. |
| F9-14 | Input Phase Loss Voltage Threshold | 60.0V | 0.0~320.0V | This parameter is used to set the voltage threshold required for phase loss detection, generally no modification is needed. |

3.8 Monitoring

The monitoring function displays the status and parameter information of the inverter on the display area of the digital operator. The parameters to be displayed can be selected by setting function code F7-21. The monitoring function related parameters are shown in Table 3-81:

Table 3-81 Monitoring Function Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| F7-21 | Page display selection | 3 | 0: Output current 1: PG card feedback frequency 2: Motor actual operating frequency 3: DC Bus Voltage 4: Output voltage 5: Power Factor Angle 6: Output power 7: Actual motor operating speed 8: Output Torque % 9: PG feedback value 10: PID Feedback Value % 11: AI1 % 12: AI2 % 13: AI3 % 14: IGBT temperature 15: Ambient temperature 16: Digital input terminal status 17: Digital output terminal status 18: Multi-speed status 19: CPU input terminal status 20: CPU output terminal status 21: Actual motor position 22: Pulse input frequency value 23: Pulse input position 24: Position tracking error 25: Overload count value 26: Ground short-circuit current threshold 27: Bus voltage fluctuation value 28: PLC Buffer D1043 Value 29: PM motor pole sector 30: User physical quantity 31: H page value multiplied by coefficient K 32: Encoder Z phase count 33: Motor pulse count 34: Reserved 35: Speed/torque mode 36: Current carrier frequency 37: Reserved 38: Inverter status 39: Output Torque Nt-m 40: Torque command 41: kWh 42: PID target value 43: PID compensation 44: PID output frequency 45: Reserved 46: Auxiliary frequency 47: Main frequency 48: Set frequency display 49: Reserved 50: Reserved 51: PMVVC torque compensation amount 52: AI10 % 53: AI11 % 54: Reserved 55: Current roll diameter 56: Current line speed 57: Tension reference value 58: MI6 count value 59: U phase current AD value 60: V phase current AD value 61: W-phase current AD value | This parameter can be used to change the LCD page display parameters during stop or operation mode. |

3.9 User Settings

3.9.1 Application Macros

By selecting application macro parameters, the system automatically imports function parameters related to the selected industry application, thus simplifying the parameter settings for users when choosing different on-site applications. Supports custom application macros, allowing users to edit50function code indexes, and retains custom function code indexes and values after power loss. Additionally, there are ten industry-specific application macros: air compressors, fans, pumps, conveyors, CNC machines, packaging, textile machinery, high-speed electric drills, processPID, and processPIDmaster-slave frequency. When different industry applications are selected, the system automatically updates relevant function parameters, eliminating the need for user settings. Application macro-related parameters are shown in Table 3-82:

Table 3-82 Application Macro-Related Parameters

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| L8-00 | Industry Application Macro Selection | 0 | 0: No Function 1: User Defined 2: Air Compressor 3: Fan 4: Water Pump 5: Conveyor Belt 6: Machine Tool Application 7: Packaging 8: Textile Application 9: High-Speed Drill Application 10: Reserved 11: PID 12: PID + Auxiliary Frequency | After selecting an application macro, some parameter default values will be automatically set according to the selected application macro type. |
| L8-01 | Application Macro Parameter 1 | 0.00 | 0.00～29.00 | The custom option allows users to add up to 50 function code parameter settings (L8-01~L8-50). For example, if a certain on-site application requires setting an asynchronous motor to operate in torque mode with a carrier frequency of 6KHz and a maximum torque of 150%, then the parameters should be set as follows: L8-01=0.03, L8-02=14.00, L8-03=0.15, L8-04=14.10; F0-03=2 (torque mode), FE-00=2 (IM open-loop torque control), F0-15=6 (carrier frequency), FE-10=150% (maximum torque command). After power-off, when powered on again, the parameters added by the custom application macro (L8-00=1) will be automatically imported. Precautions: ■ The setting values of parameters L8-01~L8-50 cannot be the index values of hidden attribute function codes (function codes reserved for display, such as L8-01 cannot be set to 25.00, i.e., L3-00). ■ The setting values of parameters L8-01~L8-50 cannot exceed the total number of members in the set function group, for example, the display range of the F0 group is F0-00~F0-18, L8-01 cannot be set to 0.19 (i.e., F0-19). ■ The setting values for parameters L8-01 to L8-50 cannot be L8 group parameters, such as L8-01 cannot be set to 29.XX (i.e., L8-XX). ■ When the setting values for parameters L8-01 to L8-50 are not 0.00, user-defined index items increase; otherwise, they decrease. For example, if L8-01=1.00, L8-02=1.01 (i.e., F1-00, F1-01), and L8-03=0.00, two custom function parameters are defined. ■ When adding application parameters, it is mandatory to set them consecutively starting from L8-01 without skipping over 0.00 to set the next parameter, such as L8-01=1.00 (i.e., F1-00), L8-02=0.00, and L8-03 cannot be set to 1.01. ■ When reducing application parameters, they must be set to 0.00 sequentially from the last non-0.00 parameter (L8-XX~L8-01). If the operation does not follow the above rules, the keyboard will display an “**ERR**” error, indicating that the setting has failed. |
| L8-02 | Application Macro Parameter 2 | 0.00 | 0.00～29.00 |  |
| L8-03 | Application Macro Parameter 3 | 0.00 | 0.00～29.00 |  |
| L8-04 | Application Macro Parameter 4 | 0.00 | 0.00～29.00 |  |
| L8-05 | Application Macro Parameter 5 | 0.00 | 0.00～29.00 |  |
| L8-06 | Application Macro Parameter 6 | 0.00 | 0.00～29.00 |  |
| L8-07 | Application Macro Parameter 7 | 0.00 | 0.00～29.00 |  |
| L8-08 | Application Macro Parameter 8 | 0.00 | 0.00～29.00 |  |
| L8-09 | Application Macro Parameter 9 | 0.00 | 0.00～29.00 |  |
| L8-10 | Application Macro Parameter 10 | 0.00 | 0.00～29.00 |  |
| L8-11 | Application Macro Parameter 11 | 0.00 | 0.00～29.00 |  |
| L8-12 | Application Macro Parameter 12 | 0.00 | 0.00～29.00 |  |
| L8-13 | Application Macro Parameter 13 | 0.00 | 0.00～29.00 |  |
| L8-14 | Application Macro Parameter 14 | 0.00 | 0.00～29.00 |  |
| L8-15 | Application Macro Parameter 15 | 0.00 | 0.00～29.00 |  |
| L8-16 | Application Macro Parameter 16 | 0.00 | 0.00～29.00 |  |
| L8-17 | Application Macro Parameter 17 | 0.00 | 0.00～29.00 |  |
| L8-18 | Application Macro Parameter 18 | 0.00 | 0.00～29.00 |  |
| L8-19 | Application Macro Parameter 19 | 0.00 | 0.00～29.00 |  |
| L8-20 | Application Macro Parameter 20 | 0.00 | 0.00～29.00 |  |
| L8-21 | Application Macro Parameter 21 | 0.00 | 0.00～29.00 |  |
| L8-22 | Application Macro Parameter 22 | 0.00 | 0.00～29.00 |  |
| L8-23 | Application Macro Parameter 23 | 0.00 | 0.00～29.00 |  |
| L8-24 | Application Macro Parameter 24 | 0.00 | 0.00～29.00 |  |
| L8-25 | Application Macro Parameter 25 | 0.00 | 0.00～29.00 |  |
| L8-26 | Application Macro Parameter 26 | 0.00 | 0.00～29.00 |  |
| L8-27 | Application Macro Parameter 27 | 0.00 | 0.00～29.00 |  |
| L8-28 | Application Macro Parameter 28 | 0.00 | 0.00～29.00 |  |
| L8-29 | Application Macro Parameter 29 | 0.00 | 0.00～29.00 |  |
| L8-30 | Application Macro Parameter 30 | 0.00 | 0.00～29.00 |  |
| L8-31 | Application Macro Parameter 31 | 0.00 | 0.00～29.00 |  |
| L8-32 | Application Macro Parameter 32 | 0.00 | 0.00～29.00 |  |
| L8-33 | Application Macro Parameter 33 | 0.00 | 0.00～29.00 |  |
| L8-34 | Application Macro Parameter 34 | 0.00 | 0.00～29.00 |  |
| L8-35 | Application Macro Parameter 35 | 0.00 | 0.00～29.00 |  |
| L8-36 | Application Macro Parameter 36 | 0.00 | 0.00～29.00 |  |
| L8-37 | Application Macro Parameter 37 | 0.00 | 0.00～29.00 |  |
| L8-38 | Application Macro Parameter 38 | 0.00 | 0.00～29.00 |  |
| L8-39 | Application Macro Parameter 39 | 0.00 | 0.00～29.00 |  |
| L8-40 | Application Macro Parameter 40 | 0.00 | 0.00～29.00 |  |
| L8-41 | Application Macro Parameter 41 | 0.00 | 0.00～29.00 |  |
| L8-42 | Application Macro Parameter 42 | 0.00 | 0.00～29.00 |  |
| L8-43 | Application Macro Parameter 43 | 0.00 | 0.00～29.00 |  |
| L8-44 | Application Macro Parameter 44 | 0.00 | 0.00～29.00 |  |
| L8-45 | Application Macro Parameter 45 | 0.00 | 0.00～29.00 |  |
| L8-46 | Application Macro Parameter 46 | 0.00 | 0.00～29.00 |  |
| L8-47 | Application Macro Parameter 47 | 0.00 | 0.00～29.00 |  |
| L8-48 | Application Macro Parameter 48 | 0.00 | 0.00～29.00 |  |
| L8-49 | Application Macro Parameter 49 | 0.00 | 0.00～29.00 |  |
| L8-50 | Application Macro Parameter 50 | 0.00 | 0.00～29.00 |  |

3.9.2 Sleep and Wakeup

The sleep and wakeup functions are used to achieve constant pressure water supply applications. During the sleep period, the inverter stops running. After the wake-up delay within the sleep zone, the inverter starts running, ending the sleep mode. The sleep and wake functions require setting parameters such as sleep threshold, sleep delay, wake threshold, and wake delay. In general, the wake-up frequency (L5-02) should be greater than or equal to the sleep frequency (L5-01). When the sleep frequency is 0, the sleep and wake-up functions are invalid.

Sleep and wake-up are divided into three cases:

1、Frequency command (without using processPID, parameterFA-00 = 0, only valid inVFcontrol, i.e.,VFsleep and wake-up)

After the output frequency reaches the sleep frequency (parameterL5-01), the inverter maintains operation at the sleep frequency and begins the sleep delay (parameterL5-03). After the delay time has elapsed, it stops directly at0 Hz.

When the frequency command reaches the wake-up frequency (parameterL5-02), after the wake-up delay (parameterL5-04), the inverter starts to accelerate to the set frequency according to the specified acceleration time.

This process is shown in Figure 3-56:

Figure 3-56 frequency command

2、PIDoutput frequency command (using thePIDprocess, parameterFA-00 ≠ 0 and parameterL5-00 = 0, i.e.,PIDsleep and wake-up)

When thePIDoutput frequency command reaches the sleep frequency (parameterL5-01), the inverter begins to enter sleep mode. After the sleep delay (parameterL5-03) has elapsed, it stops directly at0 Hz. If the sleep delay time has not been reached, the output frequency remains at the lower limit frequency (parameterF0-11, andF0-11 ≠ 0) or the minimum output frequency (parameterF2-04, if the lower limit frequencyF0-11 = 0), waiting for the sleep time to elapse before entering sleep mode.

When thePIDoutput frequency command reaches the wake-up frequency (parameterL5-02), the inverter starts the wake-up delay (parameterL5-04). After the delay time has elapsed, the inverter begins to accelerate according to the set acceleration time to thePIDoutput frequency.

The process is shown in Figure 3-57:

Figure 3-57 PID output frequency command

3、PIDfeedback value (when using processPID, parameterFA-00 ≠ 0 and parameterL5-00 = 1, also forPIDsleep and wake-up)

When thePIDfeedback value reaches the sleep threshold (parameterL5-01), the inverter begins to enter sleep mode. After the sleep delay (parameterL5-03) has elapsed, it stops directly at0 Hz. If the sleep delay time has not been reached, the output frequency remains at the lower limit frequency (parameterF0-11, andF0-11 ≠ 0) or the minimum output frequency (parameterF2-04, if the lower limit frequencyF0-11 = 0), waiting for the sleep time to elapse before entering sleep mode.

When thePIDfeedback value reaches the wake-up threshold (parameterL5-02), the inverter starts the wake-up delay (parameterL5-04). After the delay time has elapsed, the inverter begins to accelerate according to the set acceleration time to thePIDoutput frequency.

The process is shown in Figure 3-58:

Figure 3-58 PID feedback value

When the sleep function (cases 2 and 3 above) is controlled by the PID setpoint and feedback value, the inverter frequency source must be selected as PID (parameter F0-06 = 9)

Parameters related to the sleep and wake functions are shown in Table 3-83:

Table 3-83 Parameters related to the sleep and wake functions

| Parameter | Function Definition | Default Value | Setting Range | Parameter Description |
| --- | --- | --- | --- | --- |
| L5-00 | Sleep Method Reference Selection | 0 | 0: PID Command Arrival 1: PID Feedback Arrival | When parameter L5-00 = 0, the units for parameters L5-01 and L5-02 automatically change to frequency, and the setting range automatically changes to 0.00~599.00 Hz. When parameter L5-00 = 1, parameters L5-01 and L5-02 units automatically change to percentage, and the reference base becomes the percentage of feedback, with the setting range automatically changing to 0.00~200.00%. |
| L5-01 | Sleep Threshold | 0.00Hz | 0.00Hz~599.00Hz | During the operation of the inverter, when the set frequency is less than the sleep frequency (parameter L5-01, L5-00 = 0), or the PID negative feedback value is greater than the sleep threshold (parameter L5-01, L5-00 = 1), the inverter begins to enter sleep mode. After the sleep delay (parameter L5-03) has elapsed, it stops directly at 0 Hz. |
| L5-02 | Wake Threshold | 0.00Hz | 0.00Hz~599.00Hz |  |
| L5-03 | Sleep Delay | 0.0s | 0.0s~6000.0s |  |

4 Fault Diagnosis and Countermeasures

4.1 Fault Handling

If a fault occurs during system operation, the inverter will immediately stop output to protect the motor from further damage. At the same time, the inverter will trigger the corresponding fault relay contacts, and the control panel will display the fault code for quick diagnosis of the problem. For each fault code, there is a corresponding fault type and common troubleshooting method, which can be found in Table4-1. Please note that the information listed in the table is for reference only and should not be altered arbitrarily. If you cannot resolve the fault, please contact our technical support or product distributor for assistance.

Table 4-1 Fault Alarms and Countermeasures

| Fault Number | Fault Name | Fault Description | Possible Cause | Reset Method and Conditions |
| --- | --- | --- | --- | --- |
| E001 | Overcurrent During Acceleration | During acceleration, the output current exceeds the inverter's overcurrent threshold. When E001 occurs, the inverter immediately stops output, and the motor will free stop. | 1. The set acceleration time is too short; 2. Check if poor insulation of the motor wiring causes an output short circuit; 3. Check if the motor is burned out or has insulation aging; 4. The torque compensation amount is too large; 5. The load is too heavy; 6. Abnormal V/F curve setting; 7. Hardware fault. | Manual reset; after the status disappears, it can be reset after 5 seconds. |
| E002 | Overcurrent during deceleration | During deceleration or stopping, the output current exceeds the inverter's overcurrent threshold. When E002 occurs, the inverter immediately stops output, and the motor will free stop. | Same as above | Manual reset; after the status disappears, it can be reset after 5 seconds. |
| E003 | Overcurrent during constant speed | During constant speed operation, the output current exceeds the inverter's overcurrent threshold. When E003 occurs, the inverter immediately stops output, and the motor will free stop. | Same as above | Manual reset; after the status disappears, it can be reset after 5 seconds. |
| E004 | Grounding Short circuit | The inverter detects a ground short circuit at the output terminals (U / V / W), and the inverter immediately stops output, causing the motor to free stop. | 1. Motor burnout or insulation aging; 2. Ground short circuit due to cable damage, large stray capacitance between cables and terminals leading to false tripping, hardware fault. | Manual reset; after the status disappears, it can be reset after 5 seconds. |
| E005 | IGBT upper and lower bridge short circuit | The inverter has detected a short circuit between the upper and lower bridges of the IGBT module. | 1. IGBT fault 2. Short circuit between the upper and lower bridges of the IGBT | Manual reset; after the status disappears, it can be reset after 5 seconds. |
| E006 | Overcurrent during stop | Overcurrent occurs during stop or there is an anomaly in the hardware circuit. After E006 occurs, power off and then power on again. If there is a hardware issue, E033, E034, or E035 will appear. | 1. Misoperation due to interference 2. Hardware fault | Manual reset; after the status disappears, it can be reset after 5 seconds. |
| E007 | Overvoltage during acceleration | The inverter detects that the bus voltage is too high during acceleration. When E007 occurs, the inverter immediately stops output, and the motor will free stop. | 1. Acceleration is too small (e.g., when accelerating downward with a lifting load); 2. Whether the setting for stall prevention action is less than the no-load current; 3. Power supply voltage is too high; 4. Switching actions of capacitors in the same power system; 5. The motor is in a generating state; 6. Acceleration time is too short; 7. Ground short circuit occurs in the motor; 8. Incorrect connection of the braking resistor or braking unit; 9. Malfunction due to interference. | Manual reset; The bus voltage must be below approximately 90% of the overvoltage threshold (810V) to reset. |
| E008 | Overvoltage during deceleration | The inverter detects an excessively high bus voltage during deceleration. When E008 occurs, the inverter immediately stops output, and the motor will free stop. | 1. A deceleration time that is too short causes excessive regenerative energy from the load; 2. As above | Manual reset; The bus voltage must be below approximately 90% of the overvoltage threshold (810V) to reset. |
| E009 | Overvoltage at Constant Speed | During constant speed operation, the inverter detects an overvoltage on the bus. When E009 occurs, the inverter immediately stops output, and the motor will free stop. | 1. Sudden changes in load; 2. As above | Manual reset; The bus voltage must be below approximately 90% of the overvoltage threshold (810V) to reset. |
| E010 | Overvoltage During Stop | Overvoltage occurs when the inverter stops. | 1. The supply voltage is too high; 2. Switching actions of capacitors in the same power system; 3. Incorrect wiring of the braking resistor or braking unit; 4. Hardware fault (abnormal voltage sampling circuit); 5. Ground short circuit in the motor. | Manual reset; The bus voltage must be below approximately 90% of the overvoltage threshold (810V) to reset. |
| E011 | Undervoltage During Acceleration | During acceleration, the inverter detects that the bus voltage is below the set value of parameter L2-18. | 1. Power outage occurs; 2. Power supply voltage fluctuates; 3. Large-capacity motors start up; 4. Overload; 5. Shared DC bus; 6. Consider installing a DC reactor. | Manual reset; The inverter can be reset when the bus voltage exceeds the set value of parameter L2-18 + 60 V. |
| E012 | Undervoltage During Deceleration | During deceleration, the inverter detects that the bus voltage is below the set value of parameter L2-18. | Same as above | Same as above |
| E013 | Undervoltage During Constant Speed | At constant speed, the inverter detects that the bus voltage is below the set value of parameter L2-18. | Same as above | Same as above |
| E014 | Undervoltage During Stop | 1. When stopping, the inverter detects that the bus voltage is below the value set in parameter L2-18. | Same as above | Same as above |
| E015 | Input Phase Loss Protection | Power Input Phase Loss Protection | 1. Input power phase loss occurs; 2. Single-phase power input for three-phase models; 3. Power voltage fluctuation occurs; 4. Loose connections at the input power terminals; 5. Whether the input cables of the three-phase power supply have been cut; 6. Unbalanced three-phase input power. | Manual Reset |
| E016 | IGBT Temperature Too High | The inverter detects that the IGBT temperature is too high, exceeding 95°C. | 1. Whether the ambient temperature or the temperature inside the control cabinet is too high, and whether the cooling holes of the cabinet are blocked by foreign objects; 2. Check for foreign objects on the heat sink and whether the fan is rotating; 3. Insufficient ventilation space for the inverter; 4. Whether the load matches the inverter; 5. Continuous operation at 100% or more than 100% of the rated output. | Manual reset; IGBT temperature must be below 85°C before reset can be performed. |
| E017 | Ambient temperature too high | The inverter has detected that the internal critical components' temperature is too high, exceeding 55°C. | Ambient temperature too high | Manual reset; the ambient temperature sensor's temperature must be below 45°C before reset can be performed. |
| E021 | Inverter overload | Output current exceeds the inverter's rated current, 150% of rated output current for 1 minute. | 1. Overload; 2. Acceleration and deceleration times and working cycle times are too short; 3. V/F control voltage is too high; 4. Inverter capacity is too small; 5. Overload occurs during low-speed operation; 6. Torque compensation is too high; 7. Whether the settings for stall prevention are correct; 8. Output phase loss; 9. Speed tracking function parameters are set improperly. | Manual reset; can only be reset after the error has disappeared for 5 seconds. |
| E022 | Motor 1 Overload Protection | Motor 1 overload protection activated; after activation, free stop occurs. | 1. Overload; 2. Acceleration and deceleration times and working cycle times are too short; 3. When using a dedicated inverter motor, set parameter F9-01 Motor 1 Overload Protection Selection = 0 for constant torque output motor; 4. The motor overload action value is incorrect; 5. The set value for the maximum motor frequency is low; 6. Driving multiple motors with one inverter; 7. Whether the settings for stall prevention are correct; 8. The torque compensation amount is too large; 9. The motor fan operation is abnormal; 10. The three-phase impedance of the motor is unbalanced. | Manual reset; can be reset after the error disappears for 5 seconds. |
| E023 | Motor 2 Overload Protection | Motor 2 overload protection has been activated, and free stop will occur after activation. | 1. When using a dedicated inverter motor, set parameter H3-03 Motor 2 overload protection selection = 0 for constant torque output motor; 2. Same as above 1~2 and 4~10. | Same as above |
| E024 | Motor Overheat | Motor PTC Overtemperature Warning, when a PTC is installed on the motor and this function is enabled (Parameter F5-21/27/33 = 6 Thermistor PTC Input), if the PTC input exceeds the set value of Parameter F9-49, it will be processed according to the setting of Parameter F9-48. | 1. Motor Stalled; 2. Excessive Load; 3. High Ambient Temperature; 4. Abnormal Motor Cooling System/Fan; 5. Frequent Low-Speed Operation; 6. Acceleration/Deceleration Time and Working Cycle Time Too Short; 7. V/F Control Voltage Too High; 8. Whether the Setting of Motor Rated Current Matches the Motor Nameplate; 9. Whether the PTC settings and wiring are appropriate; 10. Whether the stall prevention settings are correct; 11. Unbalanced three-phase impedance of the motor; 12. Excessive harmonic components. | Parameter F9-48 0: Warning and continue running 1: Fault and decelerate to stop 2: Fault and free stop 3: No warning When parameter F9-48 = 0, it is a warning, and automatic reset; when parameter F9-48 = 1 or 2, it is a fault, and manual reset is required. Immediate reset is possible. |
| E026 | Over Torque 1 | When the output current exceeds the torque detection value F9-37 and exceeds the over-torque detection time set by parameter F9-38, and parameter F9-36 is set to 2 or 4, E026 will be displayed. | 1. Incorrect parameter settings; 2. Mechanical failure; 3. Overload; 4. Acceleration/deceleration time or working cycle time is too short; 5. V/F control voltage is too high; 6. Motor capacity is too small; 7. Overload occurs during low-speed operation; 8. The torque compensation amount is too large; 9. Speed tracking function parameters are set improperly (including instantaneous power failure restart and abnormal restart conditions). | Parameter F9-36 0: Not Detected 1: Over-torque detection during constant speed operation, continue running; 2: Over-torque detection during constant speed operation, stop operation; 3: Over-torque detection during operation, continue running; 4: Over-torque detection during operation, stop operation; Reset method: Parameter F9-36 = 1 or 3, when the output current is less than parameter F9-36, it will automatically clear. When parameter F9-36 = 2 or 4, manual reset is required. Reset condition: Can be reset immediately. |
| E027 | Over Torque 2 | When the output current exceeds the torque detection value H3-01 and exceeds the over-torque detection time H3-02, and parameter H3-00 is set to 2 or 4, E027 will be displayed. | 1. Incorrect parameter settings; 2. Mechanical failure; 3. Overload; 4. Acceleration/deceleration time or working cycle time is too short; 5. V/F control voltage is too high; 6. Motor capacity is too small; 7. Overload occurs during low-speed operation; 8. The torque compensation amount is too large; 9. Speed tracking function parameters are set improperly (including instantaneous power failure restart and abnormal restart conditions). | Parameter H3-00 0: No detection; 1: Over-torque detection during constant speed operation, continue running; 2: Over-torque detection during constant speed operation, stop operation; 3: Over-torque detection during operation, continue running; 4: Over-torque detection during operation, stop operation; Reset method: Parameter H3-00 = 1 or 3, when the output current is less than parameter H3-00, it will automatically clear. When parameter H3-00 = 2 or 4, manual reset is required. Reset condition: Can be reset immediately. |
| E028 | Low Current | Low current condition detected | 1. Motor cable disconnected; 2. Low current protection setting is inappropriate; 3. Load too low. | Parameter F9-24 0: No Function; 1: Error and free stop; 2: Error and stop according to second deceleration time; 3: Warning and continue running. When Parameter F9-24 = 3, it is 'Warning'. When the output current exceeds Parameter F9-22 + 0.1 A, the warning will be automatically cleared; When Parameter F9-24 = 1 or 2, it is 'Fault', requiring manual reset; can be reset immediately. |
| E029 | Arrival Limit | In non-PG vector control mode, when the inverter operates in speed mode, this fault is reported after the reverse operation limit or forward operation limit on the DI terminal is activated. | 1. Return-to-Origin Mode: Set not to use forward or reverse limit as origin, and set to report a fault when reaching the limit. When the input terminal function is selected as PL forward limit or NL reverse limit and the terminal status is valid, report a limit reached fault. 2. Non-Return-to-Origin Mode: When the input terminal function is selected as PL forward limit or NL reverse limit and the terminal status is valid, report a limit reached fault. | Manual reset when the limit terminal status is invalid. |
| E031 | Abnormal memory readout | Abnormal EEPROM data readout | Abnormal EEPROM data readout | Manual reset; Can be immediately reset. |
| E033 | U-phase current detection error | When powered on, the inverter's U-phase current detection circuit is abnormal. | Hardware fault | Power off required |
| E034 | V-phase current detection error | During power-up, the V-phase current detection circuit of the inverter is abnormal | Hardware fault | Power off required |
| E035 | W-phase current detection error | During power-up, the W-phase current detection circuit of the inverter is abnormal | Hardware fault | Power off required |
| E036 | cc (current limiting)hardware circuit abnormal | During power-up, the cc hardware protection circuit of the inverter is abnormal | Hardware fault | Power off required |
| E037 | oc (overcurrent)hardware circuit abnormal | During power-up, the oc hardware protection circuit of the inverter is abnormal | Hardware fault | Power off required |
| E038 | ov (overvoltage)hardware circuit abnormal | During power-up, the ov hardware protection circuit of the inverter is abnormal | Hardware fault | Power off required |
| E040 | Motor Parameter Identification Error | Motor Parameter Identification Error | 1. The STOP key was pressed during self-learning; 2. Incorrect motor capacity (too large or too small) and parameter settings; 3. Incorrect motor wiring; 4. Motor locked rotor; 5. Using an output contactor, but the output contactor has not engaged; 6. Overload; 7. Acceleration and deceleration times are too short. | Manual reset; Can be immediately reset. |
| E041 | PID Open Circuit | PID Feedback Error | 1. Whether the analog feedback value is abnormal; 2. Whether the negative feedback type is set correctly; 3. PID deviation threshold is set too low causing false alarms. | FA-50 Feedback Signal Open Circuit Handling: 0: Warning and continue running 1: Fault and deceleration stop 2: Fault and free stop 3: Warning and operation at the frequency before the open circuit. |
| E042 | PG Feedback Setting Error | The actual rotation direction of the motor is opposite to the frequency command direction. | 1. Incorrect encoder parameter settings; 2. Check for broken wires in the encoder connections; 3. PG card or PG encoder damage; 4. Malfunction due to interference. | ParameterF9-28 0: Warning and continue running 1: Fault and deceleration stop 2: Fault and free stop Manual reset; Can be immediately reset. |
| E043 | PG feedback disconnection | In the control mode with PG, if parameters F4-27 and F4-29 are not set, an E043 error will be reported after pressing RUN. | 1. Incorrect encoder parameter settings; 2. Incorrect selection of control mode. | Manual reset; Can be immediately reset. |
| E044 | PG feedback overspeed | In the mode with PG, when the motor frequency value exceeds the encoder stall threshold (parameter F9-30), it starts to accumulate time. If the error time exceeds the encoder timeout detection time (parameter F9-31), an E044 error occurs. | 1. Incorrect encoder parameter settings; 2. Parameter F4-02 is set too low; 3. Speed loop related parameters and acceleration/deceleration settings are inappropriate; 4. PG feedback timeout protection function parameters are set improperly. | Parameter F9-32 0: Warning and continue running 1: Fault and deceleration stop 2: Fault and free stop Manual reset; Can be immediately reset. |
| E045 | PG slip abnormality | In modes with PG, when the difference between the output frequency and the motor frequency exceeds the encoder deviation range (parameter F9-33), the cumulative time starts. If the error time exceeds the encoder deviation detection time (parameter F9-34), an E045 fault occurs. | 1. PG feedback deviation abnormality function parameters are set improperly; 2. Speed loop related parameters and acceleration/deceleration settings are inappropriate; 3. Encoder parameters are set incorrectly; 4. Acceleration and deceleration time too short; 5. Torque limit parameters set incorrectly; 6. Motor locked rotor; 7. Mechanical brake not released. | Parameter F9-35: 0: Warning and continue running 1: Fault and deceleration stop 3: Fault and free stop When parameter F9-35 = 0, it is a warning; when the difference between the output frequency and the motor frequency is less than the encoder deviation threshold, the 'warning' will be automatically cleared. When parameter F9-35 = 1 or 2, it is a 'fault', requiring manual reset. It can be reset immediately. |
| E048 | AI current signal open circuit | When the 4~20mA open circuit action (parameter F5-42) is selected as 3, and the signal type of any one of the AI1, AI2, or AI1 analog input terminals is a 4~20mA current signal, and the analog input function is not zero, if the input current signal at that terminal is less than the 4~20mA open circuit threshold (parameter F5-43), an E048 fault will be reported. | Analog Input Current Signal Disconnection | Reset Method: Manual Reset; Reset Condition: When the disconnection fault condition is not met, manual reset can be performed. |
| E049 | External Fault | External Fault, the inverter decelerates according to the set value of parameter F1-23. | DI Terminal Function = 10 “External Fault”, and the signal is valid | Parameter F1-23 0: Stop in Free Run Mode; 1: Decelerate According to First Deceleration Time; 2: Decelerate According to Second Deceleration Time; 3: Decelerate According to Third Deceleration Time; 4: Decelerate According to Fourth Deceleration Time; 5: System Deceleration (According to Original Deceleration Time) 6: Automatic Deceleration Manual reset; Manual reset can only be performed after the external fault has disappeared. |
| E050 | External emergency stop | When the DI terminal function is set to “External Fault Free Stop”, if the terminal signal is valid, the inverter immediately stops output, and the motor freely stops. | DI terminal function = 28 “External Fault Free Stop”, and the signal is valid | Manual reset; “External Fault Free Stop” signal must disappear before manual reset can be performed. |
| E051 | External Interrupt | When the DI terminal function is set to “Base Block”, the inverter immediately stops output, and the motor freely stops. | DI terminal function = 11 “Base Block”, and the signal is valid | The “Base Block Signal” disappears, and the fault is automatically cleared. |
| E052 | Password entered incorrectly three times | Password decoding failed three consecutive times | Parameter F7-33 password entry error | Manual reset; Reset condition requires power off. |
| E054 | Invalid communication command | Invalid communication command | 1. The communication command transmitted by the higher-level machine is incorrect; 2. Misoperation due to interference; 3. Communication conditions differ from those of the higher-level machine; 4. Communication cable break or poor contact. | F8-05 Communication Error Handling Method 0: Warning and continue running 1: Error and decelerate to stop 2: Error and free stop 3: No warning and continue operation Reset method: Manual reset; Reset condition: Can be immediately reset. |
| E055 | Invalid Communication Address | Invalid Communication Data Address | Same as above | Reset method: Manual reset; Reset condition: Can be immediately reset. |
| E056 | Communication Data Error | Invalid Communication Data Value | Same as above | Reset method: Manual reset; Reset condition: Can be immediately reset. |
| E057 | Communication Write to Read-Only Address | Writing Data to Read-Only Address | Same as above | Reset method: Manual reset; Reset condition: Can be immediately reset. |
| E058 | Modbus Transmission Timeout | Modbus Transmission Timeout | 1. The master device fails to transmit the communication command within the time set by parameter F8-04; 2. Misoperation due to interference; 3. Communication conditions differ from those of the higher-level machine; 4. Communication cable break or poor contact. | Parameter F8-05 Error Handling Method 0: Warning and continue running 1: Error and decelerate to stop 2: Error and free stop 3: No warning and continue operation Reset method: Manual reset; Reset condition: Can be immediately reset. |
| E062 | Deceleration Regenerative Braking Action | As long as parameter F1-34 is not zero, and a power interruption or outage causes the bus voltage to drop below the deceleration regenerative braking action threshold, the deceleration regenerative braking function will activate, causing the motor to begin decelerating and stopping. During this process, the E062 fault will be displayed. | 1. Unstable power supply or power outage; 2. There are other large loads starting in the power system. | Reset Conditions: Automatic: Select 2 for parameter F1-34; the fault will be automatically cleared after power recovery. Manual: Select 1 for parameter F1-34; the inverter can be manually reset after decelerating to 0 Hz. |
| E063 | Excessive slip | Slip abnormality, using the maximum slip (parameter F2-13) as the reference. When the inverter output is stable, if the deviation between the set speed and the actual speed exceeds the value set in parameter F9-25, and this condition persists for longer than the time set in parameter F9-26, an E063 fault will occur. The E063 fault only occurs when driving induction motors. | 1. Whether the motor parameters are correct; 2. Excessive Load; 3. Whether the settings for parameters F9-25, F9-26, and F2-13 are appropriate. | Parameter F9-27 0: Warning and continue running 1: Error and decelerate to stop 2: Warning and Free Stop 3: No warning Automatic: Parameter F9-27 = 0 is “Warning”, when the inverter output is stable, and the deviation between the set speed and the actual speed does not exceed the value set in parameter F9-25, the warning will be automatically cleared. Manual: Parameter F9-27 = 1 or 2 is “Fault”, manual reset is required. |
| E064 | Please reset the machine type code | Incorrect machine type code setting | Incorrect machine type code setting | Set product signal F0-00 according to the inverter specification table |
| E065 | PG card hardware error | PG card hardware error | 1. Wiring error; 2. Encoder parameter selection error; 3. Incorrect PG card selected. | Reset method: Manual reset; Reset condition: The error must be reset by re-powering the device. |
| E069 | Feedback speed divergence | Feedback speed exceeds the set value: maximum speed * parameter F9-30. | 1. Incorrect encoder parameter settings; 2. Incorrect motor parameters; 3. Motor parameter identification not performed; | Manual Reset |
| E070 | Excessive feedback speed deviation | The difference in feedback speed is too large, estimated value differs from command by ±F9-33. | 1. Incorrect encoder parameter settings; 2. Incorrect motor parameter settings; 3. Motor parameter identification not performed. | Manual Reset |
| E072 | STO1 Fault | An abnormality has been detected in the internal loop between S1 and +24V | The shorting wire between S1 and +24V is not connected Hardware fault | Reset method: Hardware fault, cannot reset, power off and then power on again; Reset condition: None |
| E076 | STO | Safety Torque Off function activation | Switch operation of S1 / +24V, S2 / +24V | Reset method: Automatic: When parameter F9-54=1, the system can automatically reset after the STO status disappears; Manual: When parameterF9-54=0, manual reset is required after the STO status disappears; Reset condition: Reset can only occur after the STO status has disappeared. |
| E077 | STO2 Fault | An abnormality is detected in the internal circuit between S2 and +24V | The shorting wire between S2 and +24V is not connected Hardware fault | Reset method: Hardware fault, cannot reset, power off and then power on again; Reset condition: None |
| E079 | U-phase overcurrent | U-phase short circuit detected before the inverter starts | 1. Incorrect motor wiring; 2. Poor insulation of motor wiring causing output short circuit; 3. Check if the motor is burned out or has insulation aging; 4. Malfunction due to interference 5. The wiring length of the motor cable is too long; 6. Hardware failure. | Reset method: Manual reset; Reset condition: After the status disappears, it can be reset after 5 seconds. |
| E080 | Overcurrent on V phase | Short circuit detected on V phase before the inverter starts | Same as above | Same as above |
| E081 | Overcurrent on W phase | Short circuit detected on W phase before the inverter starts | Same as above | Same as above |
| E082 | Missing U phase output | Missing U phase output | 1. Unbalanced three-phase impedance of the motor; 2. Check if there is a problem with the wiring; 3. Whether the motor is a single-phase motor; 4. Whether the current sensor is faulty; 5. Whether the inverter capacity is much larger than the motor capacity. | Parameter F9-15 0: Warning and continue running 1: Error and decelerate to stop 2: Error and free stop 3: No warning Reset method: Manual reset; Reset condition: Can be immediately reset. |
| E083 | V Phase Output Missing | V Phase Output Missing | Same as above | Same as above |
| E084 | W Phase Output Missing | W Phase Output Missing | Same as above | Same as above |
| E087 | Low Frequency Overload Protection | Inverter Low Frequency Overload Operation (<5Hz) for Too Long | 1. Overload; 2. Acceleration and deceleration times and working cycle times are too short; 3. V/F control voltage is too high; 4. Inverter capacity is too small; 5. Overload occurs during low-speed operation; 6. Torque compensation is too high; 7. Whether the settings for stall prevention are correct; 8. Output phase loss; 9. Speed Tracking Function Parameter Setting Inappropriate | Reset method: Manual reset; Reset condition: Can be immediately reset. |
| E101 | CANopen Disconnection | CANopen Software Disconnection 1 | 1. Communication Timeout Time Set Too Short; 2. Misoperation due to interference; 3. Communication Cable Disconnected or Poor Contact. | Reset method: Manual reset; Reset Condition: Clear This Error by Sending a Reset Signal from the Higher-Level Machine. |
| E102 | CANopen Disconnection | CANopen Software Disconnection 2 | 1. Communication Timeout Time Set Too Short; 2. Misoperation due to interference; 3. Communication Cable Disconnected or Poor Contact. | Reset method: Manual reset; Reset Condition: Clear This Error by Sending a Reset Signal from the Higher-Level Machine. |
| E104 | CANopen Hardware Disconnection | CANopen Hardware Disconnection | 1. Confirm whether the CANopen card is installed; 2. Confirm whether the communication format is correct; 3. Misoperation due to interference; 4. Communication cable disconnection or poor contact; | Reset method: Manual reset; Reset Condition: Power must be cycled. |
| E105 | CANopen Index Error | CANopen Communication Index Error | Communication Index Setting Error | Reset method: Manual reset; Reset Condition: Clear This Error by Sending a Reset Signal from the Higher-Level Machine. |
| E106 | CANopen Node Address Error | CANopen Communication Node Address Error (Only supports 1~127) | Communication Node Address Setting Error | Reset Method: Manual reset (Parameter F0-18=7); Reset condition: None |
| E107 | CANopen Memory Error | CANopen Memory Error | CANopen Internal Memory Error | Reset method: Manual reset; Reset Condition: Parameter F0-18 = 7. |
| E120 | EMS Overcurrent | Output current exceeds the inverter overcurrent threshold. When an EMS overcurrent fault occurs, the inverter immediately stops output, and the motor will free stop. | 1. The set acceleration time is too short; 2. Check if poor insulation of the motor wiring causes an output short circuit; 3. Check if the motor is burned out or has insulation aging; 4. The torque compensation amount is too large; 5. The load is too heavy; 6. Abnormal V/F curve setting; 7. Hardware fault. | Manual reset; after the status disappears, it can be reset after 5 seconds; |
| E128 | Over Torque 3 | When the output current exceeds the torque detection value H3-06, and exceeds the over-torque detection time set by parameter H3-07, E128 will occur if parameter H3-05 is set to 2 or 4. | 1. Incorrect parameter settings; 2. Mechanical failure; 3. Overload; 4. Acceleration/deceleration time or working cycle time is too short; 5. V/F control voltage is too high; 6. Motor capacity is too small; 7. Overload occurs during low-speed operation; 8. The torque compensation amount is too large; 9. Speed tracking function parameters are set improperly (including instantaneous power failure restart and abnormal restart conditions). | Parameter H3-05 0: Not Detected 1: Over-torque detection during constant speed operation, continue running; 2: Over-torque detection during constant speed operation, stop operation; 3: Over-torque detection during operation, continue running; 4: Over-torque detection during operation, stop operation; Reset Method: When parameter H3-05 = 1 or 3, it is a 'warning', which will automatically clear when the output current is less than parameter H3-06. When parameter H3-05 = 2 or 4, it is a 'fault', requiring manual reset. Reset condition: Can be reset immediately. |
| E129 | Over Torque 4 | When the output current exceeds the torque detection value H3-11, and exceeds the over torque detection time set by parameter H3-12, E129 will occur if parameter H3-10 is set to 2 or 4. | Same as above | Parameter H3-10 0: No detection; 1: Over-torque detection during constant speed operation, continue running; 2: Over-torque detection during constant speed operation, stop operation; 3: Over-torque detection during operation, continue running; 4: Over-torque detection during operation, stop operation; Reset Method: When parameter H3-10 is set to 1 or 3, it is 'Warning', and the warning will automatically clear when the output current is less than parameter H3-11. When parameter H3-10 is set to 2 or 4, it is 'Fault', and manual reset is required. Reset Condition: Can be immediately reset |
| E134 | Motor 3 Overload Protection | Motor 3 overload protection has been activated, and after activation, free stop occurs. | 1. Overload; 2. Acceleration and deceleration times and working cycle times are too short; 3. When using a VFD-dedicated motor, set parameter H3-08 Motor 3 Overload Protection Selection = 0 for constant torque output motor; 4. The motor overload action value is incorrect; 5. The set value for the maximum motor frequency is low; 6. Driving multiple motors with one inverter; 7. Whether the settings for stall prevention are correct; 8. The torque compensation amount is too large; 9. The motor fan operation is abnormal; 10. The three-phase impedance of the motor is unbalanced. | Reset method: Manual reset; Reset Condition: Can only be reset after 5 seconds of the fault disappearing. |
| E135 | Motor 4 Overload Protection | Motor 4 Overload Protection Action, after action, free stop. | 1. When using a dedicated inverter motor, set parameter H3-13 Motor 4 Overload Protection Selection = 0 for constant torque output motor; 2. Same as above 1~2 and 4~10. | Same as above |
| E141 | Pre-Operation Ground Fault | During the output wiring detection before the inverter operation, a ground short circuit is detected. | 1. Incorrect motor wiring; 2. Whether the motor wiring has poor insulation causing an output short circuit; 3. Check if the motor is burned out or has insulation aging; | Reset method: Manual reset; Reset condition: After the status disappears, it can be reset after 5 seconds. |
| E142 | Parameter Identification Error 1 | Motor parameter identification without feedback current error | 1. Motor not wired; 2. At the inverter output side (U / V / W); 3. Electromagnetic contactor is in an open state. | Reset method: Manual reset. Reset condition: Can be immediately reset. |
| E143 | Parameter identification error 2 | Motor phase missing error during motor parameter identification | 1. Incorrect motor wiring; 2. Motor fault; 3. Electromagnetic contactor at the inverter output side (U / V / W) is in an open state; 4. Abnormality in the motor U/V/W lines. | Same as above |
| E144 | Tension Belt breakage | Belt breakage detected through speed recognition | 1. Whether the analog feedback value is abnormal; 2. Whether the negative feedback type is set correctly. | Reset method: Manual reset. |
| E147 | Excessive tension PID deviation | Excessive deviation between tension feedback value and set value | 1. Whether the analog feedback value is abnormal; 2. Tension control PID parameter settings are inappropriate; | FB-50 Tension Error Abnormal Handling: 0: Warning and continue running 1:Faultand Free Stop 2:Faultand decelerate to stop. |

4.2 Warning Handling

For some errors that have a minor impact on the system, the inverter only performs warning handling and continues to operate, while displaying the warning name and corresponding warning code on the LCD panel. After the warning reset conditions are met, the warning will be automatically cleared, and the motor will return to normal operation. The warning codes and corresponding warning handling are shown in Table4-2.

Table 4-2 Warning Codes and Warning Handling

| Warning Code | Warning Name | Fault Description | Possible Cause | Reset Method and Conditions |
| --- | --- | --- | --- | --- |
| A001 | Communication Command Error | RS-485 Modbus, invalid communication command | 1. The communication command transmitted by the higher-level machine is incorrect; 2. Malfunction due to interference and different communication conditions with the higher-level machine; 3. Communication Cable Disconnected or Poor Contact. | Parameter F8-05 = 0 warning and continue running when it is “warning”, automatically reset upon receiving a correct communication command. |
| A002 | Communication address error | RS-485 Modbus, invalid communication data address | Same as above | Same as above |
| A003 | Communication Data Error | RS-485 Modbus, invalid communication data value | Same as above | Same as above |
| A004 | Inverter cannot process | RS-485 Modbus, writing data to read-only address | Same as above | Same as above |
| A005 | Communication transmission timeout | RS-485 Modbus, transmission timeout | 1. The higher-level machine fails to transmit the communication command within the time set in parameter F8-04; 2. Misoperation due to interference; 3. Communication conditions differ from those of the higher-level machine; 4. Communication cable break or poor contact. | Parameter F8-05 = 0 warning and continue running when it is 「warning」, automatically reset upon receiving the next communication packet. |
| A006 | Parameter Copy Error | Digital Operator to Drive COPY Function Error Warning 1 | 1. Communication Abnormality; 2. Panel Abnormality; 3. Control Board Abnormality. | Manual Reset |
| A007 | Parameter Copy Error | Digital Operator to Drive COPY Function Error Warning 2 | 1. Whether there are new parameters added to the inverter software; 2. Misoperation due to interference. | Manual Reset |
| A009 | IGBT Overheating Warning | The inverter detects that the IGBT temperature is too high, exceeding the IGBT overheating warning protection level (if parameter F9-41 is higher than the IGBT overheating protection level, there will be no IGBT overheating warning, and it will directly trip due to IGBT temperature being too high). | 1. Whether the ambient temperature or the temperature inside the control cabinet is too high, and whether the cooling holes of the cabinet are blocked by foreign objects; 2. Check for foreign objects on the heat sink and whether the fan is rotating; 3. Insufficient ventilation space for the inverter; 4. Whether the load matches the inverter; 5. Long-term operation at 100% or greater than 100% of rated output. | Automatic Reset; IGBT temperature below 85°C, automatic reset. |
| A010 | Overheating Warning | The inverter detects that the internal critical component temperature is too high, exceeding the protection level. | Same as above | Manual reset; the ambient temperature sensor's temperature must be below 45°C before reset can be performed. |
| A011 | PID Feedback Signal Warning | PID Feedback Signal Loss Warning (Effective only when PID function is enabled for analog feedback signals) | 1. Loose or broken PID feedback wiring; 2. Faulty feedback device; 3. Hardware fault. | Automatic: When parameter FA-50 = 0 or 3, it is “Warning”. The warning will be automatically cleared when the feedback signal exceeds 4 mA. Manual: When parameter FA-50 = 1 or 2, it is “Fault”, requiring manual reset. |
| A012 | AI current signal open circuit | When the 4～20mA break action (parameter F5-42) is set to 1 or 2, and the signal type of any one of the AI1, AI2, or AI1 analog input terminals is a 4～20mA current signal, and the analog input function is not zero, if the input current signal at that terminal is less than the 4～20mA break threshold (parameter F5-43), an A012 alarm will be triggered. | Analog Input Current Signal Disconnection | The alarm will automatically clear when the break alarm conditions are not met. |
| A015 | PG Feedback Error | PG Feedback Error Alarm | 1. Incorrect encoder parameter settings; 2. Encoder wiring break; 3. PG card or PG encoder damage; 4. Malfunction due to interference. | The alarm will automatically reset after the machine stops. |
| A017 | Overspeed Warning | Over-speed Warning | 1. When using open-loop control, the setting of parameter F3-36 speed observer bandwidth is inappropriate; 2.The bandwidth setting of the ASR speed controller is inappropriate; 3.The motor parameter settings are incorrect; 4. Malfunction due to interference. | The warning is automatically cleared after stopping. |
| A018 | Excessive speed deviation | Excessive speed deviation warning | 1.The feedback deviation abnormal function parameter settings are inappropriate; 2.The ASR related parameters and acceleration/deceleration settings are inappropriate; 3.The acceleration/deceleration time is too short; 4.Motor stall; 5.Mechanical brake not released; 6.Incorrect torque limit related parameter settings cause malfunction. | The warning is automatically cleared after stopping. |
| A019 | Input Phase loss | Inverter input phase loss | 1.Input power phase loss occurs 2. Single-phase power input for three-phase models; 3.The power supply voltage has fluctuated; 4. Loose connections at the input power terminals; 5. Whether the three-phase power input cables have been cut; 6. Unbalanced three-phase input power. | The warning is automatically cleared after stopping. |
| A020 | Over-torque | Over-torque 1 warning | 1. Incorrect parameter settings; 2. Mechanical failure; 3. Overload; 4. Acceleration and deceleration times and working cycle times are too short; 5. V/F control voltage is too high; 6. Motor capacity is too small; 7. Overload occurs during low-speed operation; 8. Torque compensation amount is too large; 9. Speed tracking function parameters are set improperly (including instantaneous power failure restart and abnormal restart conditions). | When the output current is less than the value set by parameter F9-37, the A020 warning will be automatically cleared. |
| A021 | Over-torque | Over-torque 2 warning | Same as above | When the output current is less than the value set by parameter H3-01, the A021 warning will be automatically cleared. |
| A022 | Motor Overheat | Motor overheating | 1. Motor Stalled; 2. Excessive Load; 3. High Ambient Temperature; 4. Abnormal Motor Cooling System/Fan; 5. Frequent Low-Speed Operation; 6. Acceleration/Deceleration Time and Working Cycle Time Too Short; 7. V/F Control Voltage Too High; 8. Whether the rated motor current setting matches the motor nameplate 9. Whether the PTC settings and wiring are appropriate; 10. Whether the stall prevention settings are correct; 11. Three-phase impedance imbalance of the motor 12. Excessive harmonic components. | Parameter F9-48 = 0 for warning, when the temperature is less than or equal to the set value of parameter F9-49, A022 warning will be automatically cleared. |
| A024 | Over slip | Use the maximum slip (parameter F2-13) as the base, when the inverter output is at a steady speed, if F > H or F < H exceeds the level set by parameter F9-25 and lasts longer than the time set by parameter F9-26, an A024 warning will occur. | 1. Whether the motor parameters are correct; 2. Excessive Load; 3. Whether the settings for parameters F9-25, F9-26, and F2-13 are appropriate. | Parameter F9-27 = 0 for 'warning', when the inverter output is at a steady speed, and the deviation between the given speed and the actual speed does not exceed the set value of parameter F9-25, the A024 warning will be automatically cleared. |
| A025 | Parameter Identification in Progress | Automatic Parameter Identification in Progress. When performing automatic parameter measurement, the panel will display an A025 warning. | The inverter is executing motor parameter identification. | If parameter identification completes without errors, this warning will be automatically cleared. |
| A028 | Output Phase Loss Warning | Inverter Output Phase Loss | 1. Unbalanced three-phase impedance of the motor; 2. Check if there is a problem with the wiring; 3. Whether the motor is a single-phase motor; 4. Whether the current sensor is faulty; 5. Whether the inverter capacity is much larger than the motor capacity. | If parameter F9-15 is set to 0, the A028 warning will be automatically cleared after the inverter stops. |
| A030 | Model Different Parameter Copy Error | Digital Operator to Drive COPY Function Error Warning 3 | Copying parameters to a different model inverter. | Manual Reset |
| A031 | Over-torque | Over Torque 3 Warning | 1. Incorrect parameter settings; 2. A mechanical fault has occurred (e.g., over torque, mechanical lockup, etc.); 3. Overload; 4. Acceleration/deceleration time or working cycle time is too short; 5. V/F control voltage is too high; 6. Motor capacity is too small; 7. Overload occurs during low-speed operation; 8. The torque compensation amount is too large; 9. Speed tracking function parameters are set improperly; 10. (including automatic restart after instantaneous power failure and restart after abnormal conditions). | When the output current is less than parameter H3-06, the A031 warning will be automatically cleared. |
| A032 | Over-torque | Over-torque 4 warning | Same as above | When the output current is less than parameter H3-11, the A032 warning will be automatically cleared. |
| A036 | CANopen software disconnection | CANopen Software Disconnection 1 | 1. The communication timeout setting is too short; 2. Malfunction due to interference. | Manual reset, send a reset signal from the higher-level machine to clear this error. |
| A037 | CANopen software disconnection | CANopen Software Disconnection 2 | 1. The communication timeout setting is too short; 2. Misoperation due to interference; 3. Communication Cable Disconnected or Poor Contact. | Manual reset, send a reset signal from the higher-level machine to clear this error. |
| A039 | CANopen Hardware Disconnection | CAN BUS hardware disconnection | 1. Verify if the communication format is correct; 2. Misoperation due to interference; 3. Communication Cable Disconnected or Poor Contact. | Manual reset, power must be disconnected and then reconnected |
| A040 | CANopen Index Error | CANopen Communication Index Error | Communication Index Setting Error | Manual reset, the upper computer sends a reset signal to clear this error |
| A041 | CANopen Node Address Error | CANopen communication station number error (only supports 1～127) | Communication Node Address Setting Error | Manual reset, parameter F0-18=7 |
| A042 | CANopen Memory Error | CANopen memory error | CANopen Internal Memory Error | Manual reset, parameter F0-18=7 |
| A043 | CANopen SDO Transmission timeout | SDO transmission timeout (this warning is only for the master station) | 1. Slave not connected; 2. Synchronization cycle set too short; 3. Misoperation due to interference; 4. Communication cable break or poor contact. | When the master station resends an SDO and receives a response, this warning will be automatically cleared. |
| A044 | CANopen SDO Receive overflow | CANopen internal stack buffer overflow | The master device sends too many SDOs at once | Manual reset, send a reset signal from the higher-level machine to clear this error. |
| A045 | CANopen startup error | CANopen startup error warning | 1. Severe hardware interference; 2. Incorrect communication speed setting; 3. Communication card not connected or loose. | Disable CANopen, power off and then power on again. |
| A046 | CANopen format error | CANopen protocol error | The master device sends incorrect communication packets | Manual reset, send a reset signal from the higher-level machine to clear this error. |

Appendix English Abbreviation Meanings

| Abbreviation | English Description | Chinese Meaning |
| --- | --- | --- |
| VVC | Voltage Vector Control | Voltage Vector Control |
| SVC | Sensorless Vector Control | Sensorless Vector Control (without encoder) |
| FVC | Flux Vector Control | Magnetic Flux Vector Control (closed-loop control with encoder) |
| ASR | Automatic Speed Regulator | Speed Loop Regulator |
| APR | Automatic Position Regulator | Position Loop Regulator |
| dEB | Deceleration Energy Backup | Deceleration Energy Feedback |
| PDFF | Pseudo Differential Feedforward | Differential Feedforward Control |
| DOB | Disturbance Observer | Disturbance Observer |
| OC | Over Current | Overcurrent |
| OV | Over Voltage | Overvoltage |
| CC | Cycle by Cycle | Cycle-by-Cycle Current Limiting |
| EF | External Fault | External Fault (Fault occurs in external equipment of the inverter) |
| B.B. | Base Block | Base Block (Stop PWMOutput) |
| AI/AO | Analog Input/Output | Analog Signal Input/Output |
| DI/DO | Digital Input/Output | Digital Signal Input/Output |
| EMS | Emergency Stop | Emergency Shutdown (Hardware Detection of Severe Overcurrent) |
| RLY | Relay | Relay Output |